# 3GPP TSG-RAN WG4 Meeting #116	R4-2510088

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.5

Source:	China Telecom

Title:	Discussion on RRM performance requirements for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, RRM performance requirements for evolution of NR duplex operation were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for evolution of NR duplex operation captured in the WF [1] and summary [2].

- Discussion

Accuracy requirements

Status in the summary [2]:

- Issue 3-2-4: L1-SRS-RSRP – accuracy requirements - Proposals - Proposal 1 (CATT, HW): - Accuracy requirements apply for both intra- and inter-cell scenarios. - Proposal 2 (Nokia): - RAN4 to introduce performance requirements for CLI measurements for L1-SRS-RSRP measurements. - Proposal 3 (HW): - Define L1-SRS-RSRP accuracy requirements based on simulation results for the agreed side conditions. - Recommended WF - Suggest to agree the following: - RAN4 to define accuracy requirements for L1-SRS-RSRP measurements. - The accuracy requirements apply for both intra- and inter-cell scenarios. - The accuracy requirements are defined based on simulation results for the agreed side conditions.

For L1-SRS-RSRP accuracy requirements, in our views, L1-SRS-RSRP accuracy requirements should be defined based on simulation results for the agreed side conditions.

Proposal 1: RAN4 to define accuracy requirements for L1-SRS-RSRP measurements, and the accuracy requirements are defined based on simulation results for the agreed side conditions.

Status in the summary [2]:

- Issue 3-2-5: L1-CLI-RSSI – accuracy requirements - Proposals - Proposal 1a (Samsung, CATT, HW): - Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Proposal 1b (Nokia, CTC): - Same accuracy requirements are defined for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Proposal 2 (Nokia): - RAN4 to introduce performance requirements for CLI measurements for L1-CLI-RSSI measurements. - Proposal 3 (MTK): - Further discuss the side conditions and accuracy requirements of L1-CLI-RSSI. - Recommended WF - Suggest to agree the following: - RAN4 to define accuracy requirements for L1-CLI-RSSI measurements. - FFS on how to define the requirements - Option 1: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Option 2: other

For L1-CLI-RSSI accuracy requirements, R16 accuracy requirements for CLI-RSSI can be reused for L1-CLI-RSSI accuracy requirements since there is no signal power estimation, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Proposal 2: RAN4 to define accuracy requirements for L1-CLI-RSSI measurements, and RAN4 to re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Test cases

Status in the summary [2]:

- Issue 3-4-2: Test case list - Proposals Samsung; Nokia; CTC; CMCC; E///; HW; ZTE. L1-SRS-RSRP measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y. L1-CLI-RSSI measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y. SSB L1 measurement; Y; Y. SSB L3 measurement; Y. CSI-RS RLM; Y; Y; Y; Y; Y; Y; Y. CSI-RS BFR; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-RSRP measurement; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-SINR measurement; Y; Y; Y; Y; Y; Y. CSI-RS L3 measurement; Y; Y; Y. RACH; Y. - Recommended WF - Suggest to agree to define at least following test cases - L1-SRS-RSRP measurement (delay and accuracy) - L1-CLI-RSSI measurement (delay and accuracy) - CSI-RS RLM - CSI-RS BFR - CSI-RS L1-RSRP measurement (delay) - Discuss whether to define following test cases - CSI-RS L1-RSRP measurement (accuracy) - CSI-RS L1-SINR measurement (delay and accuracy) - SSB L1 measurement - SSB L3 measurement - CSI-RS L3 measurement - RACH

Based on the core part discussion in RAN4 previous meetings, there are two objectives been discussed and defined, i.e., RRM requirements for UE-to-UE CLI handling and RRM impacts for SBFD operation.

For RRM requirements for UE-to-UE CLI handling, L1-SRS-RSRP measurements and L1-CLI-RSSI measurements are considered for UE-to-UE CLI handling, it’s proposed to design UE-to-UE CLI handling test cases based on L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

For RRM impacts for SBFD operation, scenarios including CSI-RS colliding with UL transmission, CSI-RS puncturing in frequency domain in SBFD symbols and CSI-RS distributed over SBFD and non-SBFD symbols are considered in the core part discussion, and impact of SBFD operation for CSI-RS based L1-RSRP/ L1-SINR measurement and CSI-RS based RLM/ BFD/ CBD evaluation are to be defined in the core part, related test cases can be designed in performance part.

Proposal 3: It’s proposed to design the following test cases

- L1-SRS-RSRP measurement (delay and accuracy)

- L1-CLI-RSSI measurement (delay and accuracy)

- CSI-RS RLM

- CSI-RS BFR

- CSI-RS L1-RSRP measurement

Status in the summary [2]:

- Issue 3-4-4: Test case scenarios - Proposal 1a (Nokia): - RAN4 to define the test case list for SBFD in SA operation and wait for the LS response from RAN1 to the LS sent by RAN2 in the last RAN2 meeting for decision on test case list for DC. - Proposal 1b (E///): - Define test cases for single cell operation. - Proposal 2 (Nokia, CTC, E///, HW): - Define test cases for FR1 and FR2. - Proposal 3 (CTC): - For RRM impacts for SBFD operation, test cases with DRX and non-DRX mode can be considered. - Recommended WF - Suggest to agree to define SBFD RRM test cases for both FR1 and FR2. - Further discuss whether to define test cases for NR SA only or both NR SA and EN-DC. - Further discuss whether to define test cases for both DRX and non-DRX.

In our views, test cases for FR1 and FR2 can be considered.

Proposal 4: RAN4 to define SBFD RRM test cases for both FR1 and FR2.

- Conclusion

The contribution presents our views on RRM performance requirements for evolution of NR duplex operation, with the following proposals:

Proposal 1: RAN4 to define accuracy requirements for L1-SRS-RSRP measurements, and the accuracy requirements are defined based on simulation results for the agreed side conditions.

Proposal 2: RAN4 to define accuracy requirements for L1-CLI-RSSI measurements, and RAN4 to re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Proposal 3: It’s proposed to design the following test cases

- L1-SRS-RSRP measurement (delay and accuracy)

- L1-CLI-RSSI measurement (delay and accuracy)

- CSI-RS RLM

- CSI-RS BFR

- CSI-RS L1-RSRP measurement

Proposal 4: RAN4 to define SBFD RRM test cases for both FR1 and FR2.

- References

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505568, Topic summary for [115][219] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #115, 19 – 23 May, 2025.