# 3GPP TSG-RAN WG4 Meeting #116	R4-2510092

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.23.4

Source:	China Telecom

Title:	Discussion on RRM performance requirements of NES for NR

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, RRM performance requirements of NES for NR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements of NES for NR captured in the WF [1] and summary [2].

- Discussion

OD-SSB performance part

Status in the WF [1]:

Issue 3-1-3: Test case list RAN4 to discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 1 - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

For performance requirements of NES part 1, based on the core part discussion in RAN4 previous meetings, the requirements on OD-SSB based deactivated SCell measurement with FMW, OD-SSB based SCell activation, and OD-SSB based L1 measurement are to be defined, therefore, related test cases need to be designed. Besides, two cases including Case 1 and Case 2 need to be considered for test case design.

For OD-SSB based deactivated SCell measurement with FMW, OD-SSB based NR intra-frequency measurements requirements are to be defined in the core part, Case 1 and Case 2 are also considered for OD-SSB based NR intra-frequency measurements.

For OD-SSB based SCell activation, scenarios including OD-SSB based normal SCell Activation, OD-SSB based direct SCell Activation at SCell addition, OD-SSB based PUCCH SCell Activation, and OD-SSB based SCell Activation with the L3 reporting during activation are considered in core part, therefore, it’s proposed to design test cases for these scenarios. Considering the workload for these scenarios, the test cases for Case 1 and Case 2 for each scenario can be designed both two or one of them, which can be further discussed.

For OD-SSB based L1 measurement, it’s proposed to design test case for L1-RSRP measurements based on OD-SSB.

Proposal 1: RAN4 to design the following OD-SSB test cases

- OD-SSB based deactivated SCell intra-frequency measurement in Case 1 and Case 2

- OD-SSB based SCell Activation in Case 1 and Case 2

- OD-SSB based direct SCell Activation at SCell addition in Case 1 and Case 2

- OD-SSB based PUCCH SCell Activation in Case 1 and Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Proposal 2: For OD-SSB based SCell activation, OD-SSB based direct SCell Activation at SCell addition, OD-SSB based PUCCH SCell Activation, and OD-SSB based SCell Activation with the L3 reporting during activation, considering the workload, whether to design one test case or two test cases for Case 1 and Case 2 can be further discussed.

Status in the WF [1]:

Issue 3-1-4: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - At least test OD-SSB periodicities with 5ms, 20ms. FFS 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms

For general principles, FR1 and FR2 can be considered for test case design. Besides, the principles on only testing non-DRX mode, the test without SSB time index detection, following the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting can be considered. The periodicities of OD-SSB, SMTC and measCycleSCell can be further discussed.

Proposal 3: RAN4 to design the OD-SSB test case with the following principles

- Both FR1 and FR2

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

OD-SIB1 performance part

Status in the WF [1]:

Issue 3-2-1: Test case - Proposals - Option 1: RAN4 not to define OD-SIB1 RRM performance test case. - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

For performance requirements of NES part 2, SSB adaptation and On-demand SIB1 are discussed in the core part, related test cases need to be designed.

For On-demand SIB1, it was agreed in the WF on NES part 2 in last meeting that RAN4 to define test case with longer TSI-NR with reasonable value in performance part.

Proposal 4: RAN4 to define test case with longer TSI-NR with reasonable value in OD-SIB1 performance part.

SSB adaptation performance part

Status in the WF [1]:

Issue 3-3-1: Test case list RAN4 to discuss whether the following SSB adaptation test case list can be agreed. - L1-RSRP - L1-SINR - CBD - L3 measurement - Deactivated SCell measurement - normal SCell activation - direct SCell activation - PUCCH SCell activation

For SSB adaptation, test cases for L1 measurement with SSB adaptation and L3 measurement with SSB adaptation can be designed, moreover, test case for SCell activation with SSB adaptation also need to be designed.

Proposal 5: RAN4 to design the following SSB adaptation test cases

- L1-RSRP

- L1-SINR

- CBD

- L3 measurement

- MAC CE based SCell activation

- Direct SCell activation

- PUCCH SCell activation

Status in the WF [1]:

Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection

For general principles, FR1 and FR2 can be considered for test case design. Besides, the principles on only testing non-DRX mode, without SSB time index detection can be considered.

Proposal 6: RAN4 to design the SSB adaptation test case with the following principles

- Both FR1 and FR2

- Only test non-DRX mode

- Without SSB time index detection

- Conclusion

The contribution presents our views on RRM performance requirements of NES for NR, with the following proposals:

Proposal 1: RAN4 to design the following OD-SSB test cases

- OD-SSB based deactivated SCell intra-frequency measurement in Case 1 and Case 2

- OD-SSB based SCell Activation in Case 1 and Case 2

- OD-SSB based direct SCell Activation at SCell addition in Case 1 and Case 2

- OD-SSB based PUCCH SCell Activation in Case 1 and Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Proposal 2: For OD-SSB based SCell activation, OD-SSB based direct SCell Activation at SCell addition, OD-SSB based PUCCH SCell Activation, and OD-SSB based SCell Activation with the L3 reporting during activation, considering the workload, whether to design one test case or two test cases for Case 1 and Case 2 can be further discussed.

Proposal 3: RAN4 to design the OD-SSB test case with the following principles

- Both FR1 and FR2

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

Proposal 4: RAN4 to define test case with longer TSI-NR with reasonable value in OD-SIB1 performance part.

Proposal 5: RAN4 to design the following SSB adaptation test cases

- L1-RSRP

- L1-SINR

- CBD

- L3 measurement

- MAC CE based SCell activation

- Direct SCell activation

- PUCCH SCell activation

Proposal 6: RAN4 to design the SSB adaptation test case with the following principles

- Both FR1 and FR2

- Only test non-DRX mode

- Without SSB time index detection

- References

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505570, Topic summary for [115][221] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #115, 19 – 23 May, 2025.