3GPP TSG-RAN WG4 Meeting #116	R4-2510224

# Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on performance metric for XR OTA

Source:	Samsung

Agenda item:	7.10.3.1

Document for:	Discussion

1.	Introduction

In the Rel-19 XR OTA discussion until RAN4#114bis meeting, legacy full sphere TRP TRS is adopted as performance metric and several test methods have been agreed. Potential new performance metric for XR OTA was proposed last meeting, and the agreement are as following [1, R4-2508782]:

Issue 1-2-3: New performance metric for XR devices Agreements: RAN4 further study new metric and corresponding test procedure/data processing for XR devices, e.g.,. ⦁ Weighted TRP & TRS ⦁ Near horizontal part radiated power (NHPRP) and Near horizontal part Isotropic Sensitivity (NHPIS) ⦁ Others If no consensus reached, the WI can also be closed with agreed TRP/TRS metric for XR devices.

In this contribution, we look into this issue in details in both mobile scenarios perspective and test method perspective, and propose to close the WI with agreed TRP/TRS metric for XR devices.

2. 	Discussion

XR is a mobile device even when wearing on head. People wearing XR may look up or down at objectives in the augmented reality or playing games in the virtual reality. Consequently, there is no clear boundary which part of the XR device should be prioritized and other parts are not.

Observation 1:	XR as a mobile device is used in every kind of position, and consequently there is no clear boundary which part of the XR device should be prioritized and other parts are not.

From network signal perspective, the incoming signals may come from any directions rather than horizontal directions when users are in indoor, urban streets, or transportation and other scenarios. That means, even users wearing the XR in static manner, it can not be guaranteed that signals come from near horizon only.

Observation 2:	network signals are often complicated in many scenarios, and consequently it can not be guaranteed that signals come from near horizon only.

Combing both observation 1 and observation 2, it can be concluded that a directional performance metric is not suitable for XR devices, which is different from the NTN case. To complete this WI on time, it would be better to maintain the agreed full sphere TRP TRS metric.

In addition, one of the arguments for adopting potential new performance metric at such late stage of Rel-19 core part is that the potential new performance metric does not affect the test method. However, this is not 100% true. As we know, Reverberation Chamber test method is one of the important test methods and we have spent great efforts in AC&RC harmonization. When specifying performance metric for new device types, it is beneficial for the industry that the performance metric is applicable for both AC and RC. The newly proposed performance metric for XR devices unfortunately only works for AC chamber but not applicable for RC chamber.

Observation 3:	The newly proposed performance metric for XR devices unfortunately only works for AC chamber but not applicable for RC chamber.

Taking all above into consideration, we have following proposals:

Proposal 1:	Conclude XR part of this WI with the agreed full sphere TRP TRS metric

Proposal 2:	when discussing performance metric for new devices, the applicability to both AC and RC should be considered unless there is strong justification to consider AC method only

3. 	Conclusion

Observation 1:	XR as a mobile device is used in every kind of position, and consequently there is no clear boundary which part of the XR device should be prioritized and other parts are not.

Observation 2:	network signals are often complicated in many scenarios, and consequently it can not be guaranteed that signals come from near horizon only.

Observation 3:	The newly proposed performance metric for XR devices unfortunately only works for AC chamber but not applicable for RC chamber.

Proposal 1:	Conclude XR part of this WI with the agreed full sphere TRP TRS metric

Proposal 2:	when discussing performance metric for new devices, the applicability to both AC and RC should be considered unless there is strong justification to consider AC method only

4. 	References

- R4-2508782    “Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3”, vivo, RAN4#115