3GPP TSG-RAN4 Meeting #116	R4-2510295

# Bangalore, India, 25 Aug - 29 Aug, 2025

Agenda item:	 7.24.3.2

Source:	 vivo

Title:	Proposals on concluding LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

RAN4 made good progress on target SNR, REFSENS, RMC, ASCS requirements, ACS guard RB, but ACS requirement is still open.

Issue 2-1-1: SNR for LP-WUR two types

Agreements:

- There is no repetition of information bits to derive OFDM-based receiver SNR.

- SNR=-4.5dB for envelope detection and OFDM-based receiver.

Issue 2-1-2: Whether update the equation for REFSENS

Agreements:

- Use legacy REFSENS approach to derive requirements in spec. For TR, some statement of this issue may be presented.

Issue 2-1-3: RMC for  LP-WUR requirements

Agreements:

- In the FR1 spec, there should be two tables for 15kHz and 30kHz;

- The following parameter is agreed. Further adding other parameter (if necessary) is not precluded.

- Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | bits | 16bits |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 |
| WUS duration for OOK |  | 8 OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 5bits |

Issue 2-3-1: proposals to update the agreed ASCS requirements for LP-WUR last meeting

WF:

- Prepare Draft CR directly based on last meeting agreement of ASCS.

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for LP-WUR

Agreement:

- 0 guard RB is assumed for deriving both type-1 and type-2 ACS requirement.

In this contritbution, we provide views on concluding REFSENS, ASCS and ACS requirements.

- LP-WUS UE RF

  - ASCS requirements

It was agreed to prepare Draft CR directly based on last meeting agreement of ASCS, the only issue is the detailed wording in the CR on how to reflect the same PSD of NR RBs and LP-WUS RBs in the spec. There are two approaches to slove this problem:

- Option 1: use LP-WUS power boosting definition.

For this option, given the definition of LP-WUS power boosting is already definied in the BS session, reuse the same iin-band LP-WUS power boosting definition is an easy way to specify the UE RF requirements.

- Option 2: just state same PSD of NR RBs and LP-WUS RBs.

Given PSD is aready used in TS 38101-1, this approach is also workable as a specification languge:

For sidelink in 6.2E.4A: “The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.”

Therefore, in general we think both ways are workable to be spec languge for ASCS. Option 1 + Option 2 is also a way to go. The suggested ASCS parameter table is as following:

Table 7.5M.3-1: ASCS parameters for LP-WUR type 1 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS+0.5 |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 1 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration as interferer, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. 0 power boost means equal PSD between NR RBs and LP-WUS RBs. |  |  |

Proposal 1: For ASCS, to reflect the equal PSD, the spec can use LP-WUS power boost definition and further clarify that it means equal PSD between inband NR RBs and LP-WUS RBs.

  - REFSENS requirements

The target snr was agreed as -4.5dB. The two set of NFLR + IMLR, were agreed as 18.0dB and 13.5dB for Type 1 and Type 2 UEs, respectively. Using the agreed equation for REFSENS, the REFSENS_LP-WUS can be calculated:

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR.

Proposal 2: To simply the REFSENS requirements, based on agreed equation and NF+IM, the REFSENS value in Table below can be used. The requrement applies to all CBW of each supported NR band.

Table 7.3M.2-1: One antenna port reference sensitivity for FDD and TDD bands for LP-WUS

| RX parameter | SCS kHz | Value (dBm) |
| --- | --- | --- |
| REFSENSLP-WUS for UEs supporting Type1 LR | 15 | -97.5 |
|  | 30 | -94.5 |
| REFSENSLP-WUS for UEs supporting Type2 LR | 15 | -102.0 |
|  | 30 | -99 |

  - ACS requirements

For ACS, RAN4 agreed to use same interference level as MR ACS case for both type 1 and type 2 LP-WUR, and the guard RB is 0 for both type-1 and type-2 ACS requirements.

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for LP-WUR

Agreement:

- 0 guard RB is assumed for deriving both type-1 and type-2 ACS requirement.

Issue 2-4-2: case 1 ACS test parameter

WF:

- Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Issue 2-4-3: case 2 ACS test parameter

WF:

- Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Based on the simulated ACS performance in our contribution last meeting, the wanted signal level should be REFSENS_LPWUS + 18.5dB.

Table 3 Sumamrized ACS performance of type 1 and type 2 with different consideration of wanted signal level (same interference as MR)

| NF legacy | SNR legacy | NF LPWUS | ADC | SNR LPWUS | ACSLPWUS | ACS Wanted signal level | Guard RB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 dB | -1 dB | type1: 15.5 dB | 4 bit | OOK/OFDM = -4.5 dB | 22.5 dB | LR REFSENS+18.5 dB | 1 |
|  |  | type2: 11 dB | 8 bit | OOK/OFDM = -4.5 dB | 27 dB | LR REFSENS + 18.5 dB | 0 |

For type-2, the sensitivity level of LR is nearly the same as MR, considering type-2 would be mainly OFDM-based receiver, it could be acceptable that same ACS requirements as MR is considered. However, for type-1, based on our simulation, there is significant performance impact of phase noise, so a relexation is needed compared to type-2.

Proposal 3: Specify the same wanted signal level as MR for type-2. For type-1, the wanted signal can be relaxed 3dB.

Table 7.5M.2-1: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + [-6] + [3] +14dB |
| Pinterferer | dBm | REFSENSLP-WUS + [-6] +45.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-3: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + [-1.5] +14 dB |
| Pinterferer | dBm | REFSENSLP-WUS + [-1.5] +45.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Proposal 1: For ASCS, to reflect the equal PSD, the spec can use LP-WUS power boost definition and further clarify that it means equal PSD between inband NR RBs and LP-WUS RBs.

Proposal 2: To simply the REFSENS requirements, based on agreed equation and NF+IM, the REFSENS value in Table below can be used. The requrement applies to all CBW of each supported NR band.

Proposal 3: Specify the same wanted signal level as MR for type-2. For type-1, the wanted signal can be relaxed 3dB.

A draft CR is prepared in [2].

# References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo, RAN4#115.

- R4-2510296, Draft CR to 38.101-1 on LP-WUS REFSENS ACS and ASCS, vivo, RAN4#116.