# 3GPP TSG-RAN WG4 Meeting #116                                 R4-2510399

# Bengaluru, India, 25th-29th Aug., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on BS RF requirements for LP-WUS/WUR

# Agenda Item:   7.24.4

# Document for:  Approval

- Introduction

In last meeting, RAN4 has achieved the agreement on manufacturer declaration on LP-WUS power boosting. The issues left to be finally decided are as follows [1].

Issue 1-2: Whether/how to define EVM requirements for LP-WUS - Options - Option 1: - No BS LP-WUS EVM requirement is specified. - The background and consideration of LP-WUS transmitted signal quality are captured in TR 38.774. - Option 2: - BS LP-WUS EVM requirement as follows, no conformance test for this requirement. - For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK-4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2. Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier Modulation scheme for LP-WUS; Required EVM. OOK-4; [FFS] %. - WF - Down select the options in next meeting Issue 1-4: Restriction on the applicability of LP-WUS operation - WF - Option 1: - It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance. - Option 2 - No restriction of LP-WUS operation, this is BS implementation issue. - WF - Down select the options in next meeting

- Discussion

## 2.1 EVM requirements for LP-WUS

Based on the latest RAN1 conclusions on LP-WUS design and the LS from RAN1 [1], we updated our simulation assumptions as shown in Table 2 to check the the impact of EVM on LP-WUS MDR.

Table 2	Link level simulation assumptions for LP-WUS BS transmission

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 2.6GHz |
| Channel structure | 5 information bits with 16bit RM coding No CRC is configured |
| Chip rate | M = 4 |
| Coding | 1/2 rate Manchester coding |
| Repetition | None |
| Candidate overlaid OFDM sequences | 4 |
| WUS duration | OOK: RM coding length*2/M = 8 OFDM: ceil(5/log2(4)/(M/2)) = 2 |
| Time error | 0 us |
| Residual Frequency error | 0 ppm |
| SCS | 30kHz |
| Channel BW | 20MHz |
| WUS RB | - 11 RB |
| Position within channel | - Center |
| Guard RB | - 1 RB between NR signal and OOK-based LP-WUS - 0 RB between NR signal and OFDM-based LP-WUS |
| Non-linearity | Not modeled |

Traditional Tx EVM is tested based on frequency-domain constellations. For LP-WUS, the symbols before RB mapping can be regarded as “constellations”. We added EVM on the LP-WUS frequency-domain symbols before RE mapping, as shown in Fig. 1, and then check the impact of EVM to Rx MDR. The simulation result is shown in Fig. 2. It can be found that frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR.

Fig. 1  Add EVM to LP-WUS frequency-domain symbols

Fig. 2  Impact of EVM on OOK-4 M=4 receiving MDR for OOK-based receiver and OFDM-based receiver

As the simulation results shown, frequency-domain EVM on LP-WUS has minor impact on the receiving MDR.

Observation 1: Frequency-domain EVM on LP-WUS has minor impact on the receiving MDR.

Therefore, we prefer not to define EVM requirements for LP-WUS, but BS still needs to satisfy the NR EVM requirements on NR signal if the BS supports LP-WUS operation.

Proposal 1: It is proposed not to define EVM requirements for LP-WUS, but BS still needs to satisfy the NR EVM requirements on each NR carrier for different modulation schemes on PDSCH if the BS supports LP-WUS operation.

## 2.2 Restrictions on LP-WUS Operation

Whether LP-WUS can be transmitted together with 256QAM signal depends on BS implementation. RAN4 should not limit BS implementation.

Proposal 2: Support Option 2: No restriction of LP-WUS operation.

- Conclusion

Observation 1: Frequency-domain EVM on LP-WUS has minor impact on the receiving MDR.

Proposal 1: It is proposed not to define EVM requirements for LP-WUS, but BS still needs to satisfy the NR EVM requirements on each NR carrier for different modulation schemes on PDSCH if the BS supports LP-WUS operation.

Proposal 2: Support Option 2: No restriction of LP-WUS operation.

- Reference

- R4-2508668 WF on NR LP-WUS BS RF, Huawei, RAN4#115, St. Julian’s, Malta, May 2025.

- R1-2504943 Reply LS on LP-WUS UE RF, Vivo, RAN1#121, St. Julian’s, Malta, May 2025.