3GPP TSG-RAN4 Meeting #116	R4-2510503

# Bengaluru, India, 25th August 2025 – 29th August 2025

Source:			Ericsson

Title:			NTN Ku bands –VSAT RF

Agenda item:		7.7.5

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In last RAN4 meetings, RAN4 agreed on many aspects of VSAT RF requirements ([2], [3] and [4]) but few of them still need further discussion. This contribution is addressing those remaining VSAT RF open issues.

# Discussion

## PFD limits

In last RAN4#114bis and RAN4#115 meetings, the need to capture PFD limits was still left opened ([4]), pending on companies to check if the conformance testing in the EN is based on real measurement or only a theorical  analysis:

As we already explained in our previous contribution ([5]), if PFD limits for SAN might not be relevant for 3GPP as they are depending on SAN deployment, the situation is not the same for the VSAT as the PFD limits are usually not depending on the VSAT location.

Also, the PFD limits being part of the Regulation, they are anyway mandatory to support for any VSAT operating in the corresponding NTN Ku-bands.

In last RAN4#114bis and RAN#115 meetings, companies were still concerned with the testing aspects of such requirements, how to test PFD requirements as this type of requirement has not yet been specified in 3GPP.

As we already mentioned in our RAN4#115 contribution ([5]), the PFD requirements have already been captured in EN 302 186 ([6]). This EN aiming to be a Harmonized Standard, the corresponding test methods have been specified as well in this specification, proving that PFD testability is not an issue. Referring to EN 302 186, the PFD requirement is tested in 3 steps:

- Measuring the antenna radiation pattern.

- Measuring the on-axis EIRP.

- And then computing the PFD at the surface of the Earth.

Based on these clarifications, we make the following proposal:

Proposal: Considering that PFD limits are already specified in ETSI hENs and the corresponding testing procedure is specified in those hENs, capture PFD limits for NTN VSAT when specified by the Regulation.

To optimize the drafting work, our companion draft CR ([7]) is capturing those PFD aspects in TS 38.101-5.

# Conclusion

In this contribution, we analyzed the remaining issue related to PFD requirements for NTN VSAT. We made the following proposal:

Proposal: Considering that PFD limits are already specified in ETSI hENs and the corresponding testing procedure is specified in those hENs, capture PFD limits for NTN VSAT when specified by the Regulation.

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2508778, Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2506144, NTN Ku-bands – VSAT, Ericsson

- EN 302 186, Satellite mobile Aircraft Earth Stations (AESs) operating in the 11/12/14 GHz frequency bands; Harmonised Standard for access to radio spectrum.

- R4-2510508, Draft CR to TS 38.101-5 – Regional PFD requirements, Ericsson