3GPP TSG-RAN WG4 Meeting #116				R4-2510525

Bengaluru, India, 25th – 29th Aug, 2025

Agenda item:	8

Source: 	Huawei, HiSilicon

Title: 	View on Rel-19 RAN4 UE feature list

Document for:	Discussion

- Introduction

This contribution provides our view of the RAN4 UE feature list for Rel-19 NR.

- NR_ENDC_RF_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. |  | Yes | N/A | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per BC | No | FR1 only | N/A |  | Optional with capability signaling |
|  | 46-2 | FR2 MPR-Improvement Downlink Independent | Indicates whether UE supports reduced MPR by removing dependence on DL CA configuration. |  | Yes | N/A | The FR2 UE only supports MPR requirement with dependence of DL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-3 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs and when all activated CCs form a contiguous block in both UL and DL for intra-band contiguous CA |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*NRB extension for dual-sided symmetric and single-sided case NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1 |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | Indicates whether UE supports MPR reduction for single UL carrier with X* NRB extension for dual-sided symmetric/asymmetric and single-sided case, where X={1/2, 1/4} |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA UE. |  | Yes | N/A | FWA does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | FR1 only | N/A |  | Optional with capability signaling |

- NonCol_intraB_ENDC_NR_CA_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NonCol_intraB_ENDC_NR_CA_Ph2 | 47-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table [7.5.x.x]/Table [7.6.x-x] in TS 38.133 and UE RF requirements for intra-band non-collocated NR-CA including [7.10A] in TS 38.101-1. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table [7.6.x-x] in TS 38.133 and UE RF requirements for intra-band NR-CA except for [7.10A] in TS 38.101-1. | 33-1 (RAN4 R18 feature) | Yes | N/A | Intra-band non-collocated NR CA operation with up to 4layer per CC is not supported. | Per BC | N/A | FR1 only | N/A | Supported for band n77/n78 only | Optional with capability signaling |
|  | 47-2 | Support of inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation | Indicates the UE supports TDD-TDD inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation with MTTD/MRTD requirements according to Table [7.5.x.x]/Table [7.6.x-x] in TS 38.133 and UE RF requirements for inter-band non-contiguous EN-DC with overlapping or partially overlapping bands including [7.10B] in TS 38.101-3. And the UE also supports inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation with MRTD according to Table [7.6.x-x] in TS 38.133 and UE RF requirements for inter-band non-contiguous EN-DC with overlapping or partially overlapping bands except for [7.10B] in TS 38.101-3. | 33-2 (RAN4 R18 feature) and 2-19 (R16 RAN4 feature) | Yes | N/A | Inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation with up to 4layer per CC is not supported. | Per BC | N/A | FR1 only | N/A | Supported for band B42 and n77/n78 only | Optional with capability signaling |

- NR_ATG_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_ATG_enh | 48-1 | Rx beam Type | Indicates whether the UE supports one common Rx beam or two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

- NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-1 | [Simultaneous] L3 measurement on three carriers for measurements without measurement gap under CA/DC operation. | Support of measuring serving cell and neighbor cells measurement on three carriers [simultaneously] for measurements without measurement gap. - Support of separate indications for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA PCell is FR1 only}, and {FR1+FR2 NR-DC PCell is FR1 only}. |  | Yes |  | UE does not support measuring serving cell and neighbor cells on three carriers [simultaneously] for measurements without measurement gap. | Per UE | No | No |  | UE indicating this capability shall meet the corresponding enhanced requirements defined in TS38.133 [Clause 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3] if the feature is supported. | Optional with capability signaling |
|  | [49-2] | [Fast Rx beam sweeping for FR2-1 L3 measurement delay reduction] | [Supports beam sweeping factor reduction for L3 measurement for FR2-1.] |  | [Yes] | [N/A] |  | [Per band] | [TDD only] | [FR2-1 only] |  | Candidate values: {2,4,6} for FR2-1 Note 1: It is only supported for power class 3. Note 2: UE supporting this FG uses one additional bit to indicate whether UE can apply this FG for FR2-1 L3 SSB-based inter-frequency measurement with or without MG. | [Optional with capability signalling] |
|  | [49-3] | [One serving carrier measurement per-band for intra-frequency measurements without measurement gap] | [Support of measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band] |  | [Yes] |  | [UE does not support measuring serving cell and neighbor cells on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured for measurement in a same band.] | [Per-UE] | [No] | [Yes] |  | [UE is required to meet the serving cell and neighbor cells measurement requirements in TS38.133 [section 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4] if the feature is supported.] | [Optional with capability signaling] |

- Netw_Energy_NR_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. Netw_Energy_NR_enh | 50-1 | Lower bound of measurement periodicity of 10ms for the deactivated measurement requirement in fast measurement window on OD-SSB SCell | Indicates a lower bound of measurement periodicity as 10ms. The lower bound is applied to the deactivated SCell measurement requirement in fast measurement window on OD-SSB SCell | At least one among RAN1 feature of 61-1, 61-2, 61-2a, 61-3, 61-4, 61-4a | Yes | n/a | UE performs on-demand SSB based deactivated SCell measurement without a lower bound within fast measurement window | Per BC | n/a | n/a | n/a | UE indicates whether the lower bound of 10ms is needed as the minimum supported measurement periodicity. | Optional with capability signaling |
|  | [50-2] | [Additional processing time for OD-SSB activation] | [Indicates an shorter additional processing time of 2ms in addition to T_min = m+3Nslotsubframe,μ+1 for reception of on-demand SSB bursts from the time when UE receives OD-SSB activation MAC CE command.] | [At least one among RAN1 feature of 61-3, 61-4, 61-4a] | [Yes] | [n/a] | [Additional processing time of 5ms in addition to T_min = m+3Nslotsubframe,μ+1 for reception of on-demand SSB bursts from the time when UE receives OD-SSB activation MAC CE command. ] | [Per band] | [n/a] | [n/a] | [n/a] | [Note 1: Default value of additional processing time is 5 ms Note 2: The additional processing time only applies to deactivated SCell measurement] | [Optional with capability signaling] |

- NR_LPWUS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - NR_LPWUS | 51-1 | LP-WUR types | Support of different LP-WUR UE type, aiming to accommodate different LP-WUR implementations by applying distinct Rx requirements: a) Type 1: LP-WUR implementation assuming a combined noise figure (NF) and implementation margin (IM) of 18dB. b) Type 2: LP-WUR implementation assuming a combined noise figure (NF) and implementation margin (IM) of 13.5dB. | N/A | Yes | N/A |  | Per UE | No | Yes (FR1 only) | N/A |  | Optional |

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_Mob_Ph4 | 52-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot [across candidate cells] for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} - [Note: FFS It is also counted in FG 2-24] | Optional with capability signaling |
|  | 52-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and [candidate cells] [across all CCs] for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in [LTM-CSI-ResourceConfig-r18] or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighbouring cells across all CCs for L1 measurement. | Per BC | No | No | N/A | Candidate values: [{2,4,8,12,16,32,64}] - Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 52-3 | Skip SSB based L1-RSRP measurement for [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per UE | no | FR2-1 only | N/A |  | Optional with capability signalling |

- NR_XR_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | [53-1] | [Recommended ratio of gap occasions for cancelation report. ] | [Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation. ] | [64-1] | [Yes] | [N/A] | [The UE does not support reporting recommended ratio for gap cancelation] | [Per UE] | [no] | [no] |  |  | [Optional with capability signalling] |

- NR_FR1_lessthan_5MHz_BW_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_FR1_lessthan_5MHz_BW_Ph2 | 54-1 | Support of less than 5MHz CBW operation in CA/DC | - Support for 3 MHz symmetric channel bandwidth in DL and UL in CA/DC - Support for 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL in CA/DC - Support 12 PRB CORESET0 in CA/DC - Support 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637 in CA/DC - Support 5 MHz channel bandwidth with 20 PRB CORESET0 in CA/DC | Layer-1 UE feature group 51-1, 51-1a, 51-2a, 51-2b, 51-3 | Yes |  | The UE does not support corresponding UE features in CA/DC operations | NA (see Note column) | FDD only | FR1 only | NA | All components in the feature group do not require new capabilities signalling definition in Rel-19 and respective Rel-18 capability signalling is reused based on RAN2 agreements. | Optional |

- NR_LBCA_SW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_SW | 55-1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between an FDD band and SDL band. The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported. switchingPeriodForFDD-SDL indicates the length of the switching time between an FDD band and SDL band: 35us represents 35 us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1. | N/A | Yes | N/A | Operation of Carrier aggregation between FDD and SDL via switching is not supported. | Per band pair per band combination |  | Applicable to FR1 only |  |  | Optional with capability signalling |

- NR_FR1_7MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_FR1_7MHz_BW | 56-1 | UE support of 7MHz CBW | Indicates UE support of 7MHz CBW |  | Yes |  | Network does not know if UE supports 7MHz CBW | Per UE |  | FR1 only |  |  | Optional with capability signalling |

- Others

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57. 3Tx UL switching | [57-1] | [Switch period for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band] | [Indicate the supported switching period for dynamic UL Tx switching between two bands for 3Tx UE with up to two transmit antenna connectors in each band] |  | [Yes] | [N/A] | [UE does not support dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band] | [UE signals supported switching period per pair of UL bands per UL band combination] | [No need] | [FR1 only] | [Support mixture of FDD/TDD] | [Candidate value set: {35us, 140 us, 210us} UE is mandatory to support dualUL when supporting this feature] | [Optional with capability signalling] |
|  | [57-2] | [Application of DL interruptions due to dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band] | [Capability to indicate that for the band where DL interruption is needed due to dynamic UL Tx switching between 2 band for 3Tx UE with up to 2Tx per band] | [59-1] | [Yes] | [N/A] | [UE not reporting this capability means DL interruption is not required] | [UE capability is defined as per band per band combination for each band pair supporting UL Tx switching] | [No need] | [FR1 only] | [Support mixture of FDD/TDD] | [The same capability for Rel-16 DL interruption due to Tx switching is reused.] | [Optional with capability signalling] |
|  | [57-3] | [UL-MIMO coherence capability for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band] | [Capability to indicate whether UL-MIMO coherence is supported when dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band is conducted.] | [59-1] | [Yes] | [N/A] | [Rel-15 per band capability pusch-TransCoherence is applicable] | [Per band per BC] | [No need] | [FR1 only] | [Support mixture of FDD/TDD] |  | [Optional with capability signalling] |