3GPP TSG-RAN WG4 Meeting # 116	R4-2510572

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on Event triggered L1 measurement reporting

Source: 	Huawei, HiSilicon

Agenda item:	7.25.3.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on Event triggered L1 measurement reporting.

Discussion

Some left open issues in [2] are to be discussed in the followings.

- Issue 2-1: applicability of L1-RSRP reporting delay Candidate options: - Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi) - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple) - Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

We understand the assumptions in option 1 come from the below RAN1 and RAN2 agreements: UE will reset the counting of beams when the measured current beam RS is updated. And TTT timer is not restarted even if current beam changes. The intention of Option 1 is to ensure the serving beams during L1 RSRP reporting evaluation are not impacted by some procedure (e.g., TCI change or RS configuration change).  However we don’t think such assumption shall be captured in core requirements, as these would strongly restrict network configuration and implementation. Recalling the legacy L3 RSRP measurement test case, the serving cell is kept unchanged during the whole event reporting procedure. The similar way can be followed for L1-RSRP measurement.

Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. FFS: Further details on the update (if necessary).

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

Proposal 1: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases.

- Issue 2-4: Impact on other LTM requirements Agreement in RAN4#114bis: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS Candidate options: - Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei) - For FR1: - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met - The related cell switch delay and TCI state activation requirements are applicable - The above optimization is not applicable to FR2 - Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia) - Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report. - Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) - Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple) - Number of active TCI does not exceed that UE can support. - Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK) - Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE) - Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///) - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

As per FR2 SSB based L1-RSRP measurement principle in R18 mobility, UE would prioritize L1-RSRP measurement on the TCI state which is in active TCI list. Whether UE would measurement on the TCI which is not in the active TCI list is left to UE implementation and no requirements are defined.

- PL1_sharing is defined as - When number of neighboring cells configured with SSB based L1-RSRP measurement is 1 - PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain) - PL1_sharing = 1, otherwise - When number of neighboring cells configured with SSB based L1-RSRP measurement is more than 1 - When none of TCI state(s) of the intra-frequency neighbor cells or inter-frequency neighbor cells without gap are in the active TCI state list - PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap - Otherwise - PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list is the number of neigbour cells including intra-frequency neighbor cell(s) and inter-frequency neighbor cell(s) without gap whose TCI state(s) are in the active TCI state list. No requirements are defined for any other cell(s) whose TCI state(s) is not in the active TCI state list.

So in last meeting companies raised a case that a beam which is not in active TCI list triggered an event-triggered L1 report, afterwards if NW activated some other TCI states, UE may stop measuring the beam. So when UE receives the cell switch command or TCI state activation command, the reported beam may become unknown. Therefore the optimization for FR1 is not applicable to FR2.

For FR1, we don’t think the case that only event entering condition is configured is valid. As per RAN2 configuration, both event entering and leaving conditions use the same hysteresis value, and the entering condition and leaving condition are explicitly defined with equation (see below context in RAN2).

where

Observation1: It is invalid that only event entering condition is configured and leaving condition is not configured.

Proposal 2:

- For FR1:

  - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - the related cell switch delay and TCI state activation requirements are applicable

- The above optimization is not applicable to FR2

Conclusion

The contribution provides analysis on Event triggered L1 measurement reporting. The following proposals are proposed:

Proposal 1: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases.

Observation1: It is invalid that only event entering condition is configured and leaving condition is not configured.

Proposal 2:

- For FR1:

  - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - the related cell switch delay and TCI state activation requirements are applicable

- The above optimization is not applicable to FR2

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple