3GPP TSG-RAN WG4 Meeting # 116	R4-2510604

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.29.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for IoT NTN TDD mode

Document for:	Discussion

# Introduction

The new WID of NTN IoT TDD mode was approved in [1] and was revised in [2]. RAN4 studied the impact due to periodic pattern from RAN4#113 meeting. The progress of last RAN4 meetings are captured in [3]. In this paper, we provide our views on the potential impacts on RRM impacts.

# Discussion

In last RAN4 meeting, RAN4 mainly discussed the applicable scenario and the assumption on number of samples at higher SINR conditions. The progress is summarized as follows:

Agreement: online - RRM requirements in IoT-NTN TDD mode in this WI are defined for earth-moving cells, and are not applicable for earth-fixed cells. Agreement: online - Target Coverage: Normal coverage - Target SNR in AWGN: -1 dB - Requirement applicability: Capture in specification clearly that the new requirements are applicable only for the new punctured TDD Frame structure where N=9 and D=U=8, as described in the WID Agreement: online - Is the measurement/evaluation solely based on NPSS, NSSS or NRS? - Issue 1: Cell reselection (4.6A.2) - Tdetect: NPSS, NSSS, [NPBCH] - Tmeasure: NRS - Treselection: NRS - Issue 2: Cell measurement in connected mode (8.14A) - Detection for neighbour cell (8.14A.6): NPSS, NSSS, [NPBCH] - Measurement: NRS - Further discussion on NSSS - Issue 3: RRC re-establishment (6.5A) - Detection for unknown cell: NPSS, NSSS, [NPBCH] - Measurement: NRS - Further discussion on NSSS - Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) - NA - Issue 5: RLM (7.23A) - NRS Agreement: online - Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ -1 dB? - Issue 1 & 2 & 3: Cell reselection (4.6A.2), Cell measurement in connected mode (8.14A) and RRC re-establishment - Detection: 12 NPSS subframes + 2 NSSS subframes + 1 NSSS subframe - T_SI: 8 NPBCH subframes is included - Measurement: 8 NRS subframes - Issue 5: RLM (7.23A): further discussion

Based on above assumption, we provide our analysis on detailed RRM requirements in a case by case manners.

Cell reselection

For Cell reselection requirements, taking existing intra-frequency at normal coverage as example (4.6A.2), which is shown as follows, the requirements are defined in number of DRX/eDRX. Thus, for IoT NTN TDD, the question that RAN4 needs to discussed is how many samples can be assumed for each DRX cycle.

Observation 1: For IDLE mode Cell reselection, RAN4 needs to discuss how many samples are assumed with in each DRX cycle.

We fist assume the extreme case that UE is required to monitor all available NPSS/NSSS/NRS within each DRX cycle. Then based the agreed number of samples, the table can be updated as follows:

Table I. Tdetect/measurement/evaluate which assumes that all available NPSS/NSSS/NRS are utilized within each DRX cycle

| DRX cycle length [s] | Tdetect,NB_Intra_NC [s] (number of DRX cycles) | Tmeasure,NB_Intra_NB_NC [s] (number of DRX cycles) | Tevaluate,NB_intra_NB_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.24 (7) | 0.32 (1) | 1.28 (4) |
| 0.64 | 1.92 (3) | 0.64 (1) | 2.56 (4) |
| 1.28 | 2.56 (2) | 1.28 (1) | 6.4 (5) |
| 2.56 | 2.56 (1) | 2.56 (1) | 7.68 (3) |
| 5.12 | 5.12 (1) | 5.12 (1) | 10.24 (2) |
| 10.24 | 10.24 (1) | 10.24 (1) | 20.48 (2) |

Based on the results in table I, for typical DRX cycle length, the Cell reselection delay is far below the serving dwelling time provided by satellites companies in last meeting.

Proposal 1: RAN4 to discuss whether to relax the Cell reselection delay based on Table I, which assumes that all available NPSS/NSSS/NRS are utilized within each DRX cycle. RAN4 to discuss how many samples are assumed within each DRX cycles that UE is required to measurement.

RRC Re-establishment

Based on the same principles, for RRC Re-establishment requirements, Tsearch_NB1-NC,I can be calculated as 1890 ms

Proposal 2: For RRC Re-establishment, Tsearch_NB1-NC is defined as 1890ms

Cell measurement in connected mode

For cell measurement in connected mode, we provide the analysis for intra-frequency and inter-frequency separately. For intra-frequency, Tdetect and Tmeasure are defined as 1400ms and 800/1600ms when DRX is no configured. When DRX is configured, Tdetect and Tmeasure are defined as 6 and 5 DRX cycles. Thus, based on the same assumptions achieved in last meeting, the requirements can be defined as follows:

When DRX is not configured:

- Tdetect = 1890 ms

- Tmeasure = 270 ms for NRS

When DRX is configured:

Considering the shortest DRX cycle 256 ms, there are around two 90ms pattern within each DRX cycle. Thus, to get enough number samples for NPSS/NSSS/NRS, 10 DRX cycles are needed. For measurement, 2 DRX cycles are needed.

- Tdetect = 10 DRX cycles

- Tmeasure = 2 DRX cycles

For inter-frequency, the existing requirements are defined by counting the number of samples. Thus, it is sample for updating based on the agreed number of samples. In details, N is equal to 13 and M is equal to 8.

Proposal 3: For neighbour cell measurement in connected mode, the requirements are defined as follows:

For intra-frequency when DRX is not configured:

- Tdetect = 1890 ms

- Tmeasure = 270 ms for NRS

For intra-frequency when DRX is configured:

- Tdetect = 10 DRX cycles

- Tmeasure = 2 DRX cycles

For inter-frequency:

- N is equal to 13 and M is equal to 8 for NRS

RLM

For RLM requirements, the logic is different from other requirements. For other requirements, measurement delay could be reduced at higher SINR conditions. For RLM, it will impact the UE implementation that at which SINR level that UE will trigger OOS. Though the target working condition is -1 dB, it is not clear whether we can assume that UE is expect to trigger OOS when the SINR is lower than -1dB. It may lead to that UE will easily trigger OOS. It could be observed from current RLM requirements, the existing measurement delay is not the bottleneck. Thus, we propose to reuse the existing RLM core requirements.

Proposal 4: Reuse the existing RLM requirements.

Another issue is about the UE pre-compensation behaviour as asked in RAN1 LS [4], which is shown as follows:

Working assumption For precompensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both

In last RAN1 meeting, RAN1 further achieved following agreements:

Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation

Based on latest RAN1 agreement achieved in last meeting, there is no further issue identified from RAN4 perspective.

Proposal 5: Regarding UE time pre-compensation, based on latest agreements in last meeting, there is no further issue/concerns from RAN4 RRM pespective.

# Conclusions

Observation 1: For IDLE mode Cell reselection, RAN4 needs to discuss how many samples are assumed with in each DRX cycle.

Proposal 1: RAN4 to discuss whether to relax the Cell reselection delay based on Table I, which assumes that all available NPSS/NSSS/NRS are utilized within each DRX cycle. RAN4 to discuss how many samples are assumed within each DRX cycles that UE is required to measurement.

Proposal 2: For RRC Re-establishment, Tsearch_NB1-NC is defined as 1890ms

Proposal 3: For neighbour cell measurement in connected mode, the requirements are defined as follows:

For intra-frequency when DRX is not configured:

- Tdetect = 1890 ms

- Tmeasure = 270 ms for NRS

For intra-frequency when DRX is configured:

- Tdetect = 10 DRX cycles

- Tmeasure = 2 DRX cycles

For inter-frequency:

- N is equal to 13 and M is equal to 8 for NRS

Proposal 4: Reuse the existing RLM requirements.

Proposal 5: Regarding UE time pre-compensation, based on latest agreements in last meeting, there is no further issue/concerns from RAN4 RRM perspective.

# References

[1] RP-242415 New WID on introduction of IoT-NTN TDD mode

[2] RP-243293 Revised WID on introduction of IoT-NTN TDD mode

[3] R4-2508290 WF on RRM requirements for IoT_NTN_TDD

[4] R1-2503142 LS on precompensation for NB-IoT NTN TDD mode