3GPP TSG-RAN WG4 Meeting #116	R4-2510651

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on UE CLI measurement for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	7.21.4.1

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for UE CLI measurement.

- General issues

- Specific issues for L1-SRS-RSRP

- Specific issues for L1-CLI-RSSI

In this paper, we will provide our views on RRM requirements for UE CLI measurement in SBFD.

Discussion

General issues

- Issue 1-1-3: L1 CLI measurement in non-SBFD symbols

In RAN1#121 it is agreed to support L1 CLI measurement on non-SBFD symbols.

Agreement For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

The discussions and agreements for L1 CLI measurement are based on the assumption that the measurement is on SBFD symbols. On whether to define requirements for L1 CLI measurement on non-SBFD symbols, we do not have strong view but slightly prefer to define for completeness, if the standard efforts are limited.

In our view, the existing agreements for L1 CLI measurements are also applicable for non-SBFD symbols. The only additional aspect we identified is the configuration of valid/invalid symbol type. The impact of this configuration has been discussed for CSI-RS measurement, and it is agreed to use NR-U approach to extend the measurement period due to resource occasion colliding with invalid symbol type. The same approach can be re-used for L1 CLI measurement.

Proposal 1: RAN4 to define requirements for L1 CLI measurement on non-SBFD symbols. Use NR-U approach to account for the impact of valid/invalid symbol configuration.

As to the spec change, we understand the description in clause 9.x.3.1 needs to account for L1-CLI-RSSI configured in non-SBFD symbols. In addition, the measurement period in clause 9.x.2.5 for L1-SRS-RSRP needs to be extended based on NR-U approach. Whether measurement period in clause 9.x.3.5 for L1-CLI-RSSI needs to be updated depends on conclusion of Issue 1-3-1.

Specific issues for L1-SRS-RSRP

#### Issue 1-2-3: Measurement restriction

RAN1 has agreed that

- Method #1 and Method #3 cannot be configured in the same OFDM symbol

- Method #2 and Method #3 cannot be configured in the same OFDM symbol

It is straightforward to define measurement restriction between Method #1 and Method #2 following the same principle, i.e. Method #1 and Method #2 cannot be configured in the same OFDM symbol.

Proposal 2: A UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol.

Specific issues for L1-CLI-RSSI

- Issue 1-3-1: Measurement period

In RAN4#115, 2 options on the sample number in measurement period are discussed

- Option 1: 1 sample

- Option 2: 1 or 3 samples based on network configuration

We do not have strong view, but slightly prefer option 2.

The technical benefit of option 2 is that NW can allow UE to do filtering for L1-CLI-RSSI by not configuring timeRestrictionForChannelMeasurement. On the other hand, the requirements do not mandate UE to do filtering. In addition, filtering does not fit the intention of L1 measurement which is to reflect the dynamic CLI status, especially when it does not help to improve the accuracy of RSSI measurement.

However, option 2 does not any harm. Both NW and UE can go back to option 1 with implementation:

- NW can always configure timeRestrictionForChannelMeasurement if does not want filtering.

- UE can always use 1 sample for reporting as it is allowed by the NW.

Based on above analysis, it seems the technical difference between the 2 options is not very critical, and to allow some flexibility in the spec, it is suggested to go with option 2.

Proposal 3: Define L1-CLI-RSSI measurement period with 1 or 3 samples based on network configuration.

- Issue 1-3-3: Measurement restriction

The restriction between L1-CLI-RSSI measurement and L1/L3 RRM measurement in FR1 are still open based on RAN4#114-bis agreement.

- Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

In our view, in FR1 measurement requirements apply for L1-CLI-RSSI could apply in most cases as in most cases UE would be able to measure RSSI and L1/L3 at same time because RSSI measurement is based on DL timing and performed within active DL BWP. “No restriction” should be taken as default, and RAN4 can further define the cases where restriction exists and L1-CLI-RSSI requirements cannot apply.

Based on RAN4#114-bis discussion, we identified the following two cases:

- MG: if RSSI resource collides with MG, it is clear that UE cannot measure RSSI and L1/L3 at same time

- Different SCS: if RSSI resource is configured with a different SCS from the SSB used for L1/L3 measurement, UE may not be able to measure RSSI and L1/L3 at same time

Proposal 4: Measurement requirements apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR1, except for the following cases

- If RSSI resource collides with MG

- If RSSI resource is configured with a different SCS from the SSB used for L1/L3 measurement

Conclusions

In this paper we provided our views on RRM requirements for UE CLI measurement in SBFD.

Proposal 1: RAN4 to define requirements for L1 CLI measurement on non-SBFD symbols. Use NR-U approach to account for the impact of valid/invalid symbol configuration.

Proposal 2: A UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol.

Proposal 3: Define L1-CLI-RSSI measurement period with 1 or 3 samples based on network configuration.

Proposal 4: Measurement requirements apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR1, except for the following cases

- If RSSI resource collides with MG

- If RSSI resource is configured with a different SCS from the SSB used for L1/L3 measurement

Reference

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon