3GPP TSG-RAN WG4 Meeting #116	R4-2510690

# Bangalore, India, 25 Aug - 29 Aug, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Perf part of R19 XR

Agenda Item:	7.26.4

Document for:	Discussion

- Introduction

Until last RAN4 meeting, the core part requirements discussion has already achieved good progress and only some small issues are suspending. In this meeting, the discussion on perf part would kick off and based on the progress in core part, we provide our analysis on perf part in this document.

- Discussion

For the scenarios to test, the following applicable scenarios were approved in core part discussion:

< Agreement>: 112bis meeting - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation < Agreement>: 113 meeting - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID < Agreement>: 114meeting - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements < Agreement>: 114bis meeting - Skipping requirements applies for NR-SA

- So both FR1 NR SA and FR2-1 NR SA should be verified in the test.

- Proposal 1: Both FR1 NR SA and FR2-1 NR SA should be verified in the test.

- Regarding the applicable MG types, besides the Type-1 MG, the MG skipping is still applicable to the Type-2 MG, i.e. the R17 concurrent MGs. Since one essential point for the application of MG skipping is Type-2 MG lies in it is only applicable to the survival MG occasions after the collision handling rule, which is different UE behavior compared with the application of MG skipping in Type-1 MG. As a result, both the Type-1 MG and Type-2 MG should be verified.

- Proposal 2: The skipping in both Type-1 MG and Type-2 MG should be verified in different tests.

- Besides which type of MGs should be verified, another relevant issue is to decide which type of measurements should be verified. Around this point, the following progress were achieved in core part:

<Agreement>: 113 meeting - The following measurement types are considered for XR measurement skipping - Intra-frequency - Inter-frequency - LTE-inter- RAT (NR-EUTRAN FDD/TDD) - The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS. <Agreement> 114 meeting - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported. <Agreement> 114bis meeting - Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping.

- So it is observed that the MG skipping has impact on the following types of measurements:

- Intra-frequency measurement with gap

- Inter-frequency measurement with gap

- EUTRAN measurement with gap

- L1 inter-frequency measurement with gap

- For the measurement without gap, such as L3 measurement without gap and L1 measurement without gap, when such measurements are configured colliding with Type-1 MG or Type-2 MG, including partial overlapping and fully overlapping, UE is not allowed to perform such measurement without gap at the skipped gap occasions, the accompanying scheduling restriction is not allowed at the skipped gap occasions as well. To our understanding, this type of UE behavior should also be verified since it is fully new and not be verified in legacy yet.

- Proposal 3: The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified:

- - Intra-frequency measurement with gap

- - Inter-frequency measurement with gap

- - EUTRAN measurement with gap

- - L1 inter-frequency measurement with gap

- Proposal 4: The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well.

- Furthermore, the UE processing timeline when receiving the DCI based MG skipping indication was defined with baseline and optional capability. These UE processing timeline should be verified in independently or together with other test motivation.

- Proposal 5: The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with out test motivation.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements around the performance part of NES:

- Proposal 1: Both FR1 NR SA and FR2-1 NR SA should be verified in the test.

- Proposal 2: The skipping in both Type-1 MG and Type-2 MG should be verified in different tests.

- Proposal 3: The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified:

- - Intra-frequency measurement with gap

- - Inter-frequency measurement with gap

- - EUTRAN measurement with gap

- - L1 inter-frequency measurement with gap

- Proposal 4: The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well.

- Proposal 5: The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with out test motivation.

- Reference

- R4-2508277 WF on Netw_Energy_NR_enh_part 1, Ericsson, RAN4#115.