3GPP TSG RAN WG4 Meeting #116	 R4-2510703

Bengaluru, India, 25 Aug. – 29 Aug. 2025

Agenda Item:	7.20.4.2

Source: 	Ericsson

Title:	Discussion on CJTC reporting and asymmetric UL TRP requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for the CJT calibration reporting and asymmetric UL TRP requirements.

# Discussion

## Scheduling restrictions and measurement restrictions

During the previous meeting, the following WF was agreed upon, and this section provides our views on the necessity of scheduling restrictions.

- Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

Assuming our understanding of the proponent’s intention is accurate, the rationale for scheduling restrictions originates from the following RAN1 agreement.

Furthermore, the defined range for the 1-bit indicator "dn" is [0, CP], applicable both internally and externally.

We first examine the MRTD conditions pertinent to the multi-TRP framework. The MRTD table from TS 38.133 is reproduced here for reference.

Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18 Frequency Range; Maximum receive timing difference. FR1; CP lengthnote 1. FR2-1; CP lengthnote 1. NOTE 1: CP length of the maximum SCS on the carrier.. Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of rxTimingDiff-r18 Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8.

In our understanding, all the specified requirements for the multi-TRP framework are applicable when the MRTD condition is fulfilled. Values of MRTD greater than CP are supported by a separate UE capability, namely rxTimingDiff-r18; however, its applicability to the CJTC reporting feature remains uncertain. Our intent is to specify the requirements under existing MRTD assumptions, thereby avoiding the introduction of additional scheduling restrictions.

We also believe that the cost associated with scheduling restrictions is substantial. As illustrated in the figure 1 [copied from Nokia email discussion], each TRS resource set configured for CJTC comprises 4 symbols. Implementing a scheduling restriction of one additional symbol per TRS symbol in a TRS set would lead to an substantial overhead, effectively resulting in the loss of 4 symbols across two slots. It is important to highlight that an entire OFDM symbol is lost rather than just the RE adjacent to the TRS. We are uncertain if this aligns with the original intention of RAN1 when this field was introduced as none one the scheduling restrictions are discussed in RAN1. We should note that RAN1 also defines dropping rules or scheduling restrictions.

- CJTC measurement requirements are applicable when the RTD is less than CP.

Figure 1: TRS resource set for CJTC reporting

Measurement restrictions:

The following options were discussed for the measurement restrictions.

- Option 1: Reuse the legacy measurement restriction in FR1 for delay and frequency offset measurement.

  - Measurement restriction does apply,

    - If UE does not support simultaneousRxDataSSB-DiffNumerology and

    - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols.

  - Measurement restriction does not apply,

    - If UE support simultaneousRxDataSSB-DiffNumerology or

    - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH.

- Option 2: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

Since the Delay and frequency offset measurements are based on the TRS (NZP CSI-RS), UE would be able to receive them simultaneously. We do not think there is a need for restriction for the reception of the measurement RS. For computation and processing, CJTC is part of the CSI framework and RAN1 already defined the limitation on the CSI processing units for the simultaneous CSI computation. Hence, we do not see a need for defining separate measurement restrictions in RAN4.

- RAN4 not to define additional measurement restrictions for the CJTC reporting.

## CJTC reporting delay

RAN4 has discussed the CJTC core requirements over several meetings. The agreement from the last meeting is as follows:

- RAN4 will discuss the achievable accuracy in terms of delay and frequency offsets based on 1, 3, or 5 samples in the next meeting.

- RAN4 will conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset has been deprioritized for Rel-19.

To determine whether a single sample is sufficient or if 3 or 5 measurement samples are needed, we must first understand the desired accuracy for CJTC. As discussed in the previous meeting, different applications—such as coherent combining and nulling—require varying levels of phase estimation accuracy from the UE, depending on the operating region with respect to SNR. Consequently, each use case demands a different total error budget. For example:

- Coherent combining: total error budget of 50 degrees

- Nulling: total error budget of 30 degrees (low SNR) and 10 degrees (high SNR)

For CJT, we propose adopting a total error budget of 50 degrees for coherent combining. Within this budget, we suggest allocating 60% (i.e., 30 degrees) to UE measurement error. The remaining 20 degrees would account for other contributing error sources, as outlined in our previous paper, R4-2413017. These include:

- UE reporting resolution/quantization error

- BS algorithmic compensation error

- Inter-TRP phase drift

- Potential inter-TRP mast sway

- Potential UE mobility

Additionally, we note that this approach aligns with the analysis and recommendations presented by QC in their paper R4-2503128.

Based on submitted results from different companies, we averaged the data and presented it in the table below. The table also includes the corresponding phase drift resulting from FO estimation error.

Requirements for Frequency Offset:

Phase drift over a slot (across all symbols): 2pi * FO * slot_duration_s *180/pi = 360*FO_Hz * slot_duration_s (deg).

One ppb level in Hz: 1/256 * 200ppb*3.5e9 = 2.734 Hz

Table 1: FO offset and its corresponding phase rotation

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256. Levels (Hz →degrees) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 44 (120.296 → 43.307) | 27 (73.818 → 26.574) | 19 (51.946 → 18.701) |
| 15KHz | 0 | 350 Hz | 32 (87.488 → 31.496) | 21 (57.414 → 20.669) | 13 (35.542 → 12.795) |
| 15KHz | 3 | 350 Hz | 20 (54.680 → 19.685) | 11 (30.074 → 10.827) | 8 (21.872 → 7.874) |
| 30KHz | -3 | 350 Hz | 88 (240.592 → 43.307) | 43 (117.562 → 21.161) | 38 (103.892 → 18.701) |
| 30KHz | 0 | 350 Hz | 72 (196.848 → 35.433) | 42 (114.828 → 20.669) | 30 (82.020 → 14.764) |
| 30KHz | 3 | 350 Hz | 38 (103.892 → 18.701) | 19 (51.946 → 9.350) | 16 (43.744 → 7.874) |

Based on the averaged results from different companies, we converted the data into phase rotation (in degrees) to evaluate how many samples are needed to meet the coherency budget of 30 degrees.

- For 15 kHz and 30kHz, both 3 and 5 samples are sufficient to meet the coherency budget at SNR > -3dB.

- For 15 kHz and 30 kHz, 1 sample achieve phase coherency at an SNR of 3 dB.

Based on these findings, to establish a common requirement across both 15 kHz and 30 kHz subcarrier spacings (SCS), we recommend using 3 samples and maintaining an SNR greater than -3 dB as a side condition for meeting the frequency offset requirements.

- RAN4 to agree on using 3 samples for the frequency offset requirements for SNR > -3dB.

Requirements for Delay offset (DO):

Based on submitted results from various companies, we averaged the data and presented it in the table below. The table also includes the corresponding phase drift resulting from DO estimation error. Since phase shift depends on the sub band size, we consider different sub band sizes to compute the phase drift across each sub band.

Phase drift across bandwidth: 2*pi*DO*BW_Hz*180/pi = DO BW Hz*360

Each level in µs =4.7µs/256

Table 2: DO and the sub band wise phase rotation for sub band size of 48 PRB

| SCS | SNR | DO estimation error in AD = CP and MD = 256 Levels for the case of 48 PRB (degrees after wraparound of 180 degrees) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 18 (127.89) | 11 (88.155) | 10 (31.05) |
| 15KHz | 0 | 350 Hz | 14 (79.47) | 9 (153.945) | 7 (39.735) |
| 15KHz | 3 | 350 Hz | 8 (96.84) | 5 (105.525) | 4 (48.42) |
| 30KHz | -3 | 350 Hz | 15 (93.15) | 9 (127.89) | 8 (13.68) |
| 30KHz | 0 | 350 Hz | 12 (110.52) | 7 (79.47) | 6 (145.26) |
| 30KHz | 3 | 350 Hz | 9 (127.89) | 5 (31.05) | 5 (31.05) |

Table 3: DO and the sub band wise phase rotation for sub band size of 8 PRB

| SCS | SNR | DO estimation error in AD = CP and MD = 256 Levels (degrees) for the case of 8RB |  |  |
| --- | --- | --- | --- | --- |
|  |  | #1 | #3 | #5 |
| 15KHz | -3 | 18 (171.315) | 11 (104.692) | 10 (95.175) |
| 15KHz | 0 | 14 (133.245) | 9 (85.658) | 7 (66.623) |
| 15KHz | 3 | 8 (76.140) | 5 (47.587) | 4 (38.070) |
| 30KHz | -3 | 15 (285.526) | 9 (171.316) | 8 (152.28) |
| 30KHz | 0 | 12 (228.42) | 7 (133.246) | 6 (114.21) |
| 30KHz | 3 | 9 (171.316) | 5 (95.174) | 5 (95.174) |

Table 4: DO and the sub band wise phase rotation for sub band size of 4 PRB

| SCS | SNR | DO estimation error in AD = CP and MD = 256 Levels (degrees) for the case of 4RB |  |  |
| --- | --- | --- | --- | --- |
|  |  | #1 | #3 | #5 |
| 15KHz | -3 | 18 (85.658) | 11 (52.346) | 10 (47.588) |
| 15KHz | 0 | 14 (66.622) | 9 (42.830) | 7 (33.312) |
| 15KHz | 3 | 8 (38.070) | 5 (23.794) | 4 (19.036) |
| 30KHz | -3 | 15 (142.764) | 9 (85.658) | 8 (76.140) |
| 30KHz | 0 | 12 (114.210) | 7 (66.624) | 6 (57.104) |
| 30KHz | 3 | 9 (85.658) | 5 (47.588) | 5 (47.588) |

Table 5: DO offset and the sub band wise phase rotation for sub band size of 2 PRB

| SCS | SNR | DO estimation error in AD = CP and MD = 256 Levels (degrees) for the case of 2RB |  |  |
| --- | --- | --- | --- | --- |
|  |  | #1 | #3 | #5 |
| 15KHz | -3 | 18 (42.829) | 11 (26.173) | 10 (23.794) |
| 15KHz | 0 | 14 (33.311) | 9 (21.415) | 7 (16.656) |
| 15KHz | 3 | 8 (19.035) | 5 (11.897) | 4 (9.518) |
| 30KHz | -3 | 15 (71.382) | 9 (42.829) | 8 (38.070) |
| 30KHz | 0 | 12 (57.105) | 7 (33.312) | 6 (28.552) |
| 30KHz | 3 | 9 (42.829) | 5 (23.794) | 5 (23.794) |

From these results, we observed that sub-band sizes greater than 8 PRB lead to phase rotations exceeding 180 degrees. For CJTC, phase rotations above 90 degrees typically result in worse performance than single TRP transmission. To enable CJT feature effectively, we believe NW may need to use sub-band sizes of 2 or 4 PRB, depending on the SCS. For a unified requirement across both 15 kHz and 30 kHz SCS, we recommend using a sub-band size of 2 PRB.

In the tables, we highlighted the configurations where the phase rotation across the sub-band is less than 30 degrees or less than 35 degrees. Based on these highlighted results, we recommend using 5 measurement samples under the side condition of SNR > 0 dB for both 15 kHz and 30 kHz SCS.

- RAN4 to agree on using 5 samples for the delay offset requirements for SNR > 0 dB.

Asymmetric DL sTRP/UL mTRP

Moderator’s note on the scenarios’ definition discussed in this section:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

When UE is configured with PL offset (Scenario 1)

For scenario 1, one of the open issue is gradual timing adjustment requirements.

Gradual timing adjustment requirements:

We think Rel-18 principles can be reused for the requirements definition. We copy the Rel-18 requirement below for reference.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

Since the single DL reference timing is used, the gradual adjustment UE should apply for each TAG can be same.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG. The amount of gradual timing adjustment is same for both TAG.

Timing requirements for PRACH:

Regarding how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP, following agreement is reached in previous meeting.

For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signals(s) of the reference cell associated with DL or Joint TCI state.

  - The above agreement can be revisited if not aligned with RAN1 further agreement.

We note that the agreement above pertains to PUCCH, PUSCH, and SRS. We believe it is crucial to discuss the downlink reference timing for PRACH, especially as PRACH is supported for uplink-only TRP.

- For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG Above applies for the case UE is configured with SSB-MTC-additionalPCI

To enable two TAs with respect to two TRPs, UE need to obtain DL reference timing and UL TA command towards UL-only TRP. Next, we further discuss the aspects of DL reference timing and UL TA towards the UL-only TRP. The two-TA feature in Rel-18 assumes non-ideal synchronization (i.e., TRPs slot boundaries are not perfectly time aligned and there can be synchronization error of up to e.g., 3µs) between the TRPs. As per the WID, asymmetric and non-collocated deployment is supported for UL-only TRP. That means the synchronization assumptions in Rel-18 are applied for Rel-19 UL-only TRP also as operators may not deploy separate TRPs for using the UL-only TRP feature (e.g., operators may use same deployment for Rel-18 and Rel-19).

As UL-only TRP do not transmit any SSB, UE may have to rely on Anchor TRP SSB to derive the DL reference timing. When slot boundaries of Anchor TRP and UL-only TRP are perfectly aligned in time (i.e., synchronization error is 0µs), UE may use the DL reference timing of Anchor TRP to perform random access procedure and obtain TA command to handle the asymmetric propagation delay difference between Anchor TRP to UE and UL-only TRP to UE.

However, in the practical deployments (such asymmetric deployments considered in this WI) it may be sometimes impossible to maintain perfect synchronization between different TRPs even for intra-DU deployments, since most of the NW deployments use switched fronthaul technic between non-collocated TRPs. Figure 2 illustrates the synchronization scenario where the timing of UL-only TRP is ahead of Anchor TRP. When UL-only TRP is ahead of Anchor TRP slot timing, when UE acquires DL timing from Anchor TRP and transmits PUSCH using the same DL reference timing, it may result in inter-slot interference at UL-only TRP.

Figure 2: Synchronization scenario UL-only TRP ahead of Anchor TRP slot timing

As per TS 38.133, initial transmit timing for RACH transmission, UE needs to acquire frame boundary within ±Te error (Te is much smaller compared to possible synchronization error between TRP), and transmits RACH after acquiring DL synchronization, RACH preamble is always expected to receive after the start of the RACH slot at the NW.

To analyze the issue with practical synchronization error for UL-only TRP, we consider three possible synchronization scenarios as shown in Figure 2. When the UL slot boundaries of anchor TRP and UL- only TRP are not perfectly synchronized (scenario 2 and 3) as shown in Figure 2, using DL timing of anchor TRP to transmit RACH preamble to UL TRP may result in early arrival of RACH preamble or later arrival of the RACH preamble at the UL- only TRP.

Figure 3: Different synchronization scenarios at UL- only TRP and anchor TRP

When UL- only TRP is behind anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in early arrival of RACH preamble at UL- only TRP (i.e., before the start of RACH occasion or RACH slot at the reception window at the UL TRP as shown in figure 3. This results in inter-slot interference at the UL- only TRP and UL TRP may not be able to determine the timing advance correctly for the UE.

Figure 4: RACH arrival at UL- only TRP and Anchor TRP with anchor TRP DL reference timing

When UL- only TRP is ahead of anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in late arrival of RACH preamble at UL- only TRP. Based on the below table showing guard period (GP) for different PRACH preamble formats, we think late arrival may not be an issue for the UL- only TRP deployment scenario where the UE is expected to be close to UL- only TRP and GP is large compared to synchronization error.

Table 6: Guard period for different PRACH preamble formats

| Preamble format | 0 | 1 | 2 | 3 | B1 | B2 | B3 | B4 | C0 | C2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP in µs | 97 | 716 | 953 | 97 | 2.3 | 7 | 11.7 | 25.8 | 37 | 94 |
| Propagation delay (in km) that GP can cover | 29km | 215km | 285km | 29km | 0.7700m | 2.1km | 3.7km | 7.5km | 11km | 28km |

Based on the above discussion, we have the following proposal to consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in an asymmetric deployment, which for example could be handled by introducing a DL timing offset between the Anchor TRP for the RACH transmission towards the UL-only node. The DL reference timing offset can be a fixed value, which may be of the order of TAE or 0.5 TAE. The value can be further discussed.

- UE to apply additional DL reference timing offset for the PRACH transmission. The DL reference timing offset can be [0.5*TAE].

When UE is not configured with PL offset and SSB is available (Scenario 2)

For Scenario 2, we reached the following agreement in last meeting.

For Scenario#2, inter-cell operation, PRACH reference point:

  - reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL [or UL] TCI state(s) of the reference cell associated with the second TAG.

For inter-cell multi-TRP, for PUCCH/PUSCH/SRS:

    - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

Most aspects of the agreement align with the RAN1 LS from the previous meeting. However, one company has proposed an additional change: introducing a UL TCI state alongside the existing DL or Joint TCI state for the timing reference derivation.

In Scenario 2, where only the SSB may be transmitted without other DL signals or channels from the UL TRP, certain network implementations might opt not to configure a joint TCI state. This is mainly because the UL TRP may not have DL transmissions such as PDCCH or PDSCH to support a meaningful DL TCI configuration. In such cases, configuring only the UL TCI state may be a reasonable from NW side. The UL TCI state can include the DL RS in its QCL chain, allowing the DL RS associated with the UL TCI state to serve as the reference point for PRACH, PUCCH, PUSCH, and SRS. We believe that adding the UL TCI state, as discussed in the last meeting's WF, is a reasonable option.

- For Scenario#2, inter-cell operation, PRACH reference point:

  - reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or UL TCI state(s) of the reference cell associated with the second TAG.

  - For inter-cell multi-TRP, for PUCCH/PUSCH/SRS:

    - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or UL TCI states having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- CJTC measurement requirements are applicable when the RTD is less than CP.

- RAN4 not to define additional measurement restrictions for the CJTC reporting.

CJTC reporting

- RAN4 to agree on using 3 samples for the frequency offset requirements for SNR > -3dB.

- RAN4 to agree on using 5 samples for the delay offset requirements for SNR > 0 dB.

Asymmetric DL sTRP/UL mTRP:

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG. The amount of gradual timing adjustment is same for both TAG.

- UE to apply additional DL reference timing offset for the RACH transmission. The DL reference timing offset can be [0.5*TAE].

- For Scenario#2, inter-cell operation, PRACH reference point:

  - reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or UL TCI state(s) of the reference cell associated with the second TAG.

  - For inter-cell multi-TRP, for PUCCH/PUSCH/SRS:

    - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or UL TCI states having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

# References

[1] RP-234007, New WID: NR MIMO Phase 5
[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5

[3] R4-2420209 Simulation assumptions for CJT calibration