3GPP TSG-RAN WG4 Meeting # 116	R4-2510748

# Bengaluru, India, 25th – 29th Aug, 2025

Source:	LG Electronics

Title:	Discussion for UE RF Rx requirements of NTN_NR_HPUE

Agenda item:	7.6.3.2.2

Document for:	Discussion

Introduction

In the RAN4#115, the WF on HPUE requirements for NR-NTN was approved. In this discussion, we share our point of view for the HPUE requirements for NR-NTN based on the approved WF [1].

Discussion

Table 1 Way forward for UE RF requirements for NTN_NR_HPUE [1]

Issue 2-1: RSD for PC2 Agreement: - To confirm the n255 RSD for PC2 as follows: n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). RSD@1Tx; 0.8; 0.8; 0.9; 0.9. RSD@2Tx; 1.6; 1.6; 1.7; 1.7. Issue 2-2: RSD for PC1.5/PC1 non-handheld devices Agreement: - Consider [0] dB RSD for PC1.5/PC1 non-handheld devices for the current NTN bands with the following. - To document 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD in the TR. - Revisit this value with more inputs in next meeting.

Reference sensitivity degradation for NTN HPUE

In the previous meeting, we proposed RSD analysis values based on a 66 dB filter rejection. This value was chosen under the assumption that non-handheld devices, due to their form factor, could accommodate filters with higher rejection than handheld devices. However, during the discussion, concerns were raised that achieving filter rejection above 66 dB might be challenging even for non-handheld devices. Taking this into account, we conducted further analysis using a lower filter rejection value. Additionally, RSD analysis for the 4Tx architecture was conducted based on the structure shown in Fig. 1.

Figure 1 4Tx reference architecture

- For 2Tx PC1.5

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0 | 0 | 0.1 | 0.1 |
|  | n255 | 0.3 | 0.3 | 0.3 | 0.3 |
| 55 | n256 | 0.0 | 0.0 | 0.1 | 0.1 |
|  | n255 | 0.8 | 0.8 | 0.8 | 0.8 |
| 50 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 2.0 | 2.1 | 2.1 | 2.1 |

- For 1Tx PC1.5

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0.0 | 0.0 | 0.1 | 0.1 |
|  | n255 | 0.1 | 0.1 | 0.1 | 0.2 |
| 55 | n256 | 0.0 | 0.0 | 0.1 | 0.1 |
|  | n255 | 0.3 | 0.3 | 0.3 | 0.3 |
| 50 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 0.8 | 0.8 | 0.8 | 0.8 |

- For 1Tx PC1

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0 | 0 | 0.1 | 0.1 |
|  | n255 | 0.3 | 0.3 | 0.3 | 0.3 |
| 55 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 0.7 | 0.7 | 0.7 | 0.7 |
| 50 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 1.4 | 1.4 | 1.5 | 1.5 |

- For 4Tx PC1

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 0.4 | 0.4 | 0.4 | 0.4 |
| 55 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 1.0 | 1.0 | 1.0 | 1.0 |
| 50 | n256 | 0.2 | 0.2 | 0.2 | 0.2 |
|  | n255 | 2.5 | 2.5 | 2.5 | 2.5 |

Based on the analysis, it was confirmed that with a filter rejection of up to 60 dB, the RSD values for 1Tx/2Tx PC1.5 and 1Tx/4Tx PC1 in bands n256 and n255 remain below 0.4 dB, which can be considered negligible. However, when the filter rejection falls below 60 dB, the n256 band still shows small RSD values, indicating that defining RSD may not be necessary. In contrast, the n255 band exhibits significantly larger RSD values under the same conditions, suggesting that RSD should be defined for this band.

Observation: Based on the analysis, it was confirmed that with a filter rejection of up to 60 dB, the RSD values for 1Tx/2Tx PC1.5 and 1Tx/4Tx PC1 in bands n256 and n255 remain below 0.4 dB, which can be considered negligible. However, when the filter rejection falls below 60 dB, the n256 band still shows small RSD values, indicating that defining RSD may not be necessary. In contrast, the n255 band exhibits larger RSD values under the same conditions.

Proposal: If a filter rejection lower than 60 dB is considered for non-handheld devices, it is necessary to define RSD for 1Tx/2Tx PC1.5 and 1Tx/4Tx PC1 in the n255 band.

Conclusion

Observation: Based on the analysis, it was confirmed that with a filter rejection of up to 60 dB, the RSD values for 1Tx/2Tx PC1.5 and 1Tx/4Tx PC1 in bands n256 and n255 remain below 0.4 dB, which can be considered negligible. However, when the filter rejection falls below 60 dB, the n256 band still shows small RSD values, indicating that defining RSD may not be necessary. In contrast, the n255 band exhibits larger RSD values under the same conditions.

- For 2Tx PC1.5

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0 | 0 | 0.1 | 0.1 |
|  | n255 | 0.3 | 0.3 | 0.3 | 0.3 |
| 55 | n256 | 0.0 | 0.0 | 0.1 | 0.1 |
|  | n255 | 0.8 | 0.8 | 0.8 | 0.8 |
| 50 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 2.0 | 2.1 | 2.1 | 2.1 |

- For 1Tx PC1.5

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0.0 | 0.0 | 0.1 | 0.1 |
|  | n255 | 0.1 | 0.1 | 0.1 | 0.2 |
| 55 | n256 | 0.0 | 0.0 | 0.1 | 0.1 |
|  | n255 | 0.3 | 0.3 | 0.3 | 0.3 |
| 50 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 0.8 | 0.8 | 0.8 | 0.8 |

- For 1Tx PC1

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0 | 0 | 0.1 | 0.1 |
|  | n255 | 0.3 | 0.3 | 0.3 | 0.3 |
| 55 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 0.7 | 0.7 | 0.7 | 0.7 |
| 50 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 1.4 | 1.4 | 1.5 | 1.5 |

- For 4Tx PC1

| Filter rejection (dB) | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| 60 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 0.4 | 0.4 | 0.4 | 0.4 |
| 55 | n256 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | n255 | 1.0 | 1.0 | 1.0 | 1.0 |
| 50 | n256 | 0.2 | 0.2 | 0.2 | 0.2 |
|  | n255 | 2.5 | 2.5 | 2.5 | 2.5 |

Proposal: If a filter rejection lower than 60 dB is considered for non-handheld devices, it is necessary to define RSD for 1Tx/2Tx PC1.5 and 1Tx/4Tx PC1 in the n255 band.

- References

# [1] R4-2505113, WF on HPUE requirments for NR-NTN (Samsung)

# [2] R4-2506312, Discussion for UE RF Rx requirements of NTN_NR_HPUE(LGE)