3GPP TSG-RAN WG4 Meeting #116 	    R4-2510763

Bengaluru, IN, 25 – 29 August 2025

Agenda Item:	7.19.3.1

Source: 	Ericsson

Title:	On requirements for RRM measurement prediction for AI/ML based mobility

Document for:	Approval

# Introduction

This contribution discusses issues related to the RRM measurement prediction for AI/ML aided mobility. The issues related to temporal prediction, and frequency prediction are particularly discussed.

# Discussion

## Intra-cell Temporal Prediction

Two cases, as elaborated in Figure 1 and Figure 2, are being considered in RAN2 evaluations for the temporal prediction of RRM measurements. In both cases, measurements performed on a cell is used by the UE to predict the RRM measurements for the future time instances for the same cell.

Temporal prediction case-A:

In temporal prediction case-A, UE performs measurement within the observation window (OW) and uses these measurements to perform prediction of measurements valid for the prediction window (PW). An OW consists of one or more measurement occasions and within a PW there can be one or more than one prediction occasions. A high-level overview of OW, measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 2. The prediction occasions or PW starts after the completion of the measurement occasions or OW in temporal prediction case-A.

Figure 1.  Example elaboration of temporal prediction case-A.

Temporal prediction case-B:

Like the temporal prediction case-A, an OW consists of one or more measurement occasions and within a PW there can be one or more prediction occasions in temporal prediction case-B. The UE is expected to perform measurements in measurement occasions and predict the measurements valid for the prediction occasions inside the prediction window. Unlike temporal prediction case-A, within the OW the UE can skip performing measurements on some of the measurement occasions in temporal prediction case-B. A high-level overview of OW, measurement occasions within OW, skipped measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 3. Like temporal prediction case-A, the prediction occasions or PW starts after the completion of the measurement occasions or OW in temporal prediction case-B.

Figure 2. Example elaboration of temporal prediction case-B.

Simulation results submitted by the participating companies for the intra-frequency intra-cell scenarios were analyzed in RAN2#127bis and RAN2#128 meetings and the following were concluded.

Agreements from RAN2#127bis [1]

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors.

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy. RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy.

Agreements from RAN2#128 [2]

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet.

- We can capture in the TR how the skipping pattern affects the performance.

From the RAN2 evaluation and the agreements above, it can be concluded that the accuracy of the predicted measurements for a fixed measurement reduction rate in temporal domain (MRRT) depends on the measurement skipping pattern within an OW.

Observation 1: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW.

In RAN4#114 meeting it was agreed to consider the impact of UE speed, MRRT, OW, PW and the number of measurements performed during the OW on the prediction accuracy requirements for temporal predictions. The agreement however does not consider the impact of measurement skipping pattern on the prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT. This issue shall be considered for further study in RAN4.

Agreement from RAN4#114 [3]

- RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

Proposal 1: RAN4 to consider the impact of measurement skipping pattern within the OW on prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT.

In the last meeting testing setup for intra-cell prediction was discussed. Based on the discussion it was agreed to consider two cell setup for testing of L3-RSRP predictions for intra-cell scenario, where the measurement and prediction are to be performed by the UE on the same cell. Given that it has been agreed to consider both absolute and relative accuracy as the candidate metrics for the predicted L3-RSRP measurements, definition for the relative accuracy of predicted L3-RSRP measurement for intra-frequency intra-cell scenario also needs to be defined by RAN4. For temporal domain predictions the relative accuracy requirement should be based on two measurements (measured or predicted) from the same cell. The relative accuracy definition of predicted L3-RSRP measurement should therefore be extended to also capture the intra-cell (temporal domain) prediction scenario.

Agreement on Testing setup from RAN4#115 [4]

For testing in FR1, conducted testing is considered as a baseline.

For the number of cells configured in the testing, two cells are considered including serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. In the test, the measurement and prediction are performed on the same cell.

Editor Note: RAN4 will further decide if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

Proposal 2: Relative accuracy of predicted L3-RSRP measurements for intra-cell scenario (temporal prediction) is defined as:

  - Relative accuracy of predicted L3-RSRP measurement = (reported predicted L3-RSRP1 of a cell at time T1 – reported L3-RSRP2 of the same cell at time T2) – (ground truth of L3-RSRP1 of a cell at time T1 – ground truth of L3-RSRP2 of the same cell at time T2).

    - Reported L3-RSRP2 can be predicted or measured.

    - T1 refers to time instance for predicted L3-RSRP1.

    - T2 refers to time instance for measured L3-RSRP2 or predicted L3-RSRP2.

## Inter-cell Relative RSRP Accuracy

In the last meeting, the following definitions for relative accuracy for predicted L3-RSRP measurement in inter-cell intra/inter frequency prediction were agreed [5]. Whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy for intra and inter frequency L3-RSRP predictions in inter-cell scenario was agreed for further study.

For intra-frequency L3-RSRP prediction [5]:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

  - cell 1 and cell 2 are on the same frequency

  - the reported RSRP of cell2 can be measured or predicted.

  - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

- Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

For inter-frequency L3-RSRP prediction [5]:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

  - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

  - the reported RSRP of cell2 can be measured or predicted.

  - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

In our view, relative accuracy requirement for predicted L3-RSRP measurement needs to be defined to ensure that the UE reported measurements for two cells under similar condition are unbiased and meet a specified accuracy requirement. In this regard, to calculate the relative accuracy of predicted L3-RSRP values in inter-cell scenario, the predicted L3-RSRP of a cell needs to be compared with the ground truth L3-RSRP of the same cell for the same time instance and the reported L3-RSRP of both cells are predicted for time instances that are within a certain time boundary. From RAN4 point of view, it is imperative to identify the time boundary for which the relative accuracy requirement for predicted L3-RSRP measurement needs to be specified. Given the reporting framework for predicted L3-RSRP measurements is yet to be concluded in RAN2, our view is to come back to this issue once the reporting framework for predicted L3-RSRP measurements in inter-cell scenario is concluded in RAN2.

Proposal 3: RAN4 to conclude on the time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy for intra and inter frequency L3-RSRP predictions in inter-cell scenario after the reporting framework for predicted L3-RSRP measurement is concluded in RAN2.

# Summary

In this paper issues related to temporal prediction, and frequency prediction are discussed. The observation and proposals below summarize the discussion presented in this paper.

Observation 1: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW.

Proposal 1: RAN4 to consider the impact of measurement skipping pattern within the OW on prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT.

Proposal 2: Relative accuracy of predicted L3-RSRP measurements for intra-cell scenario (temporal prediction) is defined as:

  - Relative accuracy of predicted L3-RSRP measurement = (reported predicted L3-RSRP1 of a cell at time T1 – reported L3-RSRP2 of the same cell at time T2) – (ground truth of L3-RSRP1 of a cell at time T1 – ground truth of L3-RSRP2 of the same cell at time T2).

    - Reported L3-RSRP2 can be predicted or measured.

    - T1 refers to time instance for predicted L3-RSRP1.

    - T2 refers to time instance for measured L3-RSRP2 or predicted L3-RSRP2.

Proposal 3: RAN4 to conclude on the time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy for intra and inter frequency L3-RSRP predictions in inter-cell scenario after the reporting framework for predicted L3-RSRP measurement is concluded in RAN2.

# References

- Chair’s note from RAN2#127bis, RAN2 Chair (InterDigital).

- Chair’s note from RAN2#128, RAN2 Chair (InterDigital).

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2508269, WF on FS_NR_AIML_Mob_Part2, OPPO.

- R4-2508268, WF on AI/ML Mobility Part 1, Moderator (Nokia).