3GPP TSG-RAN WG4 Meeting #116																		R4-2510810

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.18.3

Source: 	OPPO

Title: 	Testing issues for CSI compression two-sided models

Document for:	Approval

- Discussion

For a two-sided AI/ML model, the information transmitted through air-interface (e.g. PMI) is generated by AI/ML models (e.g. CSI encoder), rather than codebooks predefined by protocol. Regarding the testability of two-sided model, it is necessary to consider bring in a test model to cooperate with the model under test.

In the previous RAN4 meetings, 4 options on test decoder have been proposed and discussed, i.e.,

- Option 1: test decoder is provided by the vendor of the encoder

- Option 2: test decoder is provided by the vendor of the decoder

- Option 3: The test decoder(s) are fully specified and captured in RAN4 spec.

- Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

In RAN4 #110 meeting, RAN4 agreed to further discuss only options 3 and 4, i.e., fully or partially specify test decoder(s).

From our understanding, to define a test decoder in specification, following aspects should be taken into account.

-  Principles to define test decoder(s)

- Meet the minimum performance requirement

The test model is used to meet the minimum performance requirement in RAN4 tests, not directly to be used for commercial utilization and deployment.

- To be a simple design

As a test model to be specified in RAN4 spec, we may have a lot of proposals from different companies. How to do the merge and compromise among different designs and views from different companies would be a big challenge and a tough work. From the beginning, we can try to design the test model(s) as simple as possible, to facilitate calibration and promote consistency among different companies.

- Consider different UE capabilities

For AI/ML cases, UEs may vary in their AI/ML capabilities, including supported models, architectures, and computational complexities. When introducing test decoders and defining minimum performance requirements, RAN4 should account for potential testing limitations arising from these differing UE capabilities

Proposal 1: Principles to define test decoder(s)

- To meet the minimum performance requirement in RAN4 tests

- To be a simple design

- Consider different UE capabilities, including supported models, architectures, and computational complexities

-  Relation between RAN4 option 3 and RAN1 inter-vendor collaboration direction C

In 3GPP RAN#108, inter-vendor training collaboration for two-sided AI/ML models has been discussed and agreed as follows

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Based on our understanding, while both RAN4 option 3 and RAN1 direction C may involve defining a test model or reference model, they address different issues and require separate solutions. The following key differences should be taken into account in RAN4 following works:

1) Motivation

RAN1 aims to define a reference model that meets commercial deployment requirements (minimum performance requirements for deployment), which imposes relatively high demands on model performance. In contrast, RAN4 focuses on specifying a test decoder that fulfills testing requirements (minimum performance requirements for testing). Thus, the key question is: which of the following standardization approaches should RAN4 adopt?

- Option 1: RAN4 develops a standardized model based on RAN4’s testing requirements first, then RAN1 evaluates whether it can also be leveraged for RAN1 purposes.

- Option 2: RAN4 develops a standardized model that fulfils both RAN4’s testing requirements and RAN1’s commercial deployment requirements.

- Option 3: RAN4 develops two separate standardized models, one to meet RAN4’s testing needs and another one to fulfill RAN1’s commercial deployment requirements.

For option2 and option3 mentioned above, RAN1’s requirements should be clarified and let RAN4 know.

2) Model structure

RAN1 is considering standardizing a more complex transformer-based CSI encoder and decoder, while RAN4 current studies are still based on a CNN-based CSI decoder as the starting point for assessing the feasibility of standardizing a test decoder. The model structure to be specified(one or more, and which backbone structure to be used)should be clarified in RAN4.

3) Testing data & Performance requirement

RAN1’s CSI evaluation is conducted in UMa scenarios, and its reference model is expected to operate in realistic wireless channel conditions. For RAN4, however, PMI testing is performed over TDL channels, which also influences performance metric definitions.

4) Assumptions on generalization & Scalability consideration

RAN1 expects the standardized CSI encoder and decoder to effectively address generalization and scalability challenges. RAN4 should further check whether in RAN4 tests RAN4 should also consider the generalization and scalability issues.

Observation 1: While both RAN4 option 3 and RAN1 direction C may involve defining a test model or reference model, they address different issues and require separate solutions. The following key differences should be taken into account in RAN4 following works

- Motivation

- Model structure

- Testing data & Performance requirement

- Assumptions on generalization & Scalability consideration

Proposal 2: Regarding the inter-vendor training collaboration for two-sided AI/ML models, to fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account, how to take RAN1 requirements into account should clarified first. Following way forward could be discussed in RAN4 to check companies’ view.

- Option 1: RAN4 develops a standardized model based on RAN4’s testing requirements first, then RAN1 evaluates whether it can also be leveraged for RAN1 purposes.

- Option 2: RAN4 develops a standardized model that fulfills both RAN4’s testing requirements and RAN1’s commercial deployment requirements.

- Option 3: RAN4 develops two separate standardized models, one to meet RAN4’s testing needs and another one to fulfill RAN1’s commercial deployment requirements.

For option2 and option3, RAN1’s requirements should be clarified and let RAN4 know.

-  Updated simulation results for AI/ML CSI compression

In order to elaborate option 3 track 2 considering lower complexity encoders, companies are encouraged to contribute simulations for lower complexity encoder structures in RAN1 #115 meeting, including:

- Performance assuming training and testing with the Samsung frozen decoder

- Performance achieved with a jointly trained low complexity encoder and decoder

- Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity)

- Consider transformer, CNN, MLP

In addition to the agreed CNN-based CSI encoder, three different low-complexity encoders will be evaluated. Their corresponding FLOPs, model size, and backbone descriptions are summarized in Table 1, with detailed descriptions provided in Appendix-2.

Table 1  Model assumptions for encoder with lower complexity

|  | FLOPs | Model size | Description |
| --- | --- | --- | --- |
| Encoder-1 | 94.1M | 0.13Mbyte | CNN-based encoder |
| Encoder-2 | 3.63M | 1.6Mbyte | Transformer-based low-complexity encoder |
| Encoder-3 | 4.14M | 92Kbyte | CNN-based low-complexity encoder |
| Encoder-4 | 0.22M | 0.11Mbyte | MLP- based low-complexity encoder |

As discussed in 3GPP RAN4 Meeting #115, the verification procedures for these low-complexity encoders are outlined in Table 2 as below[1].

Table 2  Verification procedures for these low-complexity encoders[1]

SGCS-0: Based on the agreed CSI Encoder-1 structure and the agreed decoder structure, trained on the mixed dataset to obtain SGCS-0.

SGCS-1: Compared to SGCS-0, the CSI decoder is frozen. Encoder 1 is trained on the mixed dataset to obtain SGCS-1.

SGCS-2: Compared to SGCS-1, multiple encoders are evaluated, including Encoder-2 (Transformer-based), Encoder-3 (CNN-based), and Encoder-4 (MLP-based). The decoder remains the Samsung-provided frozen decoder.

SGCS-3: Compared to SGCS-2, the decoder is no longer frozen but is constrained to the agreed CSI decoder structure and participates in training. A dedicated decoder is joint-trained with each encoder using the mixed dataset, to obtain corresponding SGCS-3 per encoder.

SGCS-4: Compared to SGCS-3, one decoder is trained to match different encoders, instead of training a dedicated decoder per encoder.

The updated simulation results for low-complexity encoders with different backbones are shown in Table 3

Table 3  Simulation results for low-complexity encoders

| Option 3 low-complexity encoders with different backbones |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | （1-to-1）Joint training | New encoder training with frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 | OPPO | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.7054 | 0.7040 | 0.6871 | 0.6935 |
| Encoder-2 | OPPO | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.7175 | 0.6993 | 0.6926 | 0.7095 |
| Encoder-3 | OPPO | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.6988 | 0.6991 | 0.6851 | 0.6899 |
| Encoder-4 | OPPO | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.6775 | 0.6381 | 0.6698 | 0.6709 |

Observation 2: The updated simulation results for low-complexity encoders with different backbones are shown in Table 3.

Observation 3: For Encoder-2 and Encoder-3, the obtained SGCS-2 is similar to SGCS-1, that shows a common decoder that is compatible to encoders with different backbones(at least for Transformer/CNN based lower-complexity encoder) and with different complexities. For Encoder-3, SGCS-2 is much lower than SGCS-1, but SGCS-4 is similar to SGCS-1.

Proposal 3: Low-complexity encoders with different backbones should be used for deriving the test decoder.

- Conclusions

In this contribution, we have discussed the testing issues for CSI compression two-sided models and got the following proposals:

Proposal 1: Principles to define test decoder(s)

- To meet the minimum performance requirement in RAN4 tests

- To be a simple design

- Consider different UE capabilities, including supported models, architectures, and computational complexities

Observation 1: While both RAN4 option 3 and RAN1 direction C may involve defining a test model or reference model, they address different issues and require separate solutions. The following key differences should be taken into account in RAN4 following works

- Motivation

- Model structure

- Testing data & Performance requirement

- Assumptions on generalization & Scalability consideration

Proposal 2: Regarding the inter-vendor training collaboration for two-sided AI/ML models, to fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account, how to take RAN1 requirements into account should clarified first. Following way forward could be discussed in RAN4 to check companies’ view.

- Option 1: RAN4 develops a standardized model based on RAN4’s testing requirements first, then RAN1 evaluates whether it can also be leveraged for RAN1 purposes.

- Option 2: RAN4 develops a standardized model that fulfills both RAN4’s testing requirements and RAN1’s commercial deployment requirements.

- Option 3: RAN4 develops two separate standardized models, one to meet RAN4’s testing needs and another one to fulfill RAN1’s commercial deployment requirements.

- For option2 and option3, RAN1’s requirements should be clarified and let RAN4 know.

Observation 2: The updated simulation results for low-complexity encoders with different backbones are shown in Table 3.

Table 3  Simulation results for low-complexity encoders

| Option 3 low-complexity encoders with different backbones |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | （1-to-1）Joint training | New encoder training with frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 | OPPO | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.7054 | 0.7040 | 0.6871 | 0.6935 |
| Encoder-2 | OPPO | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.7175 | 0.6993 | 0.6926 | 0.7095 |
| Encoder-3 | OPPO | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.6988 | 0.6991 | 0.6851 | 0.6899 |
| Encoder-4 | OPPO | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  |  | 0.6775 | 0.6381 | 0.6698 | 0.6709 |

Observation 3: For Encoder-2 and Encoder-3, the obtained SGCS-2 is similar to SGCS-1, that shows a common decoder that is compatible to encoders with different backbones(at least for Transformer/CNN based lower-complexity encoder) and with different complexities. For Encoder-3, SGCS-2 is much lower than SGCS-1, but SGCS-4 is similar to SGCS-1.

Proposal 3: Low-complexity encoders with different backbones should be used for deriving the test decoder.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- Appendix

Appendix 1 – Previous simulation results

CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion are shown as below:

CNN based CSI encoder 								CNN based CSI decoder

Figure 1  CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion

FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Based on the CNN based CSI encoder and CNN based CSI decoder proposed by RAN4#111, performance evaluation results are shown as below(train by UMa channel data, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Note：Baseline etype2 SGCS is 0.71

However, after adopting the same model structure and the same simulation assumptions (based on RAN4 conclusion), different companies still obtained different simulation results(e.g., in SGCS), as summarized in RAN4#112 meeting.

In RAN4#112 meeting, companies agreed to perform a second round of alignment and refined more simulation parameters as captured in R4-2414447. Simulation results from our side are as follows:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | 2 bits quantization for CSI report | 0.717 |
| 32 floats | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.771 |

-  Test encoder Option 3: Track1

In RAN4 #113 meeting, following 2 tracks have been agreed to further evaluate option 3.

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s).

For "Track 1", the selected companies are Samsung, CATT, and Huawei. We utilized the CSI decoder provided by Samsung (without quantization) as the test decoder and conducted encoder training using our own dataset. The results(for RAN4 #114) obtained are as follows:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO (without quantization) | OPPO (without quantization) | OPPO | 0.771 |
| 32 floats | Samsung (without quantization) | Samsung (without quantization) | OPPO | 0.757 |
| 32 floats | OPPO retrain by OPPO dataset (without quantization) | Samsung (without quantization) | OPPO | 0.760 |

From the evaluation and comparison results in the RAN4 #114 meeting, there are still certain discrepancies in the submissions from different companies for Track 1. Based on the agreements of RAN4 #114 meeting, the quantization process is further clarified as follows:

Agreements on quantization:

Companies shall provide decoders with 2 bit quantization, and with no quantization

- No quantization aware training when model is unquantized

- Quantization aware training when model is quantized

For the model with quantization:

Agree on a scaling and quantization codebook

- Include Sigmoid in model file in the encoder, and inverse Sigmoid in the decoder

- Codebook (1/8, 3/8, 5/8, 7/8) in model file

- Do not including mapping to 2 bits. Quantizer function of converting to 2 bits should not be in the model file

For the model without quantization

For no quantization models, do not include sigmoid at the model output and no inverse Sigmoid in the decoder

Based on the above updates, we used Samsung’s provided CSI decoder (both with and without quantization) as the test decoder and retrained the encoder using our own dataset. The updated test results(for RAN4 #114bis) are shown as follows:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats (1/8, 3/8, 5/8, 7/8) | OPPO train by OPPO dataset (with quantization) | Samsung (with quantization) | OPPO | 0.7158 |
| 32 floats | OPPO train by OPPO dataset (without quantization) | Samsung (without quantization) | OPPO | 0.7612 |

-  Test encoder option 3: Track2

For "Track 2", we trained both the encoder and decoder using a mixed dataset (with RAN4 agreed model structure). Additionally, we employed the decoder trained on the mixed dataset as the test decoder and retrained the encoder using OPPO own dataset. The results(for RAN4 #114) are presented below:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO - train by mix dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.752(by mix dataset) 0.750(by OPPO dataset) |
| 32 floats | OPPO - retrain by OPPO dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.686(by mix dataset) 0.751(by OPPO dataset) |

Further in RAN4 #114bis meeting, companies agreed to adopt a mixed dataset comprising contributions from MediaTek, Ericsson, vivo, OPPO, and Nokia. Companies submitted updated models based on this mixed dataset along with their SGCS results. Among these, the Samsung-provided model (both quantized and non-quantized versions) with the median SGCS was selected as the frozen decoder. So, we use Samsung’s CSI decoders as the test decoder for track 2 evaluation, train the encoder on the mixed dataset, and evaluate the performance using the mixed test dataset. The results (for RAN4 #114bis) are shown as follows:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats (1/8, 3/8, 5/8, 7/8) | OPPO train by mix dataset (with quantization) | Samsung train by mix dataset (with quantization) | mix dataset | 0.7040 |
| 32 floats | OPPO train by mix dataset (without quantization) | Samsung train by mix dataset (without quantization) | mix dataset | 0.7492 |

In RAN4#114bis meeting, further simulation activity for option 3 track 2 has been suggested as below:

- Definitions

  - Training Dataset 1 is the dataset used for training of the frozen decoder.

  - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder.

- Additional results 1 for RAN4#115:

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset.

- Additional results 2 for RAN4#115:

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset.

- Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset.

  - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

For additional results 1 and additional results 2, the updated results for option 3 track 2 (for RAN4 #115bis) are shown in the following table.

|  | CSI Feedback | SGCS |
| --- | --- | --- |
| Additional results 1 | 32 floats (1/8, 3/8, 5/8, 7/8) | 0.6993 |
|  | 32 floats | 0.7491 |
| Additional results 2 | 32 floats (1/8, 3/8, 5/8, 7/8) | 0.7184 |
|  | 32 floats | 0.7616 |

UEs may vary in their AI/ML capabilities, including supported models, architectures, and computational complexities. When introducing test decoders and defining minimum performance requirements, RAN4 should account for potential testing limitations arising from these differing UE capabilities. For additional results 3, A simple MLP CSI encoder design and paired CNN based CSI decoder(proposed after RAN4 #111 email discussion) is utilized and got following simulation results

MLP based CSI encoder 								CNN based CSI decoder

Figure 2  MLP based CSI encoder and CNN based CSI decoder

FLOPs, Number of parameters, model size for above MLP based CSI encoder and CSI decoder are evaluated as below. In comparison to the CNN based CSI encoder, there's a substantial decrease in FLOPs in the MLP based CSI encoder, dropping from 94M to a mere 0.22M.

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 0.22M | 1481.6M | 0.11M | 1.80M | 455KB | 7.03MB |
| 112 bits | 0.23M | 1481.7M | 0.11M | 1.82M | 467KB | 7.11MB |

Based on the MLP based CSI encoder and Samsung decoder trained using the mixed dataset, the updated results for additional results 3 are also shown as follows.

|  | CSI Feedback | SGCS | note |
| --- | --- | --- | --- |
| Additional results 3 | 32 floats (1/8, 3/8, 5/8, 7/8) | 0.6381 | MLP based CSI encoder |
|  | 32 floats | 0.6723 |  |

Appendix 2 – Model structure assumptions for encoder with lower complexity

## (a)

## (b)                                                                                                        (c)

Figure 3  Model assumptions for encoder with lower complexity

(a) Transformer based lower-complexity encoder (Encoder 2)

(b) CNN based lower-complexity encoder (Encoder 3)

(c) MLP based lower-complexity encoder (Encoder 4)

Appendix 3 – Challenges of model conversion (ONNX to another format)

The ONNX (Open Neural Network Exchange) format has been designed to facilitate the interoperability of deep learning models across different frameworks. However, in practice(during track1/2 works), there are several challenges that can prevent a seamless conversion of ONNX models to other formats, e.g., TensorFlow or PyTorch. Some main issues are listed below:

- Lack of official conversation tools: The available ONNX to TensorFlow or PyTorch conversion tools, which are developed by the open-source community rather than officially released, may have significant limitations in their implementation and maintenance. For example, the onnx-tensorflow library, as one of the most commonly used and popular tools for converting models in ONNX format to TensorFlow format, has declared its code repository is not actively maintained and will be deprecated since two years ago.

- Version Compatibility Issues: Different versions of ONNX introduce new features and changes to operators. The conversion tools available for converting models in ONNX to TensorFlow or PyTorch format may not be fully updated to support the latest ONNX version. Similarly, TensorFlow and PyTorch themselves undergo continuous development and version updates, and changes in their internal architectures or APIs can impact how well they can import models which converted from ONNX format.

- Conversion for custom or unsupported layers: If a model in ONNX format contains custom layers that are designed specifically for the original framework in which it was developed, there may be no straightforward way to translate them to TensorFlow or PyTorch equivalents. These custom layers might encapsulate unique functionality or algorithms that are not natively available in the target frameworks, making it difficult to accurately represent the model during the conversion.

If in the end we use ONNX as the method for providing the test model, the aforementioned issues should be carefully considered and resolved. Relying on offline negotiation to address these problems has proven to be highly inefficient at present. An alternative approach would be to reconsider the format in which the test model is delivered, such as providing it in PyTorch format, thereby relax the challenges associated with version and format conversion.