3GPP TSG-RAN WG4 Meeting #116	R4-2510970

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | <CR#> | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 36102 on Flexible guard band for 249 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Core |  |  |  |  |  | Date: |  |  | 15/08/2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Additionl of guard band for emissions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of emission tables and AMPR clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.1, 6.2B.3.2, 6.2B.3.3, 6.5B.3.3.3, 6.5B.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<beginning of next change>

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

### 6.2B.3.1	General

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3.1-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3.1-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3.1-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.3.3.2, 6.5B.4.4.4 | 249, 254 | N/A |  |
| NS_04N | Table 6.2B.1-2, 6.5B.3.3.3, 6.5B.4.4.5 | 249, 254 | 6.2B.3.2 |  |
| NS_05N | Table 6.2B.1-2, 6.5B.3.3.4, 6.5B.4.4.6 | 249, 254 | 6.2B.3.2 |  |
| NS_06N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_07N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_08N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_11N | Table 6.2B.1-2, 6.5B.3.3.3, 6.5B.4.4.5 | 249, 254 | 6.2B.3.3 |  |
| NS_12N | Table 6.2B.1-2, 6.5B.3.3.4, 6.5B.4.4.6 | 249, 254 | 6.2B.3.3 |  |

#### 6.2B.3.2		AMPR for NS_04N and NS_05N when X=0 kHz

Additional maximum power reduction when NS_04N is signalled is in Table 6.2B.3.3-1 for 15 kHz and in Table 6.2B.3.2-2 for 3.75 kHz SCS when X=0 kHz, as defined in 6.5B.3.3.3 and 6.5B.3.3.4.

NOTE: When X=100 kHz as defined in clause 6.5B.3.3.3 and 6.5B.3.3.4 AMPR=0.

Table 6.2B.3.2-1: A-MPR for NS_04N, 15 kHz SCS for X=0 kHz

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 | 11 |
| A-MPR (dB) | ≤11 |  | ≤5.5 | ≤3.5 | ≤2 | ≤0.5 | ≤0.5 | ≤2 | ≤3.5 |  | ≤5.5 | ≤ 11 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  |  | 9-11 |  |  |  |
| A-MPR (dB) | ≤ 8.5 |  |  |  | ≤ 2.5 dB |  |  |  |  | ≤ 8.5 |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 5.5 |  |  |  |  |  | ≤ 5.5 |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 2 |  |  |  |  |  |  |  |  |  |  |  |  |

Table  6.2B.3.2-2: A-MPR for NS_04N, 3.75 kHz SCS for X=0 kHz

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-9 | 10-38 | 39-41 | 42-44 | 45-47 |
| A-MPR (dB) | ≤ 7.5 | ≤ 3 | ≤ 0.5 | ≤ 0 | ≤ 0.5 | ≤ 3 | ≤ 7.5 |

AMPR when NS_05N is signalled is specified in Table  6.2B.3.2-3 for 15 kHz SCS and for 3.75 kHz in Table  6.2B.3.2-4 when X=0 kHz.

Table  6.2B.3.2-3: A-MPR for NS_05N, 15 kHz SCS for X=0 kHz

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 | 11 |
| A-MPR (dB) | z |  | ≤7 | ≤5 | ≤3.5 | ≤2 | ≤2 | ≤3.5 |  | ≤5 | ≤7 | ≤ 14 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |  |
| A-MPR (dB) | ≤ 10 |  |  |  | ≤ 4 |  |  |  | ≤ 10 |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 8 |  |  |  |  |  | ≤ 8 |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 3.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table  6.2B.3.2-4: A-MPR for NS_05N, 3.75 kHz SCS for X=0 kHz

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-10 | 11-37 | 38-41 | 42-44 | 45-47 |
| A-MPR (dB) | 10.5 | 5.5 | 2.5 | 0 | 2.5 | 5.5 | 10.5 |

#### 6.2B.3.3		AMPR for NS_11N and NS_12N

Additional maximum power reduction when NS_11N or NS_12N is signalled is in Table 6.2B.3.3-1 and when X=100 kHz as well as NS_11N when X = 0 kHz, as defined in 6.5B.3.3.3 and 6.5B.3.3.4.  The AMPR applies for any position and modulation in Table 6.2B.3.3-1.

Table 6.2B.3.3-1. AMPR values for NS_11N and NS_12N for X=100 kHz, and NS_11N with X=100 kHz

| SCS and number of tones | 3.75kHz | 15 kHz 1 tone | 15 kHz 3-tone | 15 kHz 6-tone | 15 kHz 12-tone |
| --- | --- | --- | --- | --- | --- |
| A-MPR (dB) | 18.0 | 14.5 | 10.0 | 7 | 4 |

AMPR for A-MPR for NS_12N, 15 kHz SCS for X=0 kHz is specified in Table 6.2B.3.3-2 and for 3.75 kHz in Table 6.2B.3.3-2.

Table 6.2B.3.3-2: A-MPR for NS_12N, 15 kHz SCS for X=0 kHz

| Modulation | QPSK |  |
| --- | --- | --- |
| Tone positions for single tone allocation | 0-11 |  |
| A-MPR (dB) | ≤ 18 |  |
| Tone positions for 3 Tones allocation | 0-2, 3-5, 6-8 and 9-11 |  |
| A-MPR (dB) | ≤ 10 |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |
| A-MPR (dB) | ≤ 8 | ≤ 8 |
| Tone positions for 12 Tones allocation | 0-11 |  |
| A-MPR (dB) | ≤ 4 |  |

### 6.2B.4	Configured transmitted Power for category NB1 and NB2

<end of change>

<beginning of next change>

##### 6.5B.3.3.2	Requirements for network signalling value "NS_03N"

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.2-1.

Table 6.5B.3.3.2-1: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± 0.85–1.7 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

##### 6.5B.3.3.3	Requirements for network signalling value "NS_04N" and “NS_11N”

When "NS_04N" or “NS_11N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-1 for any channel bandwidth configured within 1610-1618.25MHz.

X in Table 6.5B.3.3.3-1 is either 0 or 100 kHz based on manufacturer declaration and AMPR in clause 6.2B.3.2	 and 6.2B.3.3 applies accordingly.

Table 6.5B.3.3.3-1: Additional requirements for "NS_04N" or “NS_11N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
| ± ( X to (X+160) ) | -2 |  | 30kHz |
| ± ( (X+160) to (X+2300) ) | -2 to -26 |  |  |
| ± ( (X+2300) to (X+18500) ) | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5B.3.3.4	Requirements for network signalling value “NS_05N" and  “NS_12N”

When "NS_05N" or “NS_12N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.4-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

X in Table 6.5B.3.3.4-1 is either 0 or 100 kHz based on manufacturer declaration and AMPR in clause 6.2B.3.2	 and 6.2B.3.3 applies accordingly.

Table 6.5B.3.3.4-1: Additional requirements for "NS_05N" or “NS_12N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
| ±( X to (X+160) ) | -5 |  | 30kHz |
| ±( (X+160) to (X+225) ) | -5 to -8.5 |  |  |
| ± ( (X+225) to (X+650) ) | -8.5 to -15 |  |  |
| ± ( (X+650) to (X+1365) ) | -15 |  |  |
| ± ( (X+1365) to (X+1800) ) | -23 to -26 |  |  |
| ± ( (X+1800) to (X+16500) ) | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

#### 6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2

<end of change>