3GPP TSG-RAN WG4 Meeting #116	R4-2510972

# Bengaluru, India, August 25th – 29th, 2025

Title:	REFSENS procedure

Source:	Qualcomm Incorporated

Agenda item:	7.22.3.2

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4#115 WF for Ambient IoT device [16] was agreed. In this paper we continue discussion on the REFSENS procedure.

# 2. 	Terminology

## 2.2	Process for REFSENS

WF [15] agrees for REFSENS the following:

Issue 3-4-1: Reference sensitivity

Agreement in RAN4#115:

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

The miss detection needs to be accommodated in the test procedure. RAN4 has received an LS [16] relevant for this issue saying there is no separate trigger for the MSG1 in CFRA. Use of CFRA makes sense since the DUT is known and it AS ID can be used directly. TE emulating the reader can set the response time to the device and then the response should be expected after the set time+SFO and if there is no response, a fail should be recorded for that try. What amount of repeats is needed to reach 10 % miss detection should be up to RAN5 MU discussion.

Proposal 1: Use CFRA for REFSENS test procedure

Proposal 2: Allow set time + SFO for the devices response time.

Proposal 3: Leave number of repeats to reach 10 % miss detection rate for the RAN5 to determine.

# Conclusion

Proposal 1: Use CFRA for REFSENS test procedure

Proposal 2: Allow set time + SFO for the devices response time.

Proposal 3: Leave number of repeats to reach 10 % miss detection rate for the RAN5 to determine.

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500490, “Ambient IoT device requirements”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[7]	R4-2502858, “WF on requirements for system parameters and A-IOT device”,  CMCC, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[8]	R4-2505230, “WF on A-IOT device requirements”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[10]	R4-2505227, “(new) TS 38.191 Ambient IoT device radio transmission and reception”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[11]	R4-2505228, “3GPP_TS 38.194 skeleton”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[12]	R4-2503560, “Relevance of phase noise and LLS for SNR for REFSENS”, Qualcomm, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[13]	R4-2503559, “Ambient IoT device backscattering and SFO error”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[14]	R4-2506162, “REFSENS procedure, SFO and timing aspects”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[15]	R4-2508116, “WF on A-IOT device requirements”, CMCC, 3GPP TSG-RAN WG4 Meeting#115, Malta, MT, 19th – 23th May, 2025

[16]	R4-2509019, “LS on A-IoT MSG1 resources indication”, RAN2, 3GPP TSG-RAN WG4 Meeting#116, Bengaluru, India, Aug, 2025

[17]	R4-2506162, “REFSENS procedure, SFO and timing aspects”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#115, Malta, MT, 19th – 23th May, 2025