3GPP TSG-RAN WG4 Meeting #116                                           R4-2510996

Bangalore, India, 25th – 29th August, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS

Agenda Item:	7.24.3.2

Document for:	Approval

Introduction

Based on the last meeting discussion for Rel-19 LP-WUS, the issues to be further studied for REFSNES, ACS and ASCS are captured in WF [1].

This contribution presents our further analysis on REFSENS, ACS, and ASCS.

Discussion

Definition of Type 1 and Type 2 LR

On LP-WUR Type 1 and Type 2, there was an agreement in last meeting:

Issue 2-1-5: How to map two LP-WUR types with UE capability

Agreement:

  - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type.

  - Requirement applicability should be clearly defined.

  - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

To our understanding, different LP-WUR types are defined based on different implementation assumption, targeted to different power consumption goals, each with distinct NF + IM value agreed in RAN4 compared to the MR. The requirements are specified for different LP-WUR types. The LP-WUR types are specified in Table 1 below.

Table 1: The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming combined noise figure (NF) and implementation margin (IM) of 18dB. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming combined noise figure (NF) and implementation margin (IM) of 13.5dB. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

Proposal 1: It is proposed to adopt LP-WUR type 1 and type 2 definition as Table 1.

REFSENS and SNR

As discussed in previous meetings, the legacy approach is used to derive the REFSENS requirement, with the most critical parameters being SNR and NF+IM.

The SNR evaluation in RAN1 is based on TDL model, while the usual model adopted by RAN4 in SNR simulation for purpose of RF requirements is AWGN based. It is foreseen that the OFDM based receiver has better SNR compared to OOK envelope detection receiver, and OOK-4 with chip rate M=4 is worse than other cases. RM coding is not fixed according to RAN1 discussion, which could be ranged from 8 bit to 32 bit. The specific bits could be determined by RAN4, here we propose to consider 16-bit RM coding.

The simulated SNR value for OOK based signals are performed for 8bits, 16bits and 32bits respectively with 4us timing offset. The SNR values for the simulated cases are:

-2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit)

It is well understandable that OFDM based signal is more sensitive to timing error, and with further evaluation, we see no big difference for the applicable SNR for both OOK based and OFDM based signals, especially when timing error and phase noise impairments considered.

Since it was agreed that LP-WUR Type 1 and Type 2 should be considered, which means that two sets of RF requirements applicable to Type 1 and Type 2 should be specified.

Since it was agreed that both LP-WUR Type 1 and Type 2 should be considered, two separate sets of RF requirements—applicable to each type—must be specified.

The fundamental distinction between Type 1 and Type 2 lies in their respective UE radio hardware implementations:

- Type 1 is primarily optimized for ultra-low power consumption, making it well-suited for OOK-based LP-WUS reception. However, this does not preclude its ability to receive OFDM-based LP-WUS signals, depending on the baseband implementation capability.

- Type 2, on the other hand, is mainly designed for OFDM-based LP-WUS reception and decoding. Nevertheless, the same RF receiver can also support OOK-based LP-WUS detection.

For the two separate sets of RF requirements, if the required SNR for demodulating OOK-based and OFDM-based LP-WUS were different, it would result in four distinct REFSENS requirements (two per LP-WUR type). While most other receiver (Rx) requirements are specified relative to REFSENS, this approach would effectively necessitate four sets of RF requirements—an unnecessarily complex outcome.

Since Rx requirement verification is a key metric to ensure RF designs meet performance targets, a single unified SNR value is preferred for both OOK and OFDM-based LP-WUS signals. Although minor demodulation differences may exist due to waveform characteristics and RF impairments, the impact is negligible in practice, which can be observed from the simulation. This justifies adopting a common SNR reference for both waveforms, allowing the focus to remain on two primary sets of RF requirements—aligned with the Type 1 and Type 2 receiver implementations.

Here we use the above assumptions to calculate the REFSENS for two LP-WUR types as a reference for the following Rx requirements.

| SCS (kHz) | LP-WUR/MR | CBW (RB) | NF + IM (dB) | SNR (RB) | REFSENS (dB) |
| --- | --- | --- | --- | --- | --- |
| 15 | LR Type 1 | 11 | 18 | -4.5 | -97.5 |
|  | LR Type 2 | 11 | 13.5 | -4.5 | -102 |
|  | MR | 11 | 9+2.5 | -1 | -100.5 |
| 30 | LR Type 1 | 11 | 18 | -4.5 | -94.5 |
|  | LR Type 2 | 11 | 13.5 | -4.5 | -99 |
|  | MR | 11 | 9+2.5 | -1 | -97.5 |

REFSENS values for type 1 and type 2 LP-WUR in the above table are based on fixed NF. It is noted that the NF for MR is assumed as 9dB, which is a band dependent value and could be increased for higher frequency bands even no Tx-Rx isolation is considered. For example, 10dB NF is assumed for n78 and n79, 10.5dB is assumed for n77 while 11dB and 12dB are assumed for n41 and n104 respectively. Obviously, NF is higher than normative 9dB assumption for higher TDD bands. The difference of NF for TDD bands should be reflected for LR as well if corresponding bands are to be implemented for LR.

Proposal 2: NF difference for TDD bands especially for higher frequency bands should be reflected as delta value for LP-WUR.

ACS

It has been observed that without digital domain filtering, the LP-WUR is unable to handle large ACS interferers, even in the absence of phase noise. When phase noise is modeled, it clearly becomes a key factor, influencing not only the number of guard RBs required but also the tolerable level of interference.

With phase noise modeled, even with 4 guard RBs, we observed that it is difficult to meet the SNR threshold proposed in section 2.1.2 for large adjacent interferers. While additional digital domain filtering was not considered during the SI studies in both RAN1 and RAN4, it could be left to the LP-WUR implementation but should not be factored into the requirements specification.

Generally, adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s). The said receiver filter usually is the baseband channel filter. While for LP-WUR, the filter attenuation mainly comes from the analog filter, e.g. 3rd and 5th order Butterworth filters implemented in the Rx chain before ADC, but it does not preclude the possible implementation of digital domain channel filter.

The requirement of ACS comes from the co-existence study, it is derived from ACIR with also consideration of ACLR from the BS side. The assumed BS ACLR is 45dBc for the co-existence study, however, the normal BS implementation for ACLR is better than the assumed value, in addition, the performance metric for LP-WUR ACS is not throughput as considered for the MR, it’s possible to relax the ACS requirement in exchange of smaller guard RB size for LP-WUS.

In current specification ACS is defined as 33dBc for NR at least for 5MHz CBW, by which together with REFSENS degradation, it results the Pinterferer, i.e. REFSENS + 45.5dB. The power imbalance of 31.5dBc is the level considering the ACS value as well as the implementation margin. Apparently, 5th order Butterworth filter cannot provide very high rejection to the adjacent interferer. The evaluated 5th order Butterworth filter can only provide less than 20dB rejection for the adjacent channel with 5MHz frequency offset. Furthermore, if component tolerance is considered for the filter implementation, the suppression capability of this kind of filter could be further degraded. In addition, as pointed out in [2] that with process and temperature variations, the metal–oxide–metal capacitor value in 28-nm CMOS can vary by ± 20%, and poly resistors vary by ±15%. All the factors make the filter rejection much less than the theoretical analysis. The real implementation may just provide ~10dB or even worse suppression for the adjacent interferer, which is much lower than the MR.

Supposing 10dB filtering rejection is provided, and still 14dB REFSENS degradation is allowed, with rough estimation, the interferer level LR could handle is about LR REFSENS + 22.5dB or less with IM considered. Alternatively, if keep the same interferer level as MR, the power imbalance between the interferer and LR wanted signal should consider the REFSENS degradation due to the larger NF for LR.

With RF impairments considered, it’s actually a tradeoff between power consumption and the LP-WUR's ability to handle large interferers.

The ACS value reflects the adjacent channel rejection capability of the filter and is derived from coexistence studies. However, LR operates under different coexistence scenarios compared to a typical UE, and therefore should not be constrained by the 33 dBc value.

The purpose of ACS Case 1 and Case 2 is to assess different aspects of the receiver's performance. Case 1 primarily focuses on evaluating the filtering capability of the receiver, which is demonstrated by the degradation of the desired signal in the presence of a stronger adjacent interferer. Case 2, on the other hand, is designed to assess the receiver's dynamic range, reflected by an increased desired signal level along with a corresponding rise in interferer strength.

However, if power consumption is the key consideration for LP-WUR design, the limiting factor may not be the receiver’s filtering or dynamic range capabilities, but rather its phase noise performance. In this context, even if dynamic range poses no significant challenges for receiver design, verifying this capability may be unnecessary. In other words, since dynamic range is not a critical factor for LP-WUR design, the test case may not effectively address the design's true priorities. If Case 2 is kept, the Power in transmission bandwidth configuration shall be relaxed.

With above analysis, we propose to consider the ACS requirements for Type 1 and Type 2 LR as follows:

Table 2: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS + 23 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 42.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 3: Test parameters for LP-WUR type 1 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -47.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 4: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS + 17 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 47 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 5: Test parameters for LP-WUR type 2 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -53.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

It should be noted that unlike MR, specific ACS value for LR does not reflect the co-existence study result derived from ACIR, neither it may not be linked to the filter ability assumed in the SI study, thus, whether to keep the specific ACS value while only define cases with degraded REFSENS in presence of interferer for two LP-WUR types could be considered.

Proposal 3: For ACS, with the same interferer level as MR, ACS REFSENS degradation for LP-WUR should be relaxed more compared to MR, especially for Type 1 LR.

ASCS

In last RAN4 meeting, we already have the following agreement for ASCS

Issue 2-3-1: ASCS requirements for OOK-based receiver

Agreement:

- For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted

  - 1 RB guard for ASCS and 0.5dB de-sens

- For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted

  - 0 RB guard for ASCS and 0 dB de-sens

- For Type 1 and Type2 UE are not limited to specific LP-WUS waveform.

- FFS how to define test case and requirement applicability for Type 1 and Type 2

  - FFS how type 1 and type 2 capability related to RAN1 UE feature

- A UE can only support either Type 1 or Type 2 but not both for LP-WUR

With above agreement, the ASCS requirements for Type 1 and Type LP-WUR are suggested as follows.

Table 6: ASCS parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS+0.5 |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 1 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

Table 7: ASCS parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 0 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

Conclusion

This contribution provides our further analysis of REFSENS, ACS and ASCS requirements for LP-WUR. With the analysis, we have the following observations and proposals:

Definition of LP-WUR Type 1 and Type 2:

Proposal 1: It is proposed to adopt LP-WUR type 1 and type 2 definition as follows:

Table 1: The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming combined noise figure (NF) and implementation margin (IM) of 18dB. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming combined noise figure (NF) and implementation margin (IM) of 13.5dB. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

REFSENS requirement:

Proposal 2: NF difference for TDD bands especially for higher frequency bands should be reflected as delta value for LP-WUR.

ACS requirement:

Proposal 3: For ACS, with the same interferer level as MR, ACS REFSENS degradation for LP-WUR should be relaxed more compared to MR, especially for Type 1 LR.

To facilitate the discussion, draft requirements for REFSENS, maximum input level, ACS and ASCS for LP-WUR are presented in Annex for reference. The requirements are separated for two LP-WUR types. The suffix 'M' is tentatively used here for LP-WUR and may be revised if conflicts arise with the introduction of other features.

- References

[1] R4-2505234, WF on UE RF requirements for LP-WUS, vivo

[2] A Digital Filtering ADC With Programmable Blocker Cancellation for Wireless Receivers, Qiwei Wang, etc., IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 53, NO. 3, MARCH 2018

- Annex (draft requirements for REFSENS/ACS/ASCS)

- 4.3	Specification suffix information

Unless stated otherwise, the suffixes shown in Table 4.3-1 are used for indicating at 2nd level clause. For shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause. For V2X, suffixes A and F are used for indicating at 3rd level clause.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |
| M | LP-WUR |

- 7.3M	Reference sensitivity for LP-WUR

- 7.3M.1	General

The reference sensitivity power level REFSENS for LP-WUR is the minimum mean power applied to single UE antenna port, at which the Miss Detection Rate (MDR) shall meet the requirements for the specified reference measurement channel.

Different LP-WUR types are defined based on different power consumption assumptions, each with distinct noise figure (NF) values compared to the MR. The requirements are specified for different LP-WUR types. The LP-WUR types are specified in Table 7.3M.1-1 below.

Table 7.3M.1-1: The definitions of LP-WUR types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming combined noise figure (NF) and implementation margin (IM) of 18dB. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming combined noise figure (NF) and implementation margin (IM) of 13.5dB. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

- 7.3M.2	Reference sensitivity power level for LP-WUR

For a LP-WUR UE equipped with 1 Rx antenna port, reference sensitivity for LP-WUR type 1 and type 2 UE are defined in Table 7.3M.2-1 for the applicable operating bands with LP-WUS bandwidth of 11RB. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1.

Table 7.3M.2-1: Single antenna port reference sensitivity for FDD and TDD bands

| RX parameter | SCS kHz | REFSENS (dBm) |
| --- | --- | --- |
| REFSENS for LP-WUR type 1 UE | 15 | -97.5 |
|  | 30 | -94.5 |
| REFSENS for LP-WUR type 2 UE | 15 | -102 |
|  | 30 | -99 |

Due to the different NF assumptions, reference sensitivity for LP-WUR in Table 7.3M.2-1 shall be modified by the amount given in ΔRNF in Table 7.3M.2-2 for the applicable operating bands.

Table 7.3M.2-2: Reference sensitivity allowance ΔRNF

| Operating band | ΔRNF (dB) |
| --- | --- |
| n41 | 2 |
| n77 | 1 |
| n78, n79 | 1.5 |
| n104 | 3 |

- 7.4M	Maximum input level for LP-WUR

Maximum input level is defined as the maximum mean power received at the UE antenna port. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1., with parameters as specified in Table 7.4-1.

Table 7.4M-1: Maximum input level

| Rx Parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -25 |

- 7.5M	Adjacent channel and subcarrier selectivity for LP-WUR

- 7.5M.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the configured channel frequency to the receive filter attenuation on the adjacent channel(s).

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR.

- 7.5M.2	Adjacent channel selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.2-1 and Table 7.5M.2-2 for LP-WUR type 1 UE and the minimum requirements specified in Table 7.5M.2-3 and Table 7.5M.2-4 for LP-WUR type 2 UE. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1.

Table 7.5M.2-1: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS + 23 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 42.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-2: Test parameters for LP-WUR type 1 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -47.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.5M.2-3: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS + 17 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 47 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 5: Test parameters for LP-WUR type 2 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -53.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

- 7.5M.3	Adjacent subcarrier selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 and 7.5M.3-2 for LP-WUR. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1.

Table 7.5M.3-1: ASCS parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS+0.5 |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 1 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

Table 7.5M.3-1: ASCS parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 0 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |