3GPP TSG-RAN WG4 Meeting #116															R4-2511074

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.1.2.3

Source:	Samsung

Title:	Views on remaining issues on 6Rx

Document for:	Discussion

1	Introduction

RAN4 had studied 6-layer feasibility under the objective of 6Rx for higher frequency band (>2.5GHz) targeting at support of handheld UE which was included in Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1]. After a long debate more than a year, RAN4 concluded that it was unable to conclude on a common set of simulation assumption to determine feasibility of 6 layer for handheld UE in Rel-19. The corresponding WF [2] and revised WID [3] were agreed as follows:

Way Forward: - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications. - Introduce 6 MIMO layers support as an optional feature only for FWA.

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Introduce 6 MIMO layers support as an optional feature only for FWA. - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we would like to provide our views on the remaining issues in general aspects based on the last discussion for MIMO layer evaluation for 6Rx UE.

2 Discussion

In the latest approved WF [4]. there is an agreement for an open issue on the release independency for 6Rx UE under the MIMO layer discussion as below:

Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

In RAN4#114-bis, it has been proposed to allow release independent for 6Rx RF requirements for both handheld UE and FWA from Rel-17 [5]. It is also referring to 8Rx case which are available from Rel-17 as the release independent manner. However, 6Rx is different with 8Rx case in our understanding. Although both share UE type limitation to FWA UE in terms of the supported MIMO layer, 8Rx just reused the legacy MIMO layer capability, i.e., maxNumberMIMO-LayersPDSCH, with MIMO-LayersDL ENUMERATED values {2, 4, and 8} which allowed release independent from Rel-17 as below.

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                                       OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                                           OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                                         OPTIONAL

}

Observation 1:	8Rx reused the legacy MIMO layer capability, i.e., maxNumberMIMO-LayersPDSCH, with MIMO-LayersDL ENUMERATED values {2, 4, and 8} which allowed release independent from Rel-17.

On the other hand, based on LS to RAN2 [6], new capability to enable 6 MIMO layers will be designed by RAN2. This may imply that even if a UE, e.g., Rel-17 FWA UE, supports 6Rx RF requirements, the UE would not be able to support 6 layers since there is no way for the network to be indicated. In order to avoid any unnatural situation, 6Rx requirements should not be applied in a release independent manner as Option 3 in [4].

Observation 2:	Even if a UE, e.g., Rel-17 FWA UE, supports 6Rx RF requirements, the UE would not be able to support 6 layers from previous release since there is no way for the network to be indicated as Option 3.

Moreover, in RAN2 discussion, it has been struggling to have a consensus to differentiate the device type by the supported MIMO layer even though it was agreed in RAN4. If their final conclusion is contradictory to RAN4 agreement, Option 1, which is different by the UE type, cannot be an option for the discussion. Option 2 would be the only way that RAN4 should adopt at the last meeting of the WI.

Observation 3:	Option 1 should be ruled out if RAN2 is struggling to have a consensus to differentiate the device type by the supported MIMO layer.

Proposal 1:		Option 2 would be the only way that RAN4 should adopt at the last meeting of the WI.

3	Conclusion

In this contribution, we focus on the MIMO layer evaluation for 6Rx UE under 6Rx objectives. Following observations and a proposal are provided to be considered by this meeting.

Observation 1:	8Rx reused the legacy MIMO layer capability, i.e., maxNumberMIMO-LayersPDSCH, with MIMO-LayersDL ENUMERATED values {2, 4, and 8} which allowed release independent from Rel-17.

Observation 2:	Even if a UE, e.g., Rel-17 FWA UE, supports 6Rx RF requirements, the UE would not be able to support 6 layers from previous release since there is no way for the network to be indicated as Option 3.

Observation 3:	Option 1 should be ruled out if RAN2 is struggling to have a consensus to differentiate the device type by the supported MIMO layer.

Proposal 1:		Option 2 would be the only way that RAN4 should adopt at the last meeting of the WI.

Reference

[1] RP-240828	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4	RAN#103

[2] R4-2503014	WF on requirements for 6Rx	RAN4#114

[3]	RP-250780	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4	RAN#107

[4]	R4-2508086	WF on 6Rx and SRS IL		RAN4#115

[5]	R4-2503965	On release independent for 6Rx RF requirement			RAN4#114-bis

[6]	R4-2505225	LS on DL MIMO layers capabilities for 6Rx UEs			RAN4#114-bis