3GPP TSG-RAN WG4 Meeting #115                                                             R4-2511228

Bangalore, India, 25th – 29th August 2025

Source:	Nokia

Title:	Considerations for UL time mask for NR_LBCA_Sw

Agenda item:	7.3.3

Document for:	Approval

Introduction

During RAN4#115 meeting following issues was recorded in the way forward [1]

Issue 1: Agreements related to the time mask requirement

-	A text proposal to TR38.768 in R4-2507932 captures the relevant agreements

-	How to specify the DL time mask:

-	The UE DL behaviour following the switch operation can be discussed in the RRM session

-	Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1

-	RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered.

Issue 2: Agreements related to UE capabilities

-	A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw

-	Related to concurrent Rx operation, the following aspect is included in the feature group:

-	[The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.]

-	Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

-	Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

Agreements done in RAN1#121:

Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier Agreement Draft LS R1-2504868 is endorsed in principle. Agreement Final LS R1-2504869 is endorsed. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

Discussion

Work item [2] outlines FDD CA band combination to which the semi static switching framework shall be applied. During a prior meeting, switching period values of {35 µs, 140 µs} were recorded [3]. In the previous meeting 70us was introduced as a candidate value.  The switched LBCA operates by alternating DL reception between two carriers: FDD and SDL. There is only one operational uplink carrier which maintains a constant carrier frequency even as the DL alternates between the two carriers. Consequently, switching period values concerns only DL. It is assumed that NW will schedule resources in time so that it covers both the switching period and the TTA. Unscheduled resources are referred to as a switching gap, measured in terms of number of symbols. The downlink switching gap remains constant and is based on switching period values as such, however, the UL switching gap must also consider the impact of timing advance and the transient period.  A 35us switching period corresponds to a one symbol switching gap, whereas a 140us corresponds to two symbols.

According to agreements reached at RAN1#121, no physical signals shall be permitted to overlap with any portion of the switching gap. However, an exception is allowed in the uplink when SDL carrier switches to FDD. The timing advance may cause the UL start slot to overlap with switching gap. If a UE is unable to manage the overlap of the UL physical signals with the switching gap, additional symbols may need to be blanked beyond what the switching period itself defines.

There may be varying interpretations regarding the position of the UL transient period in relation to the switching period and slot boundary, which may subsequently affect the number of symbols used for the switching gap in UL. UL resources are scarce in low bands. Consequently, the number of symbols used for the switching gap should be minimized and any ambiguity should be avoided.

In this paper, we discuss the location of the UL transient period in relation to the switching period and slot boundary in accordance with the latest agreements made by RAN1.

It is assumed that the NW will schedule resources in time so that it covers both the switching period and the TTA.

The downlink switching gap remains constant and is based on switching period values as such, however, the UL switching gap must also consider the impact of timing advance and the transient period.

A 35us switching period corresponds to a one symbol switching gap, whereas a 140us corresponds to two symbols.

According to agreements reached at RAN1#121, no physical signals shall be permitted to overlap with any portion of the switching gap. However, an exception is allowed in the uplink when SDL carrier switches to FDD.

UL time mask options for FDD-SDL transition

DL switching periods are to be positioned at the end of a slot in the FDD and SDL carriers (“switch from carrier”).

We present two options that are possible for UL time mask in figure 1. For simplicity TTA=0us is considered. The 140us switching period is more critical for the time mask than the 35us switching period, so we focus on this scenario. The option on the left aligns the last symbol of the UL with the start of the switching gap. This alignment complies with RAN1 agreements, although the transient period may extend into the switching gap, we don’t see this as a significant issue as the requirement for the on-to-off transient period is not based on actual UE behaviour. Empirical experience since the inception of 3G, including several chip suppliers, has shown that ramp down power is rapid and can be completed in 2.72us, which is a difference between 2 symbols and switching period.

Aligning the last symbol of UL with the switching gap start will result in switching gap length of two symbols which is the same as for DL and represents the minimal number of blanked UL symbols needed for 140us switching period.

The option on the right aligns a symbol boundary three symbols away from the end of the slot avoiding transient period spillover into the switching gap. Consequently, three UL symbols need to be blanked.

Figure 1 FDD to SDL UL time mask options with TTA =0us. The number of blanked UL symbols is dependent on the position of the UL transient period with respect to the switching period.

UL time mask options for SDL-FDD transition

Here we also present two options that are possible for UL time mask in figure 2. For simplicity TTA=0us is considered. The 140us switching period is more critical for the time mask than the 35us switching period, so we focus on this scenario.

The option on the left aligns the last symbol of SDL i.e. the end of the switching gap with the start of UL slot. This adheres to the RAN1 agreements; however, the transient periodis overlapping with the switching gap. With TTA=0us the overlap is 10 us. Conversely, when TTA exceeds 0us, the overlap increases necessitating the blanking of more than one UL symbol.

The option on the right aligns the last symbol of SDL i.e. the end of switching gap with the start of the transient period. This again complies with the RAN1 agreements, however, since the transient period extends beyond the switching gap, it is necessary to blank at least one UL symbol when TTA=0us. If TTA is extended there may be a need to blank more than one UL symbol.

As in practice TTA>0us it may be mandatory to consider blanking at least one symbol by default.

Figure 2 SDL to FDD UL time mask options with TTA =0us. The number of blanked UL symbols is dependent on how much the UE can tolerate the UL and switching gap overlap.

The positioning of the UL transient period in relation to the switching period necessitates the blanking of a variable number of UL symbols.

The length of the switching gap for DL is constant whereas the UL varies with the TTA and the positioning of the transient period.

For FDD to SDL transition aligning the last symbol of UL with the switching gap start will result in the minimal number of blanked UL symbols.

During the SDL to FDD transition, if the UE is unable to manage UL physical signals overlapping with the switching gap, it may be necessary to blank one or more symbols by default.

It is paramount that the UL and its transition period are consistently aligned with the switching period and slot boundary in a singular manner for both FDD-SDL and SDL-FDD transition, unless there is an indication to the network.

UL and its transition period shall be consistently aligned with the switching period and slot boundary in a singular manner for both FDD-SDL and SDL-FDD transition covering all switching periods.

The requirements for the UL time mask should be defined assuming TTA = 0us.

For FDD-SDL transition and TTA = 0us we propose that end of the last symbol of UL aligns with the start of the switching gap.

For SDL-FDD transition and TTA = 0us we propose that the first symbol of the start slot of the UL aligns with end of the switching gap.

Conclusion

In this paper, we discuss the positioning of the UL transient period in relation to the switching period and slot boundary in accordance with the latest agreements made by RAN1. The analysis emphasizes scenarios where the number of blanked symbols depends on the positioning of the UL transient period relative to the switching period. Although the switching period of 140us is considered, the principles discussed are applicable to other switch periods as well.

The following Observations and Proposals were made:

Observation 1: It is assumed that the NW will schedule resources in time so that it covers both the switching period and the TTA.

Observation 2: The downlink switching gap remains constant and is based on switching period values as such, however, the UL switching gap must also consider the impact of timing advance and the transient period.

Observation 3: A 35us switching period corresponds to a one symbol switching gap, whereas a 140us corresponds to two symbols.

Observation 4: The positioning of the UL transient period in relation to the switching period necessitates the blanking of a variable number of UL symbols.

Observation 5: The length of the switching gap for DL is constant whereas the UL varies with the TTA and the positioning of the transient period.

Observation 6: For FDD to SDL transition aligning the last symbol of UL with the switching gap start will result in the minimal number of blanked UL symbols.

Observation 7: During the SDL to FDD transition, if the UE is unable to manage UL physical signals overlapping with the switching gap, it may be necessary to blank one or more symbols by default.

Observation 8: It is paramount that the UL and its transition period are consistently aligned with the switching period and slot boundary in a singular manner for both FDD-SDL and SDL-FDD transition. , unless there is an indication to the network.

Proposal 1: UL and its transition period shall be consistently aligned with the switching period and slot boundary in a singular manner for both FDD-SDL and SDL-FDD transition covering all switching periods.

Proposal 2: The requirements for the UL time mask should be defined assuming TTA = 0us.

Proposal 3: For FDD-SDL transition and TTA = 0us we propose that end of the last symbol of UL aligns with the start of the switching gap.

Proposal 4: For SDL-FDD transition and TTA = 0us we propose that the first symbol of the start slot of the UL aligns with end of the switching gap.

- References

- R4-2508118, WF on UE RF requirements for LB CA via switching, Apple, Malta, Malta, May 19-23, 2025

- RP-243317 NR_LBCA_Sw WID_cl, RAN#106

- R4-2503016 NR_LBCA_Sw WF, RAN4#114 Athens, Greece, February 17-21, 2025.