3GPP TSG-RAN WG4 Meeting #116                                                      R4-2511273

# Bengaluru, IN, 25th – 29th August, 2025

Source:		Nokia

Title:	Reply R5-253653 LS on Beam Correspondence Initial Access

Agenda item:		9.2

Document for:	Discussion

# Introduction

Based on the LS On Beam Correspondence Initial Access from RAN5 to RAN4 in RAN4 #115 meeting [1], we’d like to present our view for related issues.

1. Overall Description: RAN5 has discussed the test procedure and feasibility of 6.6.4 Beam Correspondence in RRC_INACTIVE requirements. RAN5 seeks the clarification from TS38.101-2 sub clause 6.6.4 Beam correspondence for power class 3 to support test procedure with UE Beamlock test function in the IDLE mode and without UE Beamlock test function in the IDLE mode. As stated in 6.6.4.1, UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4. RAN5 seeks the following clarifications: - RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined? 2. Actions: To RAN4 group. ACTION: RAN5 respectfully requests RAN4 to respond to the above questions.

# Discussion

  - Question 1

-	RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

In general, RAN4 cannot prevent RAN5 from developing new test methods to measure the RAN4 defined requirements. However, when RAN4 created the requirements for initial access, RAN4 LS referred to the test methods RAN5 already specified for connected mode [2].

RAN4 cannot prevent RAN5 from developing new test methods. If UE vendors believe they can meet the already defined requirements without beam lock function, it is not precluded as long as the core requirements are met.

  - Question 2

-	Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

We assume that the UE can choose freely before applying the beam lock function within a certain time window, which is defined in test procedure. During this time window, UE should choose the best possible beam. Then, TE sends the command to lock the beam for measurement purposes since when TE tests the beam, TE does not want UE to change the beam. This can be described step by step as below:

Step 1, UE choose the beam freely  TE does not prevent UE from choosing beam.

Step2 TE sends comments to lock the beam for measurement. This is due to TE need time to do measurement.  TE indicates/signals the UE that it should not change its beam.

Step3: TE free the UE to choose any other beam.  TE does not prevent UE from choosing beam.

Step4: TE send command again to lock the beam for measurement. This is due to TE need time to do measurement.  TE indicates/signals the UE that it should not change its beam.

Therefore, beam lock does not prevent UE from automatically choosing any beams which is the UE’s behaviour in the field. But in the current test set up, TE has to take some time, which can be longer than waiting for RAR from gNB, to finish measuring one beam. TE only wants to test the UE-chosen beam during the measurement step. This is the point that TE send the beam lock indication to let UE lock the beam for measurement. But TE will always give a time window for UE to select the best beam for test first before each measurement.

Beam lock function does not prevent UE from autonomously choosing other beams in the test step 1 and step 3. But TE has to indicate/signal the UE that it should not change its beam in step 2 and step4 during measurement of that beam, this because TE need time to do measurement. Beam lock function is only an aid for the specific test case.

  - Question 3

-	RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Still, RAN4 cannot prevent RAN5 from developing new test methods to measure the RAN4 defined requirements. This is RAN5 decision. Any new test method should be verified and approved based on OTA tests.

The core requirement is independent from which test method will be used to verify the spherical coverage. As long as core requirements are met, either test method can be used.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

- RAN4 cannot prevent RAN5 from developing new test methods. If UE vendors believe they can meet the already defined requirements without beam lock function, it is not precluded as long as the core requirements are met.

- Beam lock function does not prevent UE from autonomously choosing other beams in the test step 1 and step 3. But TE has to indicate/signal the UE that it should not change its beam in step 2 and step4 during measurement of that beam, this because TE need time to do measurement. Beam lock function is only an aid for the specific test case.

- The core requirement is independent from which test method will be used to verify the spherical coverage. As long as core requirements are met, either test method can be used.

# References

- R5-253653, LS On Beam Correspondence Initial Access

- R4-2317764, LS on testability issues of beam correspondence requirements