3GPP TSG-RAN WG4 Meeting #116	R4-2511278

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Impact of DL coverage enhancement on SMTC configuration and measurement requirements

Agenda item:	7.27.5.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

The discussions of this WI in RAN4 were initiated in the meeting RAN4 #112. Discussions ensued in the following meetings, including RAN4#115 and as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].

In this document we provide our views related to current open issues on “Downlink Coverage Enhancements”.

Discussion

Increased number of configured SMTCs

In the previous meeting, RAN4 has discussed whether to support more than 4 SMTC windows configured, as RAN2 has sent indication to RAN4 that there was consideration to increase the total number of supported SMTCs from 4 to 6.

The discussion has progressed in RAN2 and the following agreements were made:

Agreements: 1. the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) 2. To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) 3. We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported 4. No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme) Working Assumption: 1. We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: Alt 1: UE provide the closest N reference locations/neighbor cells to network. Alt 2: UE reports the selected SMTCs from configured SMTC set to network. We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

It seems from the agreements above that RAN2 has agreed to increase the maximum number of SMTCs from 4 to 6. However, this time, there will be prioritization across the different configured SMTCs.

RAN2 has location-based prioritization across the different SMTCs.

Currently, the Rel-18 version of the specification establishes that:

Therefore, by introducing if the Rel-19 supports the new list, it shall support the prioritization of the SMTCs, RAN2 has decided to enhance the mobility procedures and make it less dependent on the UE capability. In order to maintain the harmony across the specification groups and reach RAN2 intended design of the feature, it is necessary that RAN4 also captures the location-based “downselection” of SMTCs.

For UEs that support prioritization across the different SMTCs, RAN4 will introduce enhanced requirements, that captures the downselection of SMTCs.

For UEs that support less SMTCs than the configured number of SMTCs, but support dowprioritization of SMTCs based on location-based selection, the requirements are based on the final number of selected SMTCs.

Limiting the configuration possibilities for earth-moving cell

In the previous meeting, RAN4 has discussed whether to introduce a limitation to earth-moving configuration to no more than 4 SMTCs. This topic was left for further discussion.

First of all, it is important to remember that from the UE perspective, there is no differentiation whether the NTN cell is Earth moving or Quasi-Earth-fixed. This information is not explicitly conveyed to the UE and a NTN UE shall equally support both scenarios. Therefore, there is no simplification of UE implementation by allowing the UE support less SMTCs in one scenario (earth-moving) as the UE will equally need to support the scenario where more SMTCs might be configured (quasi-earth-fixed).

From the network perspective, in the majority of cases, it seems there will not be any advantages to implement and configure more than 4 SMTCs in an earth-moving scenario. If that is the case, in practice, there won’t be many cases where more than 4 SMTCs are configured in Earth-moving scenarios, in a practical point of view. However, this is ultimately a network decision, and there might be special circumstances where a NW might see some advantage in configuring a large number of neighbors.

Based on the discussion above, we see no benefits to introduce a limitation in the number of SMTCs configured.

Do not limit the number of SMTCs allowed to be configured in earth-moving scenarios.

However, RAN2 has indicated that in DL-CE scenarios, there will be an increased need for more SMTCs to deal with the different cell periodicities and “intermittent” configuration of cells. This indicates that UEs will more often be configured with multiple SMTCs. In order to maintain a good mobility performance, the UE shall be capable of dealing with the typical DL-CE scenarios. Based on this, it becomes even more important that UEs that support the coverage enhancement feature also support multiple SMTC configurations.

For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support at least 4 SMTCs per frequency layer

Conclusion

In this paper we discussed the impact of the new measurement framework proposed by RAN2 and RAN1 to handle the extended SSB periodicity in NTN under the topic of DL coverage enhancements. Based on our discussion and analysis, we made the following list of proposals:

Observation 1: RAN2 has location-based prioritization across the different SMTCs.

Proposal 1: For UEs that support prioritization across the different SMTCs, RAN4 will introduce enhanced requirements, that captures the downselection of SMTCs.

Proposal 2: For UEs that support less SMTCs than the configured number of SMTCs, but support dowprioritization of SMTCs based on location-based selection, the requirements are based on the final number of selected SMTCs.

Proposal 3: Do not limit the number of SMTCs allowed to be configured in earth-moving scenarios.

Proposal 4: For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support at least 4 SMTCs per frequency layer

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2508288, " WF on RRM requirements for NR_NTN_Ph3_Part2", Moderator (Qualcomm Incorporated), RAN4 # 115 Malta, MT, 19th – 23rd May, 2025

- R2-2503068, LS on SMTC enhancements, RAN WG2 Meeting #129bis, Wuhan, China, 7 - 11 Apr, 2025