3GPP TSG-RAN WG4 Meeting #116									                          R4- 2511313

Bangaluru, India, 25th – 29th Aug. 2025

Agenda item:			7.1.2.3.2

Source:	Ericsson

Title:	Discussion on release dependency for 6Rx UE and FWA definition

Document for:	Approval

# Introduction

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1], and later on, RAN revised the WI to make it clearer to support 6Rx for FWA devices [2].

6Rx for handheld and FWA UE [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Introduce 6 MIMO layers support as an optional feature only for FWA.

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The following sections will discuss the remining issues on the 6Rx MIMO layers.

# FWA terminology

The WF on the MIMO layers for 6Rx are captured below:

## <Sub-topic 1-1> FWA terminology

Agreement:

- Remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and use “FWA CPE” or “FWA” for ΔRIB,6R requirement in the note of the table.

  - Consider adding the definition of FWA CPE or FWA, and industrial UE in the specifications

  - Focus on modifying the notes of the receiver requirements for 4Rx, 6Rx and 8Rx for Rel-19 specifications in RAN4#115

  - FFS on whether and how to align the existing notes between 4Rx, 6Rx and 8Rx since the notes were specified according to the separate WIDs.

- Further modifications will be done in the future meetings

  - Address the similar issues for previous releases under the maintenance agenda

  - Address the similar issue for transmission requirements under the maintenance agenda.

- Adding FWA definition in the UE spec: a wireless connection that provides broadband access to a specific location such as a home or enterprise premises.

# Release dependency

The WF on the MIMO layers for 6Rx are captured below [3]:

## <Sub-topic 2-1> Release independency for 6Rx UE

Agreement:

- Option 1:

  - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18

  - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19

- Option 2:

  - Support 6Rx from Rel-19

- Option 3:

  - Support 6Rx from Rel-18

Release dependency depends on whether new signaling is introduced. If no new signaling is introduced, the 6Rx can be supported in earlier releases. However, if new signaling is introduced in the current release, then it should be supported from Rel-19.

- Release dependency depends on whether new signaling is introduced.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Release dependency depends on whether new signaling is introduced.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Adding FWA definition in the UE spec: a wireless connection that provides broadband access to a specific location such as a home or enterprise premises.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2508086, WF on 6Rx and SRS IL imbalance, Huawei, HiSilicon.