# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511439

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			CW node RF impact overview

Agenda item:		7.22.3.3

Document for:		Approval

# Introduction

In this paper, we present our overview for RF requirements for a CW node.

# Discussion

### Issue 3-4: Phase noise

- Agreement:

  - For BS with phase noise cancellation capability, no phase noise requirement is specified.

  - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS.

As discussed in our companion paper for A-IoT BS REFSENS, there is a competition between CWT phase noise and BS LO phase noise on the desensitization. As the BS LO phase noise is implementation issue so no phase noise profile will be needed in specification. Because CWT node may be from another manufacture, if the CWT phase noise will not be specified, the desensitization of the backscatter signal caused by CWT phase noise would not be controlled. From this perspective, specify the phase noise of CWT will help the dimension of A-IoT network.

Specifying the phase noise of the A-IoT node would help the dimension of the A-IoT network.

The phase noise model can be referred to TR 38.803 and the phase noise is scaled from 30 GHz phase model with 20 log(f/fc) factor. The phase noise profile is illustrated in Figure 1 with the original phase noise profile of 30GHz for comparison.

Though the phase noise profile could be specified, it is component specification. What matters for de-sensitization of BS is the unwanted emission level relative to the CW signal in dBc scale. The unwanted emission can capture either phase noise, PA linearity or other RF imperfections. The SEM is in absolute level and as CWT is transmitting a single tone, as unwanted emission mainly caused by phase noise is quite low. From this aspect, the relative level is meaningful and testable. In the CWT node spectrum emission, therefore, only the frequency point where the wanted signal located (1/ Tc) will be specified.  The latest RAN1 agreement on Tc (μs) is below:

| Tchip (μs ) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |

The phase noise for the frequency points has similar performance are omitted according to Figure 1. Therefore, the emission level can be specified as below, in case other measurement bandwidth, a scaling factor can be applied.

Table 1-x: CW node spectrum emission level

| Δf (kHz) from the CW frequency | Emission limit (dBc) | Measurement bandwidth |
| --- | --- | --- |
| 7.5 | -114 | 1Hz |
| 60 | -117 | 1Hz |
| 480 | -128 | 1Hz |

And corresponding requirement text:

The spectrum emission level of the CW node applies to frequencies starting from CW signal configured frequency.

The power of CW node emission shall not exceed the levels specified in Table 1 for the specified frequency offset.

Specify the above unwanted emission for CWT node.

Figure 1: Phase noise for 1GHz

There is no specification on which frequency the CWT node can be configured and this aspect is missing from discussion. To test the CWT node and BS node properly, the same R2D channel raster could be reused for CWT node.

Reuse the R2D channel raster for CWT node channel raster.

### Issue 3-8: CW others

- Agreement:

  - CW transmission and A-IoT BS downlink data transmission are non-concurrent.

    - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

This is an issue relating to the device. As there is no OOB for device receiver, such agreement should be reflected in the specification.

Reflect the above agreement in TS 38.194.

# Conclusions

In this contribution, we present our overview on CWT node RF with below proposal and observations:

Observation 1 Specifying the phase noise of the A-IoT node would help the dimension of the A-IoT network.

Proposal-1:Specify the above unwanted emission for CWT node.

Proposal-2:Reuse the R2D channel raster for CWT node channel raster.

Proposal-3:Reflect the above agreement in TS 38.194.

# References

- R4-2508101, WF on requirements for A-IoT BS and CW, Huawei

- R4-2507498, CW node RF impact overview, Ericsson