3GPP TSG-RAN4 Meeting #116	R4-2511568

# Bangalore, India 25th – 29th August, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Prediction: Testing Framework

Agenda item:	7.17.3

Document for:	Discussion

# Introduction

In this paper, we focus on testing framework of CSI prediction.

# Prior Agreements

Following agreement happened during the last few meetings [1][5][7]

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4 Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model. Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations Issue 1-4: Generalization Agreement: - Postpone this discussion until the requirement and test definition is clear

# Discussion of Testing Framework

## Time Separation between Last CSI-RS Slot and PUSCH Report Containing Predicted PMI

In FDD tests, RAN4 is currently considering allowing 4ms to generate PUSCH containing predicted PMI after receiving the last CSI-RS occasion of the observation window [11]. This 4ms time separation is aligned with the minimum time separation between CSI-RS reference resource and P/SP CSI reporting of non-AI-ML RAN1 spec. Figure 2 shows the reference from 38,214 regarding this [12].

The time separation of 4ms between reference CSI-RS and corresponding PMI was applicable for non-AI-ML operation based CSI generation, including non-AI-ML generated Rel-18 predicted PMI codebook. 4ms time separation will not be sufficient for UE to run inference and then generate predicted PMI reports. UE will need to consume inference time to generate AI-ML based predicted PMI. RAN1 will discuss the extended inference time from August 2025 meeting. RAN4 should align with RAN1 regarding the time separation between CSI-RS and corresponding predicted PMI.

Observation 1: RAN4 is currently considering 4ms time separation between PUSCH containing predicted PMI and the last CSI-RS occasion of the observation window in tests.

Observation 2: The 4ms time separation is aligned with RAN1 spec defined minimum time separation between CSI-RS reference resource and SP reporting of non-AI-ML based CSI report.

Figure 1: Currently considered framework for AI-ML based CSI prediction test in FDD

Figure 2: CSI reference resource for P/SP CSI reporting

Observation 3: 4ms time separation may not be sufficient to process CSI-RS, run inference and generate reports containing AI-ML based predicted PMI.

Observation 4: RAN1 will discuss potential extension of 4ms time separation in RAN1 122, i.e., during August 2025.

Proposal 1: RAN4 extends the time separation between the last CSI-RS occasion of the observation window and the PUSCH slot containing SP configured predicted PMI report from 4ms to 4+X ms.

- Note: (4+X) ms will be aligned with the RAN1 spec defined minimum time separation between CSI-RS reference and corresponding AI-ML based predicted PMI in SP reporting.

# Conclusions

Observation 1: RAN4 is currently considering 4ms time separation between PUSCH containing predicted PMI and the last CSI-RS occasion of the observation window in tests.

Observation 2: The 4ms time separation is aligned with RAN1 spec defined minimum time separation between CSI-RS reference resource and SP reporting of non-AI-ML based CSI report.

Observation 3: 4ms time separation may not be sufficient to process CSI-RS, run inference and generate reports containing AI-ML based predicted PMI.

Observation 4: RAN1 will discuss potential extension of 4ms time separation in RAN1 122, i.e., during August 2025.

Proposal 1: RAN4 extends the time separation between the last CSI-RS occasion of the observation window and the PUSCH slot containing SP configured predicted PMI report from 4ms to 4+X ms.

- Note: (4+X) ms will be aligned with the RAN1 spec defined minimum time separation between CSI-RS reference and corresponding AI-ML based predicted PMI in SP reporting.

# References

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- 3GPP TR 38.843 V 18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- R1 116bis Chairman’s notes, Hunan, China, April 2024.

- R4-2502970, Baseline simulation assumptions for AIML based CSI prediction, 3GPP TSG RAN WG4 #114.

- R4-2502856, WF on the requirements for AI/ML air interface, 3GPP TSG RAN WG4 #114.

- 38.822, v 18.0.0, UE feature list.

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN WG4 #114bis.

- R4-2505153, Collection of simulation result for AI-ML CSI prediction, RAN4 114bis.

- R4-2505513, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 115.

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, RAN4 #114bis.

- R4-2508102, “WF on updated simulation assumptions for AI based CSI prediction”, RAN4 115bis.

- 38.214 V 17.3.0