# 3GPP TSG-RAN WG4 Meeting #116bis	                                                 R4-2513024

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.12

Source:		Qualcomm Incorporated

Title:			Qualcomm Views on 6G Testability and OTA

Document for:	Approval

# 1  Introduction

Following new SID for Study on 6G Radio [1] and Study on 6G Scenarios and requirements [2-3], RAN4 will start the 6G discussion from RAN4#116bis meeting. In this paper, we provide our preliminary views on 6G testability and OTA.

# 2  6G Testability aspects

## 2.1 FR3 testability

- 6G new spectrum in FR3

As illustrated in Figure 1, the emerging 6G spectrum is targeting a new frequency range between 6 and 8+ GHz

Figure 1: Spectrum landscape for 6GR [4]

RAN4 has determined that FR1 UEs (≤7.125 GHz) are verified using conductive testing, while FR2 UEs (≥24.25 GHz) require OTA testing due to their high integration levels. The appropriate test methodology for FR3 UEs remains uncertain and is highly dependent on UE architecture. Architectural choices at these frequencies should balance implementation feasibility—such as physical constraints for handheld devices—and performance considerations like uplink/downlink coverage. As a starting point, RAN4 proposes further investigation into FR3 testability by segmenting the frequency range at 8.4 GHz.

Observation 1: As the starting point, RAN4 can further study FR3 testability with the splitting FR3 assuming 8.4GHz as the boundary.

- FR3 testability for below 8.4 GHz

As concluded in [5], FR1 system parameters defined for frequencies up to 7.125 GHz are also applicable to the 7.1–8.4 GHz range. Co-located deployment of FR3 and FR1 cells is a likely target to enable reuse of existing infrastructure. However, higher frequency bands introduce increased pathloss and penetration loss compared to sub-6 GHz, presenting new challenges for system design. To mitigate these effects, uplink performance can be enhanced through multi-antenna techniques, particularly in coverage-limited scenarios. UEs may employ various multi-Tx schemes—non-coherent, partially coherent, or fully coherent UL MIMO with single-layer transmission—to achieve higher antenna gain, often requiring more Tx chains (e.g., 4T8R). Consequently, RAN4 should assess the suitability of OTA testing for evaluating UE radiated performance under multi-Tx configurations.

Proposal 1: For FR3 below 8.4 GHz frequencies, conductive test could be considered as the baseline. RAN4 needs to study whether to use OTA test method to verify UE radiated performance with multi-Tx scheme.

- FR3 testability for above 8.4GHz

Due to increased frequency and pathloss, uplink coverage is a critical consideration in UE antenna design. An FR2-like array antenna system with analog beamforming may enhance EIRP and mitigate pathloss between UE and base station. However, challenges such as panel size must be addressed. RAN4 should further evaluate system architecture, coverage targets, and physical constraints to guide UE antenna design for frequencies above 8.4 GHz. With input from UE RF discussions, if OTA testing is needed, RAN4 should assess the feasibility of leveraging existing FR2 test setups (e.g., IFF chambers) or methodologies used for Ka/Ku-band VSAT UEs.

Proposal 2: RAN4 needs futher study the UE archetechture for the frequencies above 8.4 GHz. If OTA testing is needed, RAN4 should assess the feasibility of leveraging existing FR2 test setups (e.g., IFF chambers) or methodologies used for Ka/Ku-band VSAT UEs.

## 2.2 FR1 SISO and MIMO OTA testability

- Test methodology

For SISO, RAN4 should evaluate test methodologies to verify UE radiated performance for single-layer transmission using various multi-Tx schemes, including non-coherent, partially coherent, and fully coherent UL MIMO. OTA test methods for FR3 below 8.4 GHz may be leveraged for this purpose. Additionally, UE types—such as handheld devices, FWA, and IoT—are under discussion in RAN-P and RAN1/2/4. RAN4 should also assess appropriate test methods and phantom models, if needed, across different UE categories and deployment scenarios, including TN and NTN.

Proposal 3: For SISO, RAN4 needs study the test method to verify UE radiated performance for multiple Tx scheme with one-layer transmission.

Proposal 4: RAN4 should also study the test method and new phontom if needed for different UE device types incluidng handheld UE, FWA, IoT devices, etc., and different scenarios including TN and NTN, etc.

- Performance requirements

The OTA inlcuidng both SISO OTA and MIMO OTA requirements for 6G bands will not be defined in 6G Day 1 due to the absence of commercial devices for measurement commpaign.

Observation 2: The OTA inlcuidng both SISO OTA and MIMO OTA requirements for 6G bands will not be defined in 6G Day 1 due to the absence of commercial devices.

Study can be conducted to assess whether OTA requirement can be derived without measurement results or with partial measurement resluts. Condiering OTA performanc of handheld UE would be highly relying on the factors such as antenna placement and material covering the antenna, etc. Therefore, the study for the approach of not fully relying on measurement compaign could start from other UE types which might have common antenna placement option such as glass, etc.

A study may be conducted to evaluate whether OTA requirements can be derived without full measurement data or with partial results. For handheld UEs, OTA performance is highly influenced by factors such as antenna placement and surrounding materials. Therefore, initial investigation into reduced reliance on measurement campaigns could focus on UE types with more standardized antenna configurations, such as those incorporating glass structures.

Propsoal 4: Study can be conducted in RAN4 to assess whether OTA requirement can be derived without measurement results or with partial measurement resluts. The initial investigation into reduced reliance on measurement campaigns could focus on UE types with more standardized antenna configurations, such as those incorporating glass structures.

## 2.3 AI/ML Testability

In 5G, test method for AI/ML was studied with the assumptions that keeping the test costs in a reasonable level. Therrfore, the legacy test setup of enhanced IFF and 3D-MPAC were used as the baseline in AI/ML beam mamagement testing. While in 6G, new AI/ML cases may be driven in RAN1/2/4. The the correspoding test method to verify the new AI/ML features such as reduced RX beam sweeping factor will require further investigation assuming that new test setup could be considred.

Proposal 5: The the correspoding test method to verify the new AI/ML features such as reduced RX beam sweeping factor needs to be further studied in RAN4.

## 2.4 Conductive or Radiated spurious emission

For FR1, the general spurious emission requirements are specified in Table 6.5.3.1-2 of TS38.101-1 [6] wihch is based on conductive testing.

Meanwhile, the radiated spurious emission requirements are specified in Table 8.2.4-1 of TS 38.124 [7] for UE EMC.

Table 8.2.4-1: Radiated spurious emissions requirements for UE equipment supporting operations in FR1, traffic mode

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 | 1 MHz | 4 |
|  | -25 | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz. NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz. NOTE 3: As specified in TS 38.101-1 [3]: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: As specified in TS 38.101-1 [3]: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

From above two tables, conductive spurious emission in UE RF requirements and radiated spurious emission are same for frequency range 30MHz to 26GHz. The only different is that frequency range for 9kHz to 150kHz is not tested for radiated spurious emissions. It might be duplicated for UE to pass both conductive and radiated emission requirements. RAN4 could investigate whether compliance with radiated spurious emission limits consistently implies compliance with conducted spurious emission limits and study the feasibility of verifying either radiated or conductive spurious emission requirement for 6G UE.

Proposal 6: RAN4 could investigate whether compliance with radiated spurious emission limits consistently implies compliance with conducted spurious emission limits and study the feasibility of verifying either radiated or conductive spurious emission requirement for 6G UE.

# 3. Conclusion

In summary, we have following observations and proposals.

Observation 1: As the starting point, RAN4 can further study FR3 testability with the splitting FR3 assuming 8.4GHz as the boundary.

Proposal 1: For FR3 below 8.4GHz frequencies, conductive test could be considered as the baseline. RAN4 needs to study whether to use OTA test method to verify UE radiated performance with multi-Tx scheme.

Proposal 2: RAN4 needs futher study the UE archetechture for the frequencies above 8.4GHz. If OTA testing is needed, RAN4 should assess the feasibility of leveraging existing FR2 test setups (e.g., IFF chambers) or methodologies used for Ka/Ku-band VSAT UEs.

Proposal 3: For SISO, RAN4 needs study the test method to verify UE radiated performance for multiple Tx scheme with one-layer transmission.

Proposal 4: Study can be conducted in RAN4 to assess whether OTA requirement can be derived without measurement results or with partial measurement resluts. The initial investigation into reduced reliance on measurement campaigns could focus on UE types with more standardized antenna configurations, such as those incorporating glass structures.

Observation 2: The OTA inlcuidng both SISO OTA and MIMO OTA requirements for 6G bands will not be defined in 6G Day 1 due to the absence of commercial devices.

Proposal 5: The the correspoding test method to verify the new AI/ML features such as reduced RX beam sweeping factor needs to be further studied in RAN4.

Proposal 6: RAN4 could investigate whether compliance with radiated spurious emission limits consistently implies compliance with conducted spurious emission limits and study the feasibility of verifying either radiated or conductive spurious emission requirement for 6G UE.

# 4  References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025

- R1-2506216, Overview of 6GR air interface, Qualcomm Incorporated, August 2025

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz, v19.1.0, June 2025.

- TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, Rel-19

- TS 38.124, ElectroMagnetic Compatibility (EMC) requirements for mobile terminals and ancillary equipment, Rel-18