3GPP TSG-RAN WG4 Meeting #116bis	R4-2513031

Prague, Czech Republic, 13-17 October 2025

Source:	Apple

Title:	Discussion on AI/ML-Based 6G RRM Use Cases

Agenda item:	8.9

Document for:	Discussion

Introduction

This paper presents candidate AI/ML-based use cases for 6G RRM aimed at reducing measurement gaps, lowering UE power consumption, boosting throughput, and strengthening mobility robustness. Building on 3GPP studies in TR 38.744 [1] and TR 38.843 [2], it extends prediction from cell-level to beam- and multi-domain L3 RSRP across time, frequency, and spatial dimensions, and further advances spatial-domain prediction in beam management across Rx spatial and temporal domains.

Proposed 6G AI/ML Use Cases

Temporal L3 Beam-Level Prediction

This use case focuses on Top-K prediction of beam-level L3 RSRP across time. The UE measures neighbor-cell beams over multiple frequencies and uses an AI/ML model to track a time series of the Top-K strongest beams for all measured frequencies, predicting how this ranked list will evolve in temporal domain. When the correct target frequency or cell is selected for handover, the specific Top-K beam set for that target cell is extracted from these predictions, giving the network a time-aligned view of which beams will remain optimal. Each predicted beam is accompanied by a confidence score and a validity window timer, enabling accurate and forward-looking beam selection across all relevant frequency layers and cells.

A key justification for investigating this use case is mobility enhancement. Accurate, time-aware knowledge of which beams will remain strong in the target cell at the future handover moment is essential in inter-frequency mobility, where fast fading, Doppler shifts, and user motion can change conditions rapidly between measurement and handover execution. Although the proposal does not define handover optimization procedures, it provides the critical input: reliable, temporally predicted Top-K beam-level RSRP that future mobility strategies can leverage.

Conceptually, with a trusted Top-K beam list and associated validity windows, the network could prepare multiple potential handover paths in parallel, ensuring that at least one remains optimal when the HO command is executed. For example, this could improve the likelihood of contention-free random access (CFRA) success, reduces the risk of contention-based fallback, and lowers overall handover delay and UE power consumption. By extending prediction from cell-level to detailed, time-sequenced beam-level L3 RSRPs, the approach provides the fine-grained, forward-looking channel knowledge needed to support robust and efficient mobility in 6G.

- Time-Domain L3 Beam-Level Prediction for 6G RRM
Investigate an AI/ML use case where the UE predicts a time series of L3 beam level RSRP across neighbor frequencies and cells.

Frequency-Domain L3 Prediction for Non-Collocated Scenarios

This use case focuses on frequency-domain prediction of L3 RSRP at both cell and beam level for non-collocated deployments. The UE measures its serving layer and a limited subset of neighbor frequencies and cells, and an AI/ML model learns frequency–beam relationships to predict all L3 RSRP values across the candidate frequency layers or, alternatively, to forecast a ranked Top-K list of {frequency / target-cell, beam} pairs, each with confidence scores and validity window timers. By operating across multiple layers and cells, the model can identify the most promising target frequency and its strongest beam without performing exhaustive neighbor-frequency sweeps, providing time-aligned predictions for handover preparation.

The justification for investigating this use case is mobility enhancement and significant measurement-gap (MG) reduction. In today’s NR mobility, UEs must retune from the serving frequency to every neighbor frequency layer, creating long measurement gaps and frequent interruptions. These gaps lower user throughput and drain battery power, and the delay they introduce can reduce the accuracy of contention-free random access (CFRA) preparation. By predicting in the frequency domain which carrier and beam will remain optimal, the UE can skip continuous inter-frequency measurements, cutting MG duration and periodicity. This frees valuable spectrum-time resources, improving throughput and energy efficiency while supporting timely handover decisions.

The enhancement would enable the network to prepare CFRA resources in advance on the most promising frequency/beam combinations. The target gNB can pre-arm PRACH occasions and activate parallel receive-beam (TCI) states for all predicted candidates. When the handover command is executed, the UE can attempt access on the best predicted pair and, if needed, immediately retry on the next candidates without extra signaling. This frequency + beam prediction provides frequency, spatial, and temporal diversity, helping maintain robust mobility even in fast-changing radio environments and ensuring that at least one of the predicted frequency/beam pairs remains valid at the critical instant.

By extending AI/ML prediction beyond single-frequency, cell-level methods to cross-frequency, beam-level L3 RSRP prediction, this approach provides more precise and forward-looking channel knowledge. It boosts throughput, shortens handover delays, and lowers UE power consumption, while giving operators a robust foundation for future CFRA preparation and advanced mobility enhancements in 6G.

- Frequency-Domain L3 Level Prediction for Non-Collocated Cells for 6G RRM
Investigate an AI/ML use case for non-collocated scenarios to predict all L3 RSRP values or a list of {frequency / target-cell, beam} pairs across frequency.

Spatial-Domain Joint Tx/Rx Beam-Level L3 Prediction

This use case targets spatial-domain AI/ML prediction of L3 RSRP with joint Tx/Rx beam prediction, aiming to sharply reduce measurement gaps (MGs) and increase throughput, especially by limiting time consuming Rx-beam sweeping procedures. For example, in current NR beam management, even when AI/ML reduces the number of Tx beams to probe, the UE must still perform full Rx-beam sweeps for each probed Tx beam to identify the best receive direction. Each Rx sweep requires a measurement gap, interrupts user traffic, and increases UE power consumption. The overhead grows with the number of Rx beams, making it a critical bottleneck for 6G mobility and inter-frequency scenarios.

The proposed method enables the UE to measure only a minimal Set B of Tx–Rx beam pairs, then employ AI/ML to reconstruct the full Set A L3 RSRP space. By predicting both transmit and receive beam space, the UE can skip many of the traditional Rx sweeps, thereby shortening or even eliminating measurement gaps that would otherwise be needed to discover the best Rx direction. This saves spectrum-time resources, allowing more continuous downlink and uplink traffic and improving effective throughput.

By integrating Tx and Rx beam prediction into a single spatial-domain L3 model, this approach offers a scalable way to handle the growing number of beams expected in 6G. The resulting measurement-gap reduction translates directly into higher throughput, faster mobility decisions, and more efficient use of spectrum, while remaining fully compatible with 3GPP reporting principles.

- Spatial-Domain Joint Tx/Rx Beam-Level L3 Prediction for 6G RRM
Investigate an AI/ML use case where the UE measures only a minimal Set B of Tx–Rx beam pairs to reconstruct the full Set A RSRP space.

Multi-Domain L3 level Prediction for Measurement-Gap Reduction

This use case focuses on multi-domain AI/ML prediction of L3 RSRP, spanning frequency, Tx beams, Rx beams, and time, to minimize measurement effort while sustaining accurate mobility performance. In current NR procedures, a UE must open fixed measurement gaps to sweep every configured frequency layer and all Tx–Rx beam pairs at regular intervals, even when strong cross-domain correlations mean many of these measurements are redundant. As the number of carriers and beams grows in 6G, this exhaustive process can consume more than 30 % of downlink and uplink capacity and significantly delay mobility actions.

The numerical impact in throughput is significant. In FR1, a 6 ms MG every 40 ms can occupy about 15 % of downlink and uplink resources, reducing throughput by more than 20 %. In FR2, a 5 ms MG every 20 ms can consume around 25 % of resources and reduce throughput by over 30 %. Measurement delay grows with the number of layers: 20 ms periods multiplied by 24 (=8 Rx beams x 3) and multiple frequency layers lead to long reaction times. These penalties directly counteract key 6G objectives of high spectral efficiency, low latency, and energy efficiency.

For example, Figure 1 shows that AI/ML-based prediction can cut inter-frequency measurement overhead from about 28 % to 7 %, yielding a ≈35 % net throughput increase. For this example, and for a legacy process, a UE would measure nine 5 ms MG gaps every 20 ms MGRP, consuming about 9 × 5 / (8 × 20) ≈ 28 % of DL capacity. With prediction across frequency layers and across Rx beam space, the UE would need only four gaps, and their effective MGL can be shortened through Tx beam prediction to about 1.25 ms each, so the overhead falls to roughly 4 × 1.25 / (8 × 20) ≈ 7 %. The recovered airtime directly boosts usable data resources and lowers interruption, explaining the ~35 % throughput gain illustrated in the figure.

The proposed framework allows the UE to measure only a minimal, dynamically chosen Set B of frequency layers and Tx/Rx beam pairs, together with a short time sequence of past measurements. An AI/ML model then reconstructs the complete multi-dimensional Set A of L3 RSRP values in a time series fashion allowing to predict their evolution over future time slots. This compact “fingerprint” captures spatial, spectral, and temporal continuity, enabling the network to obtain full mobility-relevant information without requiring full sweeps.

The benefits of multi-domain prediction are substantial. By cutting measurements across several domains simultaneously, the UE can shrink measurement-gap length (MGL) and reduce the number of gaps per measurement cycle. The freed spectrum-time resources directly translate to higher user throughput and lower latency, while fewer RF retuning operations reduce UE power consumption. Temporal forecasting smooths short-term fading, further eliminates measurement gaps by preventing unnecessary re-measurements, further decreasing signaling and delay.

By combining spatial, frequency, and temporal prediction in a single AI/ML model, this approach provides a scalable and power-efficient foundation for 6G radio-resource management. It enables more continuous data transmission, faster mobility decisions, and robust inter-frequency handovers.

Figure 1: Indirect measurement event prediction.

- Multi-Domain L3 level Prediction for Measurement-Gap Reduction in 6G RRM
Investigate an AI/ML use case where the UE measures only a minimal, dynamically chosen Set B of frequency layers and Tx/Rx beam pairs with a short time history, and reconstructs the full multi-dimensional Set A of L3 RSRP values and forecasts their evolution.

AI/ML based Rx Beam Prediction for Beam-Management

Current AI/ML beam-management (BM) procedures could require the UE to perform exhaustive Rx-beam sweeping for every Set B Tx beam to collect reliable L1 measurements. Although AI/ML beam-management in Release 19 can reduce the number of Tx beams that need to be measured, the UE still has to sweep all Rx beams for each Tx beam.

This creates residual overhead in three dimensions: (i) spatially, because every Tx beam still triggers a full Rx sweep; (ii) temporally, because the UE repeats those sweeps every measurement period regardless of channel stability; and (iii) with respect to device orientation, because even small rotations force a new Rx discovery. The result is unnecessary power consumption, time delay, and scheduling restrictions, which limits the real efficiency gains of AI/ML-assisted BM.

A generic Rx-beam selection methods could treat each measurement period as an isolated snapshot, overlooking the temporal continuity of the radio channel and the stability of beam direction in a global reference frame even when device orientation changes. This assumption increases sensitivity to short-term fading and measurement noise, leading to redundant sweeps and repeated identification of beams that remain essentially unchanged.

To fully realize the potential of AI/ML, a next-generation BM framework must compress measurements jointly across space (RX-TX beams) , temporal, and orientation so that the UE measures only when and where it matters, while still guaranteeing robust Set A Tx/Rx beam prediction

We propose a multi-dimensional compression framework with joint Tx–Rx beam prediction and temporal/orientation tracking:

Spatial compression lets the UE probe only a small subset of Tx–Rx pairs inside Set B. An AI/ML model reconstructs the full Tx–Rx measurement space from these partial observations and predicts the optimal Set A Tx-Rx beams together 
Temporal compression extends prediction into a time-series task. By continuously updating a hidden “beam state” using recent Top-K RSRP observations and UE motion data, the model forecasts the next likely Tx–Rx beam pairs, and RSRPs and skips whole measurement cycles when confidence is high.
Orientation compression maintains a global beam map that decouples the best Rx beam from the UE’s instantaneous pose. When the device rotates, sensor data re-projects this global map into the current local frame, eliminating repetitive Rx sweeps triggered solely by orientation changes.

Key Benefits
Our proposal directly lowers the number of Rx beam sweeps per Tx beam, cutting power consumption and air-interface interruptions. Temporal forecasting smooths noise and prevents spurious beam switches, while orientation awareness ensures stable beam association despite device motion. Together these features reduce UE scheduling restrictions, and improve end-to-end throughput, without compromising prediction accuracy, and improving measurement delays and latency.

- Rx Beam Prediction in Spatial and Time Domain for L1 AI/ML Beam Management for 6G RRM
Investigate an AI/ML use case where the UE measures only a minimal subset of Tx–Rx beam pairs and to reconstructs the full Set A L1 RSRP space, jointly performing Tx-Rx prediction in both spatial and time domains

Conclusion

In this paper, we discussed candidate AI/ML-based use cases for 6G RRM, highlighting approaches to reduce measurement gaps, lower UE power consumption, increase throughput, and enhance mobility robustness. We made the following proposals:

- Time-Domain Top-K L3 Beam-Level Prediction for 6G RRM
Investigate an AI/ML use case where the UE predicts a time series of Top-K L3 beam level RSRP across neighbor frequencies and cells.

- Frequency-Domain L3 Level Prediction for Non-Collocated Cells for 6G RRM
Investigate an AI/ML use case for non-collocated scenarios to predict all L3 RSRP values or a Top-K list of {frequency / target-cell, beam} pairs across frequency.

- Spatial-Domain Joint Tx/Rx Beam-Level L3 Prediction for 6G RRM
Investigate an AI/ML use case where the UE measures only a minimal Set B of Tx–Rx beam pairs to reconstruct the full Set A L3 RSRP space.

- Multi-Domain L3 level Prediction for Measurement-Gap Reduction in 6G RRM
Investigate an AI/ML use case where the UE measures only a minimal, dynamically chosen Set B of frequency layers and Tx/Rx beam pairs with a short time history, and reconstructs the full multi-dimensional Set A of L3 RSRP values and forecasts their evolution.

- Rx Beam Prediction in Spatial and Time Domain for L1 AI/ML Beam Management for 6G RRM
Investigate an AI/ML use case where the UE measures only a minimal subset of Tx–Rx beam pairs and to reconstructs the full Set A L1 RSRP space, jointly performing Tx-Rx prediction in both spatial and time domains

- References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, Draft Technical Report, RAN #109.

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Technical Report