3GPP TSG-RAN WG4 Meeting #116BIS						       R4-2513037

Prague, Czech, 13th – 17th October 2025

Agenda Item: 	8.4

Source: 	MediaTek Inc.

Title: 	Views on 6G general RF and UE RF

Document for: 	Approval

1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G general RF and UE RF with the focus on the following aspects:

- UL coverage and cell edge performance improvement

- MSD framework

- RF retuning time

- Spectrum aggregation

- Device specific aspects

- UE power class framework

- Band and band combination framework and related aspects

2	UL coverage and cell edge performance improvement

The objectives of 6G RAN SID state the following:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration

To achieve the same coverage as 3.5GHz (e.g. n77/n78), the following candidate techniques are proposed be considered to increase uplink cell coverage in “around 7GHz” band deployments:

- Default power class as PC2 (26dBm) at least for handheld devices

- UL coherent MIMO – ensuring this can be commercially adopted, unlike in 5G

- Increase in number of antennas: UL: 4Tx in UE and 128/256Rx in gNB

- Low PAPR waveform – under consideration in RAN1

- UL power boost via opportunistic Tx emissions limits relaxation (i.e. ACLR, SEM, Tx EVM, Occupied CBW,…) and mechanisms to enable A-MPR reduction/elimination

- Sub-Band Full Duplex

  - Network-side SBFD should not require more studies vs Rel-19, whereas UE-SBFD can offer further practical enhancements, but requires further study within 3GPP.

- UL-DL decoupling:

  - Low frequency operating band is considered as uplink spectrum to enhance cell coverage and high frequency band can be considered as DL spectrum to improve T-put.

Many of the above mechanisms can also be applicable to other frequency bands. Applicability of mechanisms may also depend on device characteristics and associated constraints (e.g. form factor etc).

Proposal 1: Investigate the RAN4 aspects of the listed techniques to achieve uplink cell coverage enhancement solutions in around 7GHz deployments to guarantee the T-put performance and achievable QoS of the edge user in the existing 5G mid-band (~3.5GHz) site grid:

- Default power class as PC2 (26dBm) at least for handheld devices

- UL coherent MIMO – ensuring this can be commercially adopted, unlike in 5G

- MIMO with beamforming

- Increase in number of antennas: UL: 4Tx in UE and 128/256Rx in gNB

- Low PAPR waveform

- UL power boost via opportunistic Tx emissions limits relaxation (i.e. ACLR, SEM, Tx EVM, Occupied CBW,…) and mechanisms to enable A-MPR reduction/elimination

- Sub-Band Full Duplex

  - Network-side SBFD should not require more studies vs Rel-19, whereas UE-SBFD can offer further practical enhancements, but requires further study within 3GPP.

- UL-DL decoupling:

  - Low frequency operating band is considered as uplink spectrum to enhance cell coverage and high frequency band can be considered as DL spectrum to improve T-put.

3	MSD framework

In 5G NR, the aggregation of lower bands (e.g., FDD bands) and higher bands (e.g., TDD bands) is typically introduced because the lower band DL spectrum is very limited and not enough to support very wide CBW or aggregated CBW operation. However, regarding the aggregation of lower bands (e.g., FDD bands) and higher bands (e.g., TDD bands) operations, the lower-band’s TX harmonic becomes very strong interference to jam high band’s RX. Regarding NR, some of the high MSD requirements for bands combination were specified in Table as shown below for quick reference.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | RX MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |
| n1 | n77 | 5 | 15 | 12 | 10 | 23.9 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 10 | 23.9 | UL2/DL1 direct-hit |
| n2 | n78 | 5 | 15 | 12 | 10 | 23.9 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 12 | 10 | 23.9 | UL2/DL1 direct-hit |
| n28 | n50 | 5 | 15 | 12 | 5 | 28.1 | UL2/DL1 direct-hit |
| n28 | n74 | 5 | 15 | 12 | 5 | 28.1 | UL2/DL1 direct-hit |
| n28 | n94 | 5 | 15 | 12 | 5 | 28.1 | UL2/DL1 direct-hit |

Observation 1: Regarding the aggregation of lower and higher bands, there is severe UE DL cell-coverage reduction due to the high RX MSD issues (e.g., MSD of 20~30 dB). Conventional NR MSD mitigation method based on limiting TX CBW does not change MSD values.

Besides, the MSD minimum requirements for the aggregation of lower and higher bands are typically evaluated based on example TX and RX frequency channels. However, some TX and RX frequency channel configurations do not have MSD issue. Regarding the bands combination with MSD issues only on some RX channels, the potential methods can be considered to avoid deactivating RX carriers/channels without MSD issues.

Observation 2: The MSD requirements for the aggregation of lower and higher bands are just evaluated based on example TX and RX frequency channels. However, some TX and RX frequency channel configurations do not have MSD issue.

Regarding MSD requirements for the aggregation of lower and higher bands, the assisted MSD reporting method can be helpful for preventing NW from deactivating the aggregation of lower and higher bands in certain RX carriers that have no significant MSD issues.

Proposal 2: Regarding the bands’ combination with high MSD issues only on some RX channels, study the feasibility of the assisted MSD reporting to avoid deactivating RX carriers/channels without significant MSD issues.

Furthermore, to specify default power class MSD requirements for FR1 bands combination usually introduces huge RAN4 workload. In Rel-19, the phase 1 MSD requirements simplification was proposed. It indicates that HPUE MSD requirements can be calculated from the default power class MSD values using a LUT table. However, there are still significant default power class MSD workload for the aggregation of lower and higher bands.

Observation 3: To specify default power class MSD requirements for FR1 bands combination usually introduces huge RAN4 workload.

Proposal 3: Study the default power class MSD requirements simplification for the aggregation of lower and higher bands to help significantly reduce the RAN4 workload.

- To specify specific bands-combination MSD requirement is not precluded if necessary.

Studying the potential MSD mitigation methodology is helpful and beneficial for managing UEs with severe DL cell-coverage reduction due to high MSD issue (e.g., 20~30 dB).

Proposal 4: In additional to restricting TX CBW in NR to reduce jammed RX spectrum range, regarding potential PC3 or default power class MSD mitigation in 6G:

- Study the UE technologies improvement such as antenna isolation, front-end components, and ICs.

- Study improvements on BS aspect such as power, time, and frequency domain configurations.

4	RF retuning time

The interruption defined in RRM requirements is associated with various procedures, such as Bandwidth Part (BWP) switching, and consists of both RF switching time and baseband (BB) processing time. This results in an overall interruption requirement of approximately 1 ms. Based on existing NR requirements, RF switching time is defined as 0.5 ms for FR1 and 0.25 ms for FR2. However, recent advancements in UE implementation suggest that RF switching time can be performed much faster than the original value of 0.5 ms from NR Rel-15. Therefore, it is recommended that RAN4 further investigate ways to reduce interruption times and define requirements that more accurately reflect the capabilities of the latest UE technologies.

Observation 4: The requirements of interruption including the RF switching time in previous mobile generation has not been optimised since NR Rel-15.

Proposal 5: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

5	Spectrum aggregation

More efficient spectrum aggregation

In 5G, the carrier-aggregation (CA) framework is critical for spectrum aggregation, yet it limits physical layer and protocol efficiencies because it enforces a one-to-one coupling of each physical carrier component and a logical cell, resulting in excess control overhead and slow carrier activation.

For 6G, we recommend decoupling this one-to-one mapping and aggregating multiple carriers with similar propagation channel properties to a single “Multi-carrier Cell”. As illustrated in Figure 1, intra-band and nearby inter-band carriers can be aggregated into a single cell when they meet specific conditions, e.g., time and frequency alignment, frequency span, and receive power difference, thereby enabling fast carrier activation and reduce the control overhead. Based on 5G experiences, for example, practical thresholds of conditions may include Maximum Receive Timing Difference (MRTD) < 3us (or CP), frequency error < 0.1ppm, and similar total received power for an intra-band-like UE receiver, where a common time/frequency synchronisation reference and AGC can be used at a UE. In addition, it is a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where these conditions are typically met.

Note that when the conditions for synchronization and power relative power level alignment noted above are not met across carriers, we can still apply the 5G legacy way to aggregate carriers, for example, when aggregating two widely separated carriers (e.g. very different bands).

While RAN1 should study Multi-carrier Cell support and procedures, RAN4 RF will need to study RF-switch time (to switch on a new carrier within a cell) requirements, acceptable Transmit timing alignment (TAE), BS frequency error, and total power differences at UE between carriers; RAN4 RRM can then assess aggregation delay, accounting for AGC resettling and fine time tracking.

Figure 1 Illustration of multi-carrier cell

Observation 5: Aggregate multiple carriers into a single cell can reduce the sigalling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 6: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 6: RAN4 RF to study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE.

Impact of wider aggregate bandwidths

The UE antenna performance due to wide aggregated BW shall be further discussed since 200MHz maximum channel bandwidth is also discussed for >2GHz spectrum.

Proposal 7: Further investigate the antenna performance aspects when applying wider aggregate bandwidths in 6G.

Carrier switching enhancements for UL and DL

Due to the limited number of Tx chains at the UE, the number of bands that the UE can be configured for is constrained. For instance, a UE with two Tx chains can either be configured with one band using UL-MIMO or with two bands without UL-MIMO (one Tx chain per band). UL Tx switching allows the UE to dynamically switch the Tx chain(s) among bands, enabling configuration with more bands and supporting UL-MIMO transmissions without requiring additional Tx chains. UL Tx switching was introduced incrementally from R16 to R19. Each release targets a different scenarios, which adds unnecessary limitation to 5G networks. For example, if more than 2 UL bands are needed, it can only be supported from Release 18. For 3Tx UE, it can only be supported in Rel-19, but it is limited in 2 UL bands. Enabling flexible carrier switching in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity by ensuring the uplink resources for the UE can be allocated where resources are available. Therefore, improvements for UL Tx switching should be investigated in 6G to simplify and streamline its operation.

Beyond UL, enabling more flexible DL carrier switching without requiring full spectrum aggregation could deliver benefits comparable to those achieved with UL carrier switching. In 5G, it was considered only in Rel-19 with limited scope, while carrier switching is especially valuable under hardware constraints (e.g., few RF chains or synthesizers) and in the presence of DL/UL interference and intermodulation distortion. Therefore, the corresponding switching-delay will need to be studied in RAN4 RF and the interruption requirements can be further studied in RAN4 RRM.

For 6G, we expect a more flexible mechanism for DL/UL switching with forward compatibility on aspects on number of bands, number of Tx/Rx, … etc. In this 6G SI, it would be good to take this as one design target and study its corresponding impact in 3GPP specs.

Observation 7: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 8: RAN4 to study support of carrier switching and simplify configurations in a forward-compatible manner; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements.

Secondary carrier activation, increase of spectrum aggregation

Since Rel-15, 5G NR has continuously refined Scell activation to address a variety of conditions (with or without measurement reports, different synchronization states, power differences), diverse RS configurations (SSB-less, A-TRS, P/SP/AP CSI-RS), and different types of UE RF architecture (intra-/inter-band, co-/non-collocated), yet only limited scenario has seen widespread adoption in commercial 5G.

For 6G, the secondary carrier activation should ensure timely CSI so the network can schedule sufficient radio resources for busty traffic, and the secondary carrier activation should be specified only for critical, high-likely deployment cases to simply the requirement design.

Based on 5G experience, the SCell/carrier activation at least involves RF switch, AGC settling, cell search, fine sync tracking, and CSI report. RAN4 RF will need to study RF switch-time requirements, and RAN4 RRM will assess secondary carrier activation interruption and the overall delay, taking other factors into account.

Observation 8: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 9: RAN4 to study critical, high-likely secondary carrier activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements.

6	Device-specific aspects

To classify the device type in 6G, RAN Plenary agreed [2], as follows:

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

- The value(s) for the identified parameters for a device type

Conclusion: proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion) in # 109 Plenary Meeting report.

Based on RAN1 and RAN plenary decision, RAN4 need to wait the RAN plenary guidance regarding the device types topic.

If some early discussion is needed regarding potential device characteristics for 7GHz, to evaluate 6G system performance and coexistence evaluation, RAN4 can consider the following device characteristics in Table 1.

Table 1: Examples of possible 6G device capabilities for around 7GHz

| Device types | Level of constraints | Example Capabilities Envelope |  |  |
| --- | --- | --- | --- | --- |
|  |  | Duplex | MIMO layers / Antennas | Default power class (dBm) |
| Reference 2, e.g. FWA form factor | Low | FDD, TDD | At least 4L/ At least #4T/#4R | 29 |
| Reference 1, e.g. handheld form factor | Medium | FDD, TDD | At least 4L/ At least #4T/#4R | 26 |

Table 2: Potential Min. Tx/Rx Capability per Handheld device and spectrum ranges

| FR1 low band (Sub-1GHz) | FR1 mid band (1.7~3GHz) | FR1 high band (3~6GHz) | U6G + around 7GHz (6~8.4GHz) | Around 15GHz (10~15GHz) |
| --- | --- | --- | --- | --- |
| 1T/2R | 2T/4R | 2T/4R | 4T/4R | 4T/4R |

The values in Table 2 are provided for information at this stage but may need more formal discussion once the Device Types discussion is further progressed by RAN plenary. These could be considered for the reference 1 device evaluations in other bands.

Proposal 10: Consider the device characteristics and parameters in Table 1 for 7GHz and Table 2 for other bands for the 6GR coexistence evaluation and system performance analysis with different antenna elements in 6G BS and default power class of 6G UE.

7	UE Power Class framework

In 5G NR, the UE power class reporting is quite complicated in existing specs. For single band, single carrier(CC), 4Tx was supported since Rel-18. Further, 3Tx is also supported from Rel-19. UE may have 1~4 Tx chains for different UL MIMO layer capability. In the beginning NR spec, Tx diversity was transparent to network. To clearly specify requirements on Tx diversity, the IE to indicate Tx diversity configuration was introduce in Rel-17. Same scheme shall be considered when it comes to 6G study.

Proposal 11: Study a new framework for 6G power class definition to enable more flexibility to different UE implementations

When it comes to 6G UE, UE implementation also needs to consider 5G NR support, until the incumbent bands operating in 5G NR will migrate fully to 6G. Therefore, it would be anticipated the 6G RF hardware may be similar to the 5G NR implementation. RAN4 need to consider these scenarios as starting point.

Proposal 12: On bands shared by 5G and 6G, the existing UE 5G implementation can be considered as a starting point

8	Band and Band combination frameworks and related aspects

In 5G(NR) cellular communication system, carrier aggregation (CA) band combinations had been widely introduced in current specs in 38.101-1 for FR1, 38101-2 for FR2 and 38.101-3 for MR-DC. In 5G, the CA framework is used where there is always a one-to-one mapping between a logical cell and a physical carrier. Plus there are thousands of CA combination in the existing specs. The large number of 5G NR CA/DC/MRDC combinations bring heavy signalling overhead in air interface due to the number of BC-specific features. UE-specific signaling and carriers configurations complexity is proportional with the number of aggregated carriers. All of these leads to system overhead. Efforts to reducing excessive UE capability signalling should be led by RAN2. However, RAN4 should also study whether there are potential approaches to simplify its band combination framework and UE capability reporting of supported combinations for 6G.

Observation 9: UE-specific signalling and complexity of carriers in 5G NR CA configurations is proportional with the number of aggregated carriers.

Proposal 13: RAN4 to study whether there are potential approaches to simplify its band combination framework and UE capability reporting of supported combinations for 6G, to reduce UE capability signalling overhead.

If UL-DL decoupling is introduced, a framework for defining and reporting the supported band combinations would be needed.

Proposal 14: If UL-DL decoupling is introduced, further study the specification framework and how to efficiently report supported UL-DL decoupled band combinations.

There are incumbent bands and legacy cellular service such as 4G and 5G networks. To accommodate legacy network, the FR1 legacy FDD/TDD band definition shall be re-used in 6G, possibly generalising more “TDD” to cover also SBFD operation. It can further be considered whether band index can be simplified when the frequency range is fully overlapped.

Observation 10: For the incumbent spectrum blocks migration from 5G to 6G, maintain the FR1 legacy FDD/TDD band definition and duplex can be considered as starting point.

Proposal 15: Incumbent frequency band/band index shall be re-used in 6G as starting point, with further study on useful adaptations to the framework.

9	Summary

In this paper, the observations and proposals for the 6G general RF and UE RF are provided.

<UL coverage enhancement>

Proposal 1: Investigate the RAN4 aspects of the listed techniques to achieve uplink cell coverage enhancement solutions in around 7GHz deployments to guarantee the T-put performance and achievable QoS of the edge user in the existing 5G mid-band (~3.5GHz) site grid:

- Default power class as PC2 (26dBm) at least for handheld devices

- UL coherent MIMO – ensuring this can be commercially adopted, unlike in 5G

- MIMO with beamforming

- Increase in number of antennas: UL: 4Tx in UE and 128/256Rx in gNB

- Low PAPR waveform

- UL power boost via opportunistic Tx emissions limits relaxation (i.e. ACLR, SEM, Tx EVM, Occupied CBW,…) and mechanisms to enable A-MPR reduction/elimination

- Sub-Band Full Duplex

  - Network-side SBFD should not require more studies vs Rel-19, whereas UE-SBFD can offer further practical enhancements, but requires further study within 3GPP.

- UL-DL decoupling:

  - Low frequency operating band is considered as uplink spectrum to enhance cell coverage and high frequency band can be considered as DL spectrum to improve T-put.

< MSD framework >

Observation 1: Regarding the aggregation of lower and higher bands, there is severe UE DL cell-coverage reduction due to the high RX MSD issues (e.g., MSD of 20~30 dB). Conventional NR MSD mitigation method based on limiting TX CBW does not change MSD values.

Observation 2: The MSD requirements for the aggregation of lower and higher bands are just evaluated based on example TX and RX frequency channels. However, some TX and RX frequency channel configurations do not have MSD issue.

Proposal 2: Regarding the bands’ combination with high MSD issues only on some RX channels, study the feasibility of the assisted MSD reporting to avoid deactivating RX carriers/channels without significant MSD issues.

Observation 3: To specify default power class MSD requirements for FR1 bands combination usually introduces huge RAN4 workload.

Proposal 3: Study the default power class MSD requirements simplification for the aggregation of lower and higher bands to help significantly reduce the RAN4 workload.

- To specify specific bands-combination MSD requirement is not precluded if necessary.

Proposal 4: In additional to restricting TX CBW in NR to reduce jammed RX spectrum range, regarding potential PC3 or default power class MSD mitigation in 6G:

- Study the UE technologies improvement such as antenna isolation, front-end components, and ICs.

- Study improvements on BS aspect such as power, time, and frequency domain configurations.

< RF retuning time >

Observation 4: The requirements of interruption including the RF switching time in previous mobile generation has not been optimised since NR Rel-15.

Proposal 5: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

< Spectrum aggregation >

Observation 5: Aggregate multiple carriers into a single cell can reduce the sigalling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 6: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 6: RAN4 RF to study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE.

Proposal 7: Further investigate the antenna performance aspects when applying wider aggregate bandwidths in 6G.

Observation 7: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 8: RAN4 to study support of carrier switching and simplify configurations in a forward-compatible manner; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements.

Observation 8: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 9: RAN4 to study critical, high-likely secondary carrier activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements.

< Device specific aspects >

Proposal 10: Consider the device characteristics and parameters in Table 1 for 7GHz and Table 2 for other bands for the 6GR coexistence evaluation and system performance analysis with different antenna elements in 6G BS and default power class of 6G UE.

Table 1: Examples of possible 6G device capabilities for around 7GHz

| Device types | Level of constraints | Example Capabilities Envelope |  |  |
| --- | --- | --- | --- | --- |
|  |  | Duplex | MIMO layers / Antennas | Default power class (dBm) |
| Reference 2, e.g. FWA form factor | Low | FDD, TDD | At least 4L/ At least #4T/#4R | 29 |
| Reference 1, e.g. handheld form factor | Medium | FDD, TDD | At least 4L/ At least #4T/#4R | 26 |

Table 2: Potential Min. Tx/Rx Capability per Handheld device and spectrum ranges

| FR1 low band (Sub-1GHz) | FR1 mid band (1.7~3GHz) | FR1 high band (3~6GHz) | U6G + around 7GHz (6~8.4GHz) | Around 15GHz (10~15GHz) |
| --- | --- | --- | --- | --- |
| 1T/2R | 2T/4R | 2T/4R | 4T/4R | 4T/4R |

< UE Power Class framework >

Proposal 11: Study a new framework for 6G power class definition to enable more flexibility to different UE implementations

Proposal 12: On bands shared by 5G and 6G, the existing UE 5G implementation can be considered as a starting point

< Band and Band combination frameworks and related aspects >

Observation 9: UE-specific signalling and complexity of carriers in 5G NR CA configurations is proportional with the number of aggregated carriers.

Proposal 13: RAN4 to study whether there are potential approaches to simplify its band combination framework and UE capability reporting of supported combinations for 6G, to reduce UE capability signalling overhead.

Proposal 14: If UL-DL decoupling is introduced, further study the specification framework and how to efficiently report supported UL-DL decoupled band combinations.

Observation 10: For the incumbent spectrum blocks migration from 5G to 6G, maintain the FR1 legacy FDD/TDD band definition and duplex can be considered as starting point

Proposal 15: Incumbent frequency band/band index shall be re-used in 6G as starting point, with further study on useful adaptations to the framework.

< Cross-WG>

Proposal 16: RAN4 to identify whether any issue has potential cross-WG impact and arrange the required action, if any.

10	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] RP-252873, Summary of 6G Diverse Device Types (including Minimum UE Bandwidth), Apple