3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513045

# Prague, Czech Republic, 13th – 17th October, 2025

Title:	Discussion on BS RF and coexistence for 6GR

Source:	Samsung

Agenda item:	8.5

Document for:	Approval

Introduction

Recognizing the need to lay the groundwork for future advancements, the 3rd Generation Partnership Project (3GPP) has initiated its early research into the sixth generation (6G) of mobile technology. As a critical part of this effort, a Study Item (SI) on "6G Radio" (6GR) was approved during a TSG RAN Plenary #108, with the SID [1], addresses the foundational radio aspects for 6G. While 3GPP began with high-level vision and use-case workshops, this 6GR SI represents a significant step towards technical investigations. In this paper, we will discuss the objectives for the Base Station (BS) Radio Frequency (RF) and 3GPP co-existence study parts, analyzing the technological opportunities and challenges presented by the 6GR study.

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

In RAN4#116 meeting, the meeting had noted the plan of RAN4 6G in [2]. According to [2], RAN4 planned to discuss non-spectrum and non-AI BS RF, MSR and coexistence study in Q4’25 and Q1’26. Thus, in this paper, we will express our views for these three aspects.

Considerations for 6G Radio co-existence study

  - General review

In the latest 6G deployment scenario discussions agreed in RAN-P #109 [3], the current targeted carrier frequencies for 6G are for 700MHz, 2GHz, 4GHz, 7GHz, 15GHz and 30GHz. And by reviewing the existing studies carried out in RAN4, we summarized the previous available RAN4 co-ex studies together with their example carrier frequencies and key assumptions from both UE and BS RF perspective in the table below.

Table 2.1-1 Summary of RAN4 co-ex studies in previous TRs

| TR# in 3GPP | Carrier frequencies studied | Deployment scenarios studied | BS assumption (TN) | UE assumption (TN) |
| --- | --- | --- | --- | --- |
| 36.942[4] | 900MHz | Rural | 43/46 dBm for LTE 15 dBi w. Non-AAS pattern | -40~23 dBm 0 dBi omni-directional atenna |
|  |  | Urban | 43/46 dBm for LTE 12 dBi w. Non-AAS pattern |  |
|  | 2GHz | Urban | 43/46 dBm for LTE; 49dBm for LTE-A 15 dBi w. Non-AAS pattern |  |
| 38.803[5] | 30 GHz | Indoor hotspot | 23 dBm Tx power 5 dBi w. (8x16x2) AAS pattern | -40~23 dBm 5dBi w. (2x2x2) AAS pattern |
|  |  | Urban macro | 43 dBm Tx power 8 dBi w. (8x16x2) AAS pattern |  |
|  |  | Dense urban | 33 dBm Tx power 8 dBi w. (8x16x2) AAS pattern |  |
|  | 45 GHz | Indoor/Dense | Same as 30GHz | Same as 30GHz |
|  | 70 GHz | Indoor/Dense |  |  |
| 38.921[6] | 7 GHz | Urban macro | 43 dBm Tx power 5.5 dBi w. (8x16x2) AAS pattern | -33 ~ 23 dBm 0 dBi omni-directional antenna |
|  |  | Indoor hotspot | 24 dBm Tx power 5.5 dBi w. (4x4x2) AAS pattern |  |
|  | 10 GHz | Indoor/Urban | Same as 7GHz | Same as 7GHz |
| 38.922[7] | 4 GHz (not co-ex study) | Rural macro | 46 dBm Tx power 6.4 dBi w. (4x8)x(3x1)x2 AAS pattern | -33 ~ 23 dBm 0 dBi omni-directional antenna |
|  |  | Suburban macro |  |  |
|  |  | Urban macro |  |  |
|  |  | Urban micro | 37 dBm Tx power 6.4 dBi w. (8x8x2) AAS pattern |  |
|  | 7 GHz (not co-ex study) | Suburban macro | 46 dBm Tx power 6.4 dBi w. (8x16)x(3x1)x2 AAS pattern | -33 ~ 23 dBm 0 dBi omni-directional antenna |
|  |  | Urban macro |  |  |
|  |  | Urban micro | 37 dBm Tx power 6.4 dBi w. (8x8x2) AAS pattern |  |
|  | 15 GHz | Urban macro | 46 dBm Tx power 6.4 dBi w. (16x24)x(4x1)x2 AAS pattern | -33 ~ 23 dBm 0 dBi omni-directional antenna |
| 38.858[8] | 4 GHz | Urban macro | 49 / 53 dBm Tx power 5 dBi w. (8x8x2) AAS pattern | -33 ~ 23 dBm 0 dBi omni-directional antenna |
|  |  | Indoor hotspot | 24 dBm Tx power 5 dBi w. (4x4x2) AAS pattern |  |
|  | 30 GHz | Urban macro | 30 / 40 dBm Tx power 5.5 dBi w. (8x16x2) AAS pattern | -40~22.4 dBm e.i.r.p 5.5 dBi w. (2x2x2) AAS pattern |
|  |  | Indoor hotspot | 24 dBm Tx power 3 dBi w. (4x8x2) AAS pattern |  |
| 38.863[9] (TN) | 2 GHz | Urban macro | 46 dBm Tx power 6.4 dBi w. (8x8x2) AAS pattern Or 17dBi w. Non-AAS pattern | -40 ~ 23 dBm 0 dBi omni-directional antenna |
|  |  | Rural macro | 46 dBm Tx power 7.1 dBi w. (8x8x2) AAS pattern Or 17dBi w. Non-AAS pattern |  |
|  | 11 GHz | Urban macro | 46 dBm Tx power 6.4 dBi w. (8x16)x(3x1)x2 AAS pattern |  |
|  | 14 GHz | Urban macro | 46 dBm Tx power 6.4 dBi w. (16x24)x(4x1)x2 AAS pattern |  |
|  | 17 GHz | Urban macro | 46 dBm Tx power 5.5 dBi w. (8x16x2) AAS pattern | -40~23 dBm 5 dBi w. (2x2x2) AAS pattern |
|  | 27 GHz | Urban macro |  |  |

And in [3] approved in RAN-P #109, it contains target scenarios and BS assumptions for different carrier frequencies as summarized in the following table. It should be noted that all UE antenna elements assumptions in [3] are marked as “TBD”, hence they were not included in the table below. And for the “Dense urban” in [3], the table below only incorporate the scenarios with carrier frequencies that marked as “micro” in [3].

Table 2.1-2 Summary of scenarios and BS assumptions in [3]

| Carrier frequencies [3] | Deployment scenarios [3] | BS assumptions [3] | From [4]~[9] |
| --- | --- | --- | --- |
| Around 700 MHz | Rural | Up to 64 Tx and Rx elements | Studied in LTE [4] w. different assumptions in terms of bandwidth, BS antenna, power, etc. |
|  | Urban macro |  |  |
|  | Sub-urban macro |  | Not available. |
| Around 2 GHz | Indoor hotspot | Up to 288 Tx and Rx elements | Not available. |
|  | Rural |  | Studied in [4] w. different LTE BS assumptions. Studied in [9], but focus on NTN-TN, not between TN, and with significantly less array. |
|  | Urban macro |  |  |
|  | Sub-urban macro |  | Not available. |
| Around 4 GHz | Indoor hotspot | Up to 576 Tx and Rx elements | Studied in [8] with significantly less antenna array. |
|  | Rural |  | Not available. |
|  | Urban macro |  | Studied in [8] with significantly less antenna array. |
|  | Sub-urban macro |  | Not available. |
| Around 7 GHz | Indoor hotspot | Up to 2304 Tx and Rx elements | Studied in [6] with significantly less antenna array and power. |
|  | Urban micro |  | Not available. |
|  | Rural |  |  |
|  | Urban macro |  | Studied in [6] with significantly less antenna array and power. |
|  | Sub-urban macro |  | Not available. |
| Around 15 GHz | Indoor hotspot | Up to 2304 Tx and Rx elements | Not available. [7] and [9] studied UMa which is not pursued in [3]. And [9] was focusing on NTN-TN. |
|  | Urban micro |  |  |
|  | Sub-urban macro |  |  |
| Around 30 GHz | Indoor hotspot | Up to 4096 Tx and Rx elements | Studied in [5], [8] and [9] with significantly less array. |
|  | Urban micro |  |  |
|  | Sub-urban macro |  |  |

From the two tables above, it is our observation that there is significant gap between what had been studies in RAN4 in [4]~[9] and what is currently pursued in [3] for 6G deployment. In summary, we have following observations for all listed deployments with target carrier frequencies listed in [3]:

Observation 1: The gaps between co-ex studies performed in RAN4 and 6GR target deployment scenarios in different carrier frequencies are quite large, and they are shown as below.

Observation 2: The previously available co-existence studies on 700MHz and 2GHz carrier frequencies are majorly from LTE study, which differs significantly with later 5G NR and the current 6G Radio in both system parameters and deployment scenarios.

Observation 3: The previously available co-existence studies on 4GHz and 7GHz carrier frequencies assumed much less antenna array elements and only partial of the deployment scenarios as given in latest 6G deployment discussion.

Observation 4: The previously available co-existence studies on 15GHz and 30GHz carrier frequencies also assumed much less antenna array elements, and different scenario (UMa) other than scenarios targeted in latest 6G deployment discussion.

In general for 6G radio, the previous co-ex studies performed in RAN4 can only provide very limited reference. And with the development of new scenarios for different carrier frequencies, larger antenna array size for 6G, different conducted power per sub-array/elements, different ISD/topology to each scenarios considered, the best way to understand the RF requirements to ensure the co-existence in adjacent channel is to perform new co-existence studies to these carrier frequencies.

Among all the candidate frequencies in [3], in our view, we can pick 7GHz with higher priority, given it has most major changes compared to previous studies in both deployment scenario and BS antenna assumption aspects.

Proposal 1: It is proposed to perform new co-existence studies to the carrier frequencies suggested for 6G Radio.

Proposal 2: The new 6G Radio co-existence study can start with 7GHz to re-evaluate ACLR and ACS given its significant difference to previous studies in both deployment scenario and BS antenna assumption aspects.

  - BS antenna array and modelling

From above Table 2.1-1 and Table 2.1-2, it can be observed that one major differences between [3] and [4]~[9] are the antenna array size of base station that assumed. In [3], the base station array size are captured in a high-level description way, which only mentions the upper bound of array size in different carrier frequencies, as follows:

BS antenna elements; Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements.

Where in the most recent study carried out from RAN4 in Rel-19 [7], for the 4GHz, 7GHz and 15GHz carrier frequencies, it concluded representable parameter set for BS antenna, including element gain, 3dB beamwidth, array size, max Tx power, etc. But in the same TR[7], the co-existence study was only performed for 15GHz carrier frequency, but the co-existence analysis were not performed for 4GHz and 7GHz carrier frequency. It was because there was legacy ACLR/ACS suggestions for 4GHz and 7GHz from previous analysis, and considering the tight timeline for the information delivery to ITU-R, those studies were skipped in Rel-19 in [7].

However, the co-existence study performed in [6] for 7GHz assumed a base station antenna with 43 dBm Tx power, 5.5 dBi element gain and 8x16x2 array for Urban Macro and Indoor Hotspot scenarios. The assumption in [6] was already different to what concluded as ”representable” array assumption in [7] for the same carrier freqeuncy, e.g. 7GHz. Where the representable array assumption for 7GHz Urban Macro is (8x16)x(3x1)x2.

Observatoin 5: For 7GHz, TR 38.921 studied co-ex with 8x16x2 array size for Urban Macro, TR 38.922 concluded (8x16)x(3x1)x2 array size for Urban Macro, while latest 6G deployment discussion suggested ”up to 2304” elements.

The figures below depicted a difference between [6] and [7] assumptions in 7GHz BS antenna. The difference will be significant larger if we consider larger array as 2304 elements.

Figure 2.2-1 Differences in antenna pattern between [6] and [7] for 7GHz

Left: Horizontal domain; Right: Vertical domain

The simulated antenna pattern from [6] and [7] simply reflects the difference in antenna gain in different directions of both horizontal and vertical domain, it does not consider the difference introduced by different Tx power. And yet, with a (3x1) sub-array assumed in [7], its side lobes in vertical domain already shows better suppression compared to [6]. With larger array size as documented in [3] in 7GHz, we are expecting larger difference in antenna pattern in new co-existence study.

Observation 6: With more elements considered, for example 3x1 sub-array in a same 8x16 antenna, the difference in antenna pattern is already significantly large. The 2304 elements proposed in latest 6G deployment discussion would introduce greater impacts in antenna pattern in new co-existence studies alone.

With above observations, we propose RAN4 to review the Rel-19 7GHz BS antenna assumption and together consider the agreements in [3], and carry out new co-existence study based on the outcomes to reflect changes in BS antenna in 7GHz carrier.

Proposal 3: RAN4 to review and discuss antenna assumptions in TR 38.921, 38.922 and RP-252888 for 7GHz, and to perform new co-existence study with the agreed typical antenna assumption for 7GHz.

In addition, in the Figure 2.2-1, the pattern from TR 38.922[7] considered a sub-array modelling, which was not usually considered in previous [4]~[9] co-existence studies in RAN4. With more and more antenna elements considered as expressed in [3], and a broader adoption of having sub-array assumed in the BS AAS antenna model in [7], we tend to suggest RAN4 to use sub-array based AAS model as a baseline assumption at least for 6G base station in 6G Radio co-existence study.

Proposal 4: In 6G Radio co-existence study, use sub-array based AAS model as a baseline assumption for 6G base stations.

  - Deployment scenario

In 6G Radio deployment scenario discussions, the current version from #109 [3] includes Indoor hotspot, Urban micro, Rural, Urban macro, Sub-urban macro. And if we continue to use 7GHz carrier as an example, the layouts for different scenarios are listed as in following table.

Table 2.3-1 Summary of layout and ISD in [3] and [6] for 7GHz carrier

| Carrier frequency | Deployment scenario [3] | Layout [3] | ISD [3] | TR 38.921[6] |
| --- | --- | --- | --- | --- |
| Around 7 GHz | Indoor hotspot | Indoor office 120m x 50m | TBD | Office: 120m x 50m. 20m ISD. |
|  | Urban micro | Single layer: Hex. Grid Two layers: - Macro Hex. Grid - Micro: Random drop | Macro: 200m Micro: 100m | N/A |
|  | Rural | Hex. Grid | 1732m 5000m | N/A |
|  | Urban macro | Single layer: Hex. Grid Two layers: - Macro Hex. Grid - Micro: Random drop | Macro: 500m TBD on Micro. | Hex. Grid. 450m ISD. |
|  | Sub-urban macro | Hex. Grid | 1299m 1732m | Hex. Grid. 900m ISD. |

It is worth noted that the TR 38.922 [7] does not include any deployment parameters for 7GHz, that is why it was not listed in Table 2.3-1 above. And from this table, we can have following observations.

Observation 7: The previous co-existence study in TR 38.921 does not study Urban micro, Rural scenarios for 7GHz carrier, which is listed in latest 6G deployment scenario discussion.

Observation 8: The previous assumptions for Urban macro in TR 38.921 assumed one layer of hexagonal grid macro stations, while 6G deployment scenario discussion suggested both one layer and two layer layouts. But two layers in latest 6G deployment discussion assumed different carriers in different layers.

Observation 9: The previous assumption for sub-urban macro ISD is 900m, which is much smaller than the assumed 1299 or 1732 meters ISD as discussed in latest 6G deployment scenario discussion.

From the above observations, together with the difference from BS parameters and antennas discussed in previous sections, the adjacent channel interference would be very much different under new 6G Radio co-existence study.

Proposal 5: RAN4 to consider the deployment related agreements (e.g. ISD, layout) from 6G deployment scenario discussion as the new 6G Radio co-existence study assumptions.

Moreover, the channel model have been further developed and updated in the Rel-19 study item “Study on channel modelling enhancements for 7-24GHz for NR”. In our view, the major changes to RAN4 co-ex study would be the introduction of Sub-urban scenario, which is also aligned with the 6G deployment discussions.

SMa In suburban macro-cells base stations are located above the surrounding environment to allow wide area coverage, and mobile stations are outdoors at street level and within commercial and residential buildings. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure. In suburban areas, vegetation is more prevalent than in urban areas with a high variability of foliage density across different suburban areas. Details on SMa scenarios are listed in Table 7.2-5.

The Sub-urban macro (SMa) related updates in [10] includes new assumptions of base station height, UE dropping and new channel models dedicated for SMa including probability and models of LoS and non-LoS models, etc. And the related contents can be found in section 7.2, 7.3 and 7.3 in TR 38.901 [10].

Observation 10: In updated TR 38.901 from Rel-19 7-24GHz channel model study, the sub-urban macro (SMa) was introduced, and aligned with the 6G deployment discussion.

Proposal 6: To adopt the updated channel model in TR 38.901, especially for SMa scenario, to the new 6G Radio co-existence study.

  - Key technologies and scenarios in 6G Radio

Duplex evolution (e.g. SBFD)

In earlier generations like HSPA and LTE, duplex mode employed TDD and FDD (also half duplex FDD). NR also introduced SBFD to address coverage issues typically associated with the high-frequency TDD bands. In 6GR, the use of the TDD bands above 6 GHz presents similar coverage challenges. Although carrier aggregation with low-frequency bands may provide coverage, low frequency band solutions are insufficient in terms of resource utilization and capacity, and require additional UE RF complexity. Therefore, SBFD should be supported from day one in 6GR in order to provide a standalone solution for operation in high frequency TDD bands. In particular, implementing macro cell SBFD for TDD bands above 6 GHz is vital for achieving comparable coverage to that of the 3.5 GHz band.

6GR SBFD should be enhanced SBFD by extending the existing single carrier operation to on/off 6G BS for multi-carrier, M-TRP, and NES. The feasibility of integration with functions that require SIC such as ISAC should also be studied. The term SBFD may also not be used and instead the more generic term of advanced duplex (AD) can be used to cover various scenarios that can be deployed in 6GR. In Table II, possible cases under the AD umbrella are listed. While single RU with multi-panel over a single carrier has been studied and specified in NR, for 6GR the AD support should be extended to other cases.

There are different study aspects for each case. For case A, a number of SIC and CLI impacts are identified in 7 GHz and motivate enhancing UE requirements. Case B is a multi-carrier extension of case A and similar analysis is required as in Case A. To use software based SBFD/AD operation in a network, integration with M-TRP/NES/Asymmetric M-TRP should also be considered. Latency requirements as well as CLI impacts should be considered together in Case C. Meanwhile, in Case D, using inter-band for SBFD/AD operation has specific issues to be identified. Since RX only uses low band frequency, TDD channel reciprocity cannot be used in TX panels. RX capability in upper band may be possible but requires additional cost without solving the UL coverage issue for SRS. None of the above cases alone can handle all different possibilities of real 6GR deployments and therefore the 6GR study should consider all above cases for 6GR coverage.

Table 2.4.1-1: Cases for Advanced duplex in 6GR

| Case | Scenario | Note |
| --- | --- | --- |
| A: single-carrier | One RU operating TX and RX panels simultaneously. | Same as NR SBFD |
| B: multi-carrier | One RU operating TX and RX panels for different carriers within same band simultaneously. | May have relaxed SIC requirement with wider filter than Case A |
| C: MTRP/NES/Asymmetric MTRP | One RU operating TX and the other RU operating RX within same or with different carriers simultaneously. | SIC is much relaxed than Case A and B and DU must handle both TRPs to avoid CLI. Due to NES operation, TX only or RX only TRPs are temporally available. |
| D: inter-band | One RU operation TX and RX panels with different band simultaneously. | For example, 3.5 or 7 GHz used in TX panel while <1 GHz in RX panel. SIC may be needed depending of band combination due to IMD. |

Meanwhile, while dynamic TDD was standardized in both LTE and NR, it has never been implemented practically and should be excluded at least from initial 6GR support.

In previous RAN4 studies, SBFD studied in TR 38.858 [8] and the Dynamic TDD studies performed in TR 38.828[11] were performed on 4 and 30 GHz carriers, and with the previous BS station antenna, layout, ISD, and other assumptions similar as TR 38.803[[5]. And which has been explained in previous sections as mis-aligned with the latest considerations for 6G Radio.

Figure 2.4.1-1 Interference modeling and considerations in TR 38.858 [8]

Observation 11: The SBFD and dynamic TDD studied prior to Rel-19 were performed under the assumption similar to TR 38.803, which has significant difference to the 6G considerations.

In Rel-20, there is an established new work item in RAN4 would focus on the adjacent channel co-existence between SBFD and SBFD operations. And given it overlaps with 6G Radio study, we believe the co-existence studies and its results in this WI should be leveraged as much as possible when considering the co-existence of SBFD in 6G Radio.

Proposal 7: The Rel-20 5GA SBFD-SBFD co-existence study and results should be leveraged as much as possible in considering co-ex of SBFD in 6G Radio co-ex related topics.

Non-terrestrial network (NTN)

For extensive coverage in areas without cellular networks, NR is increasingly relying on TN with NTN. NTN is capable of rapidly supplying basic services to remote areas and is applicable for a range of uses such as logistics, disaster management, and rescue through communication systems. Numerous TN operators have recently partnered with NTN operators and are now incorporating NTN capabilities into their services. Standards for IoT-NTN and NR-NTN are continuously evolving in LTE and NR, and it is anticipated that 6GR will support the respective functionalities within a single integrated framework.

In previous 5G NR co-existence study, the NR-NTN and IoT-NTN comes as a late comer in 3GPP environment and specifications. Thus, the co-existence studies were carried out in a way that most scenarios are conducted with the TN ACLR/ACS are fixed with existing requirements. In 6G new radio co-ex study, given NTN can be considered in early phase of developing the co-existence study and also the RF requirements for co-ex, it is possible that we can consider NTN scenario in 6G new radio co-ex study at relatively early phase and let its adjacent channel interference case to be considered in RF requirements for a harmonized TN/NTN structure/hardware.

Observation 12: In 5G NR/IoT NTN studies, the NR-NTN were introduced in a late phase where all TN requirements are ready, hence the interference transmitted and received from NTN stations are not fully addressed as the TN devices cannot be easily upgraded with new requirements.

Proposal 8: For 6G new radio co-existence study, it is possible for RAN4 to consider the NTN scenario in relatively early phase in co-ex study, and in determine requirements for better co-existence for 6G Radio on both NTN and TN.

Moreover, one of the key disputes in most NTN topics especially in Rel-19 is the isolation distance that RAN4 adopted in Rel-17 NR-NTN study in FR1-NTN carriers. In Rel-17, the assumption was that the RAN4 targeted on a common UE for both NTN and TN access. And with such target, RAN4 adopted an assumption that an UE connecting with NTN network will automatically connect to a TN network with a much better link conditions.

Isolation concepts in TR 38.863 [9]

However, in later releases, the concern from terrestrial operators is that the UE (even FR1 UE) may not be guaranteed can have the agreement between operators and even hardware support to be redirected or connect to a TN network. When that happens, the isolation distance assumption from the co-existence study will no longer be true, and so as the ACLR/ACS from the co-ex study may not guarantee the co-existence between NTN and TN operators.

In 6G Radio study, as 3GPP is pursuing a much more integrated network design for both NTN and TN, a much more general consideration for the NTN and TN co-existence may be conducted to study a more general co-existence case in adjacent channel between NTN and TN, and even NTN and NTN. Also the regulation from ITU-R and local/regional spectrum regulators could also have the limitation on the geographically overlapping of NTN and TN network in adjacent channel.

Proposal 9: The isolation distance between NTN and TN from Rel-17 should be re-evaluated when consider NTN in 6G Radio co-existence.

And, if we are targeting a more general 6G Radio co-existence with the existence of 6G NTN deployment in adjacent channel, the UE density of such 6G NTN network beam/cell may not be as many as the TN network assumptions. And it can also comes from the capacity limit of a SAN in downlink and uplink and in different beams.

Proposal 10: The UE density and its distribution in one and all beams/cell from a single SAN/satellite should be better modeled if we are targeting a more general overlapping co-ex between 6G NTN and TN.

Moreover, the NTN-NTN scenarios were not explicitly considered in RAN4 co-existence study, due to the facts that the interference and facilitation between NTN operators usually follows regulatory procedure, which is outside the 3GPP scope.

Observation 13: Previous RAN4 did not really study NTN-NTN co-existence, due to the facts that NTN-NTN interference management and negotiation usually follows regulatory framework and out of 3GPP scope.

But recently, many local/regional regulators files new rules to allow the trial to deploy the mobile service (MS) spectrum onto satellite for a better coverage purpose. Also, in ITU-R, there is an agenda item 1.13 study the regulatory framework on such activity. The candidate bands shown in the following table are from ITU-R discussion, and it shows a relatively broad impacts on both legacy TN and also NTN networks.

Table 2.4.2-1 FDD candidate bands in AI 1.13 discussion

| UL | DL |
| --- | --- |
| 807-849 | 852-894 |
| 880-915 | 925-960 |
| 832-862 | 791-821 |
| 698-716 776-798 | 716-746 746-768 |
| 698-748 | 753-803 |
| 1 427-1 470 | 1 475-1 518 |
| 1 920-1 980 | 2 110-2 170 |
| 1 710-1 785 | 1 805-1 880 |
| 1 850-1 920 | 1 930-2 000 |
| 1 710-1 780 | 2 110-2 180 |
| 2 000-2 020 | 2 180-2 200 |
| 2 305-2 320 | 2 345-2 360 |
| 2 500-2 570 | 2 620-2 690 |

With not clear regulations published at the moment, in our view, we have two possible directions at least. One is that we leave it totally to other organizations to define the requirements and then 3GPP define additional requirements to guarantee co-existence. And the other way is that 3GPP can also study the interference between NTN and TN, and even NTN and NTN to understand what RF requirements should be targeted.

Observation 14: With a more flexible trend of spectrum usage between NTN and TN, along with a common design purpose of 6G Radio for NTN and TN, the co-existence between NTN and TN, and even between NTN and NTN can be more critical than 5G.

Another concept in NTN for 6G is the consideration of a het-net structure between SAN in different orbits. The experience from previous TN network is that the performance and interference for a co-channel deployment of het-network would be very hard to optimize. However, if the operators interested in pursuing a deployment of het-net between different orbits in adjacent channel, the interference should be studied.

Observation 15: If het-net structure between different satellite orbits are pursued as a key deployment in 6G NTN, the interference between NTN and TN, NTN and TN may be also studied.

Integrated Sensing and Communication (ISAC)

Due to the added sensing service, the ISAC system may suffer from multiple kind of interference, including self-interference, neighbouring cell interference. The self-interference refers to the received undesired signal from the transmit antennas (working in either communication or sensing mode) to receive antennas (working in either communication or sensing mode) of the same sector in the same site, as is illustrated in Fig 2.1.3 (a). The neighbouring cell interference can be classified to two types, co-site inter-sector co-channel interference and inter-site TRP-TRP co-channel interference, which are shown in Fig. 2.1.3 (b), respectively. Generally, the existence of these interference signals would affect the sensing result and may undermine the performance of communication service. Thus, to figure out the influence of the interference, it is recommended RAN4 study the interference signal modelling and the corresponding impacts on the sensing and communication performance.

Figure 2.4.3-1 Left: (a) Demonstration of self-interference; Right: (b) Demonstration of neighbouring cell interference

Demonstration of self-interference			           Demonstration of neighbouring cell interference

- Observation 16: The interference suffered by ISAC system can be classified into two types: 1) self-interference, which can be defined as the received undesired signal from the transmit antennas of the same sector in the same site 2) neighboring cell interference, including 2.1) co-site inter-sector co-channel interference and 2.2) inter-site TRP-TRP co-channel interference, which can be defined as the received undesired signal from the transmit antennas of neighboring sector in the same site and the received undesired signal from the transmit antennas of neighboring sites, respectively.

Self Interference Modelling

The self-interference exists in multiple scenarios for ISAC systems. The following discussion will be expanded from the following two aspects: a) TDM case, b) FDM case.

（1）Time-division multiplexing case

Based on the discussion in section above, it can be derived that the sensing signals and the communication signals are completely isolated in the time division. Thus, there is no self-interference between the sensing service and communication service. However, for the TRP monostatic sensing mode, the sensing service requires full-duplex operation of TRPs. This full-duplex operation occurs when sensing signals are transmitted and received at the same time. In this scenario, the power of the transmitted signal can affect the receiver itself. A similar situation was studied in the Rel-18 duplex SI, where methodologies for modelling can be referenced. However, it is worth noting that the self-interference may not exist if a signal reception switch is adopted at the TRP, which means that the signal reception switch is on only if the sensing signal transmission is finished.

- Observation 17: When the ISAC system working in TDM mode, there is no self-interference between sensing and communication service. However, the self-interference between the sensing transmitter and the sensing receiver may exist under some circumstances.

（2）Frequency-division multiplexing case

For the FDM ISAC multiplexing case, due to the co-existence of sensing service and communication service at the frequency domain, the self-interference inevitably exists between the sensing signals and communication signals. In this case, the UL communication signal reception could be influenced by the transmitted sensing signal or reflected sensing signals. On the other hand, the sensing signal reception process could also be affected by either UL communication signals or the transmitted DL communication signals from BS. Further, the self-interference may also exist between the sensing transmitter and sensing receiver.

For the self-interference signal from the UL communication, the self-interference cancellation can be conducted by spatial isolation, frequency isolation, RF interference cancellation (IC), digital IC, etc. Companies can report the assumption of self-interference cancellation capability based on their own observations.

- Observation 18: When the ISAC system working in FDM mode, the self-interference exists between 1) the sensing receiver and communication transmitter, 2) the communication receiver and the sensing transmitter, as well as 3) the sensing transmitter and the sensing receiver.

Neighbouring Cell Interference Modeling

In real-world scenarios, sensing will be operated across multiple cells rather than a single cell. From a monostatic perspective, not only is self-interference encountered, but signals transmitted from other cells also acts as additional interference to the receiver, which is shown in Fig. 2.4.3-1(Right). Therefore, in this case, it is necessary to model the neighbouring cell interference and figure out the corresponding impact.

(1) co-site inter-sector co-channel interference

Receivers of each TRP responsible for different sectors in the same site may be affected by interference from sensing transmission signals from TRPs in neighbouring sectors. However, the propagation in these near-field areas is currently not considered in the existing channel model. Notice that the previous TR [6] has modelled interference in the same way as self-interference, which is that the transmission power passes through multiple isolation mechanisms and acts as interference at the receiver. Therefore, in this study, we can refer to this and apply noise boosting accordingly. However, since interference isolation methods such as antenna isolation and digital cancellation are implementation-dependent, each company can select feasible values, conduct evaluation based on them, and report the used values as assumptions.

(2) inter-site TRP-TRP co-channel interference

Another aspect of interference comes from sensing transmission signals emitted by TRPs located in different sites. This interference could have a critical impact, especially when the TRPs are facing each other. The inter-site TRP-TRP co-channel interference can be modelled using the same approach as the modelling of the bistatic background channel.

Influence of interference signals on sensing

Unlike the interference in communication system, the interference in sensing signal reception is less damaging. For the communication signal reception, the extra interference signal would lead to mis-detection of communication symbols or deteriorate channel decoding process, and thus result in poor transmission throughput / high bit error rate. However, for the sensing service, the interference signals may lead to saturation of RF components making sensing system inoperable. Otherwise, the interference can be regarded as an extra point/block in the doppler-delay diagram (sensing result). If the sensing devices have prior knowledge of this interference, the interference can be neglected in further sensing result analysis.

However, how to identify the interference and the desired target need to be further studied, and RAN4 could investigate the interference modelling and decides parameters (e.g., power strength, doppler, delay) /criteria to distinguish the interference signals from the desired target signal based on the sensing result.

- Proposal 11: RAN4 could investigate the interference modelling and decides basic criteria to distinguish the interference signals from the desired target signal based on the sensing result.

Considerations for 6G Base station RF requirements

Supporting FR2 in NR posed challenges due to significant pathloss, which reduced coverage. Analog beams were therefore implemented to enhance coverage. Efforts to combine the benefits of digital beams have further led to hybrid beamforming, integrating existing MIMO and beam technologies. In 6GR, exploring the optimal system structure for FR3, which is crucial for both 6G BS and UE antenna architecture, should be prioritized.

In 6GR FR3 band (7-24 GHz), 6G BS utilize energy-efficient antenna structures with up to 256 ports to support massive MIMO (NeX-MIMO). The subarray structure is essential to support 256 ports by utilizing more antenna elements but various aspects such as the performance and complexity of the power splitting network for the subarray network, resolution for phase array operation, and UE’s channel measurement complexity must be considered. Use of AI/ML for scalable CSI feedback reduction, dynamic spatial adaptation, and channel reconstruction, minimizing overhead while maximizing coverage should be studied.

Observation 19: The 6G BS would support 7-24 GHz carrier with potentially more ports and MIMO schemes, and likely in hybrid beamforming scheme.

For the carriers in new frequency range, a feasibility study together with co-ex study should be performed to determine the RF requirements for this new range. The question is on the carriers in legacy FR1 range, that whether the existing BS RF requirements can be fully adopted for 6G BS in FR1 range. We believe the this also depends on the feasibility and co-ex study from the FR3 6G BS.

Observation 20: The feasibility and co-ex studies on FR3 carriers, e.g. 7GHz, are the basis of 6G BS RF requirements for 6G Radio. And the study outcomes would also impact the RF requirements in the 6G BS RF requirements in FR1 carriers.

Proposal 12: The new 6G BS RF requirements should be discussed based on feasibility and new co-ex studies at least on 7GHz carrier.

Given the overlapping timeline, the RAN4 ongoing Rel-20 WI of BS RF evolution for FR1 and testing phase 2 [12] should be leveraged as much as possible. And the outcomes of this WI can be taken as starting point for the 6G BS requirements discussion.

Enhancement of co-location requirements - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

Especially considering on above Proposal 7, we propose to consider the outcomes of SBFD studies in Rel-19 and Rel-20[12], as the starting point for 6G BS requirement discussions.

Proposal 13: To be efficient and not duplicating work, the Rel-20 BS evolution WI outcomes, e.g. SBFD-capable gNB unwanted emission and Rx in-band blocking, can be taken as starting point for 6G BS requirements discussions in normative phase.

Once above discussions had been done, with a new set of 6G BS RF requirements on an example carrier/band (e.g. 7GHz), then we can discuss whether the requirements can be applied or scaled or expanded to other carriers in FR1 range.

Observation 21: The agreed 6G BS requirements from 7GHz could then be evaluated to check if they can be expanded to other carriers < 6GHz.

Another key technology/topic to be considered in 6G BS is energy efficiency topic, in which, RAN1 is discussing about to utilize part of the antenna elements and associated power amplifiers. The candidate 6GR spectrum such as around 7 GHz would make the spatial and power adaptation effective where the power usage scales with the number of antennas and associated power amplifiers. One of the potential solutions is depicted as follows:

Figure 3-1: Spatial and power adaptation (left), Joint network and UE energy efficiency (right)

By adopting the ‘energy saving mode’ as shown above, one thing that RAN4 should discuss is whether and how to apply RF requirements on the partially activated 6G BS. Especially, considering the beam would usually grow “fatter” when adopting less elements under such “energy saving mode”, and also the Tx power would be less, then whether different ACLR/ACS is needed for it could also be discussed.

Observation 22: It’s worth to discuss whether and how to apply RF requirements on “energy saving mode” when only part of the element arrays and their associated PA is activated.

For MSR requirements for 6G BS together with other RAT, we believe that the requirements architecture could be the same as TS 37.104 [13], where the applicability rule, band category, and the relationship between MSR and single-RAT requirements could remain the same.

Observation 23: In general, our observation is that the applicability rule, band category, and the relationship between MSR and single-RAT requirements from TS 37.104 could be re-used in introducing MSR requirements for 6G BS.

Conclusion

In this contribution, we proposed following for the 6G radio BS RF and co-existence:

Observation 1: The gaps between co-ex studies performed in RAN4 and 6GR target deployment scenarios in different carrier frequencies are quite large, and they are shown as below.

Observation 2: The previously available co-existence studies on 700MHz and 2GHz carrier frequencies are majorly from LTE study, which differs significantly with later 5G NR and the current 6G Radio in both system parameters and deployment scenarios.

Observation 3: The previously available co-existence studies on 4GHz and 7GHz carrier frequencies assumed much less antenna array elements and only partial of the deployment scenarios as given in latest 6G deployment discussion.

Observation 4: The previously available co-existence studies on 15GHz and 30GHz carrier frequencies also assumed much less antenna array elements, and different scenario (UMa) other than scenarios targeted in latest 6G deployment discussion.

Proposal 1: It is proposed to perform new co-existence studies to the carrier frequencies suggested for 6G Radio.

Proposal 2: The new 6G Radio co-existence study can start with 7GHz to re-evaluate ACLR and ACS given its significant difference to previous studies in both deployment scenario and BS antenna assumption aspects.

Observatoin 5: For 7GHz, TR 38.921 studied co-ex with 8x16x2 array size for Urban Macro, TR 38.922 concluded (8x16)x(3x1)x2 array size for Urban Macro, while latest 6G deployment discussion suggested ”up to 2304” elements.

Observation 6: With more elements considered, for example 3x1 sub-array in a same 8x16 antenna, the difference in antenna pattern is already significantly large. The 2304 elements proposed in latest 6G deployment discussion would introduce greater impacts in antenna pattern in new co-existence studies alone.

Proposal 3: RAN4 to review and discuss antenna assumptions in TR 38.921, 38.922 and RP-252888 for 7GHz, and to perform new co-existence study with the agreed typical antenna assumption for 7GHz.

Proposal 4: In 6G Radio co-existence study, use sub-array based AAS model as a baseline assumption for 6G base stations.

Observation 7: The previous co-existence study in TR 38.921 does not study Urban micro, Rural scenarios for 7GHz carrier, which is listed in latest 6G deployment scenario discussion.

Observation 8: The previous assumptions for Urban macro in TR 38.921 assumed one layer of hexagonal grid macro stations, while 6G deployment scenario discussion suggested both one layer and two layer layouts. But two layers in latest 6G deployment discussion assumed different carriers in different layers.

Observation 9: The previous assumption for sub-urban macro ISD is 900m, which is much smaller than the assumed 1299 or 1732 meters ISD as discussed in latest 6G deployment scenario discussion.

Proposal 5: RAN4 to consider the deployment related agreements (e.g. ISD, layout) from 6G deployment scenario discussion as the new 6G Radio co-existence study assumptions.

Observation 10: In updated TR 38.901 from Rel-19 7-24GHz channel model study, the sub-urban macro (SMa) was introduced, and aligned with the 6G deployment discussion.

Proposal 6: To adopt the updated channel model in TR 38.901, especially for SMa scenario, to the new 6G Radio co-existence study.

Observation 11: The SBFD and dynamic TDD studied prior to Rel-19 were performed under the assumption similar to TR 38.803, which has significant difference to the 6G considerations.

Proposal 7: The Rel-20 5GA SBFD-SBFD co-existence study and results should be leveraged as much as possible in considering co-ex of SBFD in 6G Radio co-ex related topics.

Observation 12: In 5G NR/IoT NTN studies, the NR-NTN were introduced in a late phase where all TN requirements are ready, hence the interference transmitted and received from NTN stations are not fully addressed as the TN devices cannot be easily upgraded with new requirements.

Proposal 8: For 6G new radio co-existence study, it is possible for RAN4 to consider the NTN scenario in relatively early phase in co-ex study, and in determine requirements for better co-existence for 6G Radio on both NTN and TN.

Proposal 9: The isolation distance between NTN and TN from Rel-17 should be re-evaluated when consider NTN in 6G Radio co-existence.

Proposal 10: The UE density and its distribution in one and all beams/cell from a single SAN/satellite should be better modeled if we are targeting a more general overlapping co-ex between 6G NTN and TN.

Observation 13: Previous RAN4 did not really study NTN-NTN co-existence, due to the facts that NTN-NTN interference management and negotiation usually follows regulatory framework and out of 3GPP scope.

Observation 14: With a more flexible trend of spectrum usage between NTN and TN, along with a common design purpose of 6G Radio for NTN and TN, the co-existence between NTN and TN, and even between NTN and NTN can be more critical than 5G.

Observation 15: If het-net structure between different satellite orbits are pursued as a key deployment in 6G NTN, the interference between NTN and TN, NTN and TN may be also studied.

Observation 16: The interference suffered by ISAC system can be classified into two types: 1) self-interference, which can be defined as the received undesired signal from the transmit antennas of the same sector in the same site 2) neighboring cell interference, including 2.1) co-site inter-sector co-channel interference and 2.2) inter-site TRP-TRP co-channel interference, which can be defined as the received undesired signal from the transmit antennas of neighboring sector in the same site and the received undesired signal from the transmit antennas of neighboring sites, respectively.

Observation 17: When the ISAC system working in TDM mode, there is no self-interference between sensing and communication service. However, the self-interference between the sensing transmitter and the sensing receiver may exist under some circumstances.

Observation 18: When the ISAC system working in FDM mode, the self-interference exists between 1) the sensing receiver and communication transmitter, 2) the communication receiver and the sensing transmitter, as well as 3) the sensing transmitter and the sensing receiver.

Proposal 11: RAN4 could investigate the interference modelling and decides basic criteria to distinguish the interference signals from the desired target signal based on the sensing result.

Observation 19: The 6G BS would support 7-24 GHz carrier with potentially more ports and MIMO schemes, and likely in hybrid beamforming scheme.

Observation 20: The feasibility and co-ex studies on FR3 carriers, e.g. 7GHz, are the basis of 6G BS RF requirements for 6G Radio. And the study outcomes would also impact the RF requirements in the 6G BS RF requirements in FR1 carriers.

Proposal 12: The new 6G BS RF requirements should be discussed based on feasibility and new co-ex studies at least on 7GHz carrier.

Proposal 13: To be efficient and not duplicating work, the Rel-20 BS evolution WI outcomes, e.g. SBFD-capable gNB unwanted emission and Rx in-band blocking, can be taken as starting point for 6G BS requirements discussions in normative phase.

Observation 21: The agreed 6G BS requirements from 7GHz could then be evaluated to check if they can be expanded to other carriers < 6GHz.

Observation 22: It’s worth to discuss whether and how to apply RF requirements on “energy saving mode” when only part of the element arrays and their associated PA is activated.

Observation 23: In general, our observation is that the applicability rule, band category, and the relationship between MSR and single-RAT requirements from TS 37.104 could be re-used in introducing MSR requirements for 6G BS.

Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

[2] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair (Apple)

[3] RP-252888, pCR on Deployment scenarios for TR 38.914, Deutsche Telekom

[4] TR 36.942, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios, 3GPP

[5] TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects, 3GPP

[6] TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz, 3GPP

[7] TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz, 3GPP

[8] TR 38.858, Study on Evolution of NR Duplex Operation, 3GPP

[9] TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects, 3GPP

[10] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, 3GPP

[11] TR 38.828, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR, 3GPP

[12] RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, 3GPP

[13] TS 37.104, NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception, 3GPP