3GPP TSG-RAN WG4 Meeting #116bis	R4-2513059

# Prague, Czech Republic, Oct. 13-17, 2025

Source: 	China Telecom

Title: 	Discussion on 6G spectrum

Agenda Item:	8.6

Document for:	Approval

Introduction

In the RAN#108 meeting, the new SID [1]: Study on 6G Radio was agreed, which aims to develop one non-backward compatible radio access technology. Then, in the later RAN4#116 meeting, the 6G Plan and Meeting Arrangement was proposed by RAN4 chair [2] to give the 6G RAN4 topics. This contribution provides our views on one of topics, i.e. spectrum.

Discussion

2.1 Candidate 6G spectrum: 6GHz

In China, On May 23, 2023, the Ministry of Industry and Information Technology issued the new version of the "Radio Frequency Allocation Regulations of the People's Republic of China." In this revision of the regulations, all or part of the 6425-7125 MHz frequency band is allocated for IMT systems.

5G has specified the 6GHz (6425 MHz - 7125 MHz) as Band n104, but it is not deployed in commercial 5G networks.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n10417,18 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |

Thus, it is proposed to introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

Proposal 1: Introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

2.2 Band combination simplification

Up to ten thousands of band combinations are proposed in RAN4 to work on the requirements specification. MSD requirement for each combination needs to be analysis case by case. Now the MSD simplification work is under discussion for HPUE combinations targeting at introduction of unified Equations/LUT/Simplified guidelines to replace the existing method using HPUE MSD tables at the end of Rel-19 or earlier if agreed. The main idea is using a delta MSD relative to the default power class 3 MSD, while keeping the current procedure of PC3 band combination unchanged.

In our view, the work of band combinations for all the power classes including PC3 need to be improved for 6G. Classify the bands to be several of groups based on frequency ranges such as low/media/high etc., and then specify the MSD requirements based on band groups, which could simplify the CA requirements dramatically.

E.g., Define the group1 as combination of low+media+high+superhigh without MSD, in which to study the specific range or band lists of low, media, high and superhigh.

Another example is to define the group 2 as combination of low+media+high+superhigh with same MSD requirements, in which to study the specific range or band lists of e.g. 2nd harmonic of low fall into media.

Based on the CA groups, when introducing new band, just first checking if can be captured to the existing band list. If not, then the specific combination may be introduced except of CA groups.

Proposal 2: Define the band combinations as several groups based on different type of MSD requirements, each group includes band lists and share the same requirements.

Conclusion

In this contribution, we give our proposals on 6G new band request and band combination simplification.

Proposal 1: Introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

Proposal 2: Define the band combinations as several groups based on different type of MSD requirements, each group includes band lists and share the same requirements.

- References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (moderator)

- R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, RAN4 Chair (Apple)