3GPP TSG-RAN WG4 #116BIS Meeting							   R4-2513077

Prague, Czech, 13th – 17th October 2025

Agenda Item: 	8.13

Source: 	MediaTek Inc.

Title: Views on 6G spec improvements

Document for: Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G spec improvements with the focus on the following aspects:

- General issues

- RF spec

- RRM spec

# 2	General issues

<Mapping between feature and requirement & test case>

Current RAN4 spec is written based on the individual requirements (e.g., Rx/Tx in RF sepc, or idle/CONNECTED in RRM spec). However, when RAN4 introduces new requirements, it is usually based on a certain feature. For example, in order to implement UE which complies to all Tx switching requirements, we need to check

- clause 5.2A.2 of TS 38.101-1 for whether DL interruption for inter-band CA is allowed,

- clause 6.3A.3.3 of TS 38.101-1 for ON/OFF time mask,

- clause 8.2 of TS 38.133 for interruption requirements and

- clause A.6.5.7 of TS 38.133 for test cases for interruptions

Observation 1: One single feature may have requirements at multiple clauses and across multiple specs.

For more complex features, how to identify the requirements could be even more difficult. For example, Rel-16 NR-U has a big impact in TS 38.133. Many new clauses were introduced, such as “5.1A Cell Re-selection with CCA” and “6.1B Handover to target cell using CCA”. (The clause suffices are not aligned.) Furthermore, there is no new clause of UL Tx timing requirements for NR-U, because RAN4 agreed that Rel-15 requirements can be directly re-used. In our view, the spec is not friendly to people who did not participate in 3GPP discussions.

Observation 2: For the same feature, RAN4 may agree to introduce new clause for one requirement and re-use legacy clause for another requirement.

There are more features which do not even have the corresponding RAN4 requirements. (This does not mean the feature is incomplete.). However, it is not very easy to directly understand whether the feature has a RAN4 requirements, and (if yes) where they are. Therefore, we suggest RAN4 to study how to document whether a feature has RAN4 requirements and where to find them in 6G.

Proposal 1: RAN4 to study how to document whether a feature has RAN4 requirement and test cases as well as where to find them.

<Meeting schedule and CR quality>

In RAN4 meeting, the sessions are arranged in the way that first round discussions happen during Monday to Tuesday. During 1st round discussions, unresolved open issues will be later treated in the 2nd or 3rd round. Usually, controversial open issues are settled on Friday or even in the afternoon, leading to insufficient time for CR drafting and review. As a result, the CR quality is not good enough. Delegates need to submit CRs in the next meeting to correct errors, which increases the workload in both Tdoc drafting and reviewing. In our view, it can be considered to arrange the final round discussion on open issues slightly early, e.g., on Wednesday or Thursday. So that delegates can have more than 1 day to work on CRs.

Except above potential in-meeting arrangement, there could also be some post-meeting arrangement that can help to improve CR quality. For example, formal CR will be “endorsed” in this meeting. Then the endorsed CR will be further check until next Tuesday by NWM flagging process in the post RAN4 Meeting. The CR will be agreed in the next Wednesday if CR do not have any NWM flagging in the post RAN4 Meeting. If CR is flagged by some companies, it can be updated and clearly clarified until Thursday. The specification editor, moderator or the rapporteur can help to prepare NWM flagging process and clarification with CR proponents.

Proposal 2: RAN4 to consider new in-meeting and post-meeting arrangement to provide sufficient time for CR drafting and review.

# 3	RF spec

In the 5G TS38.101-1 specification, RAN4 specified the multiple features as follow

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |

However, this is quite difficult to understand and how to apply the real Tx/Rx RF requirements for individual vertical/mixed feature supporting UE.

To simplify the specification structure in 6G, RAN4 can consider single carrier for minimum requirements perspectives. And RAN4 just specify the 2Tx/CA/DC/DL-UL decoupling/1Tx RF requirements with suffix. Other vertical device requirements (e.g., RedCap, NTN, ATG, UAV, … ) can be specified in other spec.

Based on this observation, we propose as follow

Proposal 3: RAN4 can define UE RF requirements for single carrier as baseline for minimum requirements and specifies the 2Tx/CA/DC/DL-UL decoupling/1Tx RF requirements with Suffix.

Proposal 4: For vertical device requirements (i.e., Vehicle Device, RedCap, NTN, ATG, UAV, …), RAN4 can consider whether to specify the corresponding requirements in different specs.

Another aspect related to RF spec improvement is the potential band combo simplification to specify the CA/DC UL/DL configuration, e.g., the multiple carrier cell concept.

Proposal 5: When discussing RF spec improvement, RAN4 needs to consider the progress in band combo simplification.

# 4	RRM spec

<Leverage conclusions of previous discussions>

In NR, the RRM spec improvement was discussed for several meetings with some agreement reached. For examples, some agreements in [2] were captured below

Agreement: Adopt RAN2 pseudo-code approach for selected Rel-19 features with independent clauses. - The exact features and applicable spec version is TBD. - Subject to further check, it can be further decided when and how to continue. Recommended drafting principles: - Put conditions before requirements. - When both new and legacy requirements are to be put in the same hierarchy level, and the order can be decided later. e.g. If (condition of new requirements) New requirements Else if (condition of legacy requirement) Legacy requirements Or If (condition of legacy requirements) legacy requirements Else if (condition of new requirement) New requirements - For requirements with multiple applicable conditions, strive to start the paragraph with UE capability, if applicable, and providing all the remaining conditions in a list.

In our view, above agreements should be treated as a starting point for 6G spec.

Proposal 6: The agreements in R4-2420107 should be treated as a starting point for 6G RRM spec.

<Problem for requirement duplication>

In current TS 38.133, there are a lot of duplication among sections, such as IDLE mode, calculation of Kp, scheduling restriction, measurement restriction, … . In some cases, 2 sections are almost identical to each other with the only difference on the target procedure (e.g, RLM, BFD or CBD), or the RS (SSB or CSI-RS). In some cases, 2 sections are very different for some technical reason, e.g.,

- In “9.2.5.3 Scheduling availability of UE during intra-frequency measurements”, scheduling restriction in TDD band in FR1 is introduced when UL and DL collision occurs.

- In “ 8.1.7 Scheduling availability of UE during radio link monitoring”, scheduling restriction in TDD band in FR1 is not needed.

However, as 3GPP evolved release by release with delegates left and new delegates came, the very original reasons resulting to this difference may become unclear/unknown to the group. For example, we have 5 clauses for intra-frequency measurements in TS 38.133, as below:

- 9.2	NR intra-frequency measurements

- 9.2A	NR intra-frequency measurements with CCA

- 9.2B	NR intra-frequency measurements for RedCap

- 9.2C	NR intra-frequency measurements for SAN

- 9.2D	NR intra-frequency measurements for ATG

Among these sections, many paragraphs are simple copy-and pasted from the original one without suffix. However, there are also many necessary changes identified for new features. It becomes difficult to identify whether a certain difference between the 2 sections is due to an editorial error or a technical reason. To clarify this, RAN4 usually needs to spend a lot of time finding the answer (if there is). Sometimes companies submitted CRs to correct those error they found, but they later realized that those were not error in fact. Or in some cases, companies only corrected the error in the original clause without suffix, but did not fix the same error in other clauses. As the spec becomes more and more complicated with a lot of features involved, the problem becomes more and more serious and more and more delegate effort/time are wasted.

Observation 3: The duplicated requirement leads to confusion and unnecessary extra effort for spec maintenance.

One quick thinking to mitigate this issue is to create a certain template which can be leveraged by multiple requirements with simple changes. For example, in below figure, we copy and paste the calculation of Kp from clause 9.2.5.1 (Intra-frequency cell identification) and Kgap from clause 9.3.4 (Inter-frequency measurement with measurement gaps) with the common parts marked in gray. With such a simple gray shadowing, it is very simple to identify the difference between the 2 paragraphs. Furthermore, it helps people to know whether the difference is an editorial issue or a technical issue. For example, in our view, the yellow-highlighted part are clearly editorial difference, and the green-highlighted are technical difference. For editorial differences, RAN4 can try to align them as much as possible. So that in the end all differences are due to technical considerations.

| - Kp from clause 9.2.5.1 Intra-frequency cell identification | - Kgap from clause 9.3.4 Inter-frequency measurement with measurement gaps |
| --- | --- |

The gray shadow used above is just a very simple example. We are open to learn more ideas to help to reduce the confusions due to requirement duplication.

Proposal 7: RAN4 to study methods to differentiate whether the difference between 2 requirements are due to editorial issue or technical issue.

# 5	Summary

In this paper, we provide our views on RAN4 spec improvement. We have the following observations and conclusions.

< General issues >

Observation 1: One single feature may have requirements at multiple clauses and across multiple specs.

Observation 2: For the same feature, RAN4 may agree to introduce new clause for one requirement and re-use legacy clause for another requirement.

Proposal 1: RAN4 to study how to document whether a feature has RAN4 requirement and test cases as well as where to find them.

Proposal 2: RAN4 to consider new in-meeting and post-meeting arrangement to provide sufficient time for CR drafting and review.

<RF spec>

Proposal 3: RAN4 can define UE RF requirements for single carrier as baseline for minimum requirements and specifies the 2Tx/CA/DC/DL-UL decoupling/1Tx RF requirements with Suffix.

Proposal 4: For vertical device requirements (i.e., Vehicle Device, RedCap, NTN, ATG, UAV, …), RAN4 can consider whether to specify the corresponding requirements in different specs.

Proposal 5: When discussing RF spec improvement, RAN4 needs to consider the progress in band combo simplification.

<RRM spec>

Proposal 6: The agreements in R4-2420107 should be treated as a starting point for 6G RRM spec.

Observation 3: The duplicated requirement leads to confusion and unnecessary extra effort for spec maintenance.

Proposal 7: RAN4 to study methods to differentiate whether the difference between 2 requirements are due to editorial issue or technical issue.

# 6	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2420107, WF on RRM_Spec_Improvement, Apple