3GPP TSG-RAN WG4 Meeting # 116bis                                      R4-2513079

Prague, CZ, 13th – 17th Oct, 2025

Agenda Item: 8.9

Source: MediaTek Inc.

Title: Views on 6G AI/ML

Document for: Discussion

- Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. According to the 6G SI, the study on AI/ML includes:

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this paper, we provide our views on the 6G AI/ML.

- Selection principle of AI/ML use cases

6G AI/ML study in Rel-20 is expected to select use cases with compelling trade-off at least between performance and complexity. We believe that there will be various use cases proposed by companies. Before starting to study those use cases, we think RAN4 should discuss the principles of selecting target use cases.

To avoid spending lots of efforts on use cases with minor or no gain, the cardinal principle should be prioritizing the use cases that have system gain. For some use cases, the gain may be straightforward, e.g., measurement delay can be reduced. Some may need the evaluation results to show the gain, e.g., use cases to improve the demodulation performance. In the evaluation, SLS performance evaluations are important. Just as an example, if one use case can enhance the performance for all modulation mode, while another can get benefit only for 256QAM or 1024QAM, we may prioritize the former as the scenario that can use 256QAM or 1024QAM is limited.

Proposal 1: Prioritize use cases with system gain, e.g., the gain is straightforward or showed by evaluation results.

RAN4 defines the minimum requirements of features, not how UE implements those features. We don’t need to differentiate whether further optimization is through AI or non-AI method. There may be some optimization that can be up to UE/NW implementation. In other words, some use cases may be supported without specification. So we suggest the proponent of each use case to analyze the part to be specified too, in order to show the necessity to specify.

Proposal 2: Prioritize use cases with clear spec impact and clear differentiation to non-AI based performance.

Checking the proposals in RAN1#122, it can be observed that there are many proposals to support the ongoing NR use cases in 6G from Day-1. Considering the status of NR use cases, it is still too early to reach such a conclusion. Take AI/ML BM as an example, how to define the requirements and how to test are still in discussion and evaluation. If it turns out that the gain is marginal after considering the impairments in the real field (e.g., prediction accuracy with measurement error, impact to non-AI UE due to skipping SSB transmission), we don’t think it is necessary to have requirement in 6G for some sub-use cases. In addition, new ideas are springing up in 6G for overhead reduction, performance improvement and energy efficiency, with AI or without AI. Those 5G use cases may be outperformed by new 6G ideas or may bring less benefit under different 6G system design. Therefore, whether the use cases can achieve similar gain in 6G as in 5G may need further evaluation.

Proposal 3: 5G use cases are not guaranteed to have requirements introduced to 6G. Still need to check the gain, feasibility, complexity/cost under 6G assumptions.

- Views on some use cases

3.1 AI based DPD/DPoD

We observed that there were some proposals on AI based DPD/DPoD in RAN1#122[2] to solve PA non-linearity issue. As RAN4 is more expertized in PA compared to RAN1, if to further study this use case, it should be led by RAN4.

Proposal 4: If to study AI based DPD/DPoD, RAN4 shall take the lead.

Usually, digital pre-distortion (DPD) is adopted at transmitter to compensate PA non-linearity. We have done some research on UE side AI based DPD. Due to there are too many dimensions to consider, such as sub-bands, CBW, RB allocation etc., it turns out that it is difficult to get a generic model. With acceptable model size and inference delay, the gain is quite marginal compared to non-AI DPD solutions. In addition, if to adopt AI based DPD, the major impact on spec would be to define a smaller MPR. As RAN4 only defines minimum requirement, UE can always do better, no matter adopting AI solution or non-AI solution. Without specification, UE can still use AI based DPD. 6G Day-1 should focus on the features that require specification. So we propose not to study AI based DPD at UE.

Observation 1: UE is still allowed to use AI based DPD even not specifying the feature.

Proposal 5: Not to study AI based DPD at UE.

On the other hand, at NW side, BS can use AI based digital post-distortion to recover the signal, even if there is some nonlinear distortion at Tx side. To get gain from the feature, NW needs to indicate to allow distortion at UE transmission. So at least another set of RF requirements, e.g., relaxed EVM, and some necessary signalling should be specified. Although how network collects data and some generalization (to different PA, UE implementation) are not clear, we are open to study in 6G SI.

Proposal 6: Open to study AI based DPoD at BS.

3.2 AI based RRM

Usually, sufficient progress in RAN1 or RAN2 is needed to carry out effective discussions on RRM in RAN4. At this stage, it may be better to focus on pure RAN4 RRM use cases which are independent to RAN1/2 designs. We think Rx beam sweeping reduction and smart measurement scheduling are of this kind.

AI based Rx beam sweeping reduction

AI based Rx beam sweeping reduction is targeting for FR2. The main idea is to measure with only a subset of UE Rx beams and predict RSRP of the other used Rx beams or RSRP of the best Rx beam & the corresponding Rx beam ID. If to support this feature, RAN4 may define new measurement delay requirements and predicted RSRP accuracy requirements. We think testability of this feature should be studied at first. For AI/ML BM, there are multiple AoA and single AoA test method on table. At least, we think single AoA can be workable for AI/ML BM. But for AI based Rx beam sweeping reduction, we don’t think it is possible to use single AoA test system. We are also not sure whether the multiple AoA test system in discussion in AI/ML BM is workable here, e.g., if the signals are always transmitted from the same 4 AoAs, how do we know UE passes the test case via the help of AI or by just simply conducting measurements only with Rx beams pointing to the 4 AoAs.

Proposal 7: Before starting the work on AI based Rx beam sweeping reduction, testability of this feature should be studied at first.

AI-assisted measurement scheduling

It is observed by our PRD team that UE may not detect any cell on some configured inter-frequency in the field. The scenario may be as shown in Fig.1. f2 is deployed for hot spots. f1 is deployed for coverage. There are some areas that f2 is not covered. But NW has no information of UE location. NW configures inter-frequency measurement on f2 regardless UE location. According to current requirements, even UE is not in the coverage area of f2, UE still needs to measure f2 periodically. The overall measurement delay is extended unnecessarily.

With the help of AI, UE could be able to identify whether it is in the coverage of f2 by simply measuring f1. With such a knowledge, UE can choose not to measure f2 or measure f2 less frequently. Instead, UE can measure other inter-frequencies more frequently or save some power by skipping some measurements. Such an AI-assisted measurement scheduling at UE side can help UE to smartly spend its measurement efforts on more important frequency layers, i.e., which frequency layers to measure more, which to measure less. In our view, it would in the end enhance the mobility performance.

Fig.1

Observation 2: It is beneficial to allow AI-assisted measurement scheduling at UE side.

We understand one key issue to specify AI-assisted measurement scheduling is how to define a propriate delay requirement, allowing enough UE flexibility and meeting mobility needs. We are open to have more discussions.

- Summary

In this paper, we provide some views on 6G RAN4 AI/ML use cases. We have the following proposals and observation:

Proposal 1: Prioritize use cases with system gain, e.g., the gain is straightforward or showed by evaluation results.

Proposal 2: Prioritize use cases with clear spec impact and clear differentiation to non-AI based performance.

Proposal 3: 5G use cases are not guaranteed to have requirements introduced to 6G. Still need to check the gain, feasibility, complexity/cost under 6G assumptions.

Proposal 4: If to study AI based DPD/DPoD, RAN4 shall take the lead.

Observation 1: UE is still allowed to use AI based DPD even not specifying the feature.

Proposal 5: Not to study AI based DPD at UE.

Proposal 6: Open to study AI based DPoD at BS.

Proposal 7: Before starting the work on AI based Rx beam sweeping reduction, testability of this feature should be studied at first.

Observation 2: It is beneficial to allow AI-assisted measurement scheduling at UE side.

4 Reference

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R1-2506455	Moderator summary #1 on AI/ML for 6GR	 Samsung (Moderator)