3GPP TSG-RAN WG4 Meeting #116bis		                         R4-2513089

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	Discussion on remaining issues of core requirements maintenance for RRM Phase 5

Source: 	OPPO

Agenda item:	4.15.1

Document for:	Approval

Introduction

In last meeting, the core requirements for RRM phase 5 were concluded and the core part is completed. But there are still some remaining issues for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor and fast SCell activation with Rel-18 EMR reporting which were captured in the approved WF [1].

In this contribution, we will provide our views on the remaining issues and give our proposals.

Discussion

- Issue 1-1-1: Value for T1 Agreement: - A fixed value of 90 seconds. - Issue 1-1-2: Re-evaluation mechanism Agreement: FFS following options in the maintenance stage: - Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be: - Alt 1: Use the values for T1 as a starting point. - Alt 2: 5 s, which is predefined in RAN4 spec. - Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS. - Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS.

In previous meeting, it was agreed to introduce timer-based solution for quit condition of FBS, i.e., UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition. And in last meeting, the T1 is agreed as a fixed value 90s. The remaining part is the value of T2 for re-evaluation.

Initially we think the timer-based solution can be used, i.e., the timer starts after UE quits from Multi-Rx mode and UE re-evaluates the activation condition after the timer expires. Because the mainly concerned case is when the condition is always met but UE has power consumption or overheating issue. In this case, UE is not required to re-enter the FBS mode again. But considering UE is not mandated to quit the FBS mode when T1 expires and UE usually will continuously evaluate the condition no matter it is during the FBS mode or not, we are OK to not define the value of T2.

Proposal 1: Do not define T2 for re-evaluation after UE quits from FBS mode.

- Issue 1-1-4: Requirements when UE reports different BSF values for different bands Way forward: Further discuss this issue in the maintenance stage: - Option 1: - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured. - Issue 1-1-5: Revision of the agreement on applicability requirement Agreement in Tuesday ad-hoc: - The FBS enhancement applies for the case when the target and serving carrier frequency are same. - FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different. Agreement in Thursday online: When the target and serving carrier frequency are different, - FBS requirement does not apply to inter-frequency measurement without measurement gap. - FFS inter-frequency measurement with per-FR measurement gap. - Note: inter-RAT is not in the scope.

According to the agreed bigCR and above agreement, the requirements for FBS only apply when only one target band is configured for L3 SSB measurement, so we do not need to discuss the case when multiple bands are involved and UE reports different BSF values for different bands.

For inter-frequency measurement, it was agreed the requirements only apply to the measurement with gap. In such case, we think it is not relevant to whether it is a per UE gap or per-FR gap since the UE measurement behavior is same for FR2 band.

Proposal 2: Do not define the requirements when multiple bands are involved and UE reports different BSF values for different bands.

Proposal 3: When the target and serving carrier frequency are different, the requirements apply to measurement with gap including per-FR gap and Per-UE gap.

- Issue 2-1-1: UE capability for EMR-based fast SCell activation Agreement in Friday online: - Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

In last meeting, it was agreed to introduce one single UE capability for R19 SCell activation based on EMR, i.e., not differentiate MAC CE based SCell activation or direct SCell activation.

For the details of this UE capability, the table discussed in last meeting can be used as baseline, and one thing to be discussed is the prerequisite feature groups. Based on the agreed bigCR, the requirements apply to the following three configurations which can be the prerequisite condition, i.e., one of measValidationReportEMR or measValidationReportReselectionMeasurements or idleInactiveNR-MeasReport-r16 is supported.

Requirements in this clause apply if the SCell is unknown, provided: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode,

Proposal 4: The feature group for R19 SCell activation based on EMR is defined as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on early measurement report | Supports fast SCell activation specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16) | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A |

Conclusions

In this paper, we provided our views on remaining issues for core requirements of RRM phase 5. And the following proposals are provided.

Proposal 1: Do not define T2 for re-evaluation after UE quits from FBS mode.

Proposal 2: Do not define the requirements when multiple bands are involved and UE reports different BSF values for different bands.

Proposal 3: When the target and serving carrier frequency are different, the requirements apply to measurement with gap including per-FR gap and Per-UE gap.

Proposal 4: The feature group for R19 SCell activation based on EMR is defined as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on early measurement report | Supports fast SCell activation specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16) | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A |

Reference

- R4-2512125 WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2509055 Topic summary for [116][212] NR_RRM_Ph5_Part2, CATT

- R4-2512348 Big CR for NR Radio Resource Management (RRM) Phase 5