3GPP TSG-RAN WG4 Meeting #116bis		 												R4-2513102

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	Maintenance for NR mobility enhancements Phase 4

Source: 	OPPO

Agenda item:	4.23.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on maintenance for NR mobility enhancements Phase 4.

Discussion

Event triggered L1 measurement reporting

- Issue 2-2: whether to introduce/update requirements for reporting criteria per measurement category Agreement: - No consensus on the necessity to introduce/update requirements for reporting criteria per measurement category. - The issue does not impact the WI core part completion, and interested companies can further discuss this issue in maintenance phase based on contribution driven.

About whether to introduce/update requirements for reporting criteria per measurement category, several candidate options were discussed. Most companies can compromise to not define new requirements, while concerned by 3 companies. They proposed, if new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM were considered, to introduce reported UE capabilities for supported number of events triggered L1 measurement reporting, as list below:

- Option 1a: {4, 8, 12, 16} in total, one event can be associated to different cells, the different cells can be on different frequency layers

- Option 1b: 4 per frequency layer

- Option 1c: 72 in total, can configure multiple cells

One concern is that if  the number is not defined per cell or per frequency layer but defined in total, it is quite hard to judge the usage of reporting criteria, like intra and inter frequency,  which is more related to UE and network implementation. In this case, it is difficult to judge which number are reasonable.

Observation 1: It is quite hard to converge to a number for event triggered L1 reporting criteria.

In previous meetings, we propose to extend existing Ecat to support LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events, to include L1 reporting criteria per measurement category into current Ecat.

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

Considering the new number separately for L1 reporting criteria, or the existing number shared by L1 and L3 reporting criteria are both difficult to convince all companies, we can also compromise to  not introduce any requirements for reporting criteria for event triggered L1 reporting.

Proposal 1: Compromise to not define any new requirements for event triggered L1 reporting criteria.

Conclusion

In the contribution we provide the following proposals:

Observation 1: It is quite hard to converge to a number for event triggered L1 reporting criteria.

Proposal 1: Compromise to not define any new requirements for event triggered L1 reporting criteria.

Reference

- R4-2512141, WF on NR mobility enhancements Phase 4_part1