3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513114

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Inputs to RAN4 6G study on system parameters

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.3 system parameters [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#109, the RAN led 6G Study objectives were updated in [1] adding the RAN4 related topics. In this contribution, we share our views on system parameters.

# Discussion

## 2.1 6G SI objectives relevant to UE RF

The portion of the 6G SI objective most relevant to the UE RF requirements is reproduced below in italics and highlighted in yellow.

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

- Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

- Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

6GR core and performance requirements

- General RF aspects [RAN4]

- Intra-3GPP co-existence studies

- Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

- BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

- Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

- UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

- Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

- RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

- Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

- Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

- Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

- Handling irregular channel bandwidths including the definition [RAN4, RAN1]

- Definition of ‘frequency range(s)’[RAN4]

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

- PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

- Aspects of integration with communication services [RAN1]

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In this contribution, we will cover the system parameters per the RAN4 chairman note associated with the corresponding agenda item:

NOTE: The scope includes waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. The aspects related to the interim milestone should be prioritized.

## 2.2 CBW, FFT and numerology for 6G

Some of the limitation and source of complexity in 5G definition of CBW resulted in additional work and less optimum spectrum support:

- Intra-band contiguous CA is required to support some operator’s spectrum:

  - n41 in the US and China, n77 in the US, n104 in China

- No real use of BW parts in the specification

- Additional work to support irregular CBW and too large of a set of CBW to be tested while not necessarily being deployed

- 3MHz, 6MHz, and 7MHz “odd” CBW require additional work.

- Regular CBW almost every 5MHz

- Use of multi-SCS in a band while dynamic TDD can’t be supported.

It is thus important that both wider channel BW and flexible BW are supported without requiring the UE to be measured for many BW that may not be deployed.

Assuming 4/8K FFT support and 2x the FR1 maximum CBW, the addressable BW and required number of RBs versus numerology is presented in Table 1.

Table 1: Addressable BW and bands versus numerology and FTT size

|  |  | 4K FFT |  | 8K FFT |  |
| --- | --- | --- | --- | --- | --- |
| NRBmax |  | 278 |  | 556 |  |
| Mu | SCS [kHz] | TxBW (MHz) | Bands or band groups | TxBW (MHz) | Bands or band groups |
| 0 | 15 | 50.04 | FDD <1.6GHz | 100.08 | all FR1 FDD, n40 |
| 1 | 30 | 100.08 | All FR1 FDD | 200.16 | n41/77/78/79 |
| 2 | 60 | 200.16 | n41/77/78/79 | 400.32 | n104/Ku UL/DL |
| 3 | 120 | 400.32 | Ku UL/DL | 800.64 | Ku UL/DL and FR2-1 |
| 4 | 240 | 800.64 |  | 1601.28 | FR2-1? |
| 5 | 480 | 1601.28 |  | 3202.56 |  |
| 6 | 960 | 3202.56 |  | 6405.12 |  |

Observation: with 8K FFT and using a single SCS it is possible to address band groups in terms of frequency ranges or band types.

Proposal on CBW, FFT and numerology for 6G:

  - Support of both 4K FTT and 8K FTT with only one SCS per band and band-groups.

  - Support of different SCS in UL and DL (for NTN? others?)

  - Band groups could be determined either in terms of:

    - Frequency sub-ranges:

- 15kHz SCS below 2.4GHz => 50MHz@4K FTT, 100MHz@8K FFT: The maximum in that range is 100MHz in band n40)

- 30kHz in 2.4GHz to 5GHz => 100MHz@4K FTT, 200MHz@8K FFT: would support n41 with one CBW in the US and China and most of band n77/78/79 operator’s spectrum

- 60kHz in 5GHz to 10GHz => 200MHz@4K FTT, 400MHz@8K FFT: similar to Wi-Fi® BW support, enable 400MHz in n104 and new spectrum.

- 60 and 120kHz in 10GHz to 52GHz depending on NTN/TN => up to 400MHz@4K FTT, 800MHz@8K FFT: less complex intra-band CA in FR2-1. Some FR2-1 band may even

    - Band types:

- 15k SCS and 4K FFT => 50MHz CBW => FDD bands below 1.6GHz

- 15k SCS and 8K FFT => 100MHz CBW => all FDD bands <3GHz

- 30k SCS and 4K FFT => 100MHz CBW => all TDD bands <2.4GHz

- 30k SCS and 8K FFT => 200MHz CBW => all TDD bands <5GHz, FR2-N UL?

- 60k SCS and 8K FFT => 400MHz CBW => all TDD bands <10GHz, FR2-N bands, FR2-1 UL?

- 120k SCS and 8K FFT => 800MHz CBW => all FR2-1 bands

- To address flexible CBW applications and bandwidth parts, transmission BW could use two granularities:

- At RB level: 1RB to ~556RB (with 4K FFT maximum NRB is 276) for 0.4 to 52GHz bands

- At SC level 1/2/4/6/8/10/12 sub-carriers for <2.5GHz?

- NRB per BW could be all equation-based with a guaranteed emissions by design thanks to increased guard-bands for intermediary BWs.

- Thus, only a limited set of CBW could be verified and at the same time, it could cover every MHz at 15kHz SCS, every 5MHz at 30kHz SCS, every 10MHz at 60kHz SCS, etc,?

## 2.3 Modulation and waveforms

For the coming meetings, we expect that, if any, new waveforms will be first e valuated in RAN1. RAN4 will mostly assess their performance in terms in-band/out-of-band requirements. As such, we do not have specific recommendations in terms of waveforms other than their attributes should be verified by including proper PA non-linearity.

Our experience from the 5G discussion is that if there were many waveform proposals claiming better SU or in-band emissions, in the end, once passing through the PA non-linearities, those differences almost vanished.

However, it is useful that RAN4 properly considers which emissions requirements should be used for waveform evaluation. In our view, it may be useful to consider a set of emissions requirements like ACLR, SEM, occupied bandwidth and spurious emissions but with possibly different targets in different frequency ranges:

- ACLR target could be different for ranges <2GHz, <5GHz and <10GHz

- The current SEM first MHz requirement is often a limitation for edge allocations or SU while there no more narrow band systems to be protected. If this is still needed for regulation for some bands or frequency ranges, these should be not impact MPR and rather correspond to A-MPR.

Also, for SU and edge allocations, the key is to find the proper trade-off between EVM impact and OOB emissions. It is thus of interest that digital filtering/windowing is:

- Sharper at low MCS to optimize SU and OOB emissions

- Smother at larger MCS to optimize EVM and if needed use edge MPR for emissions.

Proposal on waveform: For MPR and SU evaluations

- Revisit SEM and ACLR evaluation criteria for the different frequency ranges

- Apply different windowing/BB filtering tradeoffs versus MCS to maximize SU.

In our contribution on general aspects, and at least for smartphone UEs, we suggest that UL and DL QAM support is improved in 6G versus 5G like it was done for 5G compared to 4G. We suggest baseline support for UL 256 QAM and DL 1024 QAM and associated improvements in transceiver impairments. It is thus useful to investigate optional support for higher QAMs.

Proposal on modulation:

- Baseline (smartphone) support for UL 256 QAM and DL 1024 QAM

- Optional support for UL 1024 QAM and DL 4096 QAM assuming better EVM performance at the BS compared to the UE.

## 2.4 Antenna configurations and multi-T/R

Antenna configuration and types should be one of the main parameters in determining UE types as, on one end, its complexity is constrained by the UE size and on the other end, its type (antenna arrays or individual antennas) is related to frequency of operation.

Given that range of UE types and the frequency range to be supported, there should be a larger set of array sizes and nTx/nRx in 6G than in 5G. However, it is painful to derive requirements for nRx/nTx and X*Y array sizes individually. An equation-based approach should be feasible by considering, antenna correlation, antenna coupling, and beamforming impairments.

Together with the number of antennas comes the MIMO order to be supported. But very high order support may have marginal impact on aspects like cell edge data rates. In that case, output power and any option for beamforming have larger benefits.

Proposal on UE multi-Tx/Rx and MIMO support:

- Support from 1 to 4Tx depending on UE types and frequency ranges. Scalability to 8Tx

- Support from 1 to 8Rx depending on UE types and frequency ranges. Scalability to 16Rx

  - For example: For smartphone sizes, 1Tx/2Rx <1.6GHz, 2Tx/4Rx >1.6GHz

- Support from 2x1 to 8x8 antenna arrays with one or two arrays per UE depending on UE types and frequency ranges

- Look into equation-based scalable requirement versus #antennas based on antenna correlation, antenna coupling and beamforming impairments

- Support of UL MIMO with DFT-s-OFDM waveforms

- Support of up to 8x8 DL MIMO and 4x4 UL MIMO

- Support improved beamforming (smaller granularity digital beamforming) for nRx/nTx cases.

## 2.5 Device types

Rather than device types like wearables, smartphones, FWA/CPE, vehicle, that will never be exhaustive or may become obsolete, we suggest that device types are based on scalable and measurable parameters like:

- Size: largest dimension in cm: from ~1cm (wearable, dongles) to ~10cm (smartphones) several 10cm (Vehicles (shark fin), CPE/FWA, drones…)

- Number of antennas or array size and whether they are integrated or external

- Mobility/speed

- Single/regional/WW band and band combinations support

- Possibly using frequency ranges to scale some requirements

Proposal on UE types: RAN4 develops a limited set of UE types based on

- UE size (largest of the three dimensions, 1cm to several 10cm)

- Number of antennas or array size, internal versus external

- Mobility and speed

- Number of band and band combination support

- Possibly considering frequency ranges as a scaling factor.

# Conclusion

In this contribution, we provide our views on the Release 20 RAN4 6G SI system parameters.

Proposal on CBW, FFT and numerology for 6G:

  - Support of both 4K FTT and 8K FTT with only one SCS per band and band-groups.

  - Support of different SCS in UL and DL (for NTN? others?)

  - Band groups could be determined either in terms of:

    - Frequency sub-ranges:

- 15kHz SCS below 2.4GHz => 50MHz@4K FTT, 100MHz@8K FFT: The maximum in that range is 100MHz in band n40)

- 30kHz in 2.4GHz to 5GHz => 100MHz@4K FTT, 200MHz@8K FFT: would support n41 with one CBW in the US and China and most of band n77/78/79 operator’s spectrum

- 60kHz in 5GHz to 10GHz => 200MHz@4K FTT, 400MHz@8K FFT: similar to Wi-Fi® BW support, enable 400MHz in n104 and new spectrum.

- 60 and 120kHz in 10GHz to 52GHz depending on NTN/TN => up to 400MHz@4K FTT, 800MHz@8K FFT: less complex intra-band CA in FR2-1. Some FR2-1 band may even

    - Band types:

- 15k SCS and 4K FFT => 50MHz CBW => FDD bands below 1.6GHz

- 15k SCS and 8K FFT => 100MHz CBW => all FDD bands <3GHz

- 30k SCS and 4K FFT => 100MHz CBW => all TDD bands <2.4GHz

- 30k SCS and 8K FFT => 200MHz CBW => all TDD bands <5GHz, FR2-N UL?

- 60k SCS and 8K FFT => 400MHz CBW => all TDD bands <10GHz, FR2-N bands, FR2-1 UL?

- 120k SCS and 8K FFT => 800MHz CBW => all FR2-1 bands

- To address flexible CBW applications and bandwidth parts, transmission BW could use two granularities:

- At RB level: 1RB to ~556RB (with 4K FFT maximum NRB is 276) for 0.4 to 52GHz bands

- At SC level 1/2/4/6/8/10/12 sub-carriers for <2.5GHz?

- NRB per BW could be all equation-based with a guaranteed emissions by design thanks to increased guard-bands for intermediary BWs.

- Thus, only a limited set of CBW could be verified and at the same time, it could cover every MHz at 15kHz SCS, every 5MHz at 30kHz SCS, every 10MHz at 60kHz SCS, etc?

Proposal on waveform: For MPR and SU evaluations

- Revisit SEM and ACLR evaluation criteria for the different frequency ranges

- Apply different windowing/BB filtering tradeoffs versus MCS to maximize SU.

Proposal on modulation:

- Baseline (smartphone) support for UL 256 QAM and DL 1024 QAM

- Optional support for UL 1024 QAM and DL 4096 QAM assuming better EVM performance at the BS compared to the UE.

Proposal on UE multi-Tx/Rx and MIMO support:

- Support from 1 to 4Tx depending on UE types and frequency ranges. Scalability to 8Tx

- Support from 1 to 8Rx depending on UE types and frequency ranges. Scalability to 16Rx

  - For example: For smartphone sizes, 1Tx/2Rx <1.6GHz, 2Tx/4Rx >1.6GHz

- Support from 2x1 to 8x8 antenna arrays with one or two arrays per UE depending on UE types and frequency ranges

- Look into equation based scalable requirement vs #antennas based on antenna correlation, antenna coupling and beamforming impairments

- Support of UL MIMO with DFT-s-OFDM waveforms

- Support of up to 8x8 DL MIMO and 4x4 UL MIMO

- Support improved beamforming (smaller granularity digital beamforming) for nRx/nTx cases.

Proposal on UE types: RAN4 develops a limited set of UE types based on

- UE size (largest of the three dimensions, 1cm to a few 10cm)

- Number of antennas or array size, internal versus external

- Mobility and speed

- Number of band and band combination support

- Possibly considering frequency ranges as a scaling factor.

# References

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN4#109