TSG-RAN Working Group 4 (Radio) meeting #116bis	  R4-2513117

Prague, Czech Republic, 13 – 17 October 2025

Source:	Ericsson

Title:	General and UE RF considerations for 6G

Agenda item:	8.4

Document for:	Approval

# Background

This contribution contains observations and proposals for the study of RF characteristics for 6GR in RAN4: general consideration for both the BS and UE, non-spectrum and non-AI UE RF characteristics including power class and low-power receivers, spectrum aggregation, RF characteristics for the 7-15 GHz range and a possible EMC specification for the UE. We also discuss timing requirements and their relation to RF requirements. RAN4 should collaborate with RAN1 and RAN2 (when appropriate) in many of these areas already from the start of the study.

The detailed objectives of the 6G study are [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

# General overview of what to study in the RF area

## RF characteristics: general considerations

General principles

A few general remarks and proposals relevant for specifying RF requirements for 6GR first. The UE RF specification of TN and NTN should be based on applicable ITU-R recommendations for consistency with the Radio Regulations. Some of the Rel-19 enhanced features for NR deviated from ITU-R recommendations for checking compliance with unwanted emissions limits.

Proposal 2.1: the specifications should be aligned with applicable ITU-R recommendations e.g. ITU-R Rec. SM-1539 and ITU-R for emissions in the OoB domain.

The specifications of LTE and NR are also based on the ERC Recommendation 74-01 to a large extent.

Alignment with ITU-R also makes RAN4 specifications more credible outside 3GPP. Local regulatory requirements could be handled based on existing mechanisms for NR, which does not preclude improvements for 6GR. However, the general minimum requirements should be global.

Proposal 2.2: the studies should be based on minimum performance requirements applicable in all regions to allow global circulation of terminals. Local regulatory requirements can still be accommodated similarly to NR.

NS signalling has been used for indicating local regulatory unwanted emissions limits since Rel-8 but could possibly be improved further for handling cases in which the use of a band within an Administration is changed. However, formulations like “this band/feature is only applicable in [a country or region]” should not be used.

Other general principles relate to the scope of RAN4 specifications

Proposal 2.3: the specification of (RF) minimum requirements should be based on behaviour specified in RAN1 and RAN2 specifications. RAN4 specifications do not specify UE behaviour unless this is assigned  by reference in the RAN1 and RAN2 specifications.

e.g. by formulations like “as specified in 38.101-1” for NR.

What to study

The study should cover RF minimum requirements covering MBB and verticals like XR, IoT/LPWA, FWA and NRAR for TN and for NTN where applicable. A starting point could be corresponding features for which NR performance could be improved, e.g. more dynamic carrier/bandwidth aggregation schemes, even if more input and decisions are needed from RAN1/RAN2 before the RAN4 part can progress.

Relating to UE RF requirements, listed below are areas in which NR performance could be improved:

- CA performance e.g. power derating for DL-only configurations, Pcell power degradation at configuration/activation, very large power back-off for concurrent transmission

- power capability reporting inconsistent in the field (based on capabilities and field descriptions proposed by RAN4)

- reciprocity-based DL-MIMO, non-reciprocal insertion loss and SRS output power performance

- UL MIMO coherent transmissions – performance?

- UE power back-off: requirements not based on state-of-the-art while recognising considerable efforts made to derive MPR based on measurements, P-MPR, unpredictable MPR for FR2

- MSD and support of simultaneous Rx-Tx

- uncertainty to what extent requirements apply for operations in the field (e.g. MPR)

- down prioritize the definition the feature per “Feature Set” and “FSPC”.

  - Networks often try to maximize the achievable bitrate, taking the capabilities of the UE and gNB into account. This often leads to maximizing the number of carriers and MIMO layers first and then considering additional features. Per FS and FSPC features generally reduce their capability as the number of carriers increase, essentially ‘cannibalizing’ CA capability to support the feature. Such features will then generally not be used, and therefore not implemented in networks or UEs when CA and MIMO capability are highly prioritized.

Some other aspects are more related to the channel arrangement, bandwidths and configuration, but also UE RF performance requirements:

- guard bands for intra-band CA and uncertainty about the applicability for SCS deployed

- applicability of requirements for the UE-specific channel bandwidths within a wider carrier

- CA requirements applicable for configured CCs or activated CCs?

All is not bad, however: NR performance in the field is still being improved, although EN-DC operation is still prevalent. Nonetheless, the above are areas for possible improvements by corresponding features supported by 6GR and areas of improvements worth considering in the 6GR study.

## Timing and synchronization – relation to RF requirements

### Methodology of 6G time & sync requirement design

Introducing new features and functions demands a balanced approach that considers cost, complexity, and benefits. To ensure value and effectiveness, it is essential to conduct a comprehensive background analysis for each proposed feature or function. This analysis should determine whether synchronization is necessary, explain the reasons behind this need, outline the specific conditions or assumptions under which it is required, and define the appropriate level of synchronization needed.. This is shown in Figure 1.

During each UE RF requirement discussion, the first objective should be whether the feature or function could be designed to operate effectively without synchronization or strict synchronization. Additionally, synchronization or strict synchronization can be optionally leveraged to enhance the feature or function when available. The strict conditions can be optionally leveraged to enhance the feature or function when available.

To make the issue more concrete, the general methodology discussed here, for timing and synchronization, applies, for instance to TDD transient times and UE switch times.

Figure 2.1: General approach regarding the need for synchronization

This strategy can be tailored to specific user equipment (UE) capabilities. For instance, a UE with advanced capabilities might not need synchronization or strict synchronization, whereas other UE implementations might conditionally support the complete feature/function, or provide enhancements to the feature/function, if certain synchronization performance levels are met.

Observation 2.1: It is essential to conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Robustness and resilience will be crucial for many 6G features and functions. Designing features or functions that rely on strict requirements could compromise these qualities. For example, when we discuss a time synchronization requirement, we can deduce that a less strict synchronization requirement will be more tolerant to temporary loss of synchronization and help to enable a more robust network, feature or function. When RAN4 discuss the 6G requirement, RAN4 should consider the trade off between a strict requirement and robustness of the total system.

On the one hand, a more relaxed requirement leads to a feature or a function that can work in more deployment configurations which enables the operator to deploy in a flexible way. On the other hand, a stricter level of synchronization could in some cases enhance the feature or function and/or allow a larger set of UE types to be supported. Therefore, RAN4 should not always aim to design a strict requirement but also consider the flexibility of the real field deployment.

It is essential to conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

For requirement clarity link each requirement to specific features or functions (which may be optionally supported), when needed clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints, like robustness—and, where applicable, to specific UE capabilities.

Proposal 2.4: When RAN4 defines time and sync requirements, RAN4 should conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Proposal 2.5: When RAN4 defines time and synchronization requirements, it should consider system robustness and deployment flexibility. Features and functions should be designed to operate effectively with a base level of synchronization, ensuring reliable performance in common and typical scenarios, even without strict synchronization. Where possible, the design may optionally utilize strict synchronization to enhance performance or extend support to less common use cases in situations where such synchronization can be achieved.

Proposal 2.6: When RAN4 defines time and synchronization requirements, for requirement clarity link each requirement to specific features or functions, when needed clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints—and, where applicable, to specific UE capabilities.

### UE reference architecture for technical report and specification

Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility when it comes to cell planning, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Figure 2.2: Large MRTD and path loss difference use case.

Observation 2.2: Deployments can be collocated and non-colocated. Non colocated deployments, adds flexibility, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Requirements are strongly connected to UE architecture. Historically in 3GPP this balance between the use cases of a feature and what a certain UE architecture can and cannot do has been well studied. In each early new generation discussion, UE architecture was often part of the general RRM and RF requirement discussion.

From our understanding, there are several examples:

- Single or multiple LNA in RF frontend.

- Single or multiple FFT and IFFT in baseband.

- The number of searchers in baseband.

A single FFT drives all carriers towards a fraction of CP. A single LNA handling several carriers might work better of aligned at UE RX, limiting MRTD, so CP time period can be used to control LNA gain on one carrier without perturbing data throughput on the other, but this limits site to site distance for non-colocated deployments, on the other hand. The RF retuning time is highly impacted the interruption performance. The number of FFT will impact most of the performance for L1 level procedure, such as L1 inter-frequency measurement.

Another important aspect contributing to RRM requirement is the searcher assumption. In NR, RAN4 requirements assume that one searcher is dedicated to the PCell frequency, while the second searcher handles all other intra-frequency carriers. This constraint limits the UE’s ability to perform parallel measurements across multiple carriers or RATs.

Observation 2.3: Aspects of UE architecture like FFT, searcher and LNA implementations put conditions on what kind of requirements UE can be achieved.

Because of this interplay between UE architecture and feature use case requirements we think it would benefit 6G standardization if we could settle aspects of UE architecture early in the form of a reference UE architecture. The intention is not to constrain implementations to follow the particular reference architecture, but rather to allow a framework where standards discussions can meaningfully take place.

Proposal 2.7: RAN4 to discuss UE reference architectures in early stage of 6G. The UE RF requirements should be compatible with the UE reference architecture.

In NR, several advanced features—such as multi-RX chain reception, TDCP, and CJTC reporting for non-ideal backhaul—have been introduced across different releases. However, there remains a noticeable gap between the feature design in other WGs, and the corresponding requirements defined in RAN4. When other WG develops a feature, it often assumes that the UE can support a specific architecture necessary for the feature to function. In contrast, RAN4 discussions frequently rely on Rel-15 baseline UE assumptions, focusing on worst-case or basic UE implementations.

We would like to respectfully emphasize that not all features are expected to be supported by all UEs. For 6G, it would be valuable to define requirements that enable a feature to operate effectively. If there are concerns regarding a feature or the UE architecture needed to support it, these should ideally be raised during the WI approval phase. This would help to avoid situations where substantial effort is invested in other WG to design a feature, only for it to be challenged later in RAN4 due to misaligned architecture assumptions and impractical requirements.

In many cases, RAN4 discussions begin with whether requirements should be defined at all, rather than focusing on how they should be defined. Addressing these gaps earlier in the process would lead to more coherent and implementable specifications.

Proposal 2.8: RAN4 should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature.

# Transmitter characteristics

Channel arrangement and relation to guard bands for the specification of requirements

The UE output power for meeting the unwanted emissions requirement for any necessary/channel bandwidth of a 6GR carrier or the bandwidth of new dynamic spectrum aggregation schemes will (also) depend on the internal GBs. This should be discussed from the start of the study of the spectrum utilization, and relevant for specification of UE output power requirements in addition.

For NR CA, the internal GB and unwanted emissions requirements are different for the BS and UE. For the BS the unwanted emissions outside the aggregated carriers are defined w r t the outermost carrier with its internal GB, whereas for the UE the emission bandwidth is the aggregated bandwidth with symmetric internal GBs facilitated by a nominal CA spacing that is smaller than the channel spacing (BW1 + BW2)/2 when the carrier bandwidths are unequal as shown in Figure 3.1. For the BS the outer GBs of the aggregated CA bandwidth can therefore be asymmetric.

Figure 3.1: asymmetric (BS) and symmetric (UE) internal GBs for CA.

The specification of nominal CA spacing, aggregated CA bandwidth and guard bands for aggregated carriers was complicated by the support of multiple numerologies and is now also subject to different interpretations in the field. The CA spacing and the aggregated channel bandwidth are determined by the maximum SCS supported by both carriers but applies for all numerologies configured, the resulting aggregated bandwidth is

BWagg ≤ BW1 + BW2 - SCSmax

where SCSmax is the sub-carrier spacing/bandwidth of the largest subcarrier configuration supported. The actual internal GB between the aggregated bandwidth edge and the outer PRB of the resource grid then varies with the numerology – but there are different interpretations of this in the field.

The size of the GBs affect the specification of allowed UE power reduction (MPR).

For 6GR, new dynamic spectrum aggregation (multi-carrier) schemes complementary to or possibly replacing intra-band CA will be studied, a conceptual scheme with component carriers or “6GR bandwidth parts” within the total bandwidth is shown in Figure 3.2. A total bandwidth of the aggregated spectrum and emission bandwidths of components within must still be specified with appropriate internal GBs, either symmetric or asymmetric within the UE (total) emission bandwidth.

Figure 3.2: a spectrum aggregation scheme.

Dynamic spectrum aggregation schemes for 6G may be supported within a single ‘channel bandwidth’, also for non-contiguous (fragmented) spectrum. Then the outer GBs could be symmetric for both the BS and UE. For non-contiguous spectrum/sub-blocks unwanted emissions requirements must also be met in the gap, outside the OBW/necessary bandwidth of the sub-blocks as shown in Figure 3.2. Determination of the internal GBs size towards the centre gap is less obvious in case the sub-blocks are of unequal PRB size, then the GB size of each sub-block may not be compatible with symmetric outer GBs; further studies are needed.

Observation 3.1: for GBs of aggregated spectrum, contiguous or non-contiguous, the outer internal GBs of a 6GR spectrum aggregation scheme supported within a single channel bandwidth could possibly be symmetric for both the BS and UE, contrary to intra-band contiguous NR CA for which the outer GBs of the BS CA bandwidth can be asymmetric. However, this may not be compatible with the necessary internal GB size for non-contiguous sub-blocks of unequal PRB size (fragmented spectrum) within the said single channel bandwidth.

MRSS and BS aspects must also be considered in determining guard bands.

6GR inter-band spectrum aggregation parts of a band combination, including the case of decoupled UL and DL by 6GR, are likely to be specified similarly to the inter-band NR CA case.

Unwanted emission (OOBE and spurious)

Relating to unwanted emissions, the SEM for LTE and subsequently NR was specified for compliance with regulatory requirements, the general a baseline, while the ACLR requirements are specified for intra-system (3GPP) coexistence (the near-far problem essentially). The ACLR is not obtained by integrating the SEM like for UTRA.

Proposal 3.1: 6GR studies should be based on maintaining the concept of a general SEM for compliance with regulatory emissions requirements and the ACLR specified for intra-system coexistence.

The OOBE and spurious emissions should be specified w r t necessary/occupied bandwidths of transmissions, regardless if these occur within a wider channel/carrier of a multi-carrier scheme.

For NR, the general spurious emission limits for FR1 and FR2-1 are based on the ERC Rec. 74-01. We propose to keep these limits for 6GR

Proposal 3.2: to enable global circulation of terminals, the 6GR studies are based on maintaining the general NR spurious emissions limits for FR1 and FR2-1 based on ERC Rec. 74-01.

Regarding unwanted emissions limits for ensuring UE-UE coexistence, the LTE and NR emission limits are the same: a     -50 dBm/MHz limit with exceptions for harmonics. This could also be kept as a baseline for 6GR.

Output power accuracy: reference symbols for UL-MIMO and reciprocity-based DL-MIMO

For NR, the performance of reciprocity-based DL-MIMO is degraded by SRS power accuracy and unknown non-reciprocal losses in the UE. RAN4 output power requirements for 6GR should therefore also be specified and verified for reference symbols. Moreover, RAN4 requirements on coherent UL transmission should ensure performance for coherent UL-MIMO. We propose that

Proposal 3.3: RAN4 collaborates with RAN1 from the start of the 6G study on the design of any reference symbols to be used for reciprocity-based DL-MIMO, if specified, and for coherent UL-MIMO with due account of expected UE transmit performance.

Spectrum aggregation

For NR, the problems with CA from an RF and UE output power perspective are

- ambiguous reporting ion UE CA capabilities

- large MPR for concurrent transmissions

- degradation of Pcell coverage when CA is configured (UE power reduction increased at configuration)

- Scell output power when the UE is power limited

- PCMAX for CA as specified in the 38.101-1 is not related to UE behaviour as specified in 38.213 (only to the max power requirements in RAN4 conformance test)

- uncertain UL-MIMO output power capability when combined with CA, particularly when UL Tx switching is used

According to the 38.213, the total configured power PCMAX is the threshold at which the UE prioritizes

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order […]

presumably due to power limitations governed by SAR limit as determined by proprietary algorithms. RAN4 was supposed to specify the bounds of PCMAX in 38.101-1, but the specification in the latter is more related to compliance of the UE maximum output power for conformance tests.

We assume that 6GR output power for UE transmissions would also be limited by a configured maximum power per carrier similarly to NR. The power prioritization for new dynamic spectrum schemes and any equivalent PCMAX is RAN1 responsibility, but RAN4 engagement at an early stage is important. It may be more important that the NW is informed of any change of the total UE power capability relative to a total configured power more relevant for facilitating SAR compliance.

NR CA performance can still be improved, but the list above could be considered for the study of output power requirements for 6GR spectrum aggregation. To this end we propose

Proposal 3.4: for spectrum aggregation, study the feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme (cf. increase of the allowed MPR for the Pcell at NR CA configuration).

Proposal 3.5: for spectrum aggregation, the total configured power PCMAX, or equivalent power, should be related to the behaviour of the UE for any multi-carrier scheme specified. Changes of the total output power capability could be reported by (a)periodic dedicated signalling (cf. P-MPR reporting for NR).

Proposal 3.6: transmission on a single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction (cf. no power degradation for NR DL-only CA).

Tx transients and relation to RRM requirements

Due to strong inter-dependencies, TX transient times shall not be set or discussed isolated from , TX off level, TDD cell phase sync, UE transient times and UE  TX ⇔ RX  switching times.

The RRM requirementin RRM specification is related to Cell Phase Synchronization and TDD transient times in RF specifications through the formula:

NTA offset≥Cell_Phase_Synchronization+TDD_transient_time

This means that, Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together.

Observation 3.2: NTA offset , Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together, since NTA offset≥Cell_Phase_Synchronization+TDD_transient_time

# UE power class structure for 6G

## NR background

In NR FR1, just as in LTE, the default UE maximum output power is 23 dBm, which is defined as UE Power Class 3 (PC3). In Rel-15, the only other defined Power Class is PC2, corresponding to the maximum output power of 26 dBm, available only to single carrier, inter-band CA and UL MIMO with 2 Tx ports. The support for PC2 is optional and subject to UE capability – ue-PowerClass per band (mandatorily reported if the UE supports different than the default Power Class), powerClass per band combination and ue-CA-PowerClass-N. In FR2, four power classes were defined, tied to different UE types: FWA UE (PC1), Vehicular UE (PC2), Handheld UE (PC3) and High power non-handheld UE (PC4).

In the following Releases, the UE Power Class structure became (much) more complex, adding PC5 (20 dBm), PC1.5 (29 dBm) and PC1 (31 dBm) in FR1. The support for 3 Tx and 4 Tx ports has been introduced while the number of applicable scenarios with Power Class other than the default one has also grown: intra-band contiguous and non-contiguous CA, UL MIMO with up to 4 Tx, V2X, Sidelink CA, (Sidelink) shared spectrum channel access, Tx Diversity, CA with UL MIMO, eRedCap and CA with Tx Diversity.

Note that not every combination of the supported number of Tx ports and Power Class is possible/specified for a UE. In Table 3.1, we show the allowed combinations of number of Tx and Power Class for a single carrier case in FR1 Terrestrial Networks (TN) until Rel-19:

Table 3.1: Supported number of Tx ports per Power Class for single carrier case in Rel-19 FR1 TN.

|  | PC3 | PC2 | PC1.5 | PC1 |
| --- | --- | --- | --- | --- |
| 1Tx | X | X |  | X |
| 2Tx | X | X | X |  |
| 3Tx | X | X |  |  |
| 4Tx | X | X | X |  |

New UE capabilities/parameters of capabilities have been introduced as well (or the previous ones extended): ue-PowerClass-v1610, ue-PowerClass-v1700, powerClass-v1610, powerClassNRPart-r16, ue-PowerClassPerBandPerBC-r17, ue-PowerClassPerBandPerBC-v1820, intrabandConcurrentOperationPowerClass-r16, powerClassForTwoAggregatedCarriers-r18, powerClassForThreeAggregatedCarriers-r18, ue-PowerClassSidelink-r16, msd-PowerClass-r18 and sl-PowerClassUnlicensed-r18.

An ATG UE is more similar to a BS and therefore represents a distinct case in NR where the maximum output power is configurable and can be any integer value in the range 23 to 40 dBm based on reported UE capability maxOutputPowerATG-r18.

In FR2, three more Power Classes have been added after Rel-15: FWA UE (PC5), High Speed Train Roof-Mounted UE (PC6) and RedCap UE (PC7).

In Rel-19, PC2 and PC1 have also been specified for some NTN-FR1 operating bands.

In Rel-20, the work on the specification of PC1 with 4 Tx (already specified for NTN in Rel-19), PC1.5 with 2 Tx for FDD bands, PC1.5 with 1 Tx and PC1 with 2 Tx has been approved.

Observation 4.1: The Power Class structure in NR has organically grown and become complex with the number of available Power Classes, different configured states they apply to (CA, UL MIMO etc.) including the supported number of Tx ports if applicable, different UE types, frequency ranges and network types (TN/NTN).

Nevertheless, we propose that the basic principle should remain the same between NR and 6GR:

Proposal 4.1: Power classes should be defined for 6GR, but the complexities and contradictions of 5G should be improved. It is expected that 6GR power should also be limited by a configured maximum power per carrier and possibly a total UE output power for concurrent transmissions.

## Ways to improve

Different power classes should also be specified for 6GR: for FR1 power capabilities could be different for a given device type, whereas for FR2-1 the power class would depend on the device type similarly to NR. However, specification of the 6GR power capability for CA, new dynamic spectrum aggregation schemes and combinations with UL-MIMO should be such that the power capability of a UE follows from UE capability, UE behaviour specified by RAN1 or other (a)periodic reporting of modified power capability e.g. by P-MPR.

Ambiguous and unclear power capability reporting are still issues for NR. The problems are that

- there is no consensus that the per-band UE power class for non-CA also limits the per-band maximum power for UL/DL CA

- the per-band-per-BC power class is used beyond its intended purpose

- the per-BC power class powerClass that should be limiting the total power PCMAX is not limiting all transmissions of a configuration e.g. for single-cell transmissions in an inter-band configuration in some implementations and that the application of the default power class when the capability is absent varies

Moreover, added to this are power capability uncertainties for combinations with UL-MIMO, including cases with UL Tx switching, the power per layer for NR is not always consistent with the scaling rules specified in 38.213 amongst UE implementations in the field.

One example problem in NR is an ambiguity on the power class supported for the single UL configured for DL-only CA band combinations (BC), included in the supported band-combination list or fallbacks of another top-level BC, or for non-CA NR band combinations included in the band-combination to convey feature sets, as it can be indicated by three (3) different capabilities:

- the per-band ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-v1700 in the BandNR capability, the power class supported per band of a CA/DC band combination,

- the optional per-band-per-BC ue-PowerClassPerBandPerBC-r17 that can be – and is -- used for any BC from Rel-17 nowadays (no restriction in 38.306),

- the per-BC powerClass/powerClass-v1610 (up to PC1.5), can be lower than that indicated in BandNR and indicates PC3 if absent.

There is an ambiguity at the network side on which one does it apply for the UE. Absence of a per-BC capability could mean either that the HPUE only supports DL-only CA with PC3 or that this power class is unintentionally omitted for the supported BC in the BC list.

We assume that 6GR output power for UE transmissions will also be limited by a configured maximum power per carrier. The configured maximum power is limited by the device power capability/class.

However, for 6GR, the better approach would be to design the UE capabilities framework such that: (i) their number is minimized; (ii) they unambiguously indicate the supported power class per-band for single carrier case; (iii) for multiple configured UL carrier case

Observation 4.2: for a given NW configuration of the UE, the NW must be able to parse power capabilities from the UE capability transfer alone for a given configuration of the UE by the NW.

Currently, in RAN4 specifications, many important information are given in table notes and that should be avoided in 6GRAN4 specifications.

It is very important to solve such issues since the uplink has remained the bottleneck for NR, even in its advanced stage. It is critical for the network to have an accurate knowledge of the supported UE power class in order to optimise its scheduling on the uplink.

Proposal 4.2: Study how to design the UE capability framework on the supported UE power class for single carrier/CA, such that: (i) it is unambiguous for the NW and robust, so that it works for either single or multiple configured UL carriers; (ii) the number of needed UE capabilities is minimized.

Indexing of Power Classes is not very intuitive, the default one being PC3. In the current Power Class structure, higher the Power Class, lower the index number of the PC, which is not very forward-compatible given that PC1 is defined as 31dBm, the only lower integer number is ‘0’ and there is already a proposal in NR Rel-20 to specify Power Class with 32 dBm. The Power Class structure should be forward looking to even higher maximum output powers. On the other hand, PC3 and PC2 have well established definitions in the industry, dating from 3G, and the notation should remain the same for at least those two Power Classes.

Proposal 4.3: For PC3 and PC2, reuse the same notation from NR since those definitions are well established in the industry. For power classes higher than PC2, study how to come up with notation which is forward looking for even higher power classes in the future.

# Improving UE TX modelling for MPR/A-MPR

The RAN4 documentation of MPR simulation modelling approach is quite deficient. Requirements are not mentioned in the specification and impairment models are barely documented. In 6G, we must correct this and add applicable models, in order to enable repeatability and replicability of the simulations by both the companies involved in 3GPP as well as other companies not actively participating in the 3GPP standardization. This would increase our creditability.

For 5G today, the following assumptions for the simulations have been applied during the last 10 years:

- Power Class X

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -28dBc

- IQ Image: -28dB

- CIM3: -60dBc

The Power Class and NR Waveform tell us the nominal maximum UE output Power without tolerance and NR Waveform tells us that both DFT-s-OFDM and CP-OFDM have been assumed and simulated.

Unfortunately, a Power Amplifier model is not specified. Instead, the calibration point is a best try to align the simulation results.

The definition, 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0, means, setting up a

- Waveform = DFT-s-OFDM,

- Modulation = QPSK,

- Bandwidth size = 20MHz

- Number of Resource Blocks (RBs) = 100,

- RB starting index = 0,

Then the 1dB MPR should fulfill the NR ACLR for the specified Power Class X, found in TS 38.101-1, in Table 6.5.2.4.1-2.

The values for Carrier Leakage and IQ Image are extracted from the requirements for in-band emissions in TS 38.101-1. CIM3 is not documented at all in the specifications. Considering that all three of these (Carrier Leakage, IQ Image, CIM3) are impairment models, they should be explicitly defined in the specifications. Not just the difference expressed as a ratio.

Proposal 5.1: for specification of transmitter minimum requirements, RAN4 to agree and document impairment models as well as all the requirements for simulation assumptions.

To improve simulations and alignment, the simulation assumption should be as explicit as possible. Models should be documented, at least the characteristics of a new power amplifier model for 6G.

The current models used for MPR/A-MPR simulations are based on assumptions that are now outdated as state-of-the-art-performance of current products is significantly better. The initial impairment assumptions were created for the 4G standardization work, and then for 5G, these impairments were slightly improved. For instance, for carrier centre frequency < 1 GHz, the requirements for 4G for Carrier Leakage and IQ Image are -28dBc and -28dB respectively for output power > 10dBm. For carrier centre frequency ≥ 1 GHz, these ratios for Carrier Leakage and IQ Image are -25dBc and -25dB respectively. The small assumption improvement that was done for 5G was that for the entire Frequency range 1 and output power > 10dBm, the requirement for Carrier Leakage and IQ Image were set to -28dBc and -28dB respectively. A rather small improvement from one generation to another. It also means the applied impairment models have been the same for sub-Gigahertz for around 15 years. To exemplify, EVM requirement for 256QAM is set to 3.5%. If we convert this to dB, EVM should be ≤ -29.1dB. The IQ Image impairment model itself distorts the signal by -28dB. Then there are the other impairment models including the Power Amplifier, distorting the signal even more.

Observation 5.1: To meet the EVM requirement for 256QAM of 3.5%, the impairment models must not have distorted the signal more than -29.1dB in the IQ constellation. The IQ Image impairment model itself distorts the signal by -28dB.

The consequence of using outdated assumptions that increase noise, is that the defined MPR/A-MPR is set higher than it otherwise would have been needed. A lower output power of a UE directly affects UE coverage.. Some configured combinations in CA need more than 10dB backoff,  with the current assumptions and the present spectrum requirements, making them not usable in practice. The consequence is that the Network does not  know that a UE may do better, so the network does not configure the UE with CA. Lower output power increasing the risk of lowering the SNR. If the SNR is lower, the network then selects a lower MCS and data rate. In conclusion, by 3GPP using outdated simulation assumptions, we restrain the uplink performance.

Observation 5.2: use of outdated simulation assumptions for specifying transmitter requirements affect network performance in multiple aspects, and the overall performance is degraded.

To improve simulations and alignment, the simulation assumption should be as explicit as possible. Models should be documented. At least the Characteristics of a new Power Amplifier model for 6G. We should also evaluate if the MPR simulations should be with or without a Digital Pre-distortion model that.  may be incorporated into a Power Amplifier model.

Proposal 5.2: Implicitly specify a Power Amplifier model, with or without Digital Pre-Distortion (DPD).

Another aspect of inaccurate models may lead to artificial objectives of improving MPR simulations, for example the discussion for post-distortion standardized. A post-distortion discussion may still be a valid improvement of the overall end-to-end system, but perhaps we shall try to remove the symptom first.

# UE receiver characteristics

Receiver reference architecture

In Rel-18, a type 2 handheld UE was defined to support inter-band non-contiguous EN-DC and intra-band non-contiguous NR-CA with up to 2 layers. Building on this, Rel-19 introduced a type 4 FWA UE for intra-band non-contiguous non-collocated EN-DC/NR-CA, featuring the capability of handling 25 dB power imbalance and a MRTD of less than 33 µs, with up to four MIMO layers (Rank 4) per Component Carrier.

As with almost in every generation study, the early establishment of a reference UE architecture is essential as proposed in Section 2.2, particularly for supporting the non-collocated cell deployments. For instance, some reference Rx architectures supporting non-collocated deployment were discussed in R4-2217734. It should be noted, while requirements are to be specified for a supported feature, e.g. power imbalance MRTD, etc., the adoption of a particular architecture shall not be mandated.

Selectivity and blocking

6G aims to support wider bandwidths, higher frequencies, ISAC, AI-native designs, and denser deployments, all while emphasizing energy efficiency and coexistence with legacy systems. ACS and OOBB requirements defined in the current specifications like 38.101-1 ensure the UE receiver resilience against interference, which is increasingly critical in 6G. Similarly, in-band blocking requirements should be maintained as well to handle intra-band interferers in fragmented spectrum environments Particularly, OOBB protects the UE receiver from strong out-of-band interferes, while ACS ensures selectivity against adjacent channel signals e.g. in near-far scenarios.

We propose not to relax ACS, OOBB and IBB baselines, as doing so could create variable performance baselines that complicate network management, test, which will lead unpredictable desensitization. With 6G expected to involve denser deployments, more spectrum fragmentation, these aforementioned requirements will be essential to prevent receiver desensitization.

Observation 6.1: ACS and OOBB requirements defined in the current specification ensure the UE receiver resilience against interference, which is increasingly critical in 6G.

Proposal 6.1: Retain the current ACS, OOBB and IBB requirements in 6G as a baseline.

For low-capability devices targeting wearables or industrial IoT, simplification in receiver design may enhance affordability and energy efficiency. For instance, SAW-less architecture, which eliminate traditional SAW filter, could reduce costs while maintaining acceptable performance in low-interference environments. However, such relaxation should not compromise baseline ACS and OOBB resilience and must depend on the outcomes of ongoing discussion on 6G device types, minimum capabilities and bandwidth requirements.

Observation 6.2: There may be possibilities for simplification of requirements for low-capability devices, however it should depend on the outcomes of ongoing discussion on 6G device types, minimum capabilities and bandwidth requirements.

Simultaneous Rx-Tx and MSD

The specification of MSD for NR band combination has been simplified in Rel-19. MSD is verified in conducted test essentially verifying the noise factor of the receiver when the UE is configured with CA or DC. The conducted MSD might also determine whether simultaneous Rx-Tx can be mandated for band combination (relevant for cross-band isolation performance). To this end, the OTA performance is more relevant for performance in the field. We therefore propose

Proposal 6.2: MSD for band combinations are also measured OTA to complement the existing noise-factor (MSD) requirement since also relevant for support of simultaneous Rx-Tx for band combinations.

It is important that margins to the actual REFSENS performance are not too excessive as simultaneous Rx-Tx is crucial for performance regardless of any scheduler coordination.

# Low power receiver for 6G

The LP-WUS work item was introduced in 5G in a late phase and it is questionable if such feature would be implemented by UE chipset and network. As power saving is always important both for UE and network, the wake up receiver should be within the 6G from day 1.

- Proposal 7.1: Wake up receiver should be in 6G scope from day 1.

RAN1 allows the UE to autonomously enable/disable wake up receiver in 5G and this creates confusion on the testability of a wake up receiver on a LR (LP-WUS Receiver) level or on a UE level.  When companies talk about the “smart” wake up receiver with detection of the jammer presence and in such case wake the main receiver instead, it gives the impression such “wake up receiver” could never cope with a jammer but rely on main receiver to cope with this jammer. Though this complies with RAN1 design, the power saving gain is questionable as the jammer is not predictable in real network operation.

- Observation 7.2: UE autonomously enables/disables wake up receiver makes the power saving gain unpredictable.

Though during the work item phase, the same radio jammer between main receiver and LR is agreed as a working assumption in RAN4, in the end, two types of LR are specified and one type LR relaxes both noise figure more and also selectivity claiming the limited dynamic range of the filter capability.  As the LP-WUS signal is OOK overlaid signal and no legacy coexisting study has been done using this signal, it is unclear how selectivity can be relaxed without any concrete coexisting. Therefore, the coexisting study should be done at 6G at day 1. If the receiver is blocked by the jammer, this create the coverage hole in the network and the feature should ensure the coverage in a holistic sense.

- Proposal 7.2: Coexisting study should be in 6G for wake up receiver to decide selectivity to avoid the coverage hold in presence of jammer.

The power saving and noise figure are two competing performances and as noise figure matters more at the cell edge performance, it should be studied in system level how to balance between power saving and noise figure of the wake up receiver.

Observation 7.3: Power saving and noise figure should be balanced in system level.

# UE parameters in the 7-15 GHz range

RAN4 made initial studies in the 7-24 GHz range:

  - A complete study of this frequency range in Rel-16 timeframe, outcomes have been captured in TR 38.820.

  - A study of UE parameters in the 10.0-10.5 GHz frequency range, answering a LS from ITU-R in the scope of WRC-23. The outcomes have been captured in TR 38.921.

  - A study of UE parameters in the 7.12G-8.4 GHz and 14.8-15.35 GHz frequency ranges, answering a LS from ITU-R in the scope of WRC-27. The outcomes have been captured in TR 38.922.

In the scope of the 6G study, we propose that RAN4 should re-assess the outcomes of those studies, consolidating the different conclusions which are not always consistent. For example, in TR 38.820, a typical noise figure of 9 and 10 dB was agreed for respectively 10GHz and 15GHz example frequency, while in later discussions (TR 38.921 and TR 38.922), RAN4 still assumed a UE noise figure in the range of 9-13dB for 8GHz and 11-13dB for 15 GHz. As the UE noise figure has a direct impact on the UE receiver requirements and downlink performances, it should be properly studied and evaluated more accurately.

Also, as already mentioned in TR 38.922, antenna array should be considered for UE, especially in the 12-15 GHz frequency range. In the scope of 6G, RAN4 should further the characteristics of such antenna, evaluating its parameters. Considering that antenna array will help improving the DL link budget, RAN4 should also further study if such type of antenna could also be considered on UE side for frequencies lower than 12-15 GHz.

Proposal 8.1: RAN4 should further reassess the assumed UE parameters (e.g. UE noise figure) in the 7-15 GHz frequency range.

Proposal 8.2: RAN4 should further evaluate the UE antenna array parameters in the 7-15 GHz frequency range.

Proposal 8.3: RAN4 should further study considering phased array type of antenna in the 7-15  GHz frequency range for different UE type (e.g. handheld, FWA, …).

# UE EMC specification structure

The development of 6G standards presents a suitable opportunity to review the structure of Electromagnetic Compatibility (EMC) specifications. Unlike the network nodes, the current UE EMC specifications are relatively straightforward, consisting of a per-RAT structure:

- TS 36.124, Evolved Universal Terrestrial Radio Access (E-UTRA); Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

- TS 38.124, NR; Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

For 6G, there are two potential options.

One option is to consolidate all UE EMC Specifications into a single UE EMC specification that would contain EMC requirements for all RATs, which provides a single reference point of UE EMC requirements. Make maintenance work simple. A single change request can update requirements for all RATs.

Another option is to discontinue 3GPP UE EMC specification for 6G, i.e., the existing specifications (TS 36.124, TS 38.124) would remain for legacy technology support but would not be extended for 6G. This approach questions the necessity of developing a new 3GPP UE EMC specification for 6G, considering low interest from UE vendors in 5G. Of course, this would represent a significant change in 3GPP’s scope for UE specifications.

It is recommended that RAN4 seeks feedback from UE vendors and other relevant stakeholders on the necessity of a dedicated 3GPP 6G UE EMC specification. If the industry consensus confirms a need for a 3GPP specification, then option 1 would be the way forward. If the consensus confirms that external standards are sufficient and that a 3GPP specification is redundant, then option 2 should be seriously considered to avoid unnecessary standardization effort.

Observation 9.1: The following two options could be considered for 6G EMC specification structure regarding UE:

- Option 1: Create a single, consolidated 6G UE EMC specification.

- Option 2: Discontinue developing 3GPP UE EMC specification for 6G and maintain legacy UE EMC specification (TS 36.124 and TS 38.124).

Proposal 9.1: RAN4 seeks feedback from UE vendors and other relevant stakeholders on the necessity of a dedicated 3GPP 6G UE EMC specification.

# Proposal

General principles

A few general remarks and proposals relevant for specifying RF requirements for 6GR first.

Proposal 2.1: the specifications should be aligned with applicable ITU-R recommendations e.g. ITU-R Rec. SM-1539 and ITU-R for emissions in the OoB domain.

Proposal 2.2: the studies should be based on minimum performance requirements applicable in all regions to allow global circulation of terminals. Local regulatory requirements can still be accommodated similarly to NR.

Proposal 2.3: the specification of (RF) minimum requirements should be based on behaviour specified in RAN1 and RAN2 specifications. RAN4 specifications do not specify UE behaviour unless this is assigned by reference in the RAN1 and RAN2 specifications.

Timing and synchronisation and the relation to RF requirements

For each UE RF requirement discussion, the first objective should be whether the feature or function could be designed to operate effectively without synchronization or strict synchronization.

Observation 2.1: It is essential to conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Proposal 2.4: When RAN4 defines time and sync requirements, RAN4 should conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Proposal 2.5: When RAN4 defines time and synchronization requirements, it should consider system robustness and deployment flexibility. Features and functions should be designed to operate effectively with a base level of synchronization, ensuring reliable performance in common and typical scenarios, even without strict synchronization. Where possible, the design may optionally utilize strict synchronization to enhance performance or extend support to less common use cases in situations where such synchronization can be achieved.

Proposal 2.6: When RAN4 defines time and synchronization requirements, for requirement clarity link each requirement to specific features or functions, when needed clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints— and, where applicable, to specific UE capabilities.

UE reference architectures

RAN4 should specify 6GR requirements for RF and RRM based on realistic UE architecture assumptions.

Observation 2.2: Deployments can be collocated and non-colocated. Non colocated deployments, adds flexibility, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Observation 2.3: Aspects of UE architecture like FFT, searcher and LNA implementations put conditions on what kind of requirements UE can be achieved.

Proposal 2.7: RAN4 to discuss UE reference architectures in early stage of 6G. The UE RF requirements should be compatible with the UE reference architecture.

Proposal 2.8: RAN4 should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature.

Channel arrangement and relation to guard bands

The UE output power for meeting the unwanted emissions requirement for any necessary/channel bandwidth of a 6GR carrier or the bandwidth of new dynamic spectrum aggregation schemes will (also) depend on the internal GBs.

Observation 3.1: for GBs of aggregated spectrum, contiguous or non-contiguous, the outer internal GBs of a 6GR spectrum aggregation scheme supported within a single channel bandwidth could possibly be symmetric for both the BS and UE, contrary to intra-band contiguous NR CA for which the outer GBs of the BS CA bandwidth can be asymmetric. However, this may not be compatible with the necessary internal GB size for non-contiguous sub-blocks of unequal PRB size (fragmented spectrum) within the said single channel bandwidth.

Unwanted emissions for 6GR: general aspects

Proposal 3.1: 6GR studies should be based on maintaining the concept of a general SEM for compliance with regulatory emissions requirements and the ACLR specified for intra-system coexistence.

Proposal 3.2: to enable global circulation of terminals, the 6GR studies are based on maintaining the general NR spurious emissions limits for FR1 and FR2-1 based on ERC Rec. 74-01.

Output power accuracy

RAN4 output power requirements for 6GR should also be specified and verified for reference symbols.

Proposal 3.3: RAN4 collaborates with RAN1 from the start of the 6G study on the design of any reference symbols to be used for reciprocity-based DL-MIMO, if specified, and for coherent UL-MIMO with due account of expected UE transmit performance.

Spectrum aggregation for 6GR

In view of the specification of NR CA, the following proposals for the UE RF area:

Proposal 3.4: for spectrum aggregation, study the feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme (cf. the increase of the allowed MPR for the Pcell at NR CA configuration).

Proposal 3.5: for spectrum aggregation, the total configured power PCMAX, or equivalent power, should be related to the behaviour of the UE for any multi-carrier scheme specified. Changes of the total output power capability could be reported by (a)periodic dedicated signalling (cf. P-MPR reporting for NR).

Proposal 3.6: transmission on a single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction (cf. no power degradation for NR DL-only CA).

Transient times and relation to RRM requirements

Observation 3.2: NTA offset , Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together, since NTA offset≥Cell_Phase_Synchronization+TDD_transient_time

UE Power class

Different power classes should also be specified for 6GR: for FR1 power capabilities could be different for a given device type, whereas for FR2-1 the power class would depend on the device type similarly to NR.

Observation 4.1: The Power Class structure in NR has organically grown and become complex with the number of available Power Classes, different configured states they apply to (CA, UL MIMO etc.) including the supported number of Tx ports if applicable, different UE types, frequency ranges and network types (TN/NTN).

Proposal 4.1: Power classes should be defined for 6GR, but the complexities and contradictions of 5G should be improved. It is expected that 6GR power should also be limited by a configured maximum power per carrier and possibly a total UE output power for concurrent transmissions.

Observation 4.2: for a given NW configuration of the UE, the NW must be able to parse power capabilities from the UE capability transfer alone for a given configuration of the UE by the NW.

Proposal 4.2: Study how to design the UE capability framework on the supported UE power class for single carrier/CA, such that: (i) it is unambiguous for the NW and robust, so that it works for either single or multiple configured UL carriers; (ii) the number of needed UE capabilities is minimized.

Proposal 4.3: For PC3 and PC2, reuse the same notation from NR since those definitions are well established in the industry. For power classes higher than PC2, study how to come up with notation which is forward looking for even higher power classes in the future.

Improving UE TX modelling for MPR/A-MPR

Specification of MPR and A-MPR that is based on simulation assumptions relevant for state-of-the-art UE performance.

Proposal 5.1: for specification of transmitter minimum requirements, RAN4 to agree and document impairment models as well as all the requirements for simulation assumptions.

Observation 5.1: to meet the EVM requirement for 256QAM of 3.5%, the impairment models must not have distorted the signal more than -29.1dB in the IQ constellation. The IQ Image impairment model itself distorts the signal by -28dB.

Proposal 5.2:  Implicitly specify a Power Amplifier model, with or without Digital Pre-Distortion (DPD).

Receiver characteristics, general

Observation 6.1: ACS and OOBB requirements defined in the current specification ensure the UE receiver resilience against interference, which is increasingly critical in 6G.

Proposal 6.1: Retain the current ACS, OOBB and IBB requirements in 6G as a baseline.

Observation 6.2: There may be possibilities for simplification of requirements for low-capability devices, however it should depend on the outcomes of ongoing discussion on 6G device types, minimum capabilities and bandwidth requirements.

Further improvement of MSD

Proposal 6.2: MSD for band combinations are also measured OTA to complement the existing noise-factor (MSD) requirement since also relevant for support of simultaneous Rx-Tx for band combinations.

Low power receiver for 6G

Proposal 7.1: Wake up receiver should be in 6G scope from day 1.

Observation 7.2: UE autonomously enables/disables wake up receiver makes the power saving gain unpredictable.

Proposal 7.2: Coexisting study should be in 6G for wake up receiver to decide selectivity to avoid the coverage hold in presence of jammer.

Observation 7.3: Power saving and noise figure should be balanced in system level.

UE parameters for the 7-15 GHz range

In the scope of the 6G study, we propose that RAN4 should re-assess the outcomes of earlier studies for this range.

Proposal 8.1: RAN4 should further reassess the assumed UE parameters (e.g. UE noise figure) in the 7-15 GHz frequency range.

Proposal 8.2: RAN4 should further evaluate the UE antenna array parameters in the 7-15 GHz frequency range.

Proposal 8.3: RAN4 should further study considering phased array type of antenna in the 7-15 GHz frequency range for different UE type   (e.g. handheld, FWA, …).

UE EMC

Is a UE EMC specification needed for 6GNR?

Observation 9.1: The following two options could be considered for 6G EMC specification structure regarding UE:

- Option 1: Create a single, consolidated 6G UE EMC specification.

- Option 2: Discontinue developing 3GPP UE EMC specification for 6G and maintain legacy UE EMC specification (TS 36.124 and TS 38.124).

Proposal 9.1: RAN4 seeks feedback from UE vendors and other relevant stakeholders on the necessity of a dedicated 3GPP 6G UE EMC specification.

# References

1.    RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone