# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513127

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	8.7

Source: 	CMCC

Title: 			 Discussion on RRM for 6GR study

Document for:	Discussion

- Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1]. The objectives on RRM aspects for 6GR is highlighted in blue as following.

- (5)6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

In last RAN4 meeting, there was discussion about RAN4 6G topic planning based on 6G SID, and RAN4 6G plan and meeting arrangement was approved [2]. This meeting is RAN4 first meeting to discuss 6GR. This contribution provides discussion on RRM consideration for 6GR.

- Discussion

## 2.1 General consideration

There are many requirements defined in TS38.133. In general the requirements can be categorized as  RRC_IDLE state mobility, RRC_INACTIVE state mobility, RRC_CONNECTED state mobility, Timing, Signalling characteristics, Measurement Procedure, Measurement Performance requirements, V2X Requirements, NR Sidelink Measurements for Positioning, Measurement Performance Requirements for NR gNB. For 6GR, from the perspective of RRM, the focus is RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR. Based on above consideration, V2X Requirements, NR Sidelink Measurements for Positioning, etc, may not need to be considered in day-1.

Proposal 1: in general, it is proposed to consider following table as starting point for 6G study on RRM requirements and procedure aspects.

|  | Detail on RRM requirements and procedure aspects |
| --- | --- |
| RRC_IDLE state mobility | Cell re-selection, Idle Mode CA/DC Measurements, Measurement report for fast CA/DC setup, etc |
| RRC_INACTIVE state mobility | Cell re-selection, Inactive Mode CA/DC Measurements, Measurement report for fast CA/DC setup, etc |
| RRC_CONNECTED state mobility | Handover, Conditional Handover, RRC Re-establishment, Random access, L1/L2-Triggered Mobility, etc |
| Timing | UE transmit timing, UE timer accuracy, Timing advance, Cell phase synchronization accuracy, Maximum Transmission Timing Difference, Maximum Receive Timing Difference, etc |
| Signalling characteristics | Radio Link Monitoring, Interruption, SCell Activation and Deactivation Delay, Link Recovery Procedures, etc |
| Measurement Procedure | Measurement gap including gap pattern and gap type, UE Measurement capability, L3/L1 intra-frequency measurements, L3/L1 inter-frequency measurements, Inter-RAT measurements, etc |
| Measurement Performance requirements | RSRP/RSRQ/SINR accuracy requirements, etc |

## 2.2 Measurement gap

Measurement plays an important role in RAN4 RRM. Measurement enhancement, including multiple measurement gap types, introduced in different release in 5G. Per-UE gap and per-FR gap are the baseline. Inter-frequcy measurement without gap for the case that SSB is completely contained in the active BWP of the UE is introduced in Rel-16. NeedForGap is introduced by RAN2 in Rel-16, but the requirements are not specified until Rel-18. Concurrent MGs, Pre-MG, NCSG are introduced in Rel-17. Rel-18 focus on enhancements of pre-configured MGs, multiple concurrent MGs and NCSG, NR SSB-based inter-frequency and intra-frequency measurements without gaps, and inter-RAT measurement without gaps. XR transmission/reception in measurement gaps (MG skipping/cancelling) is supported in Rel-19.

Observation 1: Measurement enhancement, including multiple measurement gap types, are introduced in different release in 5G.

Some useful feaures.e.g. measurement without gaps, are fully supported in Rel-18 when the related RAN4 requirements are specified. Since the standardization support for this feature is late, as a result, the market support is not well. To avoid the same issue in 6GR, it is necessary that the feature with market demand are supported from 6G day-one.

Proposal 2: it is proposed that the feature with market demand are supported from 6G day-one.

From the perspective of operator, measurement without gaps is one of our interest, which is beneficial for network since the impact on throughput is reduced. In detail, there are two candidate solutions: NeedForGapNCSG, NeedForGap-InfoNR. The common part of these two solutions is that measurement without MG are performed by using the spare RF chain. The difference is that for the location of interruption for NCSG is clear between network and UE, which is the VIL of the NCSG pattern. While for NeedForGap, the interruption location are not specified, only maximum interruption ratio are defined.

As discussed above, NCSG is supported in Rel-17, and NeedForGap are introduced in Rel-16 but the requirements are not defined until Rel-18. As a results, the market support are not good. Another issue observed in 5G is that NeedForGapNCSG and NeedForGap-InfoNR are similar solutions, considering both solutions will  lead to WG workload increase and mentmarket fragment. It is necessary to focus on one solution.

Proposal 3: it is proposed that measurement without gaps, e.g. NCSG or NeedForGap are supported as mandatory from 6G day-one.

A related consideration is about interruption. For measurement without gap, no matter NeedForGapNCSG or NeedForGap-InfoNR, there is UE capability about whether there is interruption. Even though UE capability is introduced, the common case is assuming interruption is needed. On one hand, no measurement gap but with interruption will impact throughput. On the other hand, with or without interruption complex the specification, as duplicated as following as one example.

TS 38.133 The inter-frequency measurement requirements in clause 9.3.4 and clause 9.3.5 applies for the following scenarios: - SSB-based inter-frequency measurement object with measurement gap. - SSB-based inter-frequency measurement object without measurement gap for UE capable of interFrequencyMeas-NoGap, when - all of the SMTC occasions of this inter-frequency measurement object are overlapped with the measurement gap or associated measurement gap in concurrent measurement gaps, or - part of the SMTC occasions of this inter-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this inter-frequency measurement object are overlapped with the union of concurrent measurement gaps, or - part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or associated measurement gap in concurrent measurement gaps and the flag interFrequencyConfig-NoGap-r16 is not configured by the Network. - part of the SMTC occasions of this inter-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this inter-frequency measurement object are overlapped with the union of concurrent measurement gaps or the union of concurrent measurement gaps and MUSIM gaps if MUSIM gaps are configured. - no requirement applies if all of the SMTC occasions of this inter-frequency measurement object are overlapped with MUSIM gaps if MUSIM gaps are configured. - SSB-based inter-frequency measurement object without measurement gap for UE capable of NeedForInterruptionInfoNR-r18, when - all of the SMTC occasions of this inter-frequency measurement object are overlapped with the measurement gap or associated measurement gap in concurrent measurement gaps for the UE indicating ‘no-gap’ via NeedForGapsInfoNR and ‘no-gap-with-interruption’ or ‘no-gap-no-interruption’ via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement, or - part of the SMTC occasions of this inter-frequency measurement object are overlapped with the measurement gap or associated measurement gap in concurrent measurement gaps for the UE indicates ‘no-gap’ via NeedForGapsInfoNR and ‘no-gap-with-interruption’ via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement. The inter-frequency measurement requirements in clause 9.3.9 applies for the following scenarios: - SSB-based inter-frequency measurement with no measurement gap, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - SSB-based inter-frequency measurement with no measurement gap, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - SSB-based inter-frequency measurement with no measurement gap, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - SSB-based inter-frequency measurement with no measurement gap, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - for UE indicating NeedForInterruptionInfoNR-r18, when - none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps for the UE indicates ‘no-gap’ via NeedForGapsInfoNR and no-gap-with-interruption or no-gap-no-interruption via NeedForInterruptionInfoNR for the inter-frequency measurement. - part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, for the UE indicates ‘no-gap’ via NeedForGapsInfoNR and no-gap-no-interruption via NeedForInterruptionInfoNR for the inter-frequency measurement. - none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, for the UE indicates ‘no-gap’ via NeedForGap-InfoNR and ‘no-gap-with-interruption’ or ‘no-gap-no-interruption’ via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement. - part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, for the UE indicates ‘no-gap’ via NeedForGap-InfoNR for the inter-frequency measurement.

In our view, this is about UE implementation. We are not sure it is good to still have the same assumption as 5G, since it is expected 6GR is enhanced compared with NR. For 6GR, it is preferred not to consider interruption for measurement without gap, which is beneficial for throughput improvement, and also simplify RAN4 discussion and requirements definition.

Proposal 4: for 6GR, it is proposed to assume no interruption for measurement without gap including NCSG and NeedForGap.

Another important feature is parallel measurement. For NTN, parallel measurement is supported. In detail, parallelSMTC-r17 (Parallel measurements on multiple SMTC-s for a single frequency carrier), parallelMeasurementGap-r17 (Parallel measurements with multiple measurement gaps), parallelMeasurementWithoutRestriction-r17 (Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell), and maxNumber-NGSO-SatellitesWithinOneSMTC-r17 (Parallel measurements on multiple NGSO satellites within a SMTC) are considered. Taking parallelMeasurementGap-r17 (Parallel measurements with multiple measurement gaps) as an example, 2 is mandatory and 4 is optional. These parallel measurement for NTN need to be inherited from NR to 6GR.

For TN, concurrent measurement gap is introduced in Rel-17. Concurrent gaps enable different gap configurations for different MOs, which could avoid the resource waste due to configure one MG with long MGRP and/or short MGRP try to cover as much MOs as possible. Concurrent gaps is also helpful in reducing measurement delay. It is proposed to consider concurrent measurement gap from 6G day-1.

However, parallel measurement for the colliding cases is not supported. Two gaps are considered as colliding when there is fully or partially overlapping between two gaps or no overlapping but the distance between two gaps is less than 4ms. For the colliding case, only one gap is kept based on priority, the other gap is dropped. It is proposed to support Parallel measurement for colliding gaps. On one hand, it is beneficial for both throughput and measurement delay reduction. On the other hand, this issue is also about searcher assumptions. For 6GR, we believe the assumption about searchers will be enhanced compared with NR, which enable the parallel measurement.

Proposal 5: it is proposed to support concurrent measurement gaps from 6G day-1.

Proposal 6: it is proposed to support parallel measurement for the colliding measurement gaps.

Measurement gap has impact on throughput. During the measurement gaps, UE is not required to conduct reception/transmission from/to the serving cells except the reception of signals used for RRM measurement(s) and the signals used for random access procedure. How UE perform measurement is up to UE implementation, network has no information about whether measurement is performed in a certain measurement occasion. As a result, network will not schedule UE in any measurement gap occasion. For a typical measurement gap configuration with 40ms MGRP and 6ms MGL, the data loss is 15%.

Transmission/reception in measurement gaps (MG skipping/cancelling) is supported in Rel-19, but it is only limited to XR service. Dynamic indication is used to indicate whether to skip the first gap occasion after a minimum time offset required between the last symbol of the PDCCH carrying the indication and the start of corresponding skipped gap/restriction occasion indicated by the DCI. In addition, UAI about a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s is introduced. MG skipping/cancelling is a good mechanism which is beneficial for throughput improvement. It is proposed to consider MG skipping/cancelling in 6GR, and MG skipping/cancelling is considered as a generic approach, i.e. it is applied to all the service, not limit to XR.

Proposal 7: it is proposed to consider MG skipping/cancelling from 6G day-1, and MG skipping/cancelling is a generic approach, not limit to XR.

## 2.3 Definition on intra-frequency/ inter-frequency measurement

One lesson learned from NR is that RAN4 spend a lot effort in the definition of intra-frequency/inter-frequency measurement, but the actual impact is only about how to categorize requirements from RAN4 perspective. And the tough discussion is repeated again and again for different type of measurement. Following is the summary of definition on intra-frequency/ inter-frequency measurement.

SSB based L3 measurement A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same CSI-RS based L3 measurement A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement SSB based L1 measurement A measurement is defined as an SSB based intra-frequency L1-RSRP measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. CSI-RS based L1 measurement A measurement is defined as a CSI-RS based intra-frequency L1-RSRP measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - for SCS of 60KHz, the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

In our understanding, from RAN4 perspective, the original motivation to have definition on intra-frequency/ inter-frequency measurement is that it can be combined whether measurement gap is needed or not based on the measurement type. For LTE, for most cases, intra-frequency measurement means no measurement gap, inter-frequency measurement means measurement gap is needed. However, the situation is different for NR. Whether measurement gap is needed or not is not decided by the the intra-frequency measurement definition. The key point is whether target reference signal is within UE active BWP or not. Taking SSB based L3 measurement as an example, requirements are categorized into 4 cases: intra-frequency measurement without gap, intra-frequency measurement with gap, inter-frequency measurement without gap, inter-frequency measurement with gap. This categorization results in a lot workload when discussing enhancement in later release.

Observation 2: in NR, with the definition on intra-frequency/inter-frequency measurement, measurement requirements are categorized into 4 cases: intra-frequency measurement without gap, intra-frequency measurement with gap, inter-frequency measurement without gap, inter-frequency measurement with gap, which results in complex specification and increased RAN4 workload.

For 6GR, one general issue nee to be careful considered is how to handle the definition on intra-frequency/ inter-frequency measurement. Do we need the definition. If the definition is only about requirements categorization, maybe no definition is also OK. One option is that no definition on intra-frequency/ inter-frequency measurement. RRM requirements are categorized as measurement with gap and measurement without gap.

Proposal 8: it is proposed to discuss whether to have the definition on intra-frequency/ inter-frequency measurement in 6GR. Suggest to discuss whether following consideration is feasible

- Option 1: no definition on intra-frequency/ inter-frequency measurement. RRM requirements are categorized as measurement with gap and measurement without gap

## 2.4 Unified L1 measurement and L3 measurement

In NR, both L3 based mobility and L1/L2-Triggered Mobility are supported. L3 based mobility is robust, while LTM could reduce the latency, overhead. It is proposed to consider both L3 based mobility and L1/L2-Triggered Mobility from 6G day-1.

In NR, L3 based mobility and L1/L2-Triggered Mobility are independent. Taking intra-frequency measurements as an example, the requirements are duplicated as following. Table 9.2.5.2-1 is the requirements for L3 based measurement, Table 9.14.5.1-1 and Table 9.14.5.1-2 are the requirements for LTM.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is the configured periodicity for reporting. NOTE 2: K = 1.5. NOTE 3: The requirements apply when the max RTD between the serving cell and any candidate cell on which UE is required to perform L1-RSRP measurement on the same frequency layer is not larger than CP length of the cell on the frequency layer. |  |

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX*NLayer |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is the configured periodicity for reporting. NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with LTM-CSI-ResourceConfig-r18 + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with LTM-CSI-ResourceConfig-r18. NOTE 2: K = 1.5. NOTE 3: UE shall at least measure neighbour cells which have at least one TCI state on the LTM candidate cell active TCI state list within UE capability and additional cells to measure is up to UE implementation. |  |

It can be seen that the requirements are defined differently with different assumption, e.g. the whether to differentiate the relationship between RTD and CP, whether to scale with the number of intra-frequency layers. The L1 measurement delay may be even longer than L3 measurement delay. And it is also possible that UE supporting L3 measurement may not support L1 measurement for mobility.

Treating L3 based mobility and LTM separately results in a lot issues. The requirements are complex. UE behaviour is inconsistent. And additional UE power consumption is introduced due to redundant measurement. To avoid these issue, it is proposed to consider unified measurement framework for L3 measurement and L1 measurement for mobility.

Proposal 9:  for L3 measurement and L1 measurement for mobility, it is proposed to consider unified measurement framework and define unified measurement requirements.

## 2.5 Unified RLM/BFD/CBD

RLM, BFD, CBD are necessary functionalities to guarantee the radio link quality. However, the RLM, BFD, CBD are treated independently in NR.

In detail, RLM is performed on RLM-RS resources. On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin. The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and the out-of-sync block error rate (BLERout) is 10%. The threshold Qin is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout and the in-sync block error rate (BLERin) is 2%. Both thresholds are derived based on the hypothetical PDCCH transmission parameters. The evaluation period for in-sync is based on 5 samples, and the evaluation period for out-of-sync is based on 10 samples.

BFD resources are within set q0. On each RS resource configuration in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell beams. The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resource configuration on the set cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. The evaluation period for BFD is based on 5 samples

CBD resource are in the set q1. Upon request the UE shall deliver configuration indexes from the set to higher layers, and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR, which is indicated by higher layer parameter rsrp-ThresholdSSB. The evaluation period for CBD is based on 3 samples.

It can be seen that RLM, BFD, CBD are handled independently, even though all of them are related with radio link quality. As a result, the requirements are complex, UE behaviour is inconsistent, and introduce additional UE power consumption due to redundant measurement. To avoid these issue, it is proposed to consider unified measurement framework for RLM\BFD\CBD.

Proposal 10: it is proposed to consider unified measurement framework and define unified requirements for RLM, BFD, CBD.

## 2.6 Energy efficiency

From 6GR SID, the energy efficiency is the one of the baseline design principles of 6G technology framework.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

RAN1 achieved following related agreement, mainly about the power model, reference setting and evaluation metrics.

Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency. Agreement Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement - Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s) - Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

From RAN4 RRM perspective, from UE side, the L1/L3 measurement occupy the main energy consumption. From initial 5G design, the measurement requirements are bounded with DRX cycle configuration, which can help UE unified behavior in DRX ON/OFF period, target to energy efficiency purpose. Some further power saving features has been introduced in later releases, including the measurement relaxation in RRC IDLE/Inactive, RLM/BFD measurement relaxation, LP-WUR assistance and so on. All about measurement relax under good link quality and low mobility state.

For 6G Day1, we think all lessons from NR can be considered for measurement design of energy efficiency purpose, we list our initial consideration below. However, most of them need the PHY and higher layer support, RAN4 need to wait the RAN1/RAN2 progress and then refine our design strategy.

- Whether to continue the DRX cycle bounded measurement requirement in 6G

- Whether to integrate the link quality and mobility state with measurement requirement in 6G Day1

- Omitting repeated measurement behaviors. Unified L1/L3 measurement as we discussed in Clause 2.2 can be the starting point.

- Proposal 11: For UE energy efficiency, RAN4 need to wait the RAN1/RAN2 progress and then refine measurement design strategy, our initial thinking is as below:

- Whether to continue the DRX cycle bounded measurement requirement in 6G

- Whether to integrate the link quality and mobility state with measurement requirement in 6G Day1

- Merge repeated measurement behaviors. Unified L1/L3 measurement as we discussed in Clause 2.4 can be the starting point.

From RAN4 RRM perspective, from BS side, network energy saving attract 3GPP attention gradually, and several features have been introduced from Rel-18, including SSB-less SCell, cell DTX/DRX, OD-SSB SCell, OD-SIB1, adaptation of common signal/channel transmissions and so on, mainly about omit or sparse the transmission of common periodic reference signals, or replace periodic common reference signals with aperiodic reference signal or on demand reference signal.

We think the energy efficiency at BS side are highly related with RAN1/2 design. In RAN1#122 meeting, following agreements has been achieved. RAN4 shall continuously tracking the progress of PHY/higher layer design, and identify the feasible corresponding BS energy efficiency strategy in RAN4 scope.

Agreement

- Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

- Proposal 12: For BS energy efficiency, RAN4 need to wait the RAN1/RAN2 progress and then identify the feasible corresponding BS energy efficiency strategy which can be done within RAN4 scope

## 2.7 NTN

From 6GR SID, the harmonized 6G Radio design for TN and NTN is the one of the baseline design principles of 6G technology framework.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

RAN1 achieved following related agreement, the specific discussion on NTN agenda is still pending.

Agreement

- For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

In 5G, the RRM requirements for TN network and SAN were defined separately. Considering the large propagation delay difference and doppler difference caused by NTN network deployment, the main UE measurement difference between TN and NTN at least including the following:

- For NTN, One SMTC offset can not cover inter-frequency cells due to large propagation delay

- For NTN, on a single frequency carrier, limited number of SMTC that UE can process in parallel

- For NTN, limited number of MG that UE can process in parallel

- For NTN, within one SMTC, limited number of measurement on NGSO satellites that UE can process in parallel

- For NTN, on a single carrier, limited number of target NGSO satellites that UE can monitor

- For NTN, UE normal operation on serving cell in parallel with measurement on difference NGSO satellite depends on UE capability.

Towards 6G, the baseline UE measurement capability shall be reconsidered, which shall be further enhanced from NR assumption. From our perspective, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G.

Regarding the harmonized 6G Radio design for TN and NTN, from RAN4 RRM perspective, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control. Considering the continuous and extensive deployment of TN network, the measurement on TN carrier shall have higher priority as the default assumption.

- Proposal 13: Towards 6G, the baseline UE measurement capability shall be reconsidered, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G.

Proposal 14: Within the harmonized 6G Radio design for TN and NTN, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control, the measurement on TN carrier shall have higher priority as the default assumption.

- Conclusion

This contribution provides consideration on RRM for 6GR. The proposals are:

Proposal 1: in general, it is proposed to consider following table as starting point for 6G study on RRM requirements and procedure aspects.

|  | Detail on RRM requirements and procedure aspects |
| --- | --- |
| RRC_IDLE state mobility | Cell re-selection, Idle Mode CA/DC Measurements, Measurement report for fast CA/DC setup, etc |
| RRC_INACTIVE state mobility | Cell re-selection, Inactive Mode CA/DC Measurements, Measurement report for fast CA/DC setup, etc |
| RRC_CONNECTED state mobility | Handover, Conditional Handover, RRC Re-establishment, Random access, L1/L2-Triggered Mobility, etc |
| Timing | UE transmit timing, UE timer accuracy, Timing advance, Cell phase synchronization accuracy, Maximum Transmission Timing Difference, Maximum Receive Timing Difference, etc |
| Signalling characteristics | Radio Link Monitoring, Interruption, SCell Activation and Deactivation Delay, Link Recovery Procedures, etc |
| Measurement Procedure | Measurement gap including gap pattern and gap type, UE Measurement capability, L3/L1 intra-frequency measurements, L3/L1 inter-frequency measurements, Inter-RAT measurements, etc |
| Measurement Performance requirements | RSRP/RSRQ/SINR accuracy requirements, etc |

Observation 1: Measurement enhancement, including multiple measurement gap types, are introduced in different release in 5G.

Proposal 2: it is proposed that the feature with market demand are supported from 6G day-one.

Proposal 3: it is proposed that measurement without gaps, e.g. NCSG or NeedForGap are supported as mandatory from 6G day-one.

Proposal 4: for 6GR, it is proposed to assume no interruption for measurement without gap including NCSG and NeedForGap.

Proposal 5: it is proposed to support concurrent measurement gaps from 6G day-1.

Proposal 6: it is proposed to support parallel measurement for the colliding measurement gaps.

Proposal 7: it is proposed to consider MG skipping/cancelling from 6G day-1, and MG skipping/cancelling is a generic approach, not limit to XR.

Observation 2: in NR, with the definition on intra-frequency/inter-frequency measurement, measurement requirements are categorized into 4 cases: intra-frequency measurement without gap, intra-frequency measurement with gap, inter-frequency measurement without gap, inter-frequency measurement with gap, which results in complex specification and increased RAN4 workload.

Proposal 8: it is proposed to discuss whether to have the definition on intra-frequency/ inter-frequency measurement in 6GR. Suggest to discuss whether following consideration is feasible

- Option 1: no definition on intra-frequency/ inter-frequency measurement. RRM requirements are categorized as measurement with gap and measurement without gap

Proposal 9:  for L3 measurement and L1 measurement for mobility, it is proposed to consider unified measurement framework and define unified measurement requirements.

Proposal 10: it is proposed to consider unified measurement framework and define unified requirements for RLM, BFD, CBD.

- Proposal 11: For UE energy efficiency, RAN4 need to wait the RAN1/RAN2 progress and then refine measurement design strategy, our initial thinking is as below:

- Whether to continue the DRX cycle bounded measurement requirement in 6G

- Whether to integrate the link quality and mobility state with measurement requirement in 6G Day1

- Merge repeated measurement behaviors. Unified L1/L3 measurement as we discussed in Clause 2.4 can be the starting point.

- Proposal 12: For BS energy efficiency, RAN4 need to wait the RAN1/RAN2 progress and then identify the feasible corresponding BS energy efficiency strategy which can be done within RAN4 scope

- Proposal 13: Towards 6G, the baseline UE measurement capability shall be reconsidered, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G.

Proposal 14: Within the harmonized 6G Radio design for TN and NTN, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control, the measurement on TN carrier shall have higher priority as the default assumption.

- Reference

- RP-251881, New SID: Study on 6G Radio

- R4-2511652, RAN4 6G Plan and Meeting Arrangement