3GPP TSG-RAN WG4 Meeting #116bis	R4-2513137

Prague, Czech, Oct. 13 – 17, 2025

Source: 	ZTE Corporation, Sanechips

Title: 	Views on 6G RAN4 operation efficiency

Agenda item:	8.13

Document for:	Approval

- 1 Introduction

An SID on 6G Radio was approved and further revised in [1] to initiate the study on developing a 6G radio access technology. RAN4 6G topic planning based on 6G SID was provided in [2]. In this contribution, we give some initial views on the topic for 6GR RAN4 operation efficiency.

- 2	Discussion

  - Spec improvement and modernization

2.1.1	UE specification improvement

- Band combination tables

< Agreement >:

- RAN4 endorse also the inclusion of ENDC/NEDC band combinations in the database.

  - Meaning all combinations now captured by TS 38.101-1/2/3

- RAN4 intends to transition to fully use the database and remove the current Docx table-based representation of supported band combinations in Rel-20

In 5G, it is highlighted that tens of thousands of band combinations have been specified in UE specifications. The combinations are inefficiently stored in the tables in Word files. At RAN#102, it was decided to develop a new tool for handling CA band combinations within RAN4. In [3], a WF on band combination database was approved with the following agreements.

According to the WF, the CA/EN-DC/NE-DC band combinations will be transitioned from Docx table to band combination database (https://forge.3gpp.org/rep/ran4/carrieraggregation) in Rel-20.

Observation 1: In RAN4, it intends to transition 5G CA/EN-DC/NE-DC band combinations from Docx table in the specifications to database in Rel-20.

Proposal 1: It is suggested that the band combinations in 6GR follow Rel-20 5G not to store the band combinations in the specifications but store in the database.

- UE RF Specification structure in 6GR

In the 5G RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. The features are currently defined in the spec with the suffixes shown as Table 2.1.1-1. For shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause. For V2X, suffixes A and F are used for indicating at 3rd level clause.

Table 2.1.1-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |

The following Table 2.1.1-2 briefly summarizes the RF requirements for each feature defined in different clause suffixes in TS 38.101-1.

Table 2.1.1-2: RF requirements for each feature in different clause suffixes in TS 38.101-1

|  | Single carrier | CA | DC | SUL | UL-MIMO | V2X | - Shared spectrum channel access | TxD | CA+UL MIMO | (e)RedCap | ATG | Aerial UE | CA+TxD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section 6 Transmitter characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General | 6.1 | 6.1A |  |  |  |  | 6.1F |  |  |  | 6.1J |  |  |
| 6.2 Transmitter power |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE maximum output power | 6.2.1 | 6.2A.1 | 6.2B.1 |  | 6.2D.1 | 6.2E.1 | 6.2F.1 | 6.2G.1 | 6.2H.1/3.1 | 6.2I.1 | 6.2J.1 | 6.2K.2 | 6.2L.3.1 |
| UE maximum output power reduction | 6.2.2 | 6.2A.2 | 6.2B.2 |  | 6.2D.2 | 6.2E.2 | 6.2F.2 | 6.2G.2 | 6.2H.1/3.2 |  |  | 6.2K.3 | 6.2L.3.2 |
| UE additional maximum output power reduction | 6.2.3 | 6.2A.3 | 6.2B.3 |  | 6.2D.3 | 6.2E.3 | 6.2F.3 | 6.2G.3 | 6.2H.1/3.3 |  |  | 6.2K.4 | 6.2L.3.3 |
| Configured transmitted power | 6.2.4 | 6.2A.4 | 6.2B.4 | 6.2C.1 | 6.2D.4 | 6.2E.4 | 6.2F.4 | 6.2G.4 | 6.2H.1/3.4 |  | 6.2J.2 |  | 6.2L.3.4 |
| 6.3 Output power dynamics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum output power | 6.3.1 | 6.3A.1 | 6.3B |  | 6.3D.1 | 6.3E.1 | 6.3F.1 | 6.3G.1 | 6.3H.1/3.1 |  | 6.3J.1 |  | 6.3L.3.1 |
| Transmit OFF power | 6.3.2 | 6.3A.2 |  |  | 6.3D.2 | 6.3E.2 | 6.3F.2 | 6.3G.2 | 6.3H.1/3.2 |  | 6.3J.2 |  | 6.3L.3.2 |
| Transmit ON/OFF time mask | 6.3.3 | 6.3A.3 |  | 6.3C.3 | 6.3D.3 | 6.3E.3 | 6.3F.3 | 6.3G.3 | 6.3H.1/3.3 |  | 6.3J.3 |  | 6.3L.3.3 |
| Power control | 6.3.4 | 6.3A.4 |  |  | 6.3D.4 | 6.3E.4 | 6.3F.4 | 6.3G.4 | 6.3H.1/3.4 |  | 6.3J.4 |  | 6.3L.3.4 |
| 6.4 Transmit signal quality |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency error | 6.4.1 | 6.4A.1 | 6.4B | 6.4C | 6.4D.1 | 6.4E.1 | 6.4F.1 | 6.4G.1 | 6.4H.1/3.1 |  | 6.4J.1 |  | 6.4L.3.1 |
| Transmit modulation quality | 6.4.2 | 6.4A.2 |  |  | 6.4D.2 | 6.4E.2 | 6.4F.2 | 6.4G.2 | 6.4H.1/3.2 |  | 6.4J.2 |  | 6.4L.3.2 |
| 6.5 Output RF spectrum emission for CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Occupied bandwidth | 6.5.1 | 6.5A.1 | 6.5B |  | 6.5D.1 | 6.5E.1 | 6.5F.1 | 6.5G.1 | 6.5H.1/3.1 |  | 6.5J.1 | 6.5K.1 | 6.5L.3.1 |
| Out-of band emission | 6.5.2 | 6.5A.2 |  |  | 6.5D.2 | 6.5E.2 | 6.5F.2 | 6.5G.2 | 6.5H.1/3.2 |  | 6.5J.2 | 6.5K.2 | 6.5L.3.2 |
| Spurious emission | 6.5.3 | 6.5A.3 |  |  | 6.5D.3 | 6.5E.3 | 6.5F.3 | 6.5G.3 | 6.5H.1/3.3 |  | 6.5J.3 | 6.5K.3 | 6.5L.3.3 |
| Transmit intermodulation | 6.5.4 | 6.5C.4 |  |  | 6.5D.4 | 6.5E.4 | 6.5F.4 | 6.5G.4 | 6.5H.1/3.4 |  |  |  | 6.5L.3.4 |
| 6.6 Time alignment error |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time alignment error |  |  |  |  | 6.4D.3 | 6.6E |  |  | 6.4H.1.3 |  |  |  |  |
| Section 7 Receiver characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General | 7.1 | 7.1A |  |  |  |  | 7.1F |  |  | 7.1I | 7.1J |  |  |
| Diversity characteristics | 7.2 |  |  |  |  |  |  |  |  |  | 7.2J |  |  |
| Reference sensitivity | 7.3 | 7.3A | 7.3B | 7.3C | 7.3D | 7.3E | 7.3F | 7.3G |  | 7.3I | 7.3J |  |  |
| Maximum input level | 7.4 | 7.4A | 7.4B |  | 7.4D | 7.4E |  |  |  |  | 7.4J |  |  |
| Adjacent channel selectivity | 7.5 | 7.5A | 7.5B |  | 7.5D | 7.5E | 7.5F |  |  |  | 7.5J |  |  |
| 7.6 Blocking characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In-band blocking | 7.6.2 | 7.6A.2 | 7.6B | 7.6C.2 | 7.6D | 7.6E.2 | 7.6F.2 |  |  |  | 7.6J.2 |  |  |
| Out-of-band blocking | 7.6.3 | 7.6A.3 |  | 7.6C.3 |  | 7.6E.3 | 7.6F.3 |  |  |  | 7.6J.3 |  |  |
| Narrow band blocking | 7.6.4 | 7.6A.4 |  | 7.6C.4 |  | 7.6E.4 | 7.6F.4 |  |  |  |  |  |  |
| Spurious response | 7.7 | 7.7A | 7.7B |  | 7.7D | 7.7E | 7.7F |  |  |  | 7.7J |  |  |
| Intermodulation | 7.8 | 7.8A | 7.8B |  | 7.8D | 7.8E | 7.8F |  |  |  | 7.8J |  |  |
| Spurious emissions | 7.9 | 7.9A | 7.9B |  |  |  |  |  |  |  | 7.9J |  |  |

Using single carrier requirements as the benchmark, we observe that different approaches are used for different features:

-  Several (sub-)clause suffixes are missing for a certain feature, for example for (e)RedCap, there are only three clause suffixes defined.

-  For some other features such as NR CA/DC, each requirement has its corresponding (sub-)clause suffix.

Although some of the (sub-)clause suffixed are marked as ‘Reserved’, we are not fully understanding what is the exact meaning. We could not find the reason why the descriptions are missing in the specification. Then it would cause confusion for the reader who didn’t follow the feature discussion as there are at least two following interpretations:

a).	To reuse the existing requirements as single carrier feature.

b).	The requirements without clause suffix do not apply to that feature.

The above two interpretations are completely opposite in the understanding. We believe the interpretation a) is the main reason for not explicitly define the requirements. However, there are no such descriptions in the specification. When we look at NR DC feature, all of requirements are explicitly define although almost all of the RF requirements are reused from the NR CA requirements. But we also believe there are exceptions that a certain requirement is not applied, for example, MPR requirements are not applied to ATG, so there is no MPR sub-clause defined.

Observation 2: In 5G UE RF specification, different approaches are adopted for different features in the structure. Meanwhile, there are too many (sub-)clause suffixes not defined for RF requirements.

Observation 3: There exist different interpretations for the missing (sub-)clause suffixes requirements which causes misunderstanding.

For 6GR, we think the above issue for the specification improvement should be taken into account. When introducing a new feature into the specification, some general guidance/rules for uniform drafting are required to define the RF requirements.

Proposal 2: It is proposed in 6GR to discuss uniform drafting guidance/rules for the RF requirements definition when introduction of a new feature in the specification.

In addition to that, it is noted that there are also some second level features further recursively categorized into 3rd level features. For example, for shared spectrum channel access, suffices A, B and D are used for indicating at 3rd level sub-clause as shown in Figure 2.1.1-1. Such a specification structure causes poor readability.

Figure 2.1.1-1: An example for recursive sub-clause structure in the spec

The recursive sub-clause structure also appears in some other features such as V2X, etc. It is foreseeable that similar situations will occur more frequently in 6GR if the requirements for other features are applied recursively. One problem for such recursive structure is that it is hard to compare the requirements among the different features. For example, to compare transmitter power requirements between single carrier and the feature for CA with Tx Diversity, the number of the pages in the same document spans up to 225 pages!

Figure 2.1.1-2: Illustration of large page-span among features

Observation 4: In 5G UE RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. However, there are also some second level features further recursively categorized into 3rd level features. This causes the spec poor readability.

Proposal 3: In 6GR, it is proposed to study the UE RF spec structure by the requirements with different features.

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18,....

Additionally, in the existing 5G NR specification, we found the descriptions for some RF requirements are quite complicated and difficult to understand. For example, for the (e)RedCap UE REFSEN requirements as following. The IE names were repeated again and again although RAN2 has a clear definitions for (e)RedCap, RAN4 still use a complicated texts in the specification.

There are some other requirements have the similar situation in the specification. Usually these descriptions use too much RAN2 IE signallings/language but sometimes it may duplicated even discrepancy between RAN2 and RAN4 understanding. We think we should try to reduce the usage of RAN2 language as much as possible in RAN4 specification.

Proposal 4: In 6GR, it is proposed to reduce the usage of RAN2 language in RAN4 specification as much as possible.

2.1.2	BS specification improvement

In the Rel-19, RAN tasked RAN4 to build the framework for the simplification for co-existence and co-location requirement for RAN4 BS specification as listed in Table 2.1.2-1 in order to reduce the lots of duplicated CRs for any new NR/LTE bands introduction [4].

Table 2.1.2-1: The list of BS specifications

| Specification (TS) |  |
| --- | --- |
| 38.104 | NR BS Core |
| 38.141-1 | NR BS Conf - Conducted |
| 38-141-2 | NR BS Conf - OTA |
| 38.106 | NR Repeater - Core |
| 38.115-1 | NR Repeater -Conf Cond. |
| 38.115-2 | NR Repeater -Conf OTA No impact |
| 38.174 | IAB Core |
| 38.176-1 | IAB Conf Conducted |
| 38.176-2 | IAB Conf OTA |
| 37.105 | AAS BS Core |
| 37.145-1 | AAS BS Conf Conducted |
| 37.145-2 | AAS BS Conf OTA |
| 37.104 | MSR BS Core |
| 37.141 | MSR BS Conf |
| 36.104 | E-UTRA BS Core |
| 36.141 | E-UTRA BS Conf |
| 36.106 | E-UTRA Repeater Core |
| 36.143 | E-UTRA Repeater Conf |

Proposal 5: It is proposed to leverage the Rel-19 RAN task for the simplification for co-existence and co-location requirements for 6GR BS specification.

In addition, in the 6G phase regardless the support of these network node in 6G day1 or day2, the various network nodes are also expected to be specified similar as 5G NR, then if we still follow the legacy approach to define the RF requirements for different network nodes in the different specifications, then it would also face the similar situation as 5G NR BS specification e.g. duplicated information for TRP measurement, EVM measurement, test mode/configuration and test procedures across the different specifications, in order to simplify the workload and maintenance burden for the BS specification, we also propose to consider how to capture the same requirements or test procedures across different network nodes if there are many similarities just with some item/notation difference.

Proposal 6: It is proposed to discuss how to capture the same requirements (e.g. TRP measurement, EVM measurement, test mode/configuration, OTA test chamber) or test procedures across different network nodes specifications if there are many similarities just with some items/notation difference.

2.1.3	RRM specification improvement

- Specification framework improvement

Compared with the existing 5G framework in TS 38.133, we re-build the structure to some extent. The main consideration behind is to realize more clear structure from the perspective of RRM procedure. One obvious update on top of existing 5G framework is to combine the interruption requirements into each RRM procedure respectively, so that the requirements relevant to each RRM procedure are completed with the interruption requirements involved in. Another consideration is to separate the multi carrier/cell management from the other signalling characteristics, as the PSCell/SCell management are vital components of CA/DC feature if applicable in 6GR, it is better not to couple with other UE-specific characteristic switching operation. The traditional UE-specific characteristic switching, such as TCI state switching, CBW change, PL-RS switching etc. if applicable by PHY design in 6GR.

Proposal 7: Compared with the existing 5G framework in TS 38.133, construct the overall blueprint of RRM for 6GR with more clear structure from the perspective of RRM procedure. The following framework is preferred:

-    RRC_IDLE/INACTIVE state mobility

-    RRC_CONNECTED state mobility

-    Timing

-    Measurement procedure for RRC_CONNECTED state

-    RLM/BFD/CBD

-    PSCell/SCell management (if applicable by PHY/high layer design in 6GR)

-    Other UE-specific characteristic switching (if applicable by PHY design in 6GR)

-    Measurement performance

- Specification readability improvement

In Rel-18, Specification improvement has been discussed and made some progress on editorial modifications. When we study the operation efficiency in 6GR, the NR protocol has undergone discussions focusing on editorial modifications in Rel-18 intended to improve specification can be used as the baseline for future optimization toward 6G.

Proposal 8: Editorial modifications in NR can be used as the baseline for future optimization toward 6G.

Currently, RRM Specification is characterized by inconsistent indentation, which significantly impacts readability due to everyone can modify the protocol and easily break the correct hierarchical indentation. Moreover, redundant requirements present across various sections of the protocol, leading to unnecessary duplication and increased cognitive load for readers. To further enhance the readability of specifications, we suggest considering the following optimizations during the 6G.

(1)  Improve readability through structured indentation

The existing protocol structure suffers from a lack of clear indentation guidelines, which results in significant confusion and diminished readability for users attempting to navigate the specification. In the RAN4 RRM protocol, a large part of the indentation is chaotic in the current protocol, and it is very easy to break the original hierarchical indentation during the process of modifying the protocol, thereby increasing the difficulty of reading. To make it more clear, we need to label the indentation section, such as using RAN2 pseudo-code to indicate indentation levels through 1>, 2>, 3>, etc. In this case, even if there is no alignment on the text, the indentation level corresponding to this paragraph can be determined based on the numbering, increasing the readability of the specification.

Proposal 9: Improve readability through structured indentation, such as using RAN2 pseudo-code to indicate indentation levels through 1>, 2>, 3>, etc.

(2)  Reduce redundancy by referencing common requirements

Duplicated requirements scattered throughout different sections contribute to inefficiencies during both the writing and reading processes. In order to reduce redundant descriptions, for the core/performance requirements, if the newly introduced requirements can be applied to multiple scenarios (using identical requirements in multiple scenarios), it is recommended to provide detailed descriptions in a typical scenario, and directly refer to the typical scenario in other scenarios. This approach can avoid repeated descriptions of a certain feature in different positions of the same protocol, effectively reducing readers' reading volume.

For test cases, if modifications are made based on existing test cases, only the different parts can be reflected. For parts that are the same as legacy, it is recommended to directly refer to the original test case. Taking less than 5MHz as an example, to avoid duplicate descriptions, a simplified test case writing approach was used to reduce the overhead caused by the test case parameters and test steps. The main difference between traditional approach and simplified test approach is that only changed parameters are specified when compared to the baseline table.

The test procedure in clause A.6.6.1.5.2 applies for this test. Supported test configurations are specified in table A.6.6.1.12.2-1. General test parameters as specified in table A.6.6.1.5.2-2 with config 1 apply to this test, except those specified in table A.6.6.1.12.2-2. Cell specific test parameters as specified in table A.6.6.1.5.2-3 with config 1 apply to this test, except those specified in table A.6.6.1.12.2-3.

Proposal 10: For the same parts of the requirements, reduce redundancy by referencing common descriptions instead of repeating descriptions in multiple places.

(3)  Easier navigation with requirement-test case mapping

In the optimization of 6G specifications, we are particularly interested in mapping approach between core/performance requirement and test cases, which has been proposed in NR and the following conclusions were reached in the previous discussion on Rel-18.

In principle it is agreed to include references or mapping tables in the core part requirements that point to the relevant test cases from Rel-19 - FFS on the feasibility and the detailed guideline for drafting rules in future meetings.

We think this is a great proposal for readability. As for TS 38.133, the introduced test cases have already taken up a lot of space. For readers who are not familiar with the specific topic, it takes a long time to locate the test cases corresponding to the relevant requirements. Although the description of each test case will indicate which chapter is being validated, it will not reduce the reader's search time. If we could introduce a new chapter to indicate the labeling relationship between requirements and test cases, similar to a table of contents, it would be more convenient for searching and reading.

Proposal 11: Include references or mapping tables in the core part requirements that point to the relevant test cases in 6G.

- CR handling improvement

In previous discussions, CR is usually drafted in two meetings before the end of the project, and limited time may not guarantee that the requirements are captured correctly. So in order to improve the quality of CR writing, we need to start with more effort. To ensure consistency in the expression of the same topic, the moderator can first provide a draft CR for a certain section, and other sections can be completed the requirement description according to similar ideas and expressions by other companies, which can ensure uniformity in terminology and structure.

Proposal 12: To ensure consistency in terminology and structure within the same topic, a partial initial draft template can be provided before the overall drafting.

In addition, we found that many maintenance CRs were submitted in the maintenance phase, which brought a huge workload to the specification updates and may cause some CRs to not be captured to the latest version. In order to complete the specification update as comprehensively as possible while controlling the number of maintenance CRs, we suggest that only one maintenance CR for one source for one specification for the same WI code.

Proposal 13: To complete the specification update as comprehensively as possible while controlling the number of maintenance CRs, one maintenance CR can be made for one source within a single specification under the same work WI code.

2.2	New tools for band combinations

In 5G, new tools such as band combination database have been developed in RAN4. From the perspective of further improving the standardization efficiency, it seems more intelligent tools are expected.

- Band combination fallback checking tools

Currently in 5G, there are some guidelines on requesting new band combinations. The proponents should propose all the necessary fallback modes together with the proposed band combinations. All the next level fallback configurations shall be reported and the status of all the fallback configurations shall be declared. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination. A band combination configuration can only be considered as completed when all of the fallback configurations are completed. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Although rapporteurs and other companies are encouraged to check the status of all fallback configurations, there are still many fallback band combinations missing in the specification. To check the missing fallback band combinations is a quite time-consuming task. If an automatic checking fallback tool can be developed, the RAN4 efficiency of band combination will be greatly improved.

Proposal 14: In 6GR, it is proposed to develop an automatic checking tool for fallback band combinations in RAN4.

2.3	Other aspects related to operation efficiency

For four generations, 3GPP specifications have been developed using Microsoft Word with the guidelines outlined in TR 21.801 across different working groups. However, due to the increasing number of specifications, increasing number of Change Requests (CRs) and increasing specification files sizes, the handling of specifications is becoming more and more difficult which leads to low efficiency in readability. For example in RAN4, a mechanism of “Draft big CR” in the post meeting handling was introduced to solve the issue of large number of “Draft CR” for basket WID.

To ensure a more reliable, modern and efficient approach to the new Generation specifications in 3GPP, an SID was established in [5] for the study in 6G. The purpose of the SI is to assess the existing specification formats, working methods, and requirements for any improvements.

In RAN4, Word is currently used for writing specifications and for working on CRs/pCRs in the meetings. Additional tools such as Visio is used for drawing figures which are embedded in Word. In some cases, Excel spreadsheets are zipped together with specification in the separate files to collect the evaluation results.

Observation 5: In RAN4, Word is currently used for writing specifications and for working on CRs/pCRs in the meetings. Additional tools such as Visio for figures and Excel spreadsheets are used in some cases.

However, Word is increasingly questioned with large file size due to the Word file is actually a ZIP archive containing XML files, embedded resources (images, fonts), and extensive metadata (styles, revisions, author info), etc. Recently, some voices have occurred in 3GPP to use Markdown to write specifications instead of Word. Markdown is a lightweight markup language which uses plain text formatting with simple syntax. Furthermore, for the formulas in the specifications, if LaTeX with plain-text command input could be a potential tool instead of Microsoft Word Equation?

Observation 6: Recently some voices appeared in 3GPP to check if using

-    Markdown be feasible for specifying specifications.

-    LaTeX be feasible for writing formulas.

Proposal 15: Although final decision on whether using Markdown or LaTeX in 6GR will be decided by SA/RAN plenary, it is suggested RAN4 to study the possible influence to RAN4.

- 3	Conclusion

In this contribution, we give some initial views on the 6GR RAN4 operation efficiency. Issues on spec improvement and modernization, new tools for band combinations and other aspects related to operation efficiency have been discussed. The conclusions are summarized as below:

Observation 1: In RAN4, it intends to transition 5G CA/EN-DC/NE-DC band combinations from Docx table in the specifications to database in Rel-20.

Proposal 1: It is suggested that the band combinations in 6GR follow Rel-20 5G not to store the band combinations in the specifications but store in the database.

Observation 2: In 5G UE RF specification, different approaches are adopted for different features in the structure. Meanwhile, there are too many (sub-)clause suffixes not defined for RF requirements.

Observation 3: There exist different interpretations for the missing (sub-)clause suffixes requirements which causes misunderstanding.

Proposal 2: It is proposed in 6GR to discuss uniform drafting guidance/rules for the RF requirements definition when introduction of a new feature in the specification.

Observation 4: In 5G UE RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. However, there are also some second level features further recursively categorized into 3rd level features. This causes the spec poor readability.

Proposal 3: In 6GR, it is proposed to study the UE RF spec structure by the requirements with different features.

Proposal 4: In 6GR, it is proposed to reduce the usage of RAN2 language in RAN4 specification as much as possible.

Proposal 5: It is proposed to leverage the Rel-19 RAN task for the simplification for co-existence and co-location requirements for 6GR BS specification.

Proposal 6: It is proposed to discuss how to capture the same requirements (e.g. TRP measurement, EVM measurement, test mode/configuration, OTA test chamber) or test procedures across different network nodes specifications if there are many similarities just with some items/notation difference.

Proposal 7: Compared with the existing 5G framework in TS 38.133, construct the overall blueprint of RRM for 6GR with more clear structure from the perspective of RRM procedure. The following framework is preferred:

-    RRC_IDLE/INACTIVE state mobility

-    RRC_CONNECTED state mobility

-    Timing

-    Measurement procedure for RRC_CONNECTED state

-    RLM/BFD/CBD

-    PSCell/SCell management (if applicable by PHY/high layer design in 6GR)

-    Other UE-specific characteristic switching (if applicable by PHY design in 6GR)

-    Measurement performance

Proposal 8: Editorial modifications in NR can be used as the baseline for future optimization toward 6G.

Proposal 9: Improve readability through structured indentation, such as using RAN2 pseudo-code to indicate indentation levels through 1>, 2>, 3>, etc.

Proposal 10: For the same parts of the requirements, reduce redundancy by referencing common descriptions instead of repeating descriptions in multiple places.

Proposal 11: Include references or mapping tables in the core part requirements that point to the relevant test cases in 6G.

Proposal 12: To ensure consistency in terminology and structure within the same topic, a partial initial draft template can be provided before the overall drafting.

Proposal 13: To complete the specification update as comprehensively as possible while controlling the number of maintenance CRs, one maintenance CR can be made for one source within a single specification under the same work WI code.

Proposal 14: In 6GR, it is proposed to develop an automatic checking tool for fallback band combinations in RAN4.

Observation 5: In RAN4, Word is currently used for writing specifications and for working on CRs/pCRs in the meetings. Additional tools such as Visio for figures and Excel spreadsheets are used in some cases.

Observation 6: Recently some voices appeared in 3GPP to check if using

-    Markdown be feasible for specifying specifications.

-    LaTeX be feasible for writing formulas.

Proposal 15: Although final decision on whether using Markdown or LaTeX in 6GR will be decided by SA/RAN plenary, it is suggested RAN4 to study the possible influence to RAN4.

- 4	Reference

[1]	RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] 	R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair

[3] 	R4-2511804, WF for the ETSI MCC band combination database, Nokia

[4] 	R4-2504741, Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex

[5] 	SP-250802, New SID on Modernization of Specification Format and Procedures for 6G, Nokia, Samsung, CMCC