3GPP TSG-RAN4 Meeting #116bis	R4-2513146

# Prague, Czech Republic 13th – 17th October 2025

Title:	Views on 6G coexistence studies in RAN4

Source:	Qualcomm Incorporated

Agenda item:	8.5

Document for:	Approval

# Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on future 6G adjacent channel coexistence studies within RAN4.

# General discussion

Historically, adjacent channel coexistence studies have been carried out in RAN4 to derive some of the Tx/Rx RF requirements (e.g., ACLR/ACS, etc.). For 6G-related spectrum, RAN4 studied the different RF parameters and the BS AAS parameters for three different frequency ranges (4 GHz, 8GHz, and 15 GHz) to provide a response to WP5D regarding WRC-27 sharing studies in ITU-R in [2]. In [2], adjacent channel coexistence studies were only conducted for the 15GHz since it represents the only frequency range, where previous RAN4 coexistence work could not be reused/ extended (i.e., 4 GHz parameters were based on defined band n79, while 8GHz parameters were based on n104 adjacent channel coexistence studies that were done in Rel-17 study item, documented in TR 38.921).

Observation 1: In TR 38.922, RAN4 has documented the RF and AAS parameters for the frequency ranges 4.4-4.8 GHz, 7.125-8.4 GHz, and 14.8-15.35 GHz, where adjacent channel coexistence work was done only for 14.8-15.35 GHz.

With the start of the 6G SI [1], it is proposed to discuss within RAN4 if additional adjacent coexistence studies are needed or not. Additional adjacent channel coexistence would provide new insights if improvements to the simulation framework/ methodology are first identified.

Proposal 1: RAN4 to discuss if additional coexistence studies consistent with the work conducted in TR 38.922 are needed for the 6G SI.

In the following sections, we will highlight our views on the different aspects that could be further investigated to enhance the RAN4 adjacent channel coexistence methodology.

## Potential simulation methodology improvements

### BS modelling

Since the initial NR study item, the AAS model described in Section 7.7 of TR 38.922 has been used for numerous studies in RAN4, including AAS, NR WI, IAB, etc. The model has been adopted in other forums outside RAN4 and is also described in RAN1 and in ITU-R in recommendation M.2101. It is noteworthy that the baseline model given in M.2101 has been extended to account for subarray architectures. Beside the model, one of the main AAS parameters is base station horizontal and vertical coverage/ steering range. The horizontal steering range is typically set to +/- 60 degrees to address the typical tri-sectorization of the cell site. For the vertical steering range, it typically allows 10-20 degrees of vertical for macro urban and suburban deployments, while larger steering range is allowed for indoor wall-mounted ceiling deployments.

While utilizing the abovementioned AAS model (i.e., extended M.2101 with subarrays), the beamforming weights are generated in each snapshot typically depending on the vertical and horizontal steering angles, derived based on the location of the transmitter and receiver. Perfect location of the Tx and Rx is typically assumed (i.e., full CSI knowledge), and beamforming gain is computed according to the model. It is probable that depending on the location of either the Tx or Rx, the generated beam might be outside of the maximum steering range. Based on the adopted model, further clarification is needed within RAN4 on how to address the cases where the generated beam might be outside of the steering range. We believe that such modeling detail should be further studied and clarified in 6G coexistence studies to ensure that the generated beams from the adopted beamforming model mimics what is happening in realistic scenarios.

Observation 2: If additional coexistence studies are agreed in RAN4, RAN4 could study how to harmonize in the coexistence framework the incorporation of the parameterized BS AAS steering limits in 6G coexistence studies.

With the introduction of AAS, the topic of how to model the AAS radiation pattern in the out-of-band adjacent domain has been investigated in different study items in RAN4. In the NR study item, documented in TR 38.803, RAN4 has reached the following conclusion in section 5.2.6:

it is assumed that both ACLR (or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as (assuming ACLR, ACS and ACIR to be linear)

The above conclusion leads to the assumption within the coexistence framework that the total amount of adjacent channel power radiated that matters in the coexistence performance will be fully beamformed similar to the in-band power. Thus, modelling the OOB AAS radiation pattern is typically assumed to be fully correlated as the in-band AAS radiation pattern. 

In TR 38.922, similar discussion was initiated as WP 5D asked RAN4 about how the AAS performs in adjacent bands i.e. does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation. In this regard, the composite AAS radiation gain pattern for pure LOS channel, which is based on the ρ, the correlation factor defined in the interval 0≤ρ≤1, was further studied, and a stepwise model to quantify that correlation factor was introduced in Section 7.3.2.1.2 in TR 38.922. For that model to be utilized in coexistence studies, the quantification of different parameters, i.e., B, f1, f2, ρ1 , ρ2 still needs to be determined.

Observation 3: If additional coexistence studies are agreed in RAN4, RAN4 could study the characterization of the underlying parameters of the out-of-band AAS radiation pattern model in TR 38.922.

### UE modelling

During NR study item, as documented in TR 38.803, a UE antenna element model was proposed in Section 5.2.3.3. Such model emulates the presence of two panels with a 180 shift in horizontal orientation while the UE orientation is uniformly distributed in the azimuth domain between -180 and 180 degrees. Since there are two panels assumed, panel selection is assumed between the panels. This UE beamforming model has been used in several RAN4 coexistence studies (e.g., NR study item, ATG, SBFD, and Rel19 IMT-2030 parameters). The UE RF architecture and the beamforming model are an important aspect for discussion for both FR1 and FR3 future coexistence studies. For FR1, the focus should be on how to account for beamforming capability with increasing number of Txs (e.g., 4 or 2 Txs).

On the other hand, for FR3, it will be important to discuss within RAN4 what modelling to be used, if 6G adjacent channel coexistence studies will be carried out, for the case of the beamforming at the UE side and if advancements should be made on the existing model given in TR 38.803.

Observation 4: If additional coexistence studies are agreed in RAN4, proper modeling of UE beamforming gain could be discussed. Both the impact of increasing the number of TXs in FR1 (e.g., 4 or 2 TXs) and the extensions to UE beamforming model (as described in Section 5.2.3.3 in TR 38.803) in FR3 could be addressed.

# Conclusion

In this contribution, we provided our views on future 6G adjacent channel coexistence studies within RAN4 with the following observations and proposals:

Observation 1: In TR 38.922, RAN4 has documented the RF and AAS parameters for the frequency ranges 4.4-4.8 GHz, 7.125-8.4 GHz, and 14.8-15.35 GHz, where adjacent channel coexistence work was done only for 14.8-15.35 GHz.

Proposal 1: RAN4 to discuss if additional coexistence studies consistent with the work conducted in TR 38.922 are needed for the 6G SI.

Observation 2: If additional coexistence studies are agreed in RAN4, RAN4 could study how to harmonize in the coexistence framework the incorporation of the parameterized BS AAS steering limits in 6G coexistence studies.

Observation 3: If additional coexistence studies are agreed in RAN4, RAN4 could study the characterization of the underlying parameters of the out-of-band AAS radiation pattern model in TR 38.922.

Observation 4: If additional coexistence studies are agreed in RAN4, proper modeling of UE beamforming gain could be discussed. Both the impact of increasing the number of TXs in FR1 (e.g., 4 or 2 TXs) and the extensions to UE beamforming model (as described in Section 5.2.3.3 in TR 38.803) in FR3 could be addressed.

# References

- RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), RAN Meeting #108, Prague, Czech Republic.

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz, 3GPP.