3GPP TSG-RAN WG4 Meeting #116bis                                                                                                      R4-2513151

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Further discussion on system parameters for Ku-band for NR NTN

Agenda item:	4.2.7.1

Document for:	Approval

- Introduction

In RAN4#116 meeting, the big CRs for 38.101-5 and 38.108 on requirements of the NR NTN Ku band were agreed. But the note “Note: Same subcarrier spacing is assumed for SSB and data channels” in objectives of WID is missing.

As per the revised WID for Ku-band NTN [1], the objective concerning Note is shown as below:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

Addition of requirement that SSB SCS and data channel SCS are the same

As per WF [2], the remaining issue concerning the same SSB SCS and data channel SCS is shown as below:

As above, the Note “Note: Same subcarrier spacing is assumed for SSB and data channels” in objectives of WID indicates that the SSB SCS for synchronization raster for band n247, n248 should be aligned with that SCS for data channels. So to capture the objective in WID, it is reasonable to add a note “NOTE 2:    SS Block SCS is same with SCS for data channels in this release.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n248 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. NOTE 2: SS Block SCS is same with SCS for data channels in this release. |  |  |  |

So, we propose,

Proposal 1: Add a note “NOTE 2:    SS Block SCS is same with SCS for data channels.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 of TS 38.101-5 and TS 38.018 as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n248 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. NOTE 2: SS Block SCS is same with SCS for data channels in this release. |  |  |  |

Based on above proposals, two draft CR [3,4] for TS 38.101-5 and TS 38.108  for adding this note for FR1-NTN Ku-band are provided for approval.

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Add a note “NOTE 2:    SS Block SCS is same with SCS for data channels.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 of TS 38.101-5 and TS 38.018 as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n248 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. NOTE 2: SS Block SCS is same with SCS for data channels in this release. |  |  |  |

Based on above proposals, two draft CR [3,4] for TS 38.101-5 and TS 38.108  for adding this note for FR1-NTN Ku-band are provided for approval.

- References

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107

- R4-2508640, Way Forward for [115][312] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#115

- R4-2513150, Draft CR for 38.101-5, On NTN UE  requirement of the NR NTN Ku band, CATT, RAN4#116bis

- R4-2513153, Draft CR for 38.108, On SAN requirement of the NR NTN Ku band, CATT, RAN4#116bis