3GPP TSG RAN WG4 Meeting #116bis		R4-2513177

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on General RF and UE RF for 6G study

Agenda Item:	8.4

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a 6G RAN WG SI [1] was updated with the following objectives related to UE RF parts.

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 …… - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

Based on the RAN4 Chair’s guideline [2] on the RAN4 6G discussion and meeting arrangement in Rel-20 and the draft of RAN4#116bis meeting agenda, the general scope for UE RF requirements include the following parts.

| 6G Topics | Scope | Scope in Q4'25 and Q1'26 |
| --- | --- | --- |
| General RF and UE RF | non-spectrum UE RF, MIMO operation, duplexing, spectrum aggregation, initial access, multi-TRP, UE RF related coverage and energy efficiency and other joint UE-BS RF issues | non-spectrum and non-AI UE RF, spectrum aggregation, UE RF related coverage and energy efficiency |

The draft of RAN4#116bis meeting agenda: # 8.4 General RF and UE RF [FS_6G_Radio] # NOTE: The scope includes non-spectrum and non-AI UE RF, spectrum aggregation framework, UE RF related coverage and energy efficiency, and other joint UE-BS RF issues

Based on the inputs from 6G RAN WG SI and RAN4 Chair’s guideline, the following aspects can be summarized from RAN4 UE RF perspective.

1) Frequency Range:

| Frequency Range | FR1 (410 or FFS ~ 7125MHz) | between FR1 and FR2-1 (TBD & including around ~7GHz) | FR2-1 (24.25GHz ~ 52.6GHz) |
| --- | --- | --- | --- |
| TR 38.921(R17) | 6.425 - 7.025 GHz, 7.025 - 7.125 GHz | 10.0 - 10.5 GHz | NA |
| TR 38.922(R19) | 4400 - 4800 MHz | 7125 - 8400 MHz and 14800 - 15350 MHz | NA |

2) Single technology framework based on a stand-alone architecture (General principle):

b)	Energy efficiency and energy saving: both for network and device.

d)	Enhanced overall coverage, focus on cell-edge performance and UL coverage.

f)	Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

h)	Improved spectrum utilization and operations taking into account diverse spectrum allocations.

3) General RF and UE RF scope in Q4'25 and Q1'26

a) non-spectrum and non-AI UE RF

b) spectrum aggregation framework [General principle h)]

c) UE RF related coverage [General principle d) and f)]

d) UE RF related energy efficiency [General principle b)]

e) other joint UE-BS RF issues

Based on the draft of RAN4#116bis meeting agenda, there are five aspects to be discussed under the general RF and UE RF agenda. Meanwhile, we’d like to discuss some general rules or basic principles before diving into in-depth technical discussion.

2 Discussion on Basic principle

2.1 Single carrier VS multi carriers for UE RF requirements

If companies check the first release of LTE UE RF spec (TS 36.101) and NR UE RF spec (TS 38.101-1), the framework of contents were created based on the First Principle (single carrier operation in one band) referring to the Annex. Although inter band UL CA, SUL and intra-band contiguous CA and inter-band CA for DL were introduced together with single carrier operation in the first release during 5G phase, these features were still considered as enhanced feature instead of the basic feature from First Principle.

Table 1 Supported feature(s) in the first release of LTE and NR for UE RF spec

| 3GPP RAT | LTE UE RF (Rel-8) | NR UE RF (SA) (Rel-15) |
| --- | --- | --- |
| Supported feature in the first release of one generation for UE RF spec | 1. Single carrier operation for both DL and UL. | 1. Single carrier operation for both DL and UL. 2. inter-band CA for UL 3. SUL with both UL and DL RF requirements 4. UL MIMO 5. intra-band contiguous CA and inter-band CA for DL 6. DL MIMO, but without UE RF impacts… |

Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage.

Observation 1: Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage.

Referring to the contribution [3], it’s observed that most of the operators own at least one LEGACY FDD band below 1GHz and one LEGACY FDD band between 1.4GHz ~2.7GHz. Thus, at the 6G stage, operator could re-farm one LEGACY FDD at least for 6G usage and leave one LEGACY FDD band for the legacy RAT (LTE or NR) & legacy UE.

For large bandwidth TDD bands for 6G usage, operators may have spectrum resources in around 3 GHz to 6 GHz range (such as 3.5 GHz, 4.9 GHz, etc.). Furthermore, candidate new spectrum resources have been discussed, ranging from around 7 to 24 GHz, such as 6.425 – 7.125 GHz (identified in WRC-23 meeting [4]), 7.125 – 8.400 GHz, and 14.800 – 15.350 GHz (recommended for WRC-27), and FR2-1 (24.25GHz ~ 52.6GHz, legacy 5G NR mmWave).

As the center frequency of TDD band with large channel bandwidth become higher and higher, it’s very important for operators to choose one LEGACY FDD band below 2.7GHz to guarantee the UL capability and UL coverage especially for deep coverage for 6G usage.

Observation 2: at the 6G stage, most of the operators have the adequate spectrum resources to initiate one UL band below 2.7GHz and one “band” with large channel bandwidth above 3GHz for 6G usage. This strategy can guarantee a good UL capability and UL coverage especially for the demands of deep coverage at the 6G stage.

Proposal 1: RAN4 aim to design 6G UE RF specification based on First Principle for multi-carrier operation instead of simply reusing the UE RF framework of LTE and NR.

Based on this principle, alternatively, UE RF specification may not be split based different frequency ranges. Because FR2-1 (24.25GHz ~ 52.6GHz) bands need the help from UL band below 2.7GHz to response UL feedback/SR/CSI report. From both network and UE’s perspective, UL signal/channel transmission in legacy FDD band below 2.7GHz provide more cost/energy efficiency than UL transmission FR2-1 (24.25GHz ~ 52.6GHz) legacy TDD bands.

Proposal 1-2: UE RF specification may not be split based on different frequency ranges at the 6G stage considering UL/DL decoupling between FR2-1 bands and low frequency bands below 2.7GHz.

2.2 Conducted requirements VS OTA requirements

The following table shows the applicable frequency range for conducted requirements and OTA requirements during the LTE stage and NR stage.

Table 2 the applicable frequency range for conducted requirements and OTA requirements

| 3GPP RAT | LTE UE RF | NR UE RF (SA) |
| --- | --- | --- |
| FR1 (410MHz ~7125MHz) | Conducted requirements | Conducted requirements |
| FR2-1 (24.25GHz ~ 52.6GHz) | NA | OTA requirements |

There is a classical statement or assumption to clarify the consideration of UE antenna’s capability for conductive requirements.

TS 36.101/TS 38.101-1 For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s).

Firstly, “For UE(s) with an integral antenna only” is outdated since most of UE especially for smartphone are implemented with multiple integral antennas without any external antennas. But even for a few kinds of UEs, e.g. FWA, even if external antenna(s) are implemented, a reference antenna(s) with a gain of 0dBi is also assumed from conduced requirements’ perspective.

However, in the real field, the antenna efficiency of integral antennas for most of UEs are less than 100%. And some special antenna designs may consider the impacts from hand phantom and head phantom to improve the antenna efficiency. In other words, the antenna efficiency of UE can vary for different scenarios, different phantoms, different frequency and different UE form factor.

Thus, one view holds that the performance of UE’s antennas is considered as part of large scale path loss because of the varied UE antenna efficiency when RAN4 only specify conducted requirements from UE RF perspective. The following figure show the logic of this view.

Figure 1 The illustration for the relationship among BS/UE antenna performance, Transmission Channel, conducted requirements and OTA requirements.

From testability and deterministic perspective, the UE RF requirements at antenna connectors are under control, stable and easily verified.

Based on the views above, the conducted requirements were specified as UE RF core requirements for FR1. However, as the number of antennas is increasing, it becomes more and more important to verify some OTA requirements for FR1 UE, e.g, MIMO related performance and reference signal related performance.

Although the absolute OTA requirements for FR1 UE are not stable with large variance considering different scenarios, different frequency and different UE form factor, alternatively the relative OTA requirements for FR1 UE can be considered by comparing the same UE between different configurations. Thus, RAN4 could explore some UE RF requirements based on comparative test method considering OTA requirements in some FR1 bands.

Proposal 2: RAN4 explore some UE RF requirements based on comparative test method considering OTA requirements in some FR1 bands.

For FR2-1 (24.25GHz ~ 52.6GHz), based on UE implementation and AIP component in FR2-1, OTA requirements become the only one choice, but the whole UE RF framework for FR2-1 should be compatible with the general FR1 UE RF requirements. As UE types are specified based on different FR2 power class, large repetitive OTA requirements are copied everywhere. In order to avoid this disadvantage, RAN4 aim to evolve the framework of UE RF requirements for FR2-1 considering the lessons learned from 5G.

Proposal 3: RAN4 aim to evolve the framework of UE RF requirements for FR2-1 considering the lessons learned from 5G.

2.3 End-To-End UE RF requirements (Methodology)

The basic logic for specifying the UE RF requirements of new features are shown below based on the experience of LTE and NR. Firstly, RAN4 specified the basic requirements for single carrier operation with a clear framework where each UE RF requirement for single carrier operation has the clear purpose, applicable scenario and clear physical meaning. After that, when a new enhanced feature with new UE RF requirements need to be introduced, there are two alternatives.

Alternative one: For the enhanced feature, RAN4 copy the identical UE RF framework from single carrier operation no matter whether the purpose, applicability or physical meaning is clear or not, e.g. CA and UL MIMO.

For example (UL MIMO), a lot of receiver characteristics for UL MIMO were specified without clear purpose or physical meaning. Not sure if this is just to copy something mechanically.

Alternative two: For the enhanced feature, RAN4 only specify limited UE RF requirements which are affected and important. Some legacy general requirements for single carrier operation may or may not be applicable for the new enhanced feature, e.g. SUL, UL Tx switching and SRS enhancement, although we have a general clarification on clause 4.3. Companies may have different understanding on different features for the meaning that “A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause”.

Clause 4.3 of TS 38.101-1 A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to L) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to L) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.

For example (SUL), a clarification on Configured transmitted power, Reference sensitivity, Blocking characteristics and ΔTIB,c were specified for SUL in some specific clauses. And other requirements for SUL band were specified in the general requirements for single carrier operation, e.g. MOP, AMPR, spurious emission and so on. There are some ambiguities whether the general Transmit ON/OFF time mask are applicable to R15/R16 SUL since the Transmit ON/OFF time mask for SUL was introduced since R16.

For example (UL Tx switching), only one clause has the impacts on RF requirements, e.g. Transmit ON/OFF time mask for inter-band CA/SUL. There are some ambiguities whether the general RF requirements for single carrier operation are applicable to UL Tx switching or not. There is no clear RF requirements during switching period, neither on power requirements not off power requirements. In addition (an important absence in the RF requirements for UL Tx switching), there is no requirement to verify the system gain for UL transmission even from UE perspective, although the system gain (The uplink throughput could be further improved. Justification of [5]) was mentioned and justified in the WID [5] of UL Tx switching for each release.

Table 3 The comparison of impacted clauses between NR general requirements for single carrier and enhanced features

| Basic requirements for single carrier operation |  | 5 Operating bands and channel arrangement 6 Transmitter characteristics 7 Receiver characteristics |
| --- | --- | --- |
| Enhanced UE RF requirements / features | CA | Operating bands and channel arrangement for CA Transmitter characteristics for CA Receiver characteristics for CA |
|  | UL MIMO | Operating bands and channel arrangement for UL MIMO Transmitter characteristics for UL MIMO Receiver characteristics for UL MIMO |
|  | SUL | Operating band combination for SUL Configurations for SUL Transmitter power for SUL Configured transmitted power for SUL ΔTIB,c Reference sensitivity for SUL Blocking characteristics for SUL |
|  | …… | …… |
|  | UL Tx switching | 6.3A.3.3 Transmit ON/OFF time mask for inter-band CA |

The rooted issue of copying something mechanically, missing key RF requirements for system gain and some ambiguities about the applicability of general requirements, is that sometime RAN4 have deviated from the original purpose (motivation) of specifying these RF requirements and are now defining them merely for the sake of RF standardization.

Observation 3: Sometime RAN4 have deviated from the original purpose (motivation) of specifying these RF requirements and are now defining them merely for the sake of RF standardization, which result in the copying something mechanically, missing key RF requirements for system gain and some ambiguities about the applicability of general requirements.

Unlike BS RF/RRM/demod, there is no performance phase for UE RF parts/features because all the conformance test parts for UE RF are mainly in the charge of RAN5. This leads to some issues/gaps that some of UE RF core requirements can’t be verified, the reference measure channel (UL/DL RMC) are missing or not applicable in the UE RF specification and the applicable/typical scenarios/purpose are not clear from RAN5’s perspective.

For example (UL RMC missing for HPUE), some companies found that there is no appropriated UL RMC to verify the duty cycle requirements for HPUE especially for FDD bands.

For example (UL RMC missing for SRS enhancement), some companies found that there is no appropriated UL RMC to verify SRS enhancement as current UL RMC are only for UL PUSCH with type A mapping.

For example (Phase continuity requirements for DMRS bundling, R17 feature), RAN5 still didn’t complete the conformance testing part for this feature due to the complexity.

Clause 6.4.2.6 of TS 38.521-1-v18.6.0 6.4.2.6 Phase continuity requirements for DMRS bundling Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined: - MU/TT analysis is pending

For example (UL Tx switching, R16 feature), the testability for current RF requirements is often challenged by companies due to uncertain RF requirements during the switching period and the small switching period compared to the measurement period of on-power slots. It’s noted that RAN5 will further improve the procedure of verifying the on/off time mask for UL Tx swtiching.

Clause 6.3A.3.2 of TS 38.521-1-v18.6.0 6.3A.3.2 Time mask for switching between two uplink carriers Editor’s Note: The improvement for test procedure is FFS

As some features become more and more complicated, one sets of UL/DL RMC for general requirements can’t meet the demands of verifying the new features.

Observation 4: Unlike BS RF/RRM/demod, there is no performance phase for UE RF parts/features because all the conformance test parts for UE RF are mainly in the charge of RAN5. This leads to some issues/gaps that some of UE RF core requirements can’t be verified, the reference measure channel (UL/DL RMC) are missing or not applicable for some features in the UE RF specification and the applicable/typical scenarios/purpose are not clear from RAN5’s perspective.

Based on the issues above, it’s proposed to consider a general methodology for UE RF requirements from an End-to-End perspective considering both the standardization Purpose of UE RF requirements and testability of the minimum UE RF requirements. A general illustration is shown in the figure below.

Figure 2 The illustration on a general methodology for UE RF requirements from an End-to-End perspective

Proposal 4: RAN4 aim to explore a general methodology for UE RF requirements from an End-to-End perspective considering both the standardization Purpose of UE RF requirements and testability of the minimum UE RF requirements.

3 Discussion on New Area

3.1 UE RF related coverage

As the targeted frequency is increasing, the UL features, e.g. UL coverage enhancement, UL capability enhancement, UL throughput enhancement, SRS enhancement etc., become more and more important. However, current framework of RAN4 spec can’t support the verification of UE UL features from system/UL performance perspective referring the following figure 3. Based on the clause 6 transmitter characteristics of TS 36.101/TS 38.101-1, only some discrete UE requirements, e.g. output power, transmit signal quality etc., are specified or used to verify whether the UL enhancements meet the minimum requirements or not, but RAN4 missed the UE UL verification from system perspective. In addition, current UL enhancements including UL coverage become more and more complicated, it’s impossible to achieve the standardization purpose from system perspective by designing some discrete UE RF requirements.

Although RAN4 specified the EVM requirements as the core requirements for transmit signal quality, EVM requirements just indirectly indicate the quality of transmit signal. Here, we need a new framework to verify UE UL signal performance from system perspective or link level perspective.

F.2 Basic Error Vector Magnitude measurement The EVM is the difference between the ideal waveform and the measured waveform for the allocated RB(s) , where is a set of modulation symbols with the considered modulation scheme being active within the measurement period, are the samples of the signal evaluated for the EVM, is the ideal signal reconstructed by the measurement equipment, and is the average power of the ideal signal. For normalized modulation symbols is equal to 1. The basic EVM measurement interval is defined over one slot in the time domain for PUCCH and PUSCH and over one preamble sequence for the PRACH.

Furthermore, current UL RMC definition for Type A PUSCH can’t be applicable to the complicated UL features. And some key RMCs for SRS feature and HPUE (PC2 above) are missing in current specification. From completion perspective, the new framework for UL features including UE UL coverage should considering the testability of these UL features including appropriated UL RMC definition.

Proposal 5: RAN4 aim to explore new UE framework for the missing UE UL performance and the verification of UL features including UE RF related coverage from system perspective or verification of the whole UE RF&BB Tx chains.

Figure 3 the illustration on the missing UE UL Performance

3.2 UE RF related energy efficiency

As we are approaching the 6G Era, to achieving good energy efficiency is very important for both UE vendors and consumers. This is a new area for RAN4 since RAN4 never specify the relative requirements for UE energy efficiency. One model is proposed to illustrate the relationship between energy consumption (costs) and communication service/Throughput (gain) as the figure below.

Figure 4 Model between energy consumption (costs) and communication service/Throughput (gain)

Although the UE energy efficiency is brand new for RAN4, the relative metrics for energy efficiency are not totally brand new for the whole industry. The first metric for energy efficiency (EE) is the bits/Joule (bit/J), which directly reflect the energy consumed by a communication terminal to transmit one unit of bit information.

Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power).

If we go through the open research and discussion, the following metrics can also be found.

Bits Transmitted per Watt (bit/W): commonly used in power-constrained mobile communication terminals, it reflects how many bits of information a terminal can transmit at a fixed Power level, demonstrating the terminal’s data transmission capability under specific power conditions.

Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt

Energy Efficiency Ratio (EER): Represents the ratio between the electrical energy consumed by a communication terminal and the effective communication energy it generates per unit time. The effective communication energy refers to the energy used by the terminal for transmitting and processing information in its normal working state.

Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt)

Standby Power Consumption: Refers to the power consumed by a communication terminal when it is in standby mode, with the unit being Watt (W) or milliwatt (mW). Lower standby power consumption means the terminal consumes less energy when not performing actual communication operations, which may also extend the battery life.

Operating Power Consumption: Refers to the power consumed by a communication terminal when conducting communication services (such as calling, data transmission, video playback, etc., representing different throughputs). It can be measured separately according to different service types, such as call power consumption and internet access power consumption, and reflects the energy consumption of the terminal during actual operation.

Standby Power Consumption (Background) VS Operating Power Consumption

The whole industry aim to achieve the zero vision, i.e. zero bit zero watt, but it seems very difficult. One possible way is to specify the Standby Power Consumption, i.e. how much is Power Consumption when there is no UL/DL throughput transmission. RAN4 may need to answer How many Watt with zero bit.

Proposal 6: RAN4 study the following metrics at least for UE related energy efficiency and model for UE related energy efficiency between energy consumption and Throughput.

1) Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power)

2) Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt

3) Energy Efficiency Ratio: e.g. Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt)

4) Standby Power Consumption VS Operating Power Consumption, but it’s targeted to achieve zero vision, i.e. Zero Bit Zero Watt.

4 Summary

2.1 Single carrier VS multi carriers for UE RF requirements

Observation 1: Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage.

Observation 2: at the 6G stage, most of the operators have the adequate spectrum resources to initiate one UL band below 2.7GHz and one “band” with large channel bandwidth above 3GHz for 6G usage. This strategy can guarantee a good UL capability and UL coverage especially for the demands of deep coverage at the 6G stage.

Proposal 1: RAN4 aim to design 6G UE RF specification based on First Principle for multi-carrier operation instead of simply reusing the UE RF framework of LTE and NR.

Proposal 1-2: UE RF specification may not be split based on different frequency ranges at the 6G stage considering UL/DL decoupling between FR2-1 bands and low frequency bands below 2.7GHz.

2.2 Conducted requirements VS OTA requirements

Proposal 2: RAN4 explore some UE RF requirements based on comparative test method considering OTA requirements in some FR1 bands.

Proposal 3: RAN4 aim to evolve the framework of UE RF requirements for FR2-1 considering the lessons learned from 5G.

2.3 End-To-End UE RF requirements (Methodology)

Observation 3: Sometime RAN4 have deviated from the original purpose (motivation) of specifying these RF requirements and are now defining them merely for the sake of RF standardization, which result in the copying something mechanically, missing key RF requirements for system gain and some ambiguities about the applicability of general requirements.

Observation 4: Unlike BS RF/RRM/demod, there is no performance phase for UE RF parts/features because all the conformance test parts for UE RF are mainly in the charge of RAN5. This leads to some issues/gaps that some of UE RF core requirements can’t be verified, the reference measure channel (UL/DL RMC) are missing or not applicable for some features in the UE RF specification and the applicable/typical scenarios/purpose are not clear from RAN5’s perspective.

Proposal 4: RAN4 aim to explore a general methodology for UE RF requirements from an End-to-End perspective considering both the standardization Purpose of UE RF requirements and testability of the minimum UE RF requirements. For Example:

3.1 UE RF related coverage

Proposal 5: RAN4 aim to explore new UE framework for the missing UE UL performance and the verification of UL features including UE RF related coverage from system perspective or verification of the whole UE RF&BB Tx chains.

3.2 UE RF related energy efficiency

Proposal 6: RAN4 study the following metrics at least for UE related energy efficiency and model for UE related energy efficiency between energy consumption and Throughput.

1) Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power)

2) Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt

3) Energy Efficiency Ratio: e.g. Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt)

4) Standby Power Consumption VS Operating Power Consumption, but it’s targeted to achieve zero vision, i.e. Zero Bit Zero Watt.

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple)

[3] RP-252137, “Consideration on cell management and multi-carrier operation”, Huawei, HiSilicon

[4] https://www.gsma.com/connectivity-for-good/spectrum/wrc-series/

[5] RP-220681, revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon