3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513181

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	System parameters for 4.9GHz new band

Agenda item:	5.11.2

Document for:	Approval

- Introduction

In RAN#108 meeting, a new WI was established for 4.9GHz new band [1] and revised in RAN#109 in [2]. The objective is as below. In this contribution, we focus on system parameters on this new band.

A one quarter study phase to identify and document coexistence requirements for the new band: - Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band. - Identify adjacent spectrum protection requirements - Identify any potential emission limits more stringent than the normal BS and UE emission masks - Identify other technologies operating in the 4940-4990 MHz band - Identify potential coexistence mechanism(s) - Frequency division sharing - Geographic separation - TDD synchronization - Identify any potential emission limits more stringent than the normal BS and UE emission masks Following the completion of the study phase, the core objectives of this WI are: - Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths: SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. 15; 10; 15; 20; 25; 30; 40; 50. 30; 10; 15; 20; 25; 30; 40; 50. 60; 10; 15; 20; 25; 30; 40; 50. - Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band. - Address potential BS and UE co-existence issues, if any. - Specify band specific BS, repeater and IAB emissions requirements if needed - Specify UE additional emission limits and A-MPR if needed - Specify the BS, repeater and IAB RF requirements. - Specify the UE RF requirements for the new band. Reuse NR band n79 UE hardware assumption if possible. - Update the related technical specifications to include support for the new band - CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

- Discussion

Operating band

4.9GHz new band is overlapping with n79 and should be defined in operating bands in FR1 as in Table 1. As the current bands number is up to n110, so n111 is proposed for this new band.

Table 1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n111 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

Channel bandwidth

According to the objectives in WI, we can require the channel bandwidth for 4.9GHz band in Table 2.

Table 2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n111 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Channel raster

The applicable NR-ARFCN for new 4.9GHz operating band is listed in Table 3.

Table 3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n111 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Synchronization raster

The applicable SS raster for new 4.9GHz operating band is listed in Table 4.

Table 4  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n111 | 30 kHz | Case C | 8850 – <1> – 8877 |

Proposal: Approve Table 1 to Table 4 on system parameters for new 4.9GHz NR operating band.

- Conclusion

This contribution provides system parameters for 4.9GHz new band and the proposal is as below:

Proposal: Approve Table 1 to Table 4 on system parameters for new 4.9GHz NR operating band.

- References

- RP-251887 New WID on Introduction of NR TDD 4.9GHz Band

- RP-252929 Revised WID on Introduction of NR TDD 4.9GHz band for US operation