3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513197

# Prague, CZ, 13th – 17th Oct, 2025

Title: 	Discussion on performance requirements for CSI-RS based L1 measurement for Rel-19 LTM

Source: 	CATT

Agenda item:	6.17.2.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and the core part of WI for NR_Mob_Ph4 was completed on time. In this contribution, we discuss and provide our initial views on the performance requirements for CSI-RS based L1 measurement for Rel-19 LTM.

- Discussion

  - 2.1 Test case list

For CSI-RS based L1-RSRP requirements for neighbor cell, the agreements of measurement gap and the definition on CSI-RS based intra-frequency L1 measurement are copied as follows [1][2]:

- RAN4#114-bis: - Measurement GAP Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - RAN4#115: - Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19.

Since RAN4 has agreed not to define the core requirements for L1-RSRP measurement with measurement gap and not to define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s), so the test scope also doesn’t need to consider the measurement with measurement gap and inter-frequency measurement.

In addition, both FR1 and FR2 should be considered. For FR2, since skipping SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement is supported, so the test cases for CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with and without SSB based L1-RSRP measurement should be defined.

According to the above, we propose to define the following test cases for CSI-RS based L1 measurement:

- CSI-RS based L1 RSRP measurement for neighbour cell in FR1

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

Proposal 1: RAN4 to define the following test cases for CSI-RS based L1 measurement:

- CSI-RS based L1 RSRP measurement for neighbour cell in FR1

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

  - 2.2 Test configuration – BW

For test configuration for CSI-RS based L1 measurement, since RAN4 agreed at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP for intra-frequency L1 measurement, so setting 48 PRBs within the active BWP in test configuration can be supported.

Proposal 2: Test configuration is set to have 48 PRBs within the active BWP.

  - 2.3 Others

In the last meeting, the candidate options for test cases for early CSI acquisition are copied as below [3]:

- Issue 4-2-4: others Candidate options: - Proposal 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo) - Proposal 1a: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 1b: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 2: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP. (Nokia)

However, there is no consensus on defining new RAN4 requirements for early CSI acquisition, and the agreement of early CSI acquisition is copied as follows [3]:

- Issue 3-5: Early CSI acquisition Agreement: - No consensus on defining new RAN4 requirements for early CSI acquisition. - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached. - It is RAN4 common understanding that for baseline early CSI acquisition, - If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell. - Otherwise, UE shall start CSI acquisition at least after Tprocessing. - No need to capture above agreement in RAN4 spec.

Therefore, according to the progress in core part, we propose that RAN4 not to define the test case to verify early CSI acquisition.

Proposal 3: RAN4 not to define the test case to verify early CSI acquisition.

- Conclusion

This document discussed performance requirements for CSI-RS based L1 measurement for Rel-19 LTM and presented following proposals:

Proposal 1: RAN4 to define the following test cases for CSI-RS based L1 measurement:

- CSI-RS based L1 RSRP measurement for neighbour cell in FR1

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

Proposal 2: Test configuration is set to have 48 PRBs within the active BWP.

Proposal 3: RAN4 not to define the test case to verify early CSI acquisition.

- Reference

[1]    R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[2]    R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[3]    R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple