3GPP TSG-RAN WG4 Meeting # 116bis                                                         R4-2513208

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	                 Discussion on 6G spectrum

Source: 	CATT

Agenda item:	8.6

Document for:	Discussion

# Introduction

In SID[1] of RAN#108, the band/band combination definition and simplification, definition of frequency ranges and spectrum related regulatory survey need to be analysed. This contribution provides our analysis for these issues.

# Analysis of 6G spectrum

## Band/band combination definition and simplification

### 2.1.1 Band definition

In 6G, uplink and downlink operating band need to be decoupled, so we need a new way to define frequency bands. In NR, uplink operating bands are paired with downlink operating bands (For FDD bands, uplink and downlink frequency blocks are different, for TDD, uplink and downlink frequency blocks are overlapping). Therefore, the new frequency band definition does not involve this pairing. Instead, each band is defined simply as a contiguous segment of spectrum, which can be configured to support uplink, downlink, or both.

The pairing of a specific uplink operating band with a downlink operating band is a capability of the UE. The UE reports this pairing capability and required bandwidths to the network. Based on the UE's reported capability, the network then configures its corresponding parameters. A single UE can support multiple UL and DL pairing combinations. Consequently, a single cell can contain multiple TRPs (referred to as the Transmitting and Receiving Point) operating in different frequency bands, with each TRP handling the specific UL and DL carriers that it supports.

Proposal 1: RAN4 to consider a new frequency band definition, which is a continuous spectrum segment that can be configured to support uplink, downlink, or both, without UL operating band and DL operating band pairing.

Proposal 2: The UE can report its pairing capability and required bandwidths to the network, and network configures corresponding parameters based on the capabilities reported by the UE.

Proposal 3: A UE can support multiple UL and DL pairings. Therefore, one cell can contain multiple TRPs operating in different frequency bands, with each TRP handling the UL and DL carriers that it supports.

### 2.1.2 Band combination definition

Table 1: Frequency group definition of FR1

| Frequency group numbering | Frequency range | Definition | SCS (kHz) |
| --- | --- | --- | --- |
| 1 | Around 400MHz ~ 1695MHz | Low band | 15 |
| 2 | 1695MHz ~ 2690MHz | Mid band | 15 |
| 3 | 2690MHz ~ 3300MHz | High band | 30 |
| 4 | 3300MHz ~ 5000MHz | Low ultra band | 30 |
| 5 | 5000MHz ~ 5925MHz | L6G band | 30 |
| 6 | 5925MHz ~ 7GHz/8GHz? | U6G band | 30 |

In 6G, band definition is changed different to meet demands of decoupling uplink and downlink. Additionally, current NR UE inter-band carrier aggregation (CA) supports the aggregation of 2 to 6 operating frequency bands. As each CA case list contains combinations of multiple operating bands resulting in extensive and complex listings, the definition of band combinations requires updating. A band combination can be defined based on the new frequency band definition concept in Section 2.1.1, utilizing frequency groups and the UE's uplink/downlink pairing capability.

The uplink and downlink operating bands of each NR operating band can be within the same frequency group, and the frequency groups of FR1 are defined in Table 1. In table 1, only one type of SCS is set for each frequency group. For any frequency group with LTE and NR spectrum sharing, it is required to set the SCS to 15 kHz. For other frequency groups, SCS is set according to the possible channel bandwidth it can achieve.

Proposal 4: We recommend defining only one SCS per frequency group.

Proposal 5: A band combination can be defined based on the new frequency band definition concept by utilizing frequency groups and the UE's UL and DL pairing capability.

Proposal 6: The frequency groups and SCS of FR1 are defined in Table 1.

Table 2: Band combination definition of FR1

|  | Frequency group numbering | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency group numbering | Frequency range | Around 400MHz ~ 1695MHz | 1695MHz ~ 2690MHz | 2690MHz ~ 3300MHz | 3300MHz ~ 5000MHz | 5000MHz ~ 5925MHz | 5925MHz ~ 7GHz/8GHz? |
| 1 | Around 400MHz ~ 1695MHz |  |  |  |  |  |  |
| 2 | 1695MHz ~ 2690MHz |  |  |  |  |  |  |
| 3 | 2690MHz ~ 3300MHz |  |  |  |  |  |  |
| 4 | 3300MHz ~ 5000MHz |  |  |  |  |  |  |
| 5 | 5000MHz ~ 5925MHz |  |  |  |  |  |  |
| 6 | 5925MHz ~ 7GHz/8GHz? |  |  |  |  |  |  |

Paired UL and DL can reside within either a single frequency group or across different frequency groups. If the paired UL and DL fall within the same frequency group, they share the same SCS, as indicated by the diagonal areas in table 2. If they belong to different frequency groups, the SCS may differ, which corresponds to the non-diagonal areas in table 2.

Proposal 7:  Paired UL and DL can reside within either a single frequency group or across different frequency groups.

Proposal 8:  If the paired UL and DL are in the same frequency group, they share the same SCS. If they are in different frequency groups, the SCS may differ.

From the perspective of the network, band combination includes intra-cell band and inter-cell band combination. Intra-cell band combination corresponds to single cell, if n1 DL is paired with n28 UL, only 1 cell is counted in this case, while two cells are counted in NR. Inter-cell band combination is a new cell aggregation, i.e. new CA. For example, (n1 DL +n28 UL) and (n77+ n79) count as two cell aggregations.

Proposal 9: Band combination includes intra-cell band combination and inter-cell band combination. The former corresponds to a single cell, while the latter corresponds to multiple cells.

## Definition of frequency ranges

Table 3: Definition of frequency ranges

| FR |  | Frequency range | SCS (kHz) |
| --- | --- | --- | --- |
| FR1 | Low band | Around 400MHz ~ 1695MHz | 15 |
|  | Mid band | 1695MHz ~ 2690MHz | 15 |
|  | High band | 2690MHz ~ 3300MHz | 30 |
|  | Low ultra band | 3300MHz ~ 5000MHz | 30 |
|  | L6G band | 5000MHz ~ 5925MHz | 30 |
|  | U6G band | 5925MHz ~ 7GHz/8GHz? | 30 |
| [New FR] |  | 7GHz/8GHz? ~ 24.25GHz | 60 |
| FR2-1 |  | 24.25GHz ~ 52.6GHz | 120 |

The definition of frequency ranges is shown in table 3, based on table 2, the new FR and FR2-1 information are also added in this table.

Proposal 10: The definition of frequency ranges is shown in table 3.

## Spectrum related regulatory survey

Currently, the ITU and national regulatory agencies worldwide are actively engaged in the strategic allocation and coordination of 6G spectrum resources.

China: In May, 2023, the MIIT of China released [2], which does not explicitly designate the identified frequency bands for 6G systems. However, the regulatory interpretation provides the following explanation: New frequency allocations have been added for IMT (IMT, including 5G/6G) systems. Frequency allocations for IMT systems have been added in the 6GHz, 26GHz, 40GHz, and 70GHz bands, among others, to establish a regulatory framework for 5G/6G system frequency usage and stabilize industry expectations. This move pioneers the allocation of the 6GHz band for IMT systems globally, which helps promote global or regional harmonization of frequency allocations for this band and provides ample mid-band frequency resources for 5G/6G development.

ITU：WRC-23 and the new regulations now allow 4G, 5G and, in the future, 6G spectrums in the 3300-3400 megahertz (MHz), 3600-3800MHz, 4800-4990MHz and 6425-7125 MHz frequency bands in various countries and regions [3].

United States: The FCC announced in April 2020 that it would open up the 6 GHz spectrum (5925MHz -7125MHz) under an unlicensed approach, allocating the entire 1200 MHz bandwidth for Wi-Fi networks [4].

India: In May, 2025, India's Department of Telecommunications (DoT) has proposed draft rules to delicense a part (5925-6425MHz) of the 6GHz frequency band to improve Wi-Fi speed and capacity for the end users, and the government has reserved the 6425-7125MHz band for telcos [5].

European Union: In 2021, the European Commission required member states to open the lower 6GHz band (5945-6425 MHz) for Wi-Fi use. However, the allocation of the upper band (6425-7125 MHz) remains under intense debate between mobile network operators and the Wi-Fi sector [6].

The conclusions of 6G spectrum related regulatory survey are shown in table 4.

Table 4: Conclusions of 6G spectrum related regulatory survey

| Country | WIFI | IMT | Note |
| --- | --- | --- | --- |
| China |  | 6425MHz-7125 MHz |  |
| United States | 5925MHz -7125MHz | —— |  |
| India | 5925 MHz -6425 MHz | 6425 MHz ~7125 MHz |  |
| European Union | 5945 MHz -6425 MHz |  | The allocation of the 6425-7125 MHz band is still under discussion. |
| ITU |  | WRC-23 and the new regulations now allow 4G, 5G and, in the future, 6G spectrums in the 3300-3400 megahertz (MHz), 3600-3800MHz, 4800-4990MHz and 6425-7125 MHz frequency bands in various countries and regions. |  |

Proposal 11: The conclusions of 6G spectrum related regulatory survey are shown in table 4.

# Summary

This contribution provides our preliminary analysis for the general issues for 6G spectrum. We have the following proposals,

Proposal 1: RAN4 to consider a new frequency band definition, which is a continuous spectrum segment that can be configured to support uplink, downlink, or both, without UL operating band and DL operating band pairing.

Proposal 2: The UE can report its pairing capability and required bandwidths to the network, and network configures corresponding parameters based on the capabilities reported by the UE.

Proposal 3: A UE can support multiple UL and DL pairings. Therefore, one cell can contain multiple TRPs operating in different frequency bands, with each TRP handling the UL and DL carriers that it supports.

Proposal 4: We recommend defining only one SCS per frequency group.

Proposal 5: A band combination can be defined based on the new frequency band definition concept by utilizing frequency groups and the UE's UL and DL pairing capability.

Proposal 6: The frequency groups and SCS of FR1 are defined in Table 1.

Proposal 7:  Paired UL and DL can reside within either a single frequency group or across different frequency groups.

Proposal 8:  If the paired UL and DL are in the same frequency group, they share the same SCS. If they are in different frequency groups, the SCS may differ.

Proposal 9: Band combination includes intra-cell band combination and inter-cell band combination. The former corresponds to a single cell, while the latter corresponds to multiple cells.

Proposal 10: The definition of frequency ranges is shown in table 3.

Proposal 11: The conclusions of 6G spectrum related regulatory survey are shown in table 4.

# Reference

[1] RP-251881, “New SID: Study on 6G Radio”, RAN#108

[2] Regulations on the Radio Frequency Allocation of the People's Republic of China, 	May, 2023

[3] ITU 2024 opens up 6G bands and basestation in the sky, https://www.eenewseurope.com/en/itu-2024-opens-up-6g-bands-and-basestation-in-the-sky/

[4] FCC-24-125A1 - 6GHz

[5] https://www.lightreading.com/regulatory-politics/india-moves-to-delicense-part-of-6ghz-band-sparking-telco-tech-clash

[6] RSPG25-006final-RSPG_Report_on_6G_strategic_vision