3GPP RAN WG4 Meeting #116bis	R4-2513217

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on RRM core requirements and testing framework for BM

Source: 	CATT

Agenda item:	6.11.4

Document for:	Discussion

Introduction

In RAN4#116 meeting, RAN4 continued to discuss AI/ML-based beam management in Rel-19 AI/ML for NR air interface and the agreements were captured in [1]. In this contribution, we will further discuss some open issues summarized in [2].

Discussion

Metrics/KPIs for beam ID prediction

In RAN4#114bis and 115 meetings, this issue was discussed and RAN4 reached the following agreement:

Issue 2-1: Metrics/KPIs for beam prediction RAN4#114bis Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" RAN4#115 Proposals: - Option 1: adopt N - N=2 - Further study the actual value - N>2 - N= K2+1 - Option 2: Do not use N, remove the FFS on top-N - Option 3: x<3dB - Option 4: further study x based on simulations - Option 5: Others (e.g. different distributions, etc.) RAN4#116: Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

In RAN4#116 meeting, RAN4 discussed the case where RSRP prediction is also reported with predicted beam ID(s) and agreed that the absolute/relative RSRP requirements apply to reported beams which satisfy the SNR side conditions and Top-K beams will be considered in tests. However, the value of K is still not determined.

According to the Rel-19 higher layers parameters list provided by RAN1 after RAN1#122, the number of reported beams for both BM-Case 1 and BM-Case 2 is configured by network via RRC signallings as follows:

| Parameter name in the spec | New or existing? | Description | Value range |
| --- | --- | --- | --- |
| nrofreportedpredictedrs-r19 | new | Indicate the number (K）of predicted RS resources to be reported per report setting for BM-Case 1 Indicate the number (K）of predicted RS resources per time instance to be reported per report setting for BM-Case 2 | 1,2,3,4 |

Therefore the above value range, in our opinion, can be used in test, i.e., the related test apply to the top-K beams, where K is the valued configured by nrofreportedpredictedrs-r19.

Proposal 1: For the case where both predicted beam ID(s) and predicted RSRP(s) are reported, the related tests apply to the top-K beams, where K is the valued configured by nrofreportedpredictedrs-r19.

In previous meetings, RAN4 have agreed to use the successful rate as a metric/KPI for the correct prediction when only beam ID is reported, while there is no much progress for this case in the last meeting and some aspects still need further discussion.

The 1st FFS in RAN4#114bis agreement is to determine the value(s) of N, where each of Top-N predicted beams will be verified against the corresponding genie-aided beam. Considering that network needs to handle load-balancing and inter-layer interference, etc., network may not schedule UE with the strongest predicted beam even the AI/ML model has a high successful rate for predicting the strongest beam, and therefore at least some of the other predicted beams shall be also verified. Hence, N needs to be larger than 1, i.e., 1<N<=K. But if N is equal to K, which means that all reported predicted beams will be verified and it may be too restricted for UE. As a compromise, N can equal to K2+1, where at least half of the predicted beams will be verified in the tests. This ensures that plural beams predicted by UE-sided AI/ML model have comparable RSRPs to that of the corresponding genie-aided beams with the same rank and provide multiple high-quality candidate beams for data transmission scheduling.

Proposal 2: RAN4 to verify at least half of predicted beams, i.e., N= K2+1.

Regarding the values of x, multiple predicted beams may be verified based on our above discussion, and therefore we think the same x value can apply for each comparison between the same ranked predicted beam and genie-aided beam. The detailed x value can be determined based on simulation results.

Proposal 3: The same x value applies for each comparison between the same ranked predicted beam and genie-aided beam. Value of x is further discussed based on the simulation results.

RSRP accuracy

Discussion on RSRP accuracy is divided into absolute RSRP accuracy and relative RSRP accuracy. In RAN4#114bis meeting, both aspects were discussed and reached some agreements:

RAN4#114bis Agreements: Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Absolute RSRP accuracy applicability

In RAN4#115, the following proposals were brought up:

- Proposals - Option 1: applicable only to top predicted beam - Option 2: should apply to all K predicted beams - Option 3: postpone this discussion to the performance part as it relates to accuracy - Option 4: others - Recommended WF - Option 3

For absolute RSRP accuracy, it was agreed to verify the Top-1 predicted beam at least. For other predicted beam(s) with lower RSRP(s), the differential results will be reported. Hence there will be quantization loss during reporting. Besides, RAN4 also define the relative RSRP accuracy, which is more appropriate to be used to verify the rest of beams. Therefore we do not think the absolute RSRP accuracy requirement needs to apply to other beams.

Proposal 4: RAN4 not to apply absolute RSRP accuracy requirement to other beams, i.e., predicted beams with lower RSRPs.

Relative RSRP accuracy

According to the existing relative RSRP accuracy definition [3], the relative RSRP accuracy is derived as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell. Furthermore, in the test the Top-1 beam can be verified against the absolute RSRP accuracy requirement and the rest of predicted beams can be verified against the relative RSRP accuracy requirement, which can validate the quality of all reported results of predicted beams and meanwhile is able to avoid the impact of performance changes due to quantization procedure. Hence, the square bracket in RAN4#114bis agreement for the relative RSRP accuracy definition can be removed.

- TS 38.133 - 10.1.20.1.2 Relative Accuracy The relative SSB based L1-RSRP accuracy is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements. When the UE is configured with groupBasedBeamReporting-r17 set to 'enabled', the relative SSB based L1-RSRP accuracy is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell on which UE performs L1-RSRP measurements with different Rx beams, the all SSBs are in the same or different resource set (s) in one CSI resource setting.

Proposal 5: RAN4 to remove the square bracket in the definition of relative RSRP accuracy, i.e.,

Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Meanwhile, RAN4 need to determine whether reported L1-RSRP can be measured RSRP. In RAN1#120 meeting, RAN1 achieved an agreement on report content as follows [4]:

Agreement RAN1#120: For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

Based on RAN1 agreement, the reported contents are predicted RSRPs which are based on AI/ML output, and during RAN1 discussion, the AI/ML output could be predicted RSRP or measured RSRP. Hence, the reported L1-RSRP can be measured RSRP. But we are also okay to limit the reported L1-RSRP to predicted RSRP only.

Proposal 6: The reported L1-RSRP in the relative RSRP accuracy definition can be measured RSRP.

Another issue is whether the relative RSRP accuracy definition applies for BM-Case 2. At first, RAN1 have the following agreement in RAN1#120 [4]:

Agreement-RAN1#120: For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

According to RAN1 design for BM-Case 2 reporting, UE can report results in one report over multiple future time instances. And the largest RSRP value is used as reference RSRP to derive the differential RSRPs of other time instances. This reporting mechanism is quite similar to that of BM-Case 1. Therefore, in our understanding, the relative RSRP accuracy requirement can apply for BM-Case 2.

Proposal 7: The relative RSRP accuracy definition applies for BM-Case 2.

Measurement period for inference

In RAN4#116 meeting, RAN4 agreed the framework of prediction report delay and measurement period for inference. For measurement period for inference, the legacy measurement requirements are directly reused for BM-Case 1 and they are also used as a baseline for BM-Case 2, where a new parameter, T, is introduced to scale the number of measurement samples (M) used for inference in time domain.

RAN4#116: Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay + inference delay + time for the first available reporting occasion

In current 38.133, the value of M depends on whether the IE timeRestrictionForChannelMeasurement is configured or not:

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

Meanwhile, network can configure the number of future time instance(s) and the time gap between two consecutive future time instances by RRC according to RAN1 agreements [4]:

RAN1#120 meeting: Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

In Table 1, we approximately calculated the time duration of prediction window that covers all predicted results at corresponding future time instance(s) based on RAN1 design:

Table 1 - Time duration of prediction window (ms)

|  |  | Time gap between two consecutive future time instances (ms) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 | 20 | 40 | 80 | 160 |
| Number of future time instance(s), N | 1 | 10 | 20 | 40 | 80 | 160 |
|  | 2 | 20 | 40 | 80 | 160 | 320 |
|  | 4 | 40 | 80 | 160 | 320 | 640 |
|  | 8 | 80 | 160 | 320 | 640 | 1280 |

When timeRestrictionForChannelMeasurement is configured, i.e., M = 1, and UE is indicated to predict results for 8 future time instances with time gap between two consecutive future time instances is 160 ms, we think a large value may be needed for T to ensure the reliability of predicted results. Another example is M=3 with only one result being predicted, for which case a small T value is enough. Based on the analysis, multiple values may be needed for T for different prediction configurations.

Observation 1: Different values for T may be needed depending on the value of M and prediction configurations.

There are multiple solutions to specify the value(s) of T. One way is to specify a fixed product of T*M. Another way is to specify two different values of T for M=1 and M=3. Defining a UE capability is also a solution. However, in order to make spec concise and reduce the unnecessary UE capabilities, we slightly prefer to deprioritize the usage of UE capability for this issue.

Proposal 8: RAN4 to specify the value(s) of T considering the following alternatives:

- Alt 1: Specify a fixed product of T*M;

- Alt 2: Specify two different values for M=1 and M=3 respectively;

RAN4 deprioritizes the usage of UE capability for defining T.

Another issue is how to define the time duration of inference delay in the prediction report delay requirement. In our understanding, after UE completes the measurements on SetB beams, it can directly predict results for SetA beams for both BM-Case 1 and BM-Case 2, which is a short internally processing procedure. Therefore, we prefer to define a fixed value for the time duration of inference delay procedure in RRM spec.

Proposal 9: RAN4 to define a fixed value for the time duration of inference delay procedure.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: For the case where both predicted beam ID(s) and predicted RSRP(s) are reported, the related tests apply to the top-K beams, where K is the valued configured by nrofreportedpredictedrs-r19.

Proposal 2: RAN4 to verify at least half of predicted beams, i.e., N= K2+1.

Proposal 3: The same x value applies for each comparison between the same ranked predicted beam and genie-aided beam. Value of x is further discussed based on the simulation results.

Proposal 4: RAN4 not to apply absolute RSRP accuracy requirement to other beams, i.e., predicted beams with lower RSRPs.

Proposal 5: RAN4 to remove the square bracket in the definition of relative RSRP accuracy, i.e.,

Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Proposal 6: The reported L1-RSRP in the relative RSRP accuracy definition can be measured RSRP.

Proposal 7: The relative RSRP accuracy definition applies for BM-Case 2.

Observation 1: Different values for T may be needed depending on the value of M and prediction configurations.

Proposal 8: RAN4 to specify the value(s) of T considering the following alternatives:

- Alt 1: Specify a fixed product of T*M;

- Alt 2: Specify two different values for M=1 and M=3 respectively;

RAN4 deprioritizes the usage of UE capability for defining T.

Proposal 9: RAN4 to define a fixed value for the time duration of inference delay procedure.

References

[1] R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated, RAN4#116.

[2] R4-2509098, Topic summary for [116][131] NR_AIML_air_part1, Qualcomm Incorporated, RAN4#116.

[3] 3GPP TS 38.133, Requirements for support of radio resource management, V19.1.0.

[4] Chair notes RAN1#120 eom1, RAN1#120.