3GPP RAN WG4 Meeting #116bis	R4-2513218

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on RRM core requirements and testing framework for positioning

Source: 	CATT

Agenda item:	6.11.5

Document for:	Discussion

Introduction

Impacts of AI/ML based positioning on RRM requirements were discussed in last meeting and the agreements were captured in WF [1]. In this contribution, we will continue to discuss open issues summarized in [2] and analyse potential impacts on RRM core requirements.

Discussion

Reporting Delay Requirements for case 1

In last meeting, RAN4 discussed the reporting delay requirement for case 1 and the following agreement was reached:

RAN4#116 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements

In current positioning requirements taking DL RSTD measurement requirements as an example, measurement sample depends on UE capability, LMP request and side conditions:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

In our understanding, these UE capabilities and LMP signallings still apply for AI/ML based direct positioning case 1. Therefore we prefer to reuse the legacy values and requirements for measurement samples.

Proposal 1: RAN4 to reuse the legacy values and requirements for measurement samples.

Conclusions

This contribution discussed the potential impacts of AI/ML enabled positioning accuracy enhancement on RRM requirements and following proposals are provided:

Proposal 1: RAN4 to reuse the legacy values and requirements for measurement samples.

References

[1] R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated, RAN4#116.

[2] R4-2509098, Topic summary for [116][131] NR_AIML_air_part1, Qualcomm Incorporated, RAN4#116.