3GPP RAN WG4 Meeting #116bis	R4-2513220

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on RRM performance requirements of UE to UE CLI handling

Source: 	CATT

Agenda item:	6.14.3.1

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 approved the work plan for defining the SBFD RRM performance requirements. In this contribution, we will discuss the details of performance requirements for UE to UE CLI measurements based on the RAN4#116 WF [1] and open issue summary [2].

Discussion

L1-SRS-RSRP - side condition on time offset

The assumption of timing offset for L1 CLI measurements depends on specific scenarios and a LS was sent to RAN1 for clarification. In RAN1#119 meeting, the LS was discussed and the following agreement is sent back to RAN4 [2]:

RAN1 Agreement: Send reply LS to RAN4 with following information: When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

According to RAN1 agreement, intra-cell UE-to-UE L1 CLI measurement scenario is prioritized, and therefore the cell phase error can be removed from the Rel-16 SRS-RSRP side condition. However, one proposal was brought up to consider cell phase error for 15 kHz SCS and 30 kHz SCS because the performance degradation between the results with and without cell phase error is quite minor, which is less than 1 dB, and the requirement coverage can also be expanded. Considering that 15 kHz SCS and 30 kHz SCS are the typical configurations in the deployment and there indeed no much difference between the results with and without cell phase error, it is reasonable to define requirements with cell phase error for 15 kHz SCS and 30 kHz SCS to make the requirements applicable to inter-cell scenario. For 60 kHz SCS and 120 kHz SCS, the performance degradation is not negligible and therefore we still prefer to define the requirements without including cell phase error.

Proposal 1: RAN4 to define measurement accuracy requirements with cell phase error for or 15 kHz SCS and 30 kHz SCS. 	For SCS larger than or equal to 60 kHz, time offset condition is defined without cell phase error.

L1-SRS-RSRP - side condition on SRS Es/Iot and BW

Based on the summary of simulation results [3], we observed that the cell phase error has the most significant impact on L1 SRS-RSRP measurement results. If RAN4 agrees to remove cell phase error, which can significantly reduce the measurement error, especially for large SCS configurations. And then selection of number of measurement samples and SRS Es/Iot will have a limited effect on measurement performance. For SRS Es/Iot, it is acceptable to reuse the Rel-16 value, -1 dB.

In regards to the assumption of number of PRBs of SRS resources for L1 CLI measurements, RAN4 needs to consider the sizes of UL subband and total channel bandwidth. For most of SCS and band combinations, it is reasonable to reuse the assumption of 48 PRBs. However, when SCS equals 60 kHz in FR1 bands, there might be some issues.

According to agreements from RF groups, the channel bandwidths supporting SBFD shall be equal to or larger than 40 MHz except for n39, and the size of UL subbands will be chosen and declared from the Table 5.3.2-1 in TS 38.104 [4]. Hence, the applicable sizes of channel bandwidth for 60 kHz SCS in FR1 bands are highlighted below:

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

If SRS-RSRP measurements apply when the size of SRS BW is not smaller than 48 PRBs, which means the size of UL subband shall not be smaller than 48 PRBs. Furthermore, if the size of DL subbands is also included, assuming it is DU case and the sizes of UL subband and DL subband are the same, at least 96 PRBs are needed. And when guardband is required, larger channel bandwidths should be configured. Based on the above analysis, it is clear that up to 3 options are applicable for channel bandwidths configured with 60 kHz SCS in FR1 bands for DU case, and DUD configuration is not applicable in this case. If a UL subband smaller than 48 PRBs is configured, configuring some small bandwidths and DUD case become applicable from the network perspective, however the SRS-RSRP measurement performance requirement will not apply any more.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

As we discussed above, if cell phase error is removed, SRS-RSRP measurement error will be effectively reduced and the benefit of using a large number of PRBs is limited. Therefore in order to make performance requirements applicable for more channel BW configurations with SCS 60 kHz and larger SCS, we prefer to use 24 PRBs as one of the side conditions between two SRS BW assumptions, i.e., 24 and 48 PRBs.

Proposal 2: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot ≥ 1 dB.

- SRS BW: ≥ 24 PRBs.

L1-SRS-RSRP – accuracy requirements

Regarding the accuracy requirements for L1-SRS-RSRP measurements, RAN4 have carried out multiple rounds of simulations and the results are captured in [3]. Therefore it is quite straightforward to define accuracy requirements for L1-SRS-RSRP measurements based on the simulation results under agreed conditions. In addition, as mentioned in previous meetings, the inter-UE interference could come be intra-cell or inter-cell and it is impossible for victim UE to differentiate the location of source of interference. To make the accuracy requirements general and applicable to more scenarios, RAN4 can assume that the accuracy requirements apply for SRS resources from both intra- and inter-cell aggressor, as long as the side conditions are met.

Proposal 3: 	L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions.

Proposal 4: Accuracy requirements apply for SRS resources from both intra- and inter-cell aggressor, as long as the side conditions are met.

L1-CLI-RSSI - accuracy requirements

In RAN4#113 meeting, RAN4 already agreed to reuse 1 sample for L1-CLI-RSSI measurement, which is the same as the L3 RSSI requirement. But in RAN4#116 meeting, RAN4 revised the previous agreement based on RAN1 feedback and the latest agreement is as follows:

RAN4#116: - Agreement (Tuesday AH session) - Based on 1 sample if timeRestrictionForChannelMeasurement is configured; - Based on 3 samples if timeRestrictionForChannelMeasurement is not configured.

Though L1-CLI-RSSI measurement base on 1 or 3 sample(s) depending on timeRestrictionForChannelMeasurement, which is different from the legacy L3 CLI measurement, we think the legacy RSSI measurement accuracy requirement can be reused since the RSSI measurement only reflects the power level in a specific time and frequency range.

Proposal 5: For L1-CLI-RSSI measurement, reuse the legacy L3 RSSI measurement accuracy requirement.

Report mapping

One LS [5] was sent from RAN1 to RAN4 in RAN4#114 meeting to confirm the following RAN1 working assumption on L1 CLI measurement report mapping:

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value. … RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

From RAN4 perspective, we do not identify any issues to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

In addition, RAN1 agreed to support differential report for L1 CLI measurements, and therefore RAN4 should also define measurement report mapping requirements for differential reporting. From our perspective, the differential SS-RSRP and CSI-RSRP measurement (for L1 reporting) report mapping requirements defined in Table 10.1.6.1-2 in TS 38.133 can be reused for L1-SRS-RSRP measurement and L1-CLI-RSSI measurement, i.e.,

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

Proposal 6: RAN4 to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

Proposal 7: The following report mapping tables based on Table 10.1.6.1-2 are used for L1 CLI measurements with differential report,

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

In the rest of this contribution, we will discuss the aspects of test cases and the proposals apply for both UE-to-UE CLI handling and the impacts of SBFD operation on RRM measurements.

General test setup

In RAN4#116 meeting, proposals for general test setup were brought up but RAN4 did not find opportunity to discuss them and therefore no agreement was achieved. A recommended WF was provided to facilitate discussion:

RAN4#116: - Recommended WF - Suggest to agree on the following for general test setup for SBFD RRM test cases, - Test configuration - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode - FR2: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode - TDD UL/DL configuration - DDDSU (this means a new configuration is needed for FR1) - Time domain configuration of SBFD slots/symbols - Option 1: XXXXU - Option 2: DXXXU - Frequency domain configuration of SBFD subbands - DU: 50MHz – 50MHz (can be revisted) - DUD: 40 MHz – 20 MHz – 40 MHz

In general we are okay with the most of proposals. For time domain configuration of SBFD slots/symbols, two options are on the table and technically they can serve the same purpose of verifying UE’s SBFD capability. Between two options we slightly prefer Option 2, i.e., DXXXU.

For frequency domain configuration of SBFD subbands, the recommended configurations are okay to us in FR1. But in FR2 with 100 MHz and 120 kHz SCS, DU configuration gives us 33 PRBs for both UL subband and DL subband respectively and DUD configuration gives us 13 PRBs for UL subband and 26 and 27 PRBs for each DL subband. These test configurations cannot serve the purpose for some measurements, such as L1-SRS-RSRP measurement and CSI-RS L1-RSRP measurement. Moreover, they do not align with the RF requirements for SBFD in clause 5.3.2 in 38.104 [4] where the minimum number of PRBs for UL subband is at least 32 in FR2:

- 5.3.2 Transmission bandwidth configuration … Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz.. Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2 SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 66; 124; 248; N/A. 960; N/A; 33; 62; 124; 148. NOTE: All Tx and Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1 for FR1 and table 5.3.2-2 and table 5.3.2-3 for FR2. The transmission bandwidth configuration for NB-IoT is specified in TS 36.104 [13] clause 5.6. For SBFD operation, the transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL shall be configured according to the values provided for BS transmission bandwidth configuration NRB corresponding to bandwidth no less than 10MHz in Table 5.3.2-1 for FR1, and NRB in Table 5.3.2-2 for FR2-1. Transmission bandwidth configuration for SBFD DL subband in SBFD (i.e., NRB,SBFD,DL for SBFD DU or UD operation, and NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for SBFD DUD operation) and guardband between SBFD UL subband and DL subband shall be configured by SBFD-capable BS. For SBFD DUD operation, NRB,SBFD,DL,1 and NRB,SBFD,DL,2 shall have the same size or different sizes by only 1RB.

In order to ensure that enough PRBs are configured in DL subband(s), a lager BW for both DU and DUD cases in FR2, e.g., 400 MHz, is proposed to be used.

Proposal 8: RAN4 to adopt the following test setups for SBFD RRM test cases:

    - Test configuration

      - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode

      - FR2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode

    - TDD UL/DL configuration

      - DDDSU (this means a new configuration is needed for FR1)

    - Time domain configuration of SBFD slots/symbols

      - DXXXU

    - Frequency domain configuration of SBFD subbands

      - DU: 50MHz – 50MHz

      - DUD: 40 MHz – 20 MHz – 40 MHz

Test case list

The proposed test case list was also provided in the summary [2]:

RAN4#116: - Proposals Samsung; Nokia; CTC; CMCC; E///; HW; ZTE; MTK. L1-SRS-RSRP measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y; Y. L1-CLI-RSSI measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y; Y. SSB L1 measurement; Y; Y; Y. SSB L3 measurement; Y; Y. CSI-RS RLM; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS BFR; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-RSRP measurement; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-SINR measurement; Y; Y; Y; Y; Y. CSI-RS L3 measurement; Y; Y; Y. RACH; Y. - Recommended WF - Suggest to agree to define at least following test cases - L1-SRS-RSRP measurement (delay and accuracy) - L1-CLI-RSSI measurement (delay and accuracy) - CSI-RS RLM - CSI-RS BFR - CSI-RS L1-RSRP measurement (delay and accuracy) - Discuss whether to define following test cases - CSI-RS L1-SINR measurement (delay and accuracy) - SSB L1 measurement - SSB L3 measurement - CSI-RS L3 measurement - RACH

According to the proposals, it is not controversial to agree to define test cases for the following measurements and we are also supportive for this recommended WF:

    - L1-SRS-RSRP measurement (delay and accuracy)

    - L1-CLI-RSSI measurement (delay and accuracy)

    - CSI-RS RLM

    - CSI-RS BFR

    - CSI-RS L1-RSRP measurement (delay and accuracy)

In addition, companies have different opinions for the following 5 measurements:

    - CSI-RS L1-SINR measurement (delay and accuracy)

    - SSB L1 measurement

    - SSB L3 measurement

    - CSI-RS L3 measurement

    - RACH

For CSI-RS L1-SINR measurement, we think it is okay not to define test cases to reduce the number of test cases for SBFD feature, considering that test cases will be defined for CSI-RS L1-RSRP measurement which would be quite similar to CSI-RS L1-SINR measurement.

For SSB based L1/L3 measurement and RACH, since not many impacts are introduced by SBFD feature, we support not to define test cases for these measurements and procedure.

For CSI-RS L3 measurement, RAN4 reuses the legacy requirements assuming that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS, in which case there is not much technical difference. Therefore we prefer not to define test case for CSI-RS L3 measurement.

Proposal 9: RAN4 to define test cases for the following measurements:

    - L1-SRS-RSRP measurement (delay and accuracy)

    - L1-CLI-RSSI measurement (delay and accuracy)

    - CSI-RS RLM

    - CSI-RS BFR

    - CSI-RS L1-RSRP measurement (delay and accuracy)

Conclusions

This contribution discussed the impacts of UE-to-UE CLI measurements on RRM performance requirements and some proposals are provided:

Proposal 1: RAN4 to define measurement accuracy requirements with cell phase error for or 15 kHz SCS and 30 kHz SCS. 	For SCS larger than or equal to 60 kHz, time offset condition is defined without cell phase error.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

Proposal 2: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot ≥ 1 dB.

- SRS BW: ≥ 24 PRBs.

Proposal 3: 	L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions.

Proposal 4: Accuracy requirements apply for SRS resources from both intra- and inter-cell aggressor, as long as the side conditions are met.

Proposal 5: For L1-CLI-RSSI measurement, reuse the legacy L3 RSSI measurement accuracy requirement.

Proposal 6: RAN4 to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

Proposal 7: The following report mapping tables based on Table 10.1.6.1-2 are used for L1 CLI measurements with differential report,

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

Proposal 8: RAN4 to adopt the following test setups for SBFD RRM test cases:

    - Test configuration

      - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode

      - FR2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode

    - TDD UL/DL configuration

      - DDDSU (this means a new configuration is needed for FR1)

    - Time domain configuration of SBFD slots/symbols

      - DXXXU

    - Frequency domain configuration of SBFD subbands

      - DU: 50MHz – 50MHz

      - DUD: 40 MHz – 20 MHz – 40 MHz

Proposal 9: RAN4 to define test cases for the following measurements:

    - L1-SRS-RSRP measurement (delay and accuracy)

    - L1-CLI-RSSI measurement (delay and accuracy)

    - CSI-RS RLM

    - CSI-RS BFR

    - CSI-RS L1-RSRP measurement (delay and accuracy)

References

[1] R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#116.

[2] R4-2509061, Topic summary for [116][218] NR_duplex_evo, Moderator (Huawei, HiSilicon), RAN4#116.

[3] R4-2512190, Updated summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon, RAN4#116.

[4] R4-2511255, CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS, Samsung, RAN4#116.