3GPP RAN WG4 Meeting #116bis	R4-2513227

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on impacts of AIML RRM measurement prediction on RRM core requirements

Source: 	CATT

Agenda item:	7.12.3

Document for:	Discussion

Introduction

In RAN#109 meeting, revised WID of Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved with the following objectives [1]:

The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

From RAN4#116bis meeting, RAN4 will start to discuss the RRM impacts of this WI. In this contribution, we will analyse potential impacts of RRM measurement prediction and present our views.

Discussion

Scope of AI/ML RRM measurement prediction

In TR 38.744, there are 6 scenarios that were prioritized for evaluation in SI phase [2]:

Table 5.2.1.1-1: Prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | intra-cell |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Furthermore, sub-use cases are also listed in TR 38.744 [2] as follows:

3 sub-use cases are considered for cell-level RRM measurement prediction: - RRM sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated; - RRM sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s); - RRM sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s). 3 sub-use cases are considered for beam-level RRM measurement prediction: - Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated; - Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s); - Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

Considering that RRM group mainly specifies requirements for UEs and based on the guidance in the WID, L3 beam level prediction and spatial domain prediction are not supported for UE sided model. Therefore RAN4 only need to consider L3 cell level prediction for RRM measurement prediction. In summary, RAN4 consider the following scope for AI/ML RRM measurement prediction use case:

- Intra-frequency temporal domain prediction in both FR1 and FR2 with 2 goals;

  - The 1st goal: reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurement results.

  - The 2nd goal: improve the handover performance (e.g. HOF).

- Inter-frequency (frequency domain) for FR1 to FR1 with co-located deployment;

- L3 cell level prediction considered only.

Proposal 1: RAN4 consider the following scope for AI/ML RRM measurement prediction use case:

- Intra-frequency temporal domain prediction in both FR1 and FR2 with 2 goals:

  - The 1st goal: reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurement results.

  - The 2nd goal: improve the handover performance (e.g. HOF).

- Inter-frequency (frequency domain) for FR1 to FR1 with co-located deployment;

- L3 cell level prediction considered only.

Measurement/Inference delay requirements

Before discussing the detailed impacts on RRM core requirements, we would like to analyse the procedures of case A and case B. In the table cited in clause 2.1, we found that the goal of case A is to reduce handover failure (HOF) (2nd goal) and case B is to reduce measurement effort (1st goal). According to TR 38.744 [2], the mechanism of case A and case B is defined as follows respectively:

Case A: In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. Figure 5.2.1.1-1: Example of intra-frequency temporal domain case A Case B: In intra-frequency temporal domain case B, measurement results in PW are predicted by historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous PW is/are skipped during window sliding. Figure 5.2.1.1-2: Skipping pattern example 1 of intra-frequency temporal domain case B Figure 5.2.1.1-3: Skipping pattern example 2 of intra-frequency temporal domain case B

In case A, for a given time instance at which UE needs to produce a result in prediction window (PW), UE will predict the result based on continuous historical measurement result(s) in observation window (OW), and then UE will measure reference signals at this time instance. The predicted results will be used to determine whether handover will happen or not and we can see that there is no measurement efforts are reduced.

In case B, for a given time instance at which UE needs to produce a result, it belongs to PW or OW, i.e., UE will either predict the result based on some historical measurement result(s) in observation window (OW) or obtain the result based on measurement on reference signals. In other words, UE will have only one result for any time instance at which UE needs to produce a result.

Therefore case A and case B have different impacts on core requirements based on above analysis.

Observation 1: Case A and case B have different impacts on core requirements based on the following aspects:

- In case A, UE has two results for any time instance at which UE needs to produce a result;

- In case B, UE has only one result for any time instance at which UE needs to produce a result.

In addition, it is apparent that the configuration of OW and PW will affect UE’s measurement behaviour. For example, the configuration of OW may limit the number of measurement samples that UE can have in the OW, which will further impact the prediction accuracy. Therefore RAN4 need to consider the impacts of OW and PW lengths on RRM requirements for case A and case B respectively. But since this is the first meeting for this work item and the framework of AI/ML based mobility has not be designed by RAN2 yet, RAN4 can discuss the details later based on more concrete foundations after RAN2 have progresses. But in our understanding, a unified framework for defining core requirements for both cases can be beneficial. For example, the total delay includes measurement delay plus inference delay and other needed components.

Proposal 2: RAN4 to discuss the impacts on RRM core requirements for case A and case B separately with more RAN2 progress on framework design, e.g., supported OW/PW length.

Proposal 3: RAN4 to define requirements for both case A and case B with a unified framework, e.g., the total delay = measurement delay + inference delay + other needed components.

For scenario 3, i.e., FR1 to FR1 inter-frequency (frequency domain) prediction in co-located cases, its goal is also to reduce measurement efforts same as Case B. In our understanding, the measurement requirement defined for Case B can be used for scenario 3 as a baseline. Besides, RAN4 need to discuss the applicability requirements for scenario 3 considering this is the only one case that involves inter-frequency prediction.

Observation 2: Scenario 3 has a similar measurement/prediction procedure and goal as Case B.

Proposal 4: The requirements defined for Case B can be used for scenario 3 as a baseline. RAN4 need to discuss applicability requirements for scenario 3 considering the aspects introduced by inter-frequency prediction.

LCM

Regarding the LCM part, the achievements in Rel-19 AI/ML based beam management (BM) can be used as a baseline. In Rel-19 AI/ML based beam management, RAN4 agreed the AI/ML model activation delay as follows [3]:

RAN4#116: Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

If RAN2 agree to reuse the signalling framework for applicability report in Rel-19 AI/ML based BM, then this agreement can also apply for AI/ML based mobility.

In addition, RAN4 may need to define LCM requirements for performance monitoring, but this needs more RAN2 design. Therefore, RAN4 can discuss how to define requirements for performance monitoring in a later phase.

Another procedure for which RAN4 need to define LCM requirements is fallback scheme, i.e., when AI/ML model performance is worse than a certain level and UE is indicated by network to deactivate AI/ML models and fallback to non-AI methods, UE needs to be ready to measure and report the results in a specified time duration.

Proposal 5: For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report:

-	UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Proposal 6: RAN4 to discuss how to define LCM requirements for performance monitoring and fallback procedures after RAN2 have more concrete progress.

Conclusions

This contribution discussed the impacts of AI/ML RRM measurement prediction on RRM requirements, and some observations and proposals are provided:

Proposal 1: RAN4 consider the following scope for AI/ML RRM measurement prediction use case:

- Intra-frequency temporal domain prediction in both FR1 and FR2 with 2 goals:

  - The 1st goal: reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurement results.

  - The 2nd goal: improve the handover performance (e.g. HOF).

- Inter-frequency (frequency domain) for FR1 to FR1 with co-located deployment;

- L3 cell level prediction considered only.

Observation 1: Case A and case B have different impacts on core requirements based on the following aspects:

- In case A, UE has two results for any time instance at which UE needs to produce a result;

- In case B, UE has only one result for any time instance at which UE needs to produce a result.

Proposal 2: RAN4 to discuss the impacts on RRM core requirements for case A and case B separately with more RAN2 progress on framework design, e.g., supported OW/PW length.

Proposal 3: RAN4 to define requirements for both case A and case B with a unified framework, e.g., the total delay = measurement delay + inference delay + other needed components.

Observation 2: Scenario 3 has a similar measurement/prediction procedure and goal as Case B.

Proposal 4: The requirements defined for Case B can be used for scenario 3 as a baseline. RAN4 need to discuss applicability requirements for scenario 3 considering the aspects introduced by inter-frequency prediction.

Proposal 5: For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report:

-	UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Proposal 6: RAN4 to discuss how to define LCM requirements for performance monitoring and fallback procedures after RAN2 have more concrete progress.

References

[1] RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital, RAN#109.

[2] 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, V1.2.0.

[3] R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated, RAN4#116.