3GPP TSG-RAN WG4 Meeting # 116bis 	R4-2513251

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on 6G general RF

Agenda Item:	8.4

Document for:	Approval

- Introduction

According to the RAN4 6G guideline in R4-2511652, the following aspect of can be discussed before 2025 Q4:

In this contribution, we provide our views on the study point of 6G UE RF.

- Discussion

  - Non-spectrum UE RF

- Power class

In NR, the power class is defined in many conditions, e.g., single carrier, CA, EN-DC, MIMO, etc., and even in same power class, UE can have different implementations, e.g., PC2 UE could use single 26 dBm RF chain but also could be two 23 dBm RF chains, which make the power class indication is quite complicated. In NR, the following UE capability was introduced:

Table I Power class related UE capability in NR

| UE capability | Description |
| --- | --- |
| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 | Per band UE power class |
| powerClass, powerClass-v1610 | Power class for band combination |
| powerClassNRPart-r16 | Power class of NR in EN-DC, solve the ambiguity due to shared RF chain between LTE and NR |
| higherPowerLimit-r17 | Indicate whether the power class of inter-band CA/NR-DC can be changed to the sum of power class of all bands |
| higherPowerLimitMRDC-r17 | Indicate whether the power class of inter-band MRDC can be changed to the sum of power class of all bands |
| ue-PowerClassPerBandPerBC-r17 | Indicate the power class of each band for a band combination |

To further investigate problem in NR, we list different UE architecture and the corresponding capability reporting when UE equipped with multiple RF chain:

Table II UE architecture and capability reporting in NR

| UE architecture | Single CC |  | Note |
| --- | --- | --- | --- |
| 23+23 | ue-PowerClass (PC3) |  |  |
| 23+23 | ue-PowerClass(PC2) + txDiversity-r16 |  |  |
| 23+26 | ue-PowerClass (PC2) |  | 1.8 dB power loss |
| Band A | Band B | DC/CA_nA-nB | Note |
| 23+23, PC3 | 23, PC3 | powerClass(PC2) |  |
| 23+23, PC3 | 23, PC3 (shared) | powerClass(PC2) |  |
| 23+23, PC3 | 26, PC2 | higherPowerLimit-r17 | band A:23 band B:26 |
| 23+23, PC2 | 23, PC3(shared) | PowerClass(PC2) |  |
| 23+23, PC2 | 23, PC3 | higherPowerLimit-r17 | band A:26 band B:23 |
| 23+23, PC2 | 26, PC2 | powerClass(PC1.5) |  |
| 23+26, PC2 | 23, PC3 (shared) | higherPowerLimit-r17 | band A:26 band B:23 |
| 23+26, PC2 | 26, PC2 (shared) | higherPowerLimit-r17 + ue-PowerClassPerBandPerBC-r17 | band A:23 band B:26 |
| 23+26, PC2 | 23, PC3 | higherPowerLimit-r17 | band A:26 band B:23 |
| 23+26, PC2 | 26, PC2 | higherPowerLimit-r17 | band A:26 band B:26 |
| 26+26 PC2 | 23 PC3 | higherPowerLimit-r17 | band A:26 band B:23 |
| 26+26 PC2 | 26 PC2 | powerClass(PC1.5) |  |
| 26+26 PC1.5 | 23 PC3 | higherPowerLimit-r17 | band A:29 band B:23 |
| 26+26 PC1.5 | 26 PC2 | higherPowerLimit-r17 | band A:29 band B:26 |
| 26+26 PC1.5 | 26 PC2 (shared) | higherPowerLimit-r17+ ue-PowerClassPerBandPerBC-r17 | band A:26 band B:26 |

In addition, the MIMO power class is also complicated since RAN4 define the power class for MIMO without explicit capability indication, which lead to a long discussion for power scaling mode and full power mode in 3GPP. This also need to be considered.

Based on the analysis above, we can get the following observations:

Observation 1:

- For the combined power of multiple Tx is between two power class level, power loss exists (e.g., 23+26)

- powerClass and higherPowerLimit-r17 have similar function which may cause ambiguity if both are indicated

- HPUE enhancement always begin with single band before CA in standardization, and single band has higher power capability than CA is usual case, which make the UE power in CA could be restricted.

  - higherPowerLimit-r17 can help on this issue.

  - Signaling can also be simplified if higherPowerLimit-r17 is the default assumption in 6G

- If the power of each band under band combination still can reach the power level reported per band, it can be clearly reported by using powerclass or higherPowerLimit-r17

  - Otherwise PowerClassPerBandPerBC-r17 needs to be reported

- RAN4 define the power class for MIMO without explicit capability indication, which lead to complicated discussion for power scaling and full power mode

One of the sources that leads to the complexity of power class is the TxD, which UE can report a power class higher than any of the power ability of its RF chain, and when RF chain sharing happens, such UE has to fallback to lower power class. In 6G, we expect the power class can be decouple with UE implementation, and one possible way is that the power class is always the maximum power of UE can be achieved via single Tx. For example, in a certain band, the UE with 1Tx 26 dBm, 26+26 dBm, 23+26 dBm, the maximum power for single Tx is 26 dBm, so all these UE can only report PC2. For UE wants to enable higher power by TxD, power boosting approach can be used.

Proposal 1: Study a proper way to decouple the power class and UE implementation in 6G, and the following approach can be considered:

- UE per band power class defined as the maximum output power via single Tx

- For UE equip with multiple Tx, higher power can be achieved by power boosting approach

- higherPowerLimit-r17 in NR can be the baseline assumption in 6G

Another pain point in NR is how to maximize the utilization of the power ability of UE. For CA, a UE may in following status:

- UE is configured with DL CA, but no UL CA

- UE is configured with UL CA, but the Scell is deactivated.

- UE is configured with UL CA and Scell is activated, but no scheduling in Scell

- UE is configured with UL CA and Scell is activated, and scheduling exists in Scell

In NR, only status a and d are clear, b and c have a long discussion but no consensus achieved. The reason behind could in the later release the UE implementation is already different for vendors. In 6G, it is a chance to revisit these issues and find a way to make the power ability become clearer.

Proposal 2: The power ability under different UE condition is expected to be clarified clearly in 6G.

The last point we want to discuss here is the default power class. The PC3 23dBm was used as default power class in NR, but it could be seen that enhanced uplink coverage capability is expected in 6G. Currently, especially in the TDD frequency band, the uplink coverage capability is insufficient, mainly due to the limitation of the total uplink transmission power and the relatively limited uplink time resources of TDD bands. Moreover, the demand for higher uplink rates would grow significantly with the increase in AI services. For 6G, considering the introduction of new spectrum like ‘FR3’ which is higher than the current FR1 range, the high-frequency path losses need to be compensated, enhancing the default power class could be considered. Comparing to PC3, PC2 could increase the uplink coverage radius by more than 20%, and users at the edge of the cell could enjoy a better experience.

Observation 2: Increasing the default power class level would effectively expand the coverage radius and improve the uplink rates, significantly improving the users’ experience.

Proposal 3: The default power class in 6G can be considered set to PC2 26 dBm.

- Duty cycle

The duty cycle was introduced in NR as a possible SAR solution. However, in RAN4, the P-MPR serve as similar function from LTE to solve the SAR related issue which is universally used for handheld UE, and the relationship between P-MPR and duty cycle is not clear at least for FR1 which may cause unnecessary power restriction on UE. Besides, the duty cycle for TDD band is understandable but for FDD band whether it still can be used is not clear. A maintenance discussion is also on-going in 5G NR on the FDD duty cycle issue.

In addition, the duty cycle based SAR solution makes the UE behavior quite complicated due the possible power class fallback, but the SAR test normally will keep a long time which is typically larger than a real UE uplink scheduling. It is also doubtful whether such operation can really help UE in the field.

Observation 3: The duty cycle based SAR solution in NR causes complicated UE behavior and it is also doubtful whether the UE in the field can be benefit from such operation.

Proposal 4: RAN4 to check the necessity or possible improvement of NR duty cycle based SAR solution in 6G.

- RF requirement design in FR3

During NR, the UE architecture in FR3 was discussed for several times, but unfortunately it is hard to determine the final decision.

TR 38.922:

One of the key issues for this range is the UE RF architecture. While existing FR1 system parameters are defined for frequencies up to 7.1GHz, same FR1 principles are already assumed for e.g. 7.1-8.4GHz, which is however below the considered range of 14.8-15.3GHz. At the same time, the lower bound for the FR2 starts at 24.25GHz, which is notably higher than 15.3GHz. A choice for a particular UE RF architecture at these frequency ranges should consider implementation feasibility, e.g. physical constraints in case of handheld devices, and anticipated performance, such as coverage. The capabilities of PA technology in the FR2 frequency range motivated 3GPP to consider a UE architecture which relies on panels performing analog beam-forming, which was also considered as a feasible option from the handheld device implementation perspective. In contrast, panels and analog beam-forming are not feasible for lower FR1 frequencies.

TR 38.820:

For the sub-range 2 (i.e. fboundary, low - fboundary, high; where fboundary, low is within the frequency range 10 - 13 GHz and fboundary, high is within the frequency range 16 - 18 GHz, , as in table 5.2-1), PAs could have the considerable performance as for the frequency sub-range 1 with advanced IC process, and there is limited space for UE to place more antenna elements. Depending on the frequency boundaries of the frequency sub-range 2, more flexibility is envisioned:

-	If frequency sub-range 2 reaches to 16 GHz, the conducted RF requirements (FR1-like) can be considered,

-	If frequency sub-range 2 reaches up to 18 GHz, the radiated RF requirements (FR2-like) can be considered,

-	Some combination of both approaches is not precluded.

Observation 4: The UE architecture in FR3 was discussed several times in different SI, but it is still hard to give the final decision, especially for the frequency higher than 13GHz.

The UE architecture will impact on the RF requirement design. In NR, the FR1 UE use conductive requirement and all requirement is applied to antenna connector while the FR2 UE use radiated requirement which include the antenna performance. However, with the frequency increased, it will be questionable whether the antenna connector is still easily implemented in the device.

For Ku band NTN in NR R19, an overlapping range is defined in the spec TS 38.101-5:

Although currently only radiated requirement is applied in this overlapping range, but such approach give a possible way in 6G to avoid to touch the UE architecture again in FR3, which means we can define an overlapping range with both conductive requirement and radiated requirement. However, the drawback for such approach is we should ensure these two sets of requirements are comparable with each other within same frequency range.

Proposal 5: The following options can be considered for FR3 (7-24 GHz) RF requirements design

Option 1: Decide a clear breakpoint within FR3 to distinguish conductive requirement and radiated requirement

  - Encourage companies to provide the feasibility analysis on the upper limit for the conductive requirement.

Option 2: Define an overlapping range within FR3 where both conductive requirement and radiated requirement are applied.

- FFS how to ensure the conductive requirement and radiated requirement are comparable within overlapping range

  - Spectrum aggregation

- Single cell multi-carrier

In RAN1#122, the single cell multi-carrier was discussed as a possible improvement of multi-carrier operation.

Figure 1 Single cell multi-carrier operation in [1]

From RAN1 perspective, this operation could reduce network energy consumption/overhead of common signal transmission and configuration/scheduling overhead compared with existing carrier aggregation. However, RAN4 should discuss the feasibility and possible restriction of such operation from UE side. First restriction should be studied in RAN4 is the criteria for determining which spectrum resource can be aggregated into a single cell.  A simple illustration for NR UE is shown in Figure 2

Figure 2 Illustration of NR UE architecture [2]

To simplify the hardware and increase circuit integration, the chip design group some adjacent frequency bands as a unit, and such frequency bands could share RF component in front-end and transceiver. For single cell multi-carrier, the adjacent bands from practical chip design could be a possible principle to decide which spectrum can be aggregated.

Observation 5: From chip design perspective, some adjacent operation bands are grouped as a unit and share part of RF component in front-end and transceiver.

Proposal 6: Study the criteria for determining which spectrum resource can be aggregated into a single cell, e.g., within adjacent frequency bands based on practical chip design. The target is no additional UE RF front-end complexity to support this feature.

Another issue needs to be studied is the baseline assumption of UE architecture and related behavior. In NR, there are 3 possible ways to support multi-carrier operation, and the pros & cons are summarized in Table III

Table III Approaches to support multi-carrier in NR

|  | Pros | Cons |
| --- | --- | --- |
| RF chain switching | - Low complexity and cost - Avoid sensitivity degradation | - Low spectrum efficiency |
| Shared RF chain | - Low complexity and cost - High spectrum efficiency | - Large power backoff for Tx - Unstable reception due to interference |
| Separate RF chain | - High spectrum efficiency - Same as CA architecture | - High complexity and cost - Possible sensitivity degradation |

In NR, the UE behavior in single cell is clear and no confusion between UE and NW since only single carrier is configured and scheduled. However, even under same multi-carrier configuration for single cell, the NW scheduling could be restricted which depends on UE architecture. Some possible UE architecture and corresponding NW behavior are listed in Table IV.

Table IV Possible UE architecture and NW behavior under different CC configuration

| Conditions | UE architecture | NW behavior |
| --- | --- | --- |
|  | Shard RF chain | Simultaneous scheduling for all CC |
|  | RF chain switching | Only TDM |
|  | Shared RF chain | Simultaneous scheduling but need UE reporting or measurement |
|  | Separate RF chain | Simultaneous scheduling for all CC |
|  | RF chain switching | Only TDM |
|  | Separate RF chain | Simultaneous scheduling for all CC |

It is not clear what is the baseline assumption for UE under multi-carrier cell which may lead to incorrect NW scheduling and configuration, and we suggest RAN4 should study the baseline assumption and identify possible problems.

Observation 6: For a cell configured with multi-carrier, the baseline architecture and behavior are not clear which may cause incorrect NW configuration or scheduling.

There could be two possible ways to go in RAN4. One is RAN4 to decide a clear UE architecture and behavior under SCMC, and RF chain switching is preferred to alleviate the UE design complexity, but this may make additional restriction at NW side. Another way is to allow all possible architecture, but RAN4 need to discuss how to help NW can understand UE behavior clearly via proper capability indication.

Proposal 7: Study the baseline UE architecture and behavior under single cell multi-carrier (SCMC) and the following two options can be further discussed:

Option 1: RAN4 to determine a clear UE architecture and behavior under SCMC

- RF chain switching across all CCs is preferred to alleviate UE design complexity

- Possible NW configuration/scheduling restriction needs to be considered

Option 2: RAN4 allow all kinds of UE architecture and behavior under SCMC

- RF chain switching/shared RF chain/separate RF chain can be considered as candidate UE architecture

- RAN4 to discuss how to help NW clearly understand UE behavior via proper UE capability design

- UL/DL decouple

The benefit of UL/DL decouple from NW perspective is already studied for many years. This operation can improve the UL interference, increase the UL data rate, get better load balance, etc., as show in figure 3

Figure 3 Benefit of UL/DL from NW side [3]

However, from UE RF perspective, the UL/DL decouple means that the RF chain could be unoccupied, e.g., UE normally is capable to transmit and receive in a TDD band and if a UL-only carrier is configured, the Rx RF chains could be idle, and the UE hardware is not efficiently used under such condition.

Observation 7: From UE RF perspective, such operation may cause the RF chains are unoccupied and the usage of RF chain could be inefficiently.

Proposal 8: Study more efficient RF chain utilization under UL/DL decouple, e.g., enable higher MIMO layer or transmission/reception with more carriers.

  - Coverage

- General

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

In 6G SID, it is assumed that the 7GHz should achieve similar coverage as 3.5GHz:

Considering larger channel bandwidth and higher path loss/penetration loss, it become more difficult for UE to achieve same SNR in 7GHz under existing 3.5GHz site grid. Except increase UE output power, e.g., from 23 dBm to 26 dBm, the MPR reduction and power boosting also need to be studied in the early stage.

- MPR reduction

The MPR allow UE reduce its power and keep PA work in linear region to avoid the UE violate the emission requirement. For 200MHz channel bandwidth, one reference in NR is the intra-band contiguous CA, which is also assumed using 1Tx/2Tx, and the class C MPR for PC2 is shown in below:

Table V Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class C (1Tx) |  | MPR for bandwidth class C (2Tx) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 7 | 3.5 | 8 |
|  | QPSK | 2.5 | 7 | 3.5 | 8 |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 |
|  | 64QAM | 5 | 7 | 6 | 8 |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.5 | 8 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 8 | 4.0 | 8.5 |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 |

Even for inner case, there still around 3 dB MPR exist for PC2 200MHz contiguous CA, which make the real output power can only be PC3, which may dissipate the increased UE output power, so the MPR reduction should be studied at the beginning of 6G due to the large channel bandwidth.

Observation 8: The large MPR under 200MHz channel bandwidth could dissipate the increased UE output power, and it is necessary to study how to improve it in 6G.

Based on the experience in NR, there are 3 possible ways to reduce the MPR and we further discuss them one by one.

Approach 1:PA non-linearity improvement

This approach is quite straightforward and can reduce the MPR from UE side. For larger signal bandwidth, a problem we need to face is the memory effect, as show in Figure 4

Figure 4 AM-AM/AM-PM under different BW [4]

When signal bandwidth increases, the signal distortion become more significant which will lead to worse emission performance and require larger MPR. One classic way to improve the PA performance is to implement digital pre-distortion, as shown in Figure 5

Figure 5 PA non-linearity improvement by DPD [5]

In NR, the DPD is already universally used in UE side but the MPR evaluation is based on the PA itself only, so for 6G, we suggest to study the PA model with DPD assistance.

Observation 9: The DPD can improve PA linearity and mitigate memory effect which is universally used in NR UE, but the NR MPR evaluation was based on PA itself for simplicity.

Approach 2: RF requirement relaxation

In R19 and R20, MPR reduction based on RF requirement relaxation is discussed and the corresponding scenarios are summarized in Table VI

Table VI Scenarios of NR RF requirement relaxation

| Scenarios | Relaxed RF requirements |
| --- | --- |
| UE channel bandwidth is smaller than BS, e.g., RedCap | ACLR, SEM |
| Spectrum without adjacent operator | ACLR, SEM |
| Advanced BS receiver for EVM compensation | EVM |

In 6G, more scenario can be identified, for example, the relaxations above are focus on single carrier case, but intra-band CA MPR in NR is designed as a very high value (around at least 10 dB). Whether such relaxation could also be extended to multiple CC case can be studied in 6G.

Observation 10: RF requirement relaxation in NR only focus on single carrier case.

Another possible way to relax RF requirement is to revisit the emission requirement. The similar SEM and IBE definition are used from LTE which is already more than 10 years. Take SEM as an example, the SEM originally

considered both regulation and spectrum regrowth of signal itself, as shown in below:

Figure 6 Explanation of LTE SEM in TR36.803

In 6G we have new band, and the regulation considered in LTE may not be still valid and some regulation may change since many years past. Similar story also happened on IBE, so there is a chance to revisit the emission requirement to check whether it is feasible to be updated or relaxed.

Observation 11: Similar emission requirements keep reusing from LTE, and there is chance to check the feasibility of update and relaxation in 6G, at least for the new spectrum.

Approach 3: Improvement of MPR evaluation/definition

In NR, the MPR for QPSK/16QAM was divided in to different regions with different values based on the RB allocations, while the 64QAM and 256QAM use a single value for all RB allocations. The reason behind could be that for higher modulation order the MPR of all RB allocations are only gated by EVM, which make the change of MPR value doesn’t have clear trend. However, based on the simulation results, there are still rooms for some RB allocation to use smaller MPR rather than the worst value of all RB allocation.

Another possible improvement is the evaluation assumption, and take intra-band contiguous CA MPR as example. Currently, NR PC2 intra-band contiguous CA MPR of 2Tx is larger than 1Tx. The rationale behind is that the additional impact of reverse IMD is considered in 2Tx MPR. Considering the PC2 PA is more mature in 6G, it could be also possible for UE to equipped with two PC2 PA in 6G, and for some cases where MPR is large, UE is able to work with 2x23 dBm with two PC2 PA to reduce the MPR value. There could be a trade-off between full use of PA capabilities which facing high MPR and flexible configure the PA operation status with lower MPR.

Observation 12: There is room for improvement for NR MPR definition and 6G UE with enhanced hardware have chance to reduce the MPR by flexible using its RF ability.

Proposal 9: Study the following approaches to reduce the MPR in 6G

- Approach 1:PA non-linearity improvement, e.g.,

- PA model with DPD assistance

- Approach 2: RF requirement relaxation, e.g.,

- Identify more scenarios where the RF requirements can be relaxed, e.g., extend relaxation in single carrier to multiple carrier

- Revisit the emission requirements and check whether update is feasible.

  - New spectrum in 6G as starting point

- Enable NR coverage enhancement feature in 6G day 1

- Approach 3: Improvement MPR evaluation/definition, e.g.,

- Check the possible change of NR MPR definition

- Check the possible update of evaluation assumption based on the 6G UE architecture

- Power boosting

In NR, there are also many power boosting features were discussed and are summarized in Table VII

Table VII Power boosting in NR

| UE capability | Details | Spec impact |
| --- | --- | --- |
| powerBoostPi2BPSK | 2/pi BPSK with spectrum shaping | PEMAX,c , ΔPPowerClass |
| powerBoosting-pi2BPSK-QPSK-r18 | New RB region where MPR<0 | ΔPPowerBoost |
| powerBoosting-pi2BPSK-QPSK-Modified-r18 | FDSS | ΔPPowerBoost |
| mpr-PowerBoost-FR2-r16 | FR2 IBE relaxation | PIBE |

In our understanding, the MPR depends on UE itself which impact the lower bound of UE output power, but the power boosting can enable UE to transmit a power larger than its nominal power class, which need the allowance of NW. The spec impact is scattered for power boosting in NR and the principle of when/whether different power boosting can be used together is also ambiguous.

Observation 13: The power boosting in NR is triggered by individual topic, and the spec impact is scattered and lack of general principle.

For 6G, we prefer to have a dedicated requirement to describe the power that UE can boost, and crate a general framework and principle for these features.

Proposal 10: Study a new RF requirement, e.g., maximum power boosting, to construct a general framework for power boosting feature in 6G

  - Energy efficiency

From RF hardware perspective, the energy efficient is closely related to power amplifier. Typical way to improve the power efficacy of PA is using envelope tracking (ET) or average power tracking (APT). Both ET and APT can improve the PA efficiency by dynamically adjusting the supply voltage. The ET can behave better on lager PAPR modulation, e.g., 64QAM and 256QAM, and comparation between ET and APT are shown in Figure 7

Figure 7 Comparation between ET and APT [6]

However, for 6G with larger channel bandwidth, it is challenge for ET modulator design and may not still work efficiently. Another possible way to improve the PA efficiency is to use Doherty PA (DPA). The DPA is typically constructed by a class AB PA and a class C PA, and the class C PA is only work for upper few dB of the range (typically 6 dB), which can provide a power region with high efficiency.

Figure 8 Illustration of Doherty PA[7]

The Doherty PA have some drawbacks which make it hard to be implemented in NR UE, e.g., nonlinearity due to asymmetric AM-AM/AM-PM response, thermal management, etc. However, in recent years there are several techniques can overcome such problem and make UE use such PA is possible [8]. Even though the Doherty PA closely rely on the assistance of DPD, but in 6G, we can study such new PA to improve the energy efficiency and check the possible impact on RAN4 requirement.

Observation 14: The PAE of power amplifier in 6G will be more challenge due to the higher power and wider channel bandwidth.

Proposal 11: Study the PA model with high energy efficiency, e.g., Doherty PA and possible impact on RAN4 requirement in 6G.

Another issue related to the energy efficiency is that although 3GPP did a lot of work on UE power saving, the real gain is never quantified or verified since lack of proper performance metric. In 6G, the UE power saving will be more impact considering higher power and data rate, how to ensure UE can really get enough benefit from power saving feature can be studied.

Observation 15: The gain of power saving feature at UE side was not quantified or verified in NR.

Proposal 12: Study the feasibility of introducing performance metric to quantify the energy saving gain in 6G.

  - Tx EVM relaxation

According to guidance of RP-252952, both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement would be part of 6G study, and RAN4 will further discuss the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items.

Power enhancement with relaxed UE Tx EVM and advanced BS receiver

- Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

The DPoD at BS mainly rely on DMRS to estimate the non-linearity, but performance gain depends on both PA characteristic and receiver algorithm. In [9], the symbol error rate under different PA model with DPoD algorithm:

Figure 9 Symbol error rate of different DPoD algorithm for 4 different PA models [9]

The figure above implies there is a possible risk exist for UE, which is a UE transmit with relaxed EVM could not be compensated at BS side, instead, it causes the demodulation of the UE's own signal to be affected. It will be a problem for UE whether to support this feature since performance gain is quite unstable.

Observation 16: UEs transmit with same relaxed EVM value could not get same performance gain at BS side due to the difference of PA characteristic, which make this feature looks unstable from UE perspective.

One possible solution is that the BS receiver should be at least verified with a standardized PA model, which can provide a reference for UE design and to check whether such feature can be beneficial.

Proposal 14: The Tx EVM relaxation should be verified at BS side with a distorted signal which is generated by a standardized PA model.

- Conclusion

In this contribution, we provide our views on 6G UE RF and the observations and proposals are summarized below:

Observation 1:

- For the combined power of multiple Tx is between two power class level, power loss exists (e.g., 23+26)

- powerClass and higherPowerLimit-r17 have similar function which may cause ambiguity if both are indicated

- HPUE enhancement always begin with single band before CA in standardization, and single band has higher power capability than CA is usual case, which make the UE power in CA could be restricted.

  - higherPowerLimit-r17 can help on this issue.

  - Signaling can also be simplified if higherPowerLimit-r17 is the default assumption in 6G

- If the power of each band under band combination still can reach the power level reported per band, it can be clearly reported by using powerclass or higherPowerLimit-r17

  - Otherwise PowerClassPerBandPerBC-r17 needs to be reported

- RAN4 define the power class for MIMO without explicit capability indication, which lead to complicated discussion for power scaling and full power mode

Proposal 1: Study a proper way to decouple the power class and UE implementation in 6G, and the following approach can be considered:

- UE per band power class defined as the maximum output power via single Tx

- For UE equip with multiple Tx, higher power can be achieved by power boosting approach

- higherPowerLimit-r17 in NR can be the baseline assumption in 6G

Proposal 2: The power ability under different UE condition is expected to be clarified clearly in 6G.

Observation 2: Increasing the default power class level would effectively expand the coverage radius and improve the uplink rates, significantly improving the users’ experience.

Proposal 3: The default power class in 6G can be considered set to PC2 26 dBm.

Observation 3: The duty cycle based SAR solution in NR causes complicated UE behavior and it is also doubtful whether the UE in the field can be benefit from such operation.

Proposal 4: RAN4 to check the necessity or possible improvement of NR duty cycle based SAR solution in 6G.

Observation 4: The UE architecture in FR3 was discussed several times in different SI, but it is still hard to give the final decision, especially for the frequency higher than 13GHz.

Proposal 5: The following options can be considered for FR3 (7-24 GHz) RF requirements design

Option 1: Decide a clear breakpoint within FR3 to distinguish conductive requirement and radiated requirement

  - Encourage companies to provide the feasibility analysis on the upper limit for the conductive requirement.

Option 2: Define an overlapping range within FR3 where both conductive requirement and radiated requirement are applied.

- FFS how to ensure the conductive requirement and radiated requirement are comparable within overlapping range

Observation 5: From chip design perspective, some adjacent operation bands are grouped as a unit and share part of RF component in front-end and transceiver.

Proposal 6: Study the criteria for determining which spectrum resource can be aggregated into a single cell, e.g., within adjacent frequency bands based on practical chip design. The target is no additional UE RF front-end complexity to support this feature.

Observation 6: For a cell configured with multi-carrier, the baseline architecture and behavior are not clear which may cause incorrect NW configuration or scheduling.

Proposal 7: Study the baseline UE architecture and behavior under single cell multi-carrier (SCMC) and the following two options can be further discussed:

Option 1: RAN4 to determine a clear UE architecture and behavior under SCMC

- RF chain switching across all CCs is preferred to alleviate UE design complexity

- Possible NW configuration/scheduling restriction needs to be considered

Option 2: RAN4 allow all kinds of UE architecture and behavior under SCMC

- RF chain switching/shared RF chain/separate RF chain can be considered as candidate UE architecture

- RAN4 to discuss how to help NW clearly understand UE behavior via proper UE capability design

Observation 7: From UE RF perspective, such operation may cause the RF chains are unoccupied and the usage of RF chain could be inefficiently.

Proposal 8: Study more efficient RF chain utilization under UL/DL decouple, e.g., enable higher MIMO layer or transmission/reception with more carriers.

Observation 8: The large MPR under 200MHz channel bandwidth could dissipate the increased UE output power, and it is necessary to study how to improve it in 6G.

Observation 9: The DPD can improve PA linearity and mitigate memory effect which is universally used in NR UE, but the NR MPR evaluation was based on PA itself for simplicity.

Observation 10: RF requirement relaxation in NR only focus on single carrier case.

Observation 11: Similar emission requirements keep reusing from LTE, and there is chance to check the feasibility of update and relaxation in 6G, at least for the new spectrum.

Observation 12: There is room for improvement for NR MPR definition and 6G UE with enhanced hardware have chance to reduce the MPR by flexible using its RF ability.

Proposal 9: Study the following approaches to reduce the MPR in 6G

- Approach 1:PA non-linearity improvement, e.g.,

- PA model with DPD assistance

- Approach 2: RF requirement relaxation, e.g.,

- Identify more scenarios where the RF requirements can be relaxed, e.g., extend relaxation in single carrier to multiple carrier

- Revisit the emission requirements and check whether update is feasible.

  - New spectrum in 6G as starting point

- Enable NR coverage enhancement feature in 6G day 1

- Approach 3: Improvement MPR evaluation/definition, e.g.,

- Check the possible change of NR MPR definition

- Check the possible update of evaluation assumption based on the 6G UE architecture

Observation 13: The power boosting in NR is triggered by individual topic, and the spec impact is scattered and lack of general principle.

Proposal 10: Study a new RF requirement, e.g., maximum power boosting, to construct a general framework for power boosting feature in 6G

Observation 14: The PAE of power amplifier in 6G will be more challenge due to the higher power and wider channel bandwidth.

Proposal 11: Study the PA model with high energy efficiency, e.g., Doherty PA and possible impact on RAN4 requirement in 6G.

Observation 15: The gain of power saving feature at UE side was not quantified or verified in NR.

Proposal 12: Study the feasibility of introducing performance metric to quantify the energy saving gain in 6G.

Observation 16: UEs transmit with same relaxed EVM value could not get same performance gain at BS side due to the difference of PA characteristic, which make this feature looks unstable from UE perspective.

Proposal 13: The Tx EVM relaxation should be verified at BS side with a distorted signal which is generated by a standardized PA model.

- References

- R1-2505414, Overall design considerations on 6GR air interface, RAN1#122,	vivo

- Balteanu F. Circuits for 5G RF front-end modules. International Journal of Microwave and Wireless Technologies. 2023;15(6):909-924.

- F. Boccardi et al., "Why to decouple the uplink and downlink in cellular networks and how to do it," in IEEE Communications Magazine, vol. 54, no. 3, pp. 110-117, March 2016, doi: 10.1109/MCOM.2016.7432156.

- Tingting Yao, Songbai He, Jun Peng, Fei You, Hu Zhang, A modified broadband weighted magnitude‐selective behavioral model for digital predistortion of radio frequency power amplifiers, International Journal of RF and Microwave Computer-Aided Engineering, 10.1002/mmce.23182, 32, 7, (2022).

- Du X, You CJ, Zhao Y, et al. Wideband high-efficiency linearized PA design with reduction in memory effects and IMD3. International Journal of Microwave and Wireless Technologies. 2018;10(4):391-400.

- Kenji MUKAI, Hiroshi OKABE, Satoshi TANAKA, Recent Progress in Envelope Tracking Power Amplifiers for Mobile Handset Systems, IEICE Transactions on Electronics, 2021, Volume E104.C, Issue 10, Pages 516-525, Released on J-STAGE October 01, 2021, Advance online publication March 19, 2021

- Satoshi Tanaka, Kenji Mukai, Shohei Imai, Hiroshi Okabe, Evolution of power amplifiers for mobile phone terminals from the 2nd generation to the 5th generation, IEICE Transactions on Electronics, Advance online publication March 22, 2022

- He, Shihai & Chen, Huan. (2025). Advanced Doherty Power Amplifier Architectures for 5G Handset Applications: A Comprehensive Review of Linearity, Back-Off Efficiency, Bandwidth, and Thermal Management. Chips. 4. 20. 10.3390/chips4020020.

- Z. Alina and O. Amrani, "On Digital Post-Distortion Techniques," in IEEE Transactions on Signal Processing, vol. 64, no. 3, pp. 603-614, Feb.1, 2016, doi: 10.1109/TSP.2015.2477806.