3GPP TSG-RAN4 Meeting #116bis	R4-2513252

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			8.6

Source:	vivo

Title:	Discussion on 6G spectrum

Document for:	Discussion

- Introduction

In RAN4#116 meeting, the paper [1] captures RAN4 6G Topic Planning based on 6G SID [2]. In this paper, we discuss the spectrum related topics which include issues about band/band combination definition and simplification, definition of frequency ranges, and spectrum related regulatory survey.

- Discussion

## 2.1 Definition of frequency ranges

The frequency spectrum for 6G will serve as the foundational enabler of next generation digital transformation 6GR. Definition of frequency ranges is one of the detailed objectives for this 6G SID [2]. For this issue, two aspects can be considered for 6GR frequency ranges: 5G refarming spectrum to 6G, new 6GR spectrum.

Table 1 summarizes 5G TN and NTN frequency ranges. In 5G NR, TN and NTN frequency ranges are separately defined. For FR1 TN, the frequency range covers 410MHz-7125 MHz. For FR1-NTN, the upper bound of this frequency range extends to 14500MHz with the introduction of Ku NTN bands. For FR2 TN, the whole frequency range spans from 24250 MHz to 71000MHz. For FR2-NTN, the whole frequency range extends to 10700 MHz-30000MHz. Moreover, FR1-NTN overlaps with FR2-NTN due to the introduction of Ku bands NTN.

Table 1. Summary of 5G TN and NTN frequency ranges

| TN |  |  | NTN |  |
| --- | --- | --- | --- | --- |
| Frequency range designation |  | Corresponding frequency range | Frequency range designation | Corresponding frequency range |
| FR1 |  | 410 MHz-7125 MHz | FR1-NTN (NOTE 1) | 410 MHz-14500 MHz |
| FR2 | FR2-1 | 24250 MHz-52600 MHz | FR2-NTN (NOTE 2) | 10700 MHz-30000 MHz |
|  | FR2-2 | 52600 MHz-71000 MHz |  |  |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |  |  |  |

For this 6G SID, frequency ranges beyond 52.6 GHz are not in scope of the work [2]. For 6GR TN frequency range definition, 5G TN frequency ranges FR1 and FR2-1 can be inherited to 6GR. For 6GR NTN frequency range definition, it needs more discussion. This SID aims at a harmonized 6G Radio design for TN and NTN, including their integration. One simple way is to inherit 5G NTN frequency range definition for 6GR, in this case, 6G NTN and TN frequency ranges are different. Another way, is to define unified frequency ranges for both 6G NTN and TN.

Proposal 1: Discuss how to define 6G NTN and TN frequency ranges.

For 6GR new spectrum, the range between FR1 and FR2-1(including around ~7GHz can be studied in this SID. However, currently there is no clear agreements on 6GR new spectrum according to regulatory survey, before the decisions from WRC-27 are clear. There were some candidate promising 6GR new spectrum, 4400-4800MHz, 7125-8400MHz, 14.8-15.35GHz according to the agreements from WRC-23.

For the range between FR1 and FR2-1, one simple way is to define a new frequency range FR3 for 7.125~24.250 GHz. This can be quite straightforward and this so-called ‘FR3’ has been used by many companies. Another way to address this issue is to extend FR1 or FR2-1 to include this new frequency range. We can further discuss the RF characteristics for this range to decide how to extend FR1 and FR2-1 frequency ranges.

Proposal 2: Discuss how to introduce 6G new spectrum with the following options:

- Extend FR1 to 8400MHz, and define a new frequency range as FR3.

- FR3 from 8400MHz to 24250MHz

## 2.2 Spectrum related regulatory survey

For 6G spectrum, it needs to be discussed and finalized in WRC-27 which will be held in 2027. However, some information from WRC-23, RAN#109, technical report TR38.820 is captured in this section.

WRC-23 Outcomes are summarized as follows:

WRC-23 New candidate IMT frequency bands for 6G: 4400-4800MHz, 7125-8400MHz, 14.8-15.35GHz Candidate frequency bands： - For Region 1 and Region 3: 4400-4800MHz - For Region 2 and Region 3: 7125-8400MHz - For Region 1: 7125-7250MHz,7750-8400MHz - 14.8-15.35GHz

Summary of 6G spectrum proposals from RAN#109:

| RP-252159 Study on 6G Scenarios and Requirements | Proposal 1: RAN shall postpone any 6G study and normative work, with regard to IMT identification, on the frequency range [7250 – 7750 MHz] and [7900 – 8400 MHz], at least until WRC27 final decision. Proposal 2: Subject to WRC27 decision, RAN shall ensure consistency between the parameters provided by 3GPP to ITU and future work carried out in 3GPP in the frequency range [7250-7750 MHz] and [7900-8400 MHz]. |
| --- | --- |
| RP-251993 | • As carrier frequency of FR3 band, both 7 GHz and 15 GHz should be evaluated. |
| RP-252207 | - 6G RAT shall work in FR1 spectrum between 410 and 7125 MHz - 6G RAT shall work in FR2 spectrum between 24.25 and 52.6 GHz - 6G RAT shall work in FR3 spectrum between 7.125 and 24.25 GHz |
| RP-252466 | FR; Range; SCS (kHz); Required FFT size; Max CBW (MHz); Min CBW (MHz). FR1; 400 MHz – 6.425 GHz; 15 (FDD) 30 (TDD); 8192; 100 (FDD) 200 (TDD); 5 (FDD) 10 (TDD). 6.425 – 7.125 GHz; 30; 8192, [16384]; 200, [400]; 20. FR1-extended; 7.125 – 8.4 GHz; 30 (if extended); 8192, [16384]; 200, [400]; 20. [new FR]; 8.4 – 24.25 GHz; 60 (if separate FR) or 120 (if merged in FR2-1); 8192; 400, 800; 20. FR2-1; 24.25 – 52.6 GHz; 120; 4096, 8192; 400, 800; 50. |

Regulatory information for 7-24 GHz in TR38.820:

Table 4.1-1: Frequency bands in the range 7 000 to 10 000 MHz in radio regulation

| Frequency band (MHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 7 145 – 7 190 | Yes | Yes | EESS (passive) and SRS (passive), footnote 5.458. SRS (deep space) (Earth-to-space). EESS (passive) and SRS (passive), footnote 5.458. |
| 7 190 – 7 235 | Yes | Yes | EESS (passive) and SRS (passive) (Earth-to-space), footnote 5.458. Footnote 5.460A – EESS space stations shall not claim protection from existing and future stations in the fixed and mobile services. |
| 7 235 – 7 250 | Yes | Yes | EESS (passive) (Earth-to-space) and SRS (passive), footnote 5.458. Footnote 5.460A – EESS space stations shall not claim protection from existing and future stations in the fixed and mobile services. |
| 7 250 – 7 300 | Yes | Yes |  |
| 7 300 – 7 375 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 375 – 7 450 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 450 – 7 550 | Yes | Yes | METSAT (space-Earth) Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 550 – 7 750 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 750 – 7 900 | Yes | Yes | METSAT (Space-to-Earth) |
| 7 900 – 8 025 | Yes | Yes |  |
| 8 025 – 8 175 | Yes | Yes | EESS (space-to-Earth). Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 175 – 8 215 | Yes | Yes | EESS (space-to-Earth), METSAT (Earth-to-space) Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 215 – 8 400 | Yes | Yes | EESS (space-to-Earth) Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 400 – 8 500 | Yes | Yes | SRS (space-to-Earth), footnote 5.465 SRS is limited to deep space. |
| 8 500 – 8 550 | No | No | Radiolocation |
| 8 550 – 8 650 | No | No | EESS (active), Radiolocation, SRS (active) |
| 8 650 – 8 750 | No | No | Radiolocation |
| 8 750 – 8 850 | No | No | Radiolocation and Aeronautical Radionavigation |
| 8 850 – 9 000 | No | No | Radiolocation and Maritime Radionavigation |
| 9 000 – 9 200 | No | No | Radiolocation and Aeronautical Radionavigation |
| 9 200 – 9 300 | No | No | EESS (active), Radiolocation, Maritime Radionavigation, footnote 5.474 search and rescue transponders may be used. |
| 9 300 – 9 500 | No | No | EESS (active), Radiolocation, Radionavigation, SRS (active), footnote 5.474 search and rescue transponders may be used. |
| 9 500 – 9 800 | No | No | EESS (active), Radiolocation, Radionavigation, SRS (active) |
| 9 800 – 9 900 | No | No | Radiolocation, Fixed service secondary |
| 9 900 – 10 000 | No | No | EESS (active), Radiolocation. Fixed service secondary, footnote 5.478B EESS (active) shall not cause harmful interference to, nor claim protection from, stations in the fixed service. |

Table 4.1-2: Frequency bands in the range 10 to 24 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 10 – 10.4 | Yes, excluding R2 | Yes, excluding R2 | EESS (active), Radiolocation, footnote 5.478B EESS (active) shall not cause harmful interference to, nor claim protection from, stations in the fixed service. Also used by weather radar on a secondary basis, footnote 5.479 |
| 10.4 – 10.45 | Yes, excluding R2 | Yes, excluding R2 | Radiolocation |
| 10.45 – 10.5 | No | No | Radiolocation, amateur and amateur-satellite secondary |
| 10.5 – 10.55 | Yes | Yes | Radiolocation secondary R1 |
| 10.55 – 10.6 | Yes | Yes | Radiolocation secondary |
| 10.6 – 10.68 | Yes | Yes | EESS (passive), RAS (footnote 5.149), SRS (passive), Radiolocation secondary. Footnote 5.482A EESS (passive) and mobile and fixed services, Res. 751 applies |
| 10.68 – 10.7 | No | No | EESS (passive), RAS, SRS (passive) Footnote 5.340 – all emissions prohibited |
| 10.7 – 10.95 | Yes | Yes | Footnote 5.441 FSS Appendix 30B band |
| 10.95 – 11.2 | Yes | Yes | Footnote 5.848B Res. 155 applies – unmanned aircraft systems using FSS |
| 11.2 – 11.45 | Yes | Yes | Footnote 5.441 FSS Appendix 30B |
| 11.45 – 11.7 | Yes | Yes | Footnote 5.484B Res. 155 applies – unmanned aircraft systems using FSS. |
| 11.7 – 12.5 | Yes, except 11.7-12.2 GHz in R2 | Yes, except 12.1-12.2 in R2 | BS and BSS Footnotes 4.587, 5.488, 5.490, and 5.493 denotes Appendix 30 and 30B planned bands |
| 12.5 – 12.75 | Yes, except R1 | Yes, except R1 | R3 BSS Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS. |
| 12.75 – 13.25 | Yes | Yes | SRS (deep space) (space-to-Earth) secondary Footnote 5.441 FSS Appendix 30B |
| 13.25 – 13.4 | No | No | EESS (active), ARNS, SRS (active) |
| 13.4 – 13.65 | No | No | EESS (active), Radiolocation, SRS, Standard frequency and time signal-satellite (Earth-to-space) |
| 13.65 – 13.75 | No | No | EESS (active), Radiolocation, SRS (active), standard frequency and time signal-satellite (Earth-to-space) secondary |
| 13.75 – 14 | No | No | Radiolocation, standard frequency and time signal-satellite (Earth-space) |
| 14 – 14.25 | No | No | Radionavigation Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.25 – 14.3 | No | No | Radionavigation Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.3 – 14.4 | Yes, except R2 | Yes, except R2 | Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.4 – 14.47 | Yes | Yes | Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.47 – 14.5 | Yes | Yes | Radio astronomy – Footnote 5.149 Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.5 – 14.75 | Yes | Yes | Res 163 and 164 restriction to FSS to protect aeronautical mobile |
| 14.75 – 14.8 | Yes | Yes | Res 163 and 164 restriction to FSS to protect aeronautical mobile |
| 14.8 – 15.35 | Yes | Yes | Footnote 5.339 also allocated to EESS and SRS (passive) secondary |
| 15.35 – 15.4 | No | No | EESS (passive), Radio Astronomy, SRS (passive) Footnote 5.340 – all emission prohibited |
| 15.4 – 15.43 | No | No | Radiolocation and radionavigation |
| 15.43 – 15.63 | No | No | Radiolocation and radionavigation |
| 15.63 – 15.7 | No | No | Radiolocation and ARNS |
| 15.7 – 16.6 | No | No | Radiolocation Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 16.6 – 17.1 | No | No | Radiolocation, SRS (deep space) (Earth-to-space) Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.1 – 17.2 | No | No | Radiolocation Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.2 – 17.3 | No | No | EESS (active), Radiolocation, SRS (active) Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.3 – 17.7 | No | No | Footnote 5.515 Appendix 30A Footnote 5.516B High density FSS applications (in Region 1) |
| 17.7 – 18.1 | Yes, excluding Region 2 (17.7-17.8) | Yes | Footnote 5.515 Appendix 30A (Region 2 17.7-17.8) |
| 18.1 – 18.4 | Yes | Yes | Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.4 – 18.6 | Yes | Yes | Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.6 – 18.8 | Yes | Yes | EESS (passive), SRS (passive) Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.8 – 19.3 | Yes | Yes | 5.516B High density FSS applications (in Region 2) |
| 19.3 – 19.7 | Yes | Yes |  |
| 19.7 – 20.1 | No | No | 5.484B Res 155 applies – unmanned aircraft systems using FSS 5.516B High density FSS applications |
| 20.1 – 20.2 | No | No | 5.484B Res 155 applies – unmanned aircraft systems using FSS 5.516B High density FSS applications |
| 20.2 – 21.2 | No | No | Standard frequency and time signal-satellite (space-to-Earth) |
| 21.2 – 21.4 | Yes | Yes | EESS (passive) and SRS (passive) |
| 21.4 – 22 | Yes | Yes | BSS in Regions 1 and 3 Footnote 5.530A any station in the fixed or mobile services shall not produce a pfd in excess of -120.4 dB(W/m2 • MHz) at 3m above the ground for more than 20% of the time. (see Rec. ITU-R BO.1898) Footnote 5.530B Admins in R1 and R3 are encouraged no to deploy stations in the mobile service and are encouraged to limit the deployment of FS ptp links |
| 22 – 22.21 | Yes | Yes | Footnote 5.149 Radio astronomy |
| 22.21 – 22.5 | Yes | Yes | EESS (passive), Radio astronomy, SRS (passive), Footnote 5.149 Footnote 5.532 EESS and SRS (passive) shall not impose constraints on the FS and MS |
| 22.5 – 22.55 | Yes | Yes | Not applicable |
| 22.55 – 23.15 | Yes | Yes | SRS (Earth-to-space) Footnote 5.149 Radio astronomy Footnote 5.338A – Res 750 applies (passive emission limits) |
| 23.15 – 23.55 | Yes | Yes | Footnote 5.338A – Res 750 applies (passive emission limits) |
| 23.55 – 23.6 | Yes | Yes | WRC-19 Agenda item 1.13 determining IMT adjacent emissions limits for protection of passive services in the 23.6-24 GHz result would most likely be applicable to this band. |
| 23.6 – 24 | No | No | EESS (passive), Radio astronomy, SRS (passive) Footnote 5.340 all emissions prohibited – unwanted emissions from IMT is under review WRC-19 Agenda item 1.13 |

## 2.3 Band/Band combination definition and simplification

### Background & Motivation

For different frequency ranges, based on operator requirements a large number of bands and band combinations have been defined up to 5G. However, practical implementation is constrained by hardware complexity. However, 6G will introduce even more spectrum resources, leading to more complex new CA (Carrier Aggregation) combinations. Consequently, the number of test cases that need to be defined by RAN5 will increase considerably.

The implementation of CA combinations composed of adjacent frequency bands often requires complex combiner designs, which introduce high insertion loss at the front-end, sacrificing output power and sensitivity. Adjacent bands also impose high requirements on filter design and antenna isolation, increasing implementation costs. Therefore, simplifying CA combinations, particularly by phasing out those that are difficult to implement and offer limited benefits, has become a common aspiration for many terminal vendors.

By categorizing and simplifying bands and band combinations, and retaining those with high cost-effectiveness and significant implementation gains, we can greatly improve the utilization rate of CA combinations, reduce the cost of defining RAN5 test cases, and better optimize the 6G spec. As is well known, during the 5G NR stage, chip vendors have already begun attempting to group the bands defined by 3GPP and design based on these groups to simplify the architecture. For instance, the design of PAs, LNAs, and transceiver ports already considers a per-group granularity.

Thus, initiating discussions on this topic in the early stages of 6G can provide crucial design direction for relevant chip and device vendors earlier in the process. This allows for earlier research into chipset upgrades and minimization of RF chains, ultimately reducing costs.

Observation 1: Band/band combination simplification for 6G is a general demand and potential consensus among device vendors.

The simplification scheme should consider not only the re-farming spectrum but also the new spectrum, and the new bands introducing in the future as well. As has been proposed from some companies, ‘Band group’ could be an effective and simple method to achieve this goal. Band group is to categorize or classify part of or all the specified bands into several groups, each group contains several bands which are not far from each other in frequency range and could share the same RF devices as well.

### How to implement band/band combination simplification

Generally, there are two potential options for band group definition, take the current FR1 spectrum range as an example.

Option 1. Defined based on division of the specific frequency range.

Fig1. Low, mid, high band group definition based on division of FR1

Based on the separation of the frequency range, each band could be automatically categorized into one certain group, including the new bands introduced in the future. It provides a simple and direct scheme and would reduce the workload of spectrum categorize, especially when confronting with the need to introduce a large number of new bands or frequency ranges into the spec. Also, based on this definition, the simplification of CA combinations’ requirements would be more effective because the frequency bands are clearly divided without any overlap and each frequency band belongs to only one group. The RF performance of the CA combinations within the same group combination is similar and follows a pattern.

However, as the dividing method is kind of rough and there may be too many bands containing in one group, the bands at the two sides of one group may be not close to each other. In hence, there should be some exception cases when simplifying the RF requirements based on group-combination. Also, if CA would be implemented only between different band groups then the UE design must accommodate the fixed band group ranges, which would restrict the UE behaviour.

From another aspect, for the re-farming frequency ranges, the frequency division point between different band groups should be defined carefully to avoid dividing the UL range and DL range into different band groups.

Observation 2: Define the band groups based on division of the specific frequency range,

- The workload of band categorize could be reduced and the simplification of CA combinations’ requirements would be more effective.

- Some exception cases for specific band combs in requirements simplification are needed and the restriction of UE behaviour is inevitable.

Option 2. Defined based on operators/vendors’ request.

Fig2. Group1~n definition based on operator/vendor’s request

Based on the operator/vendor’s request, intuitively it leaves more flexibilities for UE design, as it would be more in line with the actual implementation demand and commercial use. At the same time, for different regions to define a common set of groups is kind of idealized and compromised, as the frequency bands within the same group may need to share some RF devices. For the bands at the both ends of one big group the deterioration of performance is unavoidable, and option 2 could resolve this problem to some extent.

However, from the other side, it would inevitably introduce lots of different group categories, and for a certain band it may belong to more than one group, which would make it harder and more complex to simplify the CA combs’ requirements. Maybe each group-group combination needs a general set of CA requirements. However, due to the excessive number of groups, the simplification of frequency bands and requirements may not be truly effective.

Last but not the least, anyway, the definition and division of band groups need the support and guidance from operators’ side and further study is necessary.

Observation 3: Define the band groups based on operators/vendors’ request,

- It is more in line with the implementation demand and commercial use, also leaves more flexibilities for UE design.

- It is less friendly to simplification of band/band group and CA combs’ requirements.

Table 2. Comparison of band group definitions of options 1 and 2

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | - Each band (including new bands introduced in the future) could be automatically categorized into one certain group - Requirements simplification of CA is effective when group defined based on large range | - Exemptions needed for certain band combination in requirement simplification - Inflexible UE behavior if CA is only allowed between band group |
| Option 2 | - Closer to the actual commercial needs - Flexible UE design and requirement simplification | - Hard to perform CA requirement simplification - Group increase when new band introduced |

Proposal 3: Due to commercialization needs from different operators, the legacy approach of 5G band/CA definition can be reused as a baseline.

Proposal 4: To further minimize the standardization work and RF system design, a Band group concept can be considered as a solution to simplify band/band combination and CA combs’ requirements, the specific definition principle need guidance from operators and further study.

### Simplification of other related requirements

According to the assumption of band simplification, such as band group concept, lots of band combination could be simplified and categorized into several ‘group combinations’. For example, when defining band groups based on division of the specific frequency range and categorizing the current FR1 frequencies roughly into three groups as illustrated in Fig. 1, the bands of FR1 would be classified as follows:

Table 3. Band grouping directory based on division of FR1

| Low Group | （sub1G~1.52G） | n5, n8, n12, n13, n14, n18, n20, n26, n28, n29, n31, n50, n51, n67, n68, n71, n72, n74, n75, n76, n81, n82, n83, n85, n87, n88, n89, n91, n92, n93, n94, n100, n105, n106, n109, n110 |
| --- | --- | --- |
| Mid Group | （1.52~3G） | n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n53, n54, n65, n66, n70, n80, n84, n86, n90, n95, n97, n98, n99, n101 |
| High Group | （ >3G） | n46, n47, n48, n77, n78, n79, n96, n102, n104 |

Then the simplification of the band combination can be further discussed. There are two types: one is CA within a group, and the other is CA between groups. Based on the latter, the simplification of some CA combination requirements can be based on the concept of group combination, such as the simplification of the MSD part.

- For MSD due to UL harmonic:

Table 4. The n-th harmonic interference of UL falling within DL

| DL UL with n order | Low Group | Mid Group | High Group |
| --- | --- | --- | --- |
| Low Group | 2nd, (3rd) | 2nd, 3rd, 4th | 2nd, (3rd), 4th, 5th |
| Mid Group |  | (2nd) | 2nd, (3rd), 4th |
| High Group |  |  | 2nd, (…） |
| Note: () represents there is no band combinations with corresponding harmonic interference issues in the spec currently. |  |  |  |

From the above table, it can be observed that most of the uplink harmonic interference originates from low-frequency groups, which is consistent with the concept of uplink harmonics that the n-th harmonic of the uplink frequency band interferes with the 1st harmonic of the downlink frequency band. Moreover, it could also be seen that some UL interference falls into the DL band of the same band group, such as CA_n28-n74/n75/n94 (2nd harmonic interference, Low-Low group combination), CA_n78-n104 (2nd harmonic interference, High-High group combination).

Furthermore, for the inter-group interference, the MSD could be simplified according to the group combination categories. Some example group combs with the same or similar MSD values are listed as follows, which are excerpted and inducted from the current spec. From Table 5 it could be seen that, the MSD of inter-group combs are nearly identical for the same group combinations, the same order, and identical allocations. Also, some exception cases also exist and not excerpted here.

Observation 4: Based on low, mid, high group definition for FR1, for UL harmonic the MSD of inter-group combs could be classified according to the harmonic order and allocation. Some exception cases also exist.

Table 5. Some PC3 harmonic MSD classification examples based on group combination

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |
| n1/n2/n3/n25/66/n70 | n77 | 5 | 15 | 12 | 10 | 23.9 | UL2/DL1 direct-hit |
| n2/n3/n25/n66/n70 | n78 |  |  |  |  |  |  |
| n1/n2/n3/n25/n66/n70 | n77 | 5 | 15 | 12 | 100 | 13.8 |  |
| n2/n3/n25/n66/n70 | n78 |  |  |  |  |  |  |
| n66 | n48 |  |  |  |  |  |  |
| n8 | n7 | 5 | 15 | 8 | 5 | 10 | UL3/DL1 direct-hit |
| n12/n85 | n66 |  |  |  |  |  |  |
| n28 | n1 |  |  |  |  | 10.2 |  |
| n5 | n77/n78 | 5 | 15 | 6 | 10 | 10.5 | UL4/DL1 direct-hit |
| n8/n20 | n77/n78 |  |  |  |  | 10.8 |  |
| n5 | n77/n78 | 5 | 15 | 6 | 100 | 2.9 |  |
| n8/n20 | n77/n78 |  |  |  |  | 3.1 |  |
| n5/n12/n13/n14/n18 | n77 | 5 | 15 | 6 | 10 | 10.4 | UL5/DL1 direct-hit |
| n12/n18/n28 | n78 | 5 | 15 | 5 | 10 | 10.4 |  |
| n8 | n79 |  |  |  |  | 12.0 |  |
| n5/n12/n13/n14/n18/n28 | n77 | 5 | 15 | 5 | 100 | 2.9 |  |
| n8 | n79 |  |  |  |  | 3.8 |  |

- For MSD due to harmonic mixing:

For comb groups with the same order and identical UL/DL allocations, the dispersion in MSD values resulting from harmonic mixing is generally greater than that caused by UL harmonics. Although there are more exception cases compared to those in the UL harmonic table, a significant number of MSD values remain similar and can be effectively classified.

Table 6. Some PC3 harmonic mixing MSD classification examples based on group combination

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW |  | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |
| n1 | n105 | 5 | 15 | 25 | 5 |  | 26.8 | UL1/DL3 |
| n1/n2/n25 | n71 |  |  |  |  |  |  |  |
| n2/n25 | n71 | 5 | 15 | 25 | 20 |  | 15.6 |  |
| n40 | n20 | 10 | 15 | 25 | 5 |  | 27.8 |  |
| n41 | n5 |  |  |  |  |  | 24.3 |  |
| n41 | n18 |  |  |  |  |  | 26.3 |  |
| n48/n77/n78 | n5 | 10 | 15 | 25 |  | 5 | 5.7 | UL1/DL4 |
| n48/n77/n78 | n26 |  |  |  |  |  | 5.4 |  |
| n48 | n26 | 10 | 15 | 25 |  | 20 | 1 |  |
| n77 | n5/n8 |  |  |  |  |  | 0.8 |  |
| n77/n78 | n2 | 10 | 15 | 25 |  | 5 | 6.7 | UL1/DL2 |
| n77/n78 | n3 |  |  |  |  |  | 5.7 |  |
| n77/n78 | n2 | 10 | 15 | 25 |  | 20 | 2.8 |  |
| n77/n78 | n3 |  |  |  |  |  | 2.2 |  |
| n79 | n5 | 10 | 15 | 25 |  | 5 | 27.5 | UL1/DL5 |
| n79 | n8 |  |  |  |  |  | 25 |  |
| n77/n78 | n12/n28 |  |  |  |  |  | 31 |  |
| n78 | n67 |  |  |  |  |  |  |  |
| n77/n78 | n40/n41 | 10 | 15 | 12 |  | 10 | 11.7 | UL2/DL3 |
| n77/n78 | n40/n41 | 10 | 15 | 12 |  | 100 | 3.6 |  |
| n41 | n77 | 10 | 15 | 6 |  | 10 | 8.9 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 |  | 10 | 9.3 |  |
| n41 | n48/n78 | 10 | 15 | 6 |  | 100 | 2.3 |  |
| n41 | n77 | 10 | 15 | 6 |  | 100 | 2.1 |  |

Observation 5: Based on low, mid, high group definition for FR1, for harmonic mixing the MSD of inter-group combs exhibits greater dispersion compared to UL harmonic, but to some extent, it can still be classified based on harmonic order and allocation.

- For MSD due to crossband:

For cross band interference, it is caused by “ACLR1/2, or >ACLR” which separately indicates that the first/second, or neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

The following table includes all the inter-group combos for PC3, and despite different band combination has different UL & DL Fc and BW, the MSD values are similar inside the same group combination. Some of the MSD values may need further check and revision, but fewer exception cases could be expected.

Table 7. Some PC3 crossband MSD classification examples based on group combination

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 4.3 | >ACLR2 |
| n41 | n48 | 2680 | 100 | 30 | 270 (RBstart=3) | 3552.5 | 5 | 8.3 | >ACLR2 |
| n38 | n78 | 2600 | 40 | 15 | 216 (RBstart=0) | 3305 | 10 | 8.3 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n41 | n78 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | n71 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | n38 | 3350 | 100 | 30 | 270 (RBstart=0) | 2617.5 | 5 | 3.3 | >ACLR2 |
| n78 | n38 | 3350 | 100 | 30 | 270 (RBstart=0) | 2600 | 40 | 3.3 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 4.5 | >ACLR2 |
| n78 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n78 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |

Observation 6: Based on low, mid, high group definition for FR1, for MSD due to crossband isolation the inter-group combs are finite and MSD values could be effectively classified, fewer exception cases are expected.

- For MSD due to IMD:

The following two tables show some example group combos for MSD caused by IMD2/IMD3, it could be observed that for IMD product, the dispersion in MSD values is much greater than the other three interference source. For example, for IMD2 of mid-high group combos, most of the MSD is 26dB while some of them is as high as 31dB. For IMD3, some of the low-mid and mid-high combos has the MSD value around 12dB while some others around 15~16dB. Further classification can be carried out based on the respective UL and DL groups.

At the same time, from the relevant discussion of MSD simplification of HPUE at R19 it could be found that the simplification of IMD MSD value can allow up and down to a certain degree of fluctuation and coordination due to different RF architectures. While for calculation of PC3 MSD, some RF device parameters such as filter rejection and duplexer/triplexer rejection could be shared among different adjacent/overlapping bands within the same group, e.g., n2 and n25, n77 and n78. In all, for the same group combo with the same order of IMD product, there is still some room for MSD simplification.

Table 8. Some PC3 IMD2 MSD classification examples based on group combination

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
| CA_n7-n46 | n7 | 2550 | 10 | 50 | 2670 | 26.8 | FDD | IMD24 |
|  | n46 | 5220 | 20 | 100 | 5220 | N/A | TDD | N/A |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
| CA_n70-n78 | n70 | 1705 | 5 | 25 | 2005 | 31 | FDD | IMD2 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| CA_n3-n26 | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD24 |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 | N/A | FDD | N/A |

Table 9. Some PC3 IMD3 MSD classification examples based on group combination

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n1-n102 | n1 | 1922.5 | 5 | 25 | 2112.5 | 13 | FDD | IMD3 |
|  | n102 | 5957.5 | 20 | 100 | 5957.5 | N/A | TDD | N/A |
| CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n20-n41 | n20 | 851 | 5 | 25 | 810 | 12.1 | FDD | IMD3 |
|  | n41 | 2512 | 10 | 50 | N/A | N/A | TDD | N/A |
| CA_n5_n41 | n5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| CA_n7-n20 | n20 | 851 | 5 | 25 | 810 | 12 | FDD | IMD3 |
|  | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
| CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD34 |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |
| CA_n1-n71 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |
| CA_n7-n26 | n7 | 2512 | 5 | 50 | 2632 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD34 |
| CA_n1-n105 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |

One special issue for IMD is that there are some complex combinations composed of three bands, if only restricting the CA implementation between different groups then some combinations would be deleted. Otherwise, the classification of MSD could also be effective to some extent as shown in Table 10, while more exceptions should be considered.

Table 10. Some PC3 IMD3 MSD classification examples based on group combination

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
| CA_n1-n3-n77 | n1 | N/A | 5 | N/A | 2140 | 31.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD2 |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n1-n3-n71 | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
| CA_n1-n3-n105 | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n105 | 695 | 5 | 25 | 644 | N/A | FDD | N/A |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1818.5 | 4.0 | FDD | IMD5 |
| CA_n1-n3-n71 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
| CA_n1-n3-n105 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n105 | 695 | 5 | 25 | 644 | N/A | FDD | N/A |

Observation 7: Based on low, mid, high group definition for FR1, the MSD of inter-group combs due to IMD exhibits the greatest dispersion but there is still some room for simplification.

Proposal 5: Further study requirements simplification based on the band group concept, e.g., MSD.

And for each group combination several typical band combinations and test points could be selected as the starting point for requirement simplification and related study could also be used to guide the definition of band group.

Proposal 6: For each group combination, select several typical band combinations as the starting point for requirements simplification and the related study could also be used to guide the definition of band group.

- Conclusion

Definition of frequency ranges

Proposal 1: Discuss how to define 6G NTN and TN frequency ranges.

Proposal 2: Discuss how to introduce 6G new spectrum with the following options:

- Extend FR1 to 8400MHz, and define a new frequency range as FR3.

- FR3 from 8400MHz to 24250MHz

Band/Band combination definition and simplification

Observation 1: Band/band combination simplification for 6G is a general demand and potential consensus among device vendors.

Observation 2: Define the band groups based on division of the specific frequency range,

- The workload of band categorize could be reduced and the simplification of CA combinations’ requirements would be more effective.

- Some exception cases for specific band combs in requirements simplification are needed and the restriction of UE behaviour is inevitable.

Observation 3: Define the band groups based on operators/vendors’ request,

- It is more in line with the implementation demand and commercial use, also leaves more flexibilities for UE design.

- It is less friendly to simplification of band/band group and CA combs’ requirements.

Proposal 3: Due to commercialization needs from different operators, the legacy approach of 5G band/CA definition can be reused as a baseline.

Proposal 4: To further minimize the standardization work and RF system design, a Band group concept can be considered as a solution to simplify band/band combination and CA combs’ requirements, the specific definition principle need guidance from operators and further study.

Observation 4: Based on low, mid, high group definition for FR1, for UL harmonic the MSD of inter-group combs could be classified according to the harmonic order and allocation. Some exception cases also exist.

Observation 5: Based on low, mid, high group definition for FR1, for harmonic mixing the MSD of inter-group combs exhibits greater dispersion compared to UL harmonic, but to some extent, it can still be classified based on harmonic order and allocation.

Observation 6: Based on low, mid, high group definition for FR1, for MSD due to crossband isolation the inter-group combs are finite and MSD values could be effectively classified, fewer exception cases are expected.

Observation 7: Based on low, mid, high group definition for FR1, the MSD of inter-group combs due to IMD exhibits the greatest dispersion but there is still some room for simplification.

Proposal 5: Further study requirements simplification based on the band group concept, e.g., MSD.

Proposal 6: For each group combination, select several typical band combinations as the starting point for requirements simplification and the related study could also be used to guide the definition of band group.

- References

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, RAN4#116.

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN#108.